MTP LEVEL 2

TEST NUMBER:	7.4

PAGE:	1 OF 1

REFERENCE:	Q.703 § 10.3	STD:	Fig. 9, Fig. 11, Fig. 17

TITLE:	AERM check

SUB TITLE:	Error rate at the emergency threshold

PURPOSE:	To test the AERM at the emergency threshold

PRE–TEST CONDITIONS:	Link out of service

CONFIGURATION:		1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			SIOS

1 – 0	SIOS

--------------------------->

:	start

<--------------------------

1 – 0			SIO

1 – 0	SIO

--------------------------->

<--------------------------

1 – 0			SIN

1 – 0	SIE

--------------------------->

1 – 0	½  corrupt LSSU

--------------------------->

1 – 0	½  SIE

--------------------------->

T4	½
(Pe)	½
	½

<--------------------------

<--------------------------

1 – 0			SIN

1 – 0			FISU

TEST DESCRIPTION

1.

Start link at A, check emergency proving started from B.

2.

Generate x number of corrupt LSSUs (e.g. CRC error) at B. (5 > x  Tie).

3.

Check that link aligns successfully.

MTP LEVEL 2

TEST NUMBER:	8.1

PAGE: 	1 OF 1

REFERENCE:	Q.703 § 5.2 	STD:	Fig. 13, Fig. 14

TITLE:	Transmission and reception control (Basic)

SUB TITLE:	MSU transmission and reception

PURPOSE:	To check basic MSU transmission and reception

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT, CPT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			FISU

1 – 0	FISU

--------------------------->

1 – 0	MSU
	(FIB+FSN=80)
	(BIB+BSN=FF)

--------------------------->

<--------------------------

1 – 0			FISU
			(FIB+FSN=FF)
			(BIB+BSN=80)

1 – 0	FISU
	(FIB+FSN=80)
	(BIB+BSN=FF)

--------------------------->

<--------------------------

1 – 0			MSU
			(FIB+FSN=80)
			(BIB+BSN=80)

1 – 0	FISU
	(FIB+FSN=80)
	(BIB+BSN=80)

--------------------------->

<--------------------------

1 – 0			FISU
			(FIB+FSN=80)
			(BIB+BSN=80)

TEST DESCRIPTION

1.

Generate an MSU at B.

2.

Check that A receives the MSU correctly, and returns a positive acknowledgement.

3.

Generate an MSU at A.

4.

Check that B receives the MSU correctly, and returns a positive acknowledgement.

MTP LEVEL 2

TEST NUMBER:	8.2

PAGE:	1 OF 1

REFERENCE:	Q.703 § 5.3	STD:	Fig. 13

TITLE:	Transmission and reception control (Basic)

SUB TITLE:	Negative acknowledgement of an MSU

PURPOSE:	To test the response to a negatively acknowledged MSU

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<-------------------------

1 – 0			FISU

1 – 0	FISU

-------------------------->

<-------------------------

1 – 0			MSU
			(FIB+FSN=80)

<-------------------------

1 – 0			MSU
			(FIB+FSN=81)

1 – 0	FISU
	(BIB+BSN=7F)

-------------------------->

<-------------------------

1 – 0			MSU
			(FIB+FSN=00)

<-------------------------

1 – 0			MSU
			(FIB+FSN=01)

TEST DESCRIPTION

1.

Send MSU from A.

2.

Reply with negative acknowledgement from B.

3.

Check that A retransmits the MSU.

MTP LEVEL 2

TEST NUMBER:	8.3

PAGE:	1 OF 1

REFERENCE:	Q.703 § 5.3	STD:	Fig. 13

TITLE:	Transmission and reception control (Basic)

SUB TITLE:	Check RTB full

PURPOSE:	To check that MSUs are buffered when no acknowledgements are received

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			FISU

1 – 0	FISU
	(BIB+BSN=FF)

--------------------------->

<--------------------------

1 – 0			MSU
			(FIB+FSN=80)
				· 
				·

<--------------------------

1 – 0			MSU
			(FIB+FSN=FE)

<--------------------------

1 – 0			FISU
			(FIB+FSN=FE)

1 – 0	FISU
	(BIB+BSN=7F)

--------------------------->

<---------------------------

1 – 0			MSU
			(FIB+FSN=00)
				· 
				·

<--------------------------

1 – 0			MSU
			(FIB+FSN=7E)

TEST DESCRIPTION

1.

Generate MSUs at A, at a rate of 100 per second, in order to fill the RTB before the EDA timer T7 expires.

2.

No acknowledgements are sent from B until the last message is received, then send negative acknowledgement to the first message received.

3.

Check that the complete contents of the RTB are retransmitted.

MTP LEVEL 2

TEST NUMBER:	8.4

PAGE:	1 OF 1

REFERENCE:	Q.703 § 5.2	STD:	Fig. 14

TITLE:	Transmission and reception control (Basic)

SUB TITLE:	Single MSU with erroneous FIB

PURPOSE:	To ensure correct performance when an MSU with erroneous FIB is received

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			FISU
			(BIB+BSN=7F)

1 – 0	FISU
	(FIB+FSN=7F)

--------------------------->

1 – 0	MSU
	(FIB+FSN=80)

--------------------------->

<--------------------------

1 – 0			FISU
			(BIB+BSN=7F)

1 – 0	FISU
	(FIB+FSN=00)

--------------------------->

1 – 0	FISU
	(FIB+FSN=00)

--------------------------->

<--------------------------

1 – 0			FISU
			(BIB+BSN=FF)

1 – 0	MSU
	(FIB+FSN=80)

--------------------------->

<--------------------------

1 – 0			FISU
			(BIB+BSN=80)

TEST DESCRIPTION

1.

Generate an MSU at B with FIB inverted.

2.

Check A discards the MSU.

3.

Generate 2 FISUs at B with correct FIB.

4.

Check A discards the FISU and negative acknowledgement returned.

5.

Check that B retransmits the MSU correctly, and positive acknowledgement returned.

MTP LEVEL 2

TEST NUMBER:	8.5

PAGE:	1 OF 1

REFERENCE:	Q.703 § 5.2	STD:	Fig. 14

TITLE:	Transmission and reception control (Basic)

SUB TITLE:	Duplicated FSN

PURPOSE:	To test the reception control response to duplicated FSNs

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			FISU

1 – 0	FISU

--------------------------->

1 – 0	MSU
	(FIB+FSN=80)

--------------------------->

<--------------------------

1 – 0			FISU
			(BIB+BSN=80)

1 – 0	MSU
	(FIB+FSN=80)

--------------------------->

1 – 0	FISU
	(FIB+FSN=81)

--------------------------->

<--------------------------

1 – 0			FISU
			(BIB+BSN=00)

1 – 0	MSU
	(FIB+FSN=01)

--------------------------->

<--------------------------

1 – 0			FISU
			(BIB+BSN=01)

TEST DESCRIPTION

1.

Generate an MSU at B, check A receives the MSU correctly and returns a positive acknowledgement.

2.

Duplicate the FSN at B, check that A responds with a negative acknowledgement.

3.

Retransmit the MSU with correct FSN, check that A replies with a positive acknowledgement.

MTP LEVEL 2

TEST NUMBER:	8.6

PAGE:	1 OF 1

REFERENCE:	Q.703 § 5.2	STD:	Fig. 14

TITLE:	Transmission and reception control (Basic)

SUB TITLE:	Erroneous retransmission – Single MSU

PURPOSE:	To test the reception control response to retransmission of a single MSU

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			FISU
			(BIB+BSN=FF)

1 – 0	FISU
	(FIB+FSN=FF)

--------------------------->

1 – 0	MSU
	(FIB+FSN=00)

--------------------------->

1 – 0	FISU
	(FIB+FSN=80)

--------------------------->

1 – 0	FISU
	(FIB+FSN=80)

--------------------------->

<--------------------------

1 – 0		FISU
		(BIB+BSN=7F)

1 – 0	MSU
	(FIB+FSN=00)

--------------------------->

<--------------------------

1 – 0		FISU
			(BIB+BSN=00)

TEST DESCRIPTION

1.

A single MSU with FIB inverted in error is sent to A, followed by FISUs with correct FIBs.

2.

Check that A returns a negative acknowledgement for the MSU.

3.

Retransmit the MSU correctly.

4.

Check that A receives the MSU correctly and returns a positive acknowledgement.

MTP LEVEL 2

TEST NUMBER:	8.7

PAGE: 1 OF 1

REFERENCE:	Q.703 § 5.3	STD:	Fig. 14

TITLE:	Transmission and reception control (Basic)

SUB TITLE:	Erroneous retransmission – Multiple FISUs

PURPOSE:	To test reception control response to retransmissions of multiple FISUs

PRE–TEST CONDITIONS:	Link in service

CONFIGURATION:	1

TYPE OF TEST:	VAT

EXPECTED SIGNAL UNIT SEQUENCE:

SP		B

SP		A

Link

Link

<--------------------------

1 – 0			FISU

1 – 0	FISU
	(FIB+FSN=FF)

--------------------------->

1 – 0	FISU
	(FIB+FSN=7F)

--------------------------->

1 – 0	FISU
	(FIB+FSN=FF)

--------------------------->

1 – 0	FISU
	(FIB+FSN=7F)

--------------------------->

<--------------------------

1 – 0			SIOS

TEST DESCRIPTION

1.

Generate FISUs with the FIB inverted at B.

2.

Check that A responds with link out of service.