|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | T.42 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (10/96) |

SERIES T: TERMINALS FOR TELEMATIC SERVICES

Continuous tone colour representation 
method for facsimile

ITU-T  Recommendation  T.42

(Previously  CCITT  Recommendation)

ITU-T  T-SERIES  RECOMMENDATIONS

TERMINALS FOR TELEMATIC SERVICES

For further details, please refer to ITU-T List of Recommendations.

FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation T.42 was revised by ITU-T Study Group 8 (1993-1996) and was approved by the WTSC (Geneva, October 9-18, 1996).

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

  ITU  1997

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	Field of application		1

3	References		1

4	Definitions		1

5	Conventions		1

6	Colour representation model		2

6.1	Overview		2

6.2	Colour representation recommendation		2

6.2.1	1976 CIE L* a* b* (CIELAB) space		2

6.2.1.1	Colour space specification		2

6.2.1.2	White point and illuminant data		2

6.2.1.3	Gamut range		2

7	Colour reproduction information		3

Appendix  I  –  Method for colorimetric calculation from spectral measurement		4

Appendix  II  –  Calculation of CIELAB values from CIEXYZ values		6

Reference		6

SUMMARY

This Recommendation defines a colour data representation method in order to make it possible to interchange continuous tone colour image data over facsimile communication services such as Group 4 and Group 3 facsimile. CIELAB space is selected as the basic colour space. CIE Illuminant D50 and its perfectly diffuse reflecting white point (X0  96.422; Y0  100.000; Z0  82.521) is selected as the basic illuminant and white point respectively. The default gamut range chosen is L*  [0, 100], a*  [–85, 85], b*  [–75, 125]. The exact expression is in terms of offset and scale.

Main differences from the previous version are the following:

1)	arbitrary N bit/component extension;

2)	colour reproduction information extension.

Recommendation T.42

Recommendation T.42     (10/96)

Continuous  Tone  Colour  Representation  
Method  for  Facsimile

(revised in 1996)

# 1	Scope

1.1	This Recommendation defines a continuous tone colour data representation method in order to make it possible to interchange continuous tone colour image data over a facsimile communication service such as Group 4 or Group 3 facsimile.

Its purpose is to specify a colour space, reference white point, illuminant type, gamut range, and colour reproduction information for the interchange of colour data.

1.2	This Recommendation, together with documents such as parts of Recommendations T.4 and T.30, or Recommendations T.563, T.503 and T.521, will define a colour image data format that may be used by colour facsimile service and by other telematic services.

# 2	Field of application

2.1	This Recommendation defines a colour data representation method which enables a receiver to reproduce colour image data as specified by the sender. The basic values are for hard copy (printed) colour image data.

2.2	It is assumed that when a service is performed using this Recommendation, all non-basic features are subject to negotiation.

# 3	References

The following references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the references listed below.

–	CIE Publication No. 15.2 (1986), Colorimetry, Second edition.

–	ISO 5-1~4 (1983-1985), Photography – Density measurements.

–	ISO 13655:1996, Graphic technology – Spectral measurement and colorimetric computation for graphic arts images.

# 4	Definitions

The definitions in Recommendation T.411 apply to this Recommendation.

The definitions in CIE Publication No. 15.2 apply to this Recommendation.

For the purposes of this Recommendation, the following additional definition applies:

CIELAB: This refers to CIE L*a*b* colour space.

# 5	Conventions

The conventions in CIE Publication No. 15.2 apply to this Recommendation.

The conventions in ISO 13655 apply to this Recommendation.

# 6	Colour representation model

## 6.1	Overview

In order to represent continuous tone colour data accurately and uniquely, a device-independent interchange colour space is needed. This colour image space should encode the range of hard copy image data when viewed under the specified viewing conditions.

The following represents an example for the use of this model: a Sender scans an original colour image using a specific device-dependent colour space which may depend on the illuminant and/or filters of a particular scanner system. The Sender converts the device-dependent colour data to the interchange colour representation. The Sender then encodes the data using a coding algorithm such as Recommendation T.81 (JPEG). The Receiver receives the encoded data. The data is decoded and converted to the colour space which is device dependent.

In order to define the colour representation, it is necessary to specify the white point, illuminant and gamut range used in the interchange data representation.

Certain additional information for better and/or more desirable colour reproduction may optionally be specified.

## 6.2	Colour representation recommendation

### 6.2.1	1976 CIE L* a* b* (CIELAB) space

#### 6.2.1.1	Colour space specification

In this Recommendation, CIELAB space is the basic value. For interoperability, other colour spaces may be added in the future.

Conversion from spectral measurement data to CIE XYZ is defined in ISO 13655.

Conversion from CIE XYZ colour space to CIELAB is defined as in CIE Publication No. 15.2. (See Appendix II.)

White point and illuminant data, and gamut range are specified in 6.2.1.2 and 6.2.1.3.

#### 6.2.1.2	White point and illuminant data

CIE Illuminant D50 and its perfectly diffuse reflecting white point (X0  96.422; Y0  100.000; Z0  82.521) are the basic values. Other illuminants and/or white points are optional and must be negotiated before use. Optional values are for further study.

#### 6.2.1.3	Gamut range

The basic gamut range is chosen to span the union of available hard copy device gamuts as observed under D50 illumination. This range is as follows, with the exact definition expressed below:

L*		[0, 100]
	a*		[–85, 85]
	b*		[–75, 125]

The gamut range is expressed as an OFFSET in the communication space and a RANGE, with the exception of the minimum and maximum values, which are explicitly defined in the definitions above.

The calculations from real values L*a*b* to nLnanb bit integers, which are expressed by NLNaNb, are made as follows:

NL		[(2**nL  –  1)/RANGEL]		L*  OFFSETL
	Na		[(2**na  –  1)/RANGEa]		a*  OFFSETa
	Nb		[(2**nb  –  1)/ RANGEb]		b*  OFFSETb

The RANGE, OFFSET pairs for basic range L*, a* and b* for the case where NLNaNb are nLnanb bit integers are:

| Variable | Range | Offset |
| --- | --- | --- |
| L* | 100.00 | 0 |
| a* | 170.00 | 2**(na – 1) |
| b* | 200.00 | 2**(nb – 2)  2**(nb – 3) |

In the case of eight-bit values, the calculations are as follows for the basic range:

NL		[255/100]    L*
	Na		[255/170]    a* 128
	Nb		[255/200]    b* 96

In the case of twelve-bit values, the calculations are as follows for the basic range:

NL		[4095/100]    L*
	Na		[4095/170]    a* 2048
	Nb		[4095/200]    b* 1536

Other gamut range values are optional and must be negotiated before use.

For example, the following optional range:

L*		[0, 100]
	a*		[–128, 127]
	b*		[–128, 127]

would be expressed using range and offset in the case of eight-bit values by:

NL		[255/100]    L*
	Na		[255/255]    a* 128
	Nb		[255/255]    b* 128

In the case of soft copy, optional gamut range may be used because gamut range of soft copy device is different than that of hard copy device.

Note that values of L* greater than 100 are not disallowed, but are not in general reproducible, as they correspond to colours which may be produced by fluorescence or specular reflection in the hard copy case. In the soft copy case, tristimulus values X, Y, Z of image data shall be scaled under the condition that the maximum Y should not be over 100. Usually, the maximum of a monitor or an image may be used for the scaling. Values of L* less than 0 have no physical meaning.

In addition, values of a* outside the range [–500, 500] and values of b* outside the range [–200, 200] are not representable from X, Y, Z tristimulus values, and have no meaning.

# 7	Colour reproduction information

Colour reproduction information may be optionally provided, in addition to absolute values (L*a*b*values) for colour image data. This information may be used for better and/or more desirable colour reproduction. This information is described in Table 1. Colour reproduction information is a non-basic feature.

Table 1/T.42 – Colour reproduction information list

| No. | From Receiver to Sender | From Sender to Receiver | Type of data |
| --- | --- | --- | --- |
| 1 | Device White (Paper white for hard copy, display white for soft copy) | Original* White | L*a*b* |
| 2 | Device Black (Cyan  magenta  yellow or  black colorant for hard copy, display black for soft copy) | Original* Black | L*a*b* |
| 3 | Device Cyan (Cyan colorant for hard copy, green  blue phosphor for soft copy) | Original* Cyan | L*a*b* |
| 4 | Device Magenta (Magenta colorant for hard copy, blue  red phosphor for soft copy) | Original* Magenta | L*a*b* |
| 5 | Device Yellow (Yellow colorant for hard copy, red  green phosphor for soft copy) | Original* Yellow | L*a*b* |
| 6 | Device Red (Magenta  yellow colorant for hard copy, red phosphor for soft copy) | Original* Red | L*a*b* |
| 7 | Device Green (Yellow  cyan colorant for hard copy, green phosphor for soft copy) | Original* Green | L*a*b* |
| 8 | Device Blue (Cyan  magenta colorant for hard copy, blue phosphor for soft copy) | Original* Blue | L*a*b* |
| * Original means not restricted only to input device gamut, but sometimes corresponded to original image data gamut. |  |  |  |

Appendix I

Method for colorimetric calculation from spectral measurement

The following is a brief synopsis of the material presented in ISO 13655, Graphic Technology – Spectral measurement and colorimetric computation for graphic arts images.

The data shall be measured from at least 400 nm to at least 700 nm inclusive, at not greater than 20-nm intervals. The reference for spectral data shall be based on computed data at 10-nm intervals where the spectral function is triangular with a 10-nm bandwidth at the half-power point. The measurements will be made with a sample mounted on a back backing, as defined by ISO 5, Part 4, subclause 4.7 The reflectance measurement geometry will be 45/0 or 0/45 as defined in ISO 5, Part 4. The measurement resolution shall be to the nearest 0.01% relative to a perfectly diffuse reflector.

The tristimulus values of the reference white under D50 illumination will be defined as Xn  96.422; Yn  100.000 and Zn  82.521. The spectral weights for illuminant D50 and 2° observer are given in Table I.1. These weights, Wx, Wy and Wz, will be used in the following manner to derive the tristimulus values:

X=Sλ(R(λ)Wx(λ))

summed over  ranging from 360 to 780 nm. R is the reflectance value as a function of wave length ().

Table I.1/T.42 – Spectral weights (W) for illuminant D50 and 2º observer 
for calculating tristimulus values at 10-nm intervals

| Wavelength(nm) | W(X) | W(Y) | W(Z) |
| --- | --- | --- | --- |
| 360 | 00000.000 | 00000.000 | 000000.001 |
| 370 | 00000.001 | 00000.000 | 000000.005 |
| 380 | 00000.003 | 00000.000 | 000000.013 |
| 390 | 00000.012 | 00000.000 | 000000.057 |
| 400 | 00000.060 | 00000.002 | 000000.285 |
| 410 | 00000.234 | 00000.006 | 000001.113 |
| 420 | 00000.775 | 00000.023 | 000003.723 |
| 430 | 00001.610 | 00000.066 | 000007.862 |
| 440 | 00002.453 | 00000.162 | 000012.309 |
| 450 | 00002.777 | 00000.313 | 000014.647 |
| 460 | 00002.500 | 00000.514 | 000014.346 |
| 470 | 00001.717 | 00000.798 | 000011.299 |
| 480 | 00000.861 | 00001.239 | 000007.309 |
| 490 | 00000.283 | 00001.839 | 000004.128 |
| 500 | 00000.040 | 00002.948 | 000002.466 |
| 510 | 00000.088 | 00004.632 | 000001.447 |
| 520 | 00000.593 | 00006.587 | 000000.736 |
| 530 | 00001.590 | 00008.308 | 000000.401 |
| 540 | 00002.799 | 00009.197 | 000000.196 |
| 550 | 00004.207 | 00009.650 | 000000.085 |
| 560 | 00005.657 | 00009.471 | 000000.037 |
| 570 | 00007.132 | 00008.902 | 000000.020 |
| 580 | 00008.540 | 00008.112 | 000000.015 |
| 590 | 00009.255 | 00006.829 | 000000.010 |
| 600 | 00009.835 | 00005.838 | 000000.007 |
| 610 | 00009.469 | 00004.753 | 000000.004 |
| 620 | 00008.009 | 00003.573 | 000000.002 |
| 630 | 00005.926 | 00002.443 | 000000.001 |
| 640 | 00004.171 | 00001.629 | 000000.000 |
| 650 | 00002.609 | 00000.984 | 000000.000 |
| 660 | 00001.541 | 00000.570 | 000000.000 |
| 670 | 00000.855 | 00000.313 | 000000.000 |
| 680 | 00000.434 | 00000.158 | 000000.000 |
| 690 | 00000.194 | 00000.070 | 000000.000 |
| 700 | 00000.097 | 00000.035 | 000000.000 |
| 710 | 00000.050 | 00000.018 | 000000.000 |
| 720 | 00000.022 | 00000.008 | 000000.000 |
| 730 | 00000.012 | 00000.004 | 000000.000 |
| 740 | 00000.006 | 00000.002 | 000000.000 |
| 750 | 00000.002 | 00000.001 | 000000.000 |
| 760 | 00000.001 | 00000.000 | 000000.000 |
| 770 | 00000.001 | 00000.000 | 000000.000 |
| 780 | 00000.000 | 00000.000 | 000000.000 |
| Total | X  96.421 | Y  99.997 | Z  82.524 |
| NOTE – This table is extracted from ASTM E308 – 1985. The sums are intended as check-sums for the spectral weights, and are not normative for the white point tristimulus values. |  |  |  |

Appendix II

Calculation of CIELAB values from CIEXYZ values

The CIE 1976 L*a*b* colour space is calculated from the tristimulus values X, Y, Z, where X, Y and Z represent the tristimulus values as measured using the procedure of Appendix I. Xn, Yn and Zn are tristimulus values from the reference white.

Using these values:

L*    116 (Y/Yn)1/3  –  16	for Y/Yn    0.008856
L*    903.3 Y/Yn	for Y/Yn    0.008856
a*    500 [(X/Xn)1/3  –  (Y/Yn)1/3]
b*    200 [(Y/Yn)1/3  –  (Z/Zn)1/3]

In addition, if any of the ratios X/Xn, Y/Yn or Z/Zn is equal to or less than 0.008856, it is replaced in the above formula by 7.7867F  16/116, where F is X/Xn, Y/Yn or Z/Zn, as the case may be.

Reference

–	CIE Publication No. 15.2 (1986), Colorimetry, Second edition.

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | Maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communication |
| Series Z | Programming languages |