Recommendation Q.763

xe ""§FORMATS AND CODES

1	General

ISDN user part messagesxe "ISDN user part messages"§ are carried on the signalling link by means of signal units the format of which is described in Recommendation Q.703, § 2.2.

The format of and the codes used in the service information octet are described in Recommendation Q.704, § 14.2. The service indicator for the ISDN user part is coded 0101.

The signalling information field of each message signal unit containing an ISDN user part message consists of an integral number of octets and encompasses the following parts (see Figure 1/Q.763):

a)	routing label;

b)	circuit identification code;

c)	message type code;

d)	the mandatory fixed part;

e)	the mandatory variable part;

f)	the optional part, which may contain fixed length and variable length parameter fields.

Note – The service information octet, the routing label and circuit identification code are not included in the SCCP user data parameter transferred between the ISDN user part and signalling connection control part.

A description of the various message parts is given in the following sections.

Figure 1/Q.763 - CCITT 73280

1.1	Routing label

The format and codes used for the routing label are described in Recommendation Q.704, § 2.2. For each individual circuit connection, the same routing label must be used for each message that is transmitted for that connection.

1.2	xe ""§Circuit identification code

The format of the circuit identification code (CIC) is shown in Figure 2/Q.763.

Figure 2/Q.763 - CCITT 73290

The allocation of circuit identification codes to individual circuits is determined by bilateral agreement and/or in accordance with applicable predetermined rules.

For international applications, the four spare bits of the circuit identification field are reserved for CIC extension, provided that bilateral agreement is obtained before any increase in size is performed. For national applications, the four spare bits can be used as required.

Allocations for certain applications are defined below:

a)	2048 kbit/s digital path

For circuits which are derived from a 2048 kbit/s digital path (Recommendations G.732 and G.734) the circuit identification code contains in the 5 least significant bits a binary representation of the actual number of the time slot which is assigned to the communication path.

The remaining bits in the circuit identification code are used, where necessary, to identify these circuits uniquely among all other circuits of other systems interconnecting an originating and destination point.

b)	8448 kbit/s digital path

For circuits which are derived from a 8448 kbit/s digital path (Recommendations G.744 and G.747) the circuit identification code contains in the 7 least significant bits an identfication of the circuit which is assigned to the communication path. The codes in Table 1/Q.763 are used.

The remaining bits in the circuit identification code are used, where necessary, to identify these circuits uniquely among all other circuits of other systems interconnecting an originating and destination point.

c)	Frequency division multiplex (FDM) systems in networks using the 2048 kbit/s pulse code modulation standard

For frequency division multiplex systems existing in networks that also use the 2048 kbit/s pulse code modulation standard, the circuit identification code contains in the 6 least significant bits the identification of a circuit within a group of 60 circuits carried by 5 basic frequency division multiplex groups which may or may not be part of the same supergroup. The codes in Table 2/Q.763 are used.

The remaining bits in the circuit identification code are used, where necessary, to identify these circuits uniquely among all other circuits of other systems interconnecting an originating and destination point.

TABLE 1/Q.763

0 0 0 0 0 0 0

Circuit 1

0 0 0 0 0 0 1

Circuit 2

|

|

0 0 1 1 1 1 1

Circuit 32

0 1 0 0 0 0 0

Circuit 33

|

|

1 1 1 1 1 1 0

Circuit 127

1 1 1 1 1 1 1

Circuit 128

TABLE 2/Q.763

0 0 0 0 0 0

Unallocated

0 0 0 0 0 1 
|
 0 0 1 1 0 0

Circuit 1
|
Circuit 12

1st basic (FDM) group

0 0 1 1 0 1
0 0 1 1 1 0
0 0 1 1 1 1
0 1 0 0 0 0
0 1 0 0 0 1
|
0 1 1 0 0 1

Circuit 1
Circuit 2
Circuit 3
Unallocated
Circuit 4
|
Circuit 12

2nd basic (FDM) group

0 1 1 0 1 0
|
0 1 1 1 1 1 
1 0 0 0 0 0 
1 0 0 0 0 1
|
1 0 0 1 1 0

Circuit 1
|
Circuit 6
Unallocated
Circuit 7
|
Circuit 12

3rd basic (FDM) group

1 0 0 1 1 1 
|
1 0 1 1 1 1 
1 1 0 0 0 0 
1 1 0 0 0 1
1 1 0 0 1 0 
1 1 0 0 1 1

Circuit 1
|
Circuit 9
Unallocated
Circuit 10
Circuit 11
Circuit 12

4th basic (FDM) group

1 1 0 1 0 0
|
1 1 1 1 1 1

Circuit 1
|
Circuit 12

5th basic (FDM) group

1.3	xe ""§Message type code

The message type code consists of a one octet field and is mandatory for all messages. The message type code uniquely defines the function and format of each ISDN user part message.

The allocation with reference to the appropriate descriptive section of this Recommendation is summarized in Table 3/Q.763.

1.4	Formatting principles

Each message consists of a number of PARAMETERS listed and described in § 2. Each parameter has a NAME which is coded as a single octet (see Table 4/Q.763). The length of a parameter may be fixed or variable, and a LENGTH INDICATOR of one octet for each parameter may be included as described below.

The detailed format is uniquely defined for each message type as described in § 3.

A general format diagram is shown in Figure 3/Q.763.

1.5	Mandatory fixed part

Those parameters that are mandatory and of fixed length for a particular message type will be contained in the mandatory fixed part. The position, length and order of the parameters is uniquely defined by the message type, thus the names of the parameters and the length indicators are not included in the message.

1.6	Mandatory variable part

Mandatory parameters of variable length will be included in the mandatory variable part. Pointers are used to indicate the beginning of each parameter. Each pointer is encoded as a single octet. The name of each parameter and the order in which the pointers are sent is implicit in the message type. Parameter names are, therefore, not included in the message. The details of how pointers are encoded is found in § 2.3. The number of parameters, and thus the number of pointers is uniquely defined by the message type.

A pointer is also included to indicate the beginning of the optional part. If the message type indicates that no optional part is allowed, then this pointer will not be present. If the message type indicates that an optional part is possible, but there is no optional part included in this particular message than a pointer field containing all zeros will be used. It is recommended that all future message types with a mandatory variable part indicate that an optional part is allowed.

All the pointers are sent consecutively at the beginning of the mandatory variable part. Each parameter contains the parameter length indicator followed by the contents of the parameters.

1.7	Optional part

The optional part consists of parameters that may or may not occur in any particular message type. Both fixed length and variable length parameters may be included. Optional parameters may be transmitted in any order. Each optional parameter will include the parameter name (one octet) and the length indicator (one octet) followed by the parameter contents.

1.8	End of optional parameters octet

If optional parameters are present and after all optional parameters have been sent, an “end of optional parameters” octet containing all zeros will be transmitted.

1.9	Order of transmission

Since all the fields consist of an integral number of octets, the formats are presented as a stack of octets. The first octet transmitted is the one shown at the top of the stack and the last is the one at the bottom (see Figure 3/Q.763).

Unless otherwise indicated, within each octet and subfield the bits are transmitted with the least significant bit first.

1.10	xe ""§Coding of spare bits

Spare bits are coded 0 unless indicated otherwise.

Figure 3/Q.763 - CCITT 73070

1.11	National message types and parameters

If message type codes and parameter name codes are required for national uses not included in this Recommendation, the codes chosen should be from the highest code downwards, that is, starting at code 11111111. Codes in the range 11111111 to 11100000 are reserved exclusively for this purpose.

2	xe ""§Parameter formats and codes

2.1	xe ""§Message type codes

The encoding of the message type is shown in Table 3/Q.763.

TABLE 3/Q.763

Message type

Reference (Table)

Code

Address complete

5/Q.763

00000110

Answer

6/Q.763

00001001

Blocking

23/Q.763

00010011

Blocking acknowledgement

23/Q.763

00010101

Call modification completed

24/Q.763

00011101

Call modification request

24/Q.763

00011100

Call modification reject

24/Q.763

00011110

Call progress

7/Q.763

00101100

Circuit group blocking

25/Q.763

00011000

Circuit group blocking acknowledgement

25/Q.763

00011010

Circuit group query

26/Q.763

00101010

Circuit group query response

8/Q.763

00101011

Circuit group reset

26/Q.763

00010111

Circuit group reset acknowledgement

9/Q.763

00101001

Circuit group unblocking

25/Q.763

00011001

Circuit group unblocking acknowledgement

25/Q.763

00011011

Charge information a)

(see Note)

00110001

Confusion

10/Q.763

00101111

Connect

11/Q.763

00000111

Continuity

12/Q.763

00000101

Continuity check request

23/Q.763

00010001

Delayed release a)

21/Q.763

00100111

Facility accepted

27/Q.763

00100000

Facility reject

13/Q.763

00100001

Facility request

27/Q.763

00011111

Forward transfer

21/Q.763

00001000

Information

14/Q.763

00000100

Information request

15/Q.763

00000011

Initial address

16/Q.763

00000001

Loop back acknowledgement a)

23/Q.763

00100100

Overload a)

23/Q.763

00110000

Pass–along

28/Q.763

00101000

Release

17/Q.763

00001100

Release complete

18/Q.763

00010000

Reset circuit

23/Q.763

00010010

Resume

22/Q.763

00001110

Subsequent address

19/Q.763

00000010

Suspend

22/Q.763

00001101

Unblocking

23/Q.763

00010100

Unblocking acknowledgement

23/Q.763

00010110

Unequipped CIC a)

23/Q.763

00101110

User–to–user information

20/Q.763

00101101

Reserved (used in 1984 version)

00001010 00001011 00001111 00100010 00100011 00100101 00100110

a) For national use only

Note – The format of this message is a national matter.

2.2	xe ""§Coding of the length indicator

The length indicator field is binary coded to indicate the number of octets in the parameter content field. The length indicated does not include the parameter name octet or the length indicator octet.

2.3	xe ""§Coding of the pointers

The pointer value (in binary) gives the number of octets between the pointer itself (included) and the first octet (not included) of the parameter associated with that pointer.

The pointer value all zeros is used to indicate that, in the case of optional parameters, no optional parameter is present.

3	xe ""§ISDN user part parameters

3.1	xe ""§Parameter names

The parameter name codes are given in Table 4/Q.763 together with references to the subsections in which they are described.

3.2	xe ""§Access transport

The format of the access transport parameter field is shown in Figure 4/Q.763.

Figure 4/Q.763 - T1110030-88

The information element is coded as described in Recommendation Q.931, § 4.5. Multiple Q.931 information elements can be included within the access transport parameter. The information elements applicable to a particular usage of the access transport parameter are dependent on, and will be determined by, the relevant procedures.

3.3	xe ""§Automatic congestion level

The format of the automatic congestion level parameter field is shown in Figure 5/Q.763.

Figure 5/Q.763 - T1110040-88

TABLE 4/Q.763

Parameter name

Reference (§)

Code

Access transport

3.2

00000011

Automatic congestion level

3.3

00100111

Backward call indicators

3.4

00010001

Call modification indicators

3.5

00010111

Call reference

3.6

00000001

Called party number

3.7

00000100

Calling party number

3.8

00001010

Calling party's category

3.9

00001001

Cause indicators

3.10

00010010

Circuit group supervision message type indicator

3.11

00010101

Circuit state indicator

3.12

00100110

Closed user group interlock code

3.13

00011010

Connected number

3.14

00100001

Connection request

3.15

00001101

Continuity indicators

3.16

00010000

End of optional parameters

3.17

00000000

Event information

3.18

00100100

Facility indicator

3.19

00011000

Forward call indicators

3.20

00000111

Information indicators

3.21

00001111

Information request indicators

3.22

00001110

Nature of connection indicators

3.23

00000110

Optional backward call indicators

3.24

00101001

Optional forward call indicators

3.25

00001000

Original called number

3.26

00101000

Range and status

3.27

00010110

Redirecting number

3.28

00001011

Redirection information

3.29

00010011

Redirection number

3.30

00001100

Signalling point code a)

3.31

00011110

Subsequent number

3.32

00000101

Suspend/Resume indicators

3.33

00100010

Transit network selection a)

3.34

00100011

Transmission medium requirement

3.35

00000010

User service information

3.36

00011101

User–to–user indicators

3.37

00101010

User–to–user information

3.38

00100000

Reserved (used in 1984 version, Red Book)

00010100 00011001 00011011 00011100 00011111

Reserved for multi–slot identifier

00100101

a)  For national use only

- The following codes are used in the automatic congestion level parameter field:

- 00000000		spare

- 00000001		congestion level 1 exceeded

- 00000010		congestion level 2 exceeded

- 00000011	ü

- to			ý	spare

- 11111111	þ

- 3.4	xe ""§Backward call indicators

- The format of the backward call indicators parameter field is shown in Figure 6/Q.763.

- Figure 6/Q.763 - CCITT 73375

- The following codes are used in the backward call indicators parameter field:

- bits	B	A:	Charge indicator

- 0	0	no indication

- 0	1	no charge

- 1	0	charge

- 1	1	spare

- bits	D	C:	Called party's status indicator

- 0	0	no indication

- 0	1	subscriber free

- 1	0	connect when free

- 1	1	spare

- bits	F	E:	Called party's category indicator

- 0	0	no indication

- 0	1	ordinary subscriber

- 1	0	payphone

- 1	1	spare

- bits	H	G:	End–to–end method indicator (Note)

- 0	0	no end–to–end method available (only link–by–link method available)

- 0	1	pass along method available

- 1	0	SCCP method available

- 1	1	pass along and SCCP methods available

- bit	I:		Interworking indicator (Note)

- 0		no interworking encountered

- 1		interworking encountered

- bit	J:		End–to–end information indicator (Note)

- 0		no end–to–end information available

- 1		end–to–end information available

- bit	K:		ISDN User Part indicator (Note)

- 0		ISDN User Part not used all the way

- 1		ISDN User Part used all the way

- bit	L:		Holding indicator (national use)

- 0		holding not requested

- 1		holding requested

- bit	M:		ISDN access indicator

- 0		terminating access non–ISDN

- 1		terminating access ISDN

- bit	N:		Echo control device indicator

- 0		incoming half echo control device not included

- 1		incoming half echo control device included

- bits	P	O:	SCCP method indicator

- 0	0	no indication

- 0	1	connectionless method available

- 1	0	connection oriented method available

- 1	1	connectionless and connection oriented methods available

- Note – Bits G–K and O–P constitute the protocol control indicator.

- 3.5	xe ""§Call modification indicators

- The format of the call modification indicators parameter field is shown in Figure 7/Q.763.

- Figure 7/Q.763 - CCITT 73370

- The following codes are used in the call modification indicators parameter field:

- bits	B	A:	Modification indicator

- 0	0	spare

- 0	1	modify to service 1

- 1	0	modify to service 2

- 1	1	spare

- bits	H	C:	Spare

- Note – Service 1 and 2 are described by the transmission medium requirement.

- 3.6	xe ""§Call reference

- The format of the call reference parameter is shown in Figure 8/Q.763.

- Figure 8/Q.763 - T1121550-89

- The following codes are used in the subfields of the call reference parameter field:

- a)	Call identity

- A code expressing in pure binary representation the identification number allocated to the call.

- b)	Point code

- The code of the signalling point in which the call identity is relevant.

- 3.7	xe ""§Called party number

- The format of the called party number parameter field is shown in Figure 9/Q.763.

- Figure 9/Q.763 - T1110050-88

- The following codes are used in the subfields of the called party number parameter field:

- a)	Odd/even indicator

- 0	even number of address signals

- 1	odd number of address signals

- b)	Nature of address indicator

- 0000000		spare

- 0000001		subscriber number

- 0000010		spare, reserved for national use

- 0000011		national (significant) number

- 0000100		international number

- 0000101	ü

- to		ý	spare

- 1101111	þ

- 1110000	ü

- to		ý	reserved for national use

- 1111110	þ

- 1111111		spare

- c)	Internal network number indicator (INN ind.)

- 0	routing to internal network number allowed

- 1	routing to internal network number not allowed

- d)	Numbering plan indicator

- 000	spare

- 001	ISDN (Telephony) numbering plan (Recommandation E.164, E.163)

- 010	spare

- 011	Data numbering plan (Recommandation X.121)

- 100	Telex numbering plan (Recommendation F.69)

- 101	reserved for national use

- 110	reserved for national use

- 111	spare

- e)	Address singal

- 0000	digit 0

- 0001	digit 1

- 0010	digit 2

- 0011	digit 3

- 0100	digit 4

- 0101	digit 5

- 0110	digit 6

- 0111	digit 7

- 1000	digit 8

- 1001	digit 9

- 1010	spare

- 1011	code 11

- 1100	code 12

- 1101	spare

- 1110	spare

- 1111	ST

- The most significant address signal is sent first. Subsequent address signals are sent in successive 4–bit fields.

- f)	Filler

- In case of an odd number of address signals, the filler code 0000 is inserted after the last address signal.

- 3.8	xe ""§Calling party number

- The format of the calling party number parameter field is shown in Figure 10/Q.763.

- Figure 10/Q.763 - T1110060-88

- The following codes are used in the calling party number parameter field.

- a)	Odd/even indicator:

- See § 3.7 a).

- b)	Nature of address indicator

- 0000000		spare

- 0000001		subscriber number

- 0000010		spare, reserved for national use

- 0000011		national (significant) number

- 0000100		international number

- 0000101	ü

- to		ý	spare

- 1101111	þ

- 1110000	ü

- to		ý	reserved for national use

- 1111110	þ

- 1111111		spare

- Note – Other types of nature of address indications (e.g. transit exchange identity) are for further study.

- c)	Calling party number incomplete indicator (NI)

- 0	complete

- 1	incomplete

- d)	Numbering plan indicator

- See § 3.7 d).

- e)	Address presentation restricted (Pres. Restric.) indicator

- 00	presentation allowed

- 01	presentation restricted

- 10	address not available (Note)

- 11	spare

- Note – When the address is unavailable, the subfields in items a), b), c) and d) are coded with 0's.

- f)	Sceening indicator

- 00	reserved (Note)

- 01	user provided, verified and passed

- 10	reserved (Note)

- 11	network provided

- Note – Code 00 and 10 are reserved for “user provided, not verified” and “user provided, verified and failed” respectively.

- g)	Address signal

- 0000		digit 0

- 0001		digit 1

- 0010		digit 2

- 0011		digit 3

- 0100		digit 4

- 0101		digit 5

- 0110		digit 6

- 0111		digit 7

- 1000		digit 8

- 1001		digit 9

- 1010		spare

- 1111		code 11

- 1100		code 12

- 1101	ü

- to		ý	spare

- 1111	þ

- h)	Filler

- See § 3.7 f).

- 3.9	xe ""§Calling party's category

- The format of the calling party's category parameter field is shown in Figure 11/Q.763.

- Figure 11/Q.763 - CCITT 73380

- The following codes are used in the calling party's category parameter field.

- 00000000		calling party's category unknown at this time

- 00000001		operator, language French

- 00000010		operator, language English

- 00000011		operator, language German

- 00000100		operator, language Russian

- 00000101		operator, language Spanish

- 00000110	ü	available to Administrations for

- 00000111	ý	selecting a particular language

- 00001000	þ	by mutual agreement

- 00001001		reserved (see Recommendation Q.104) (Note)

- 00001010		ordinary calling subscriber

- 00001011		calling subscriber with priority

- 00001100		data call (voice band data)

- 00001101		test call

- 00001110		spare

- 00001111		payphone

- 00010000	ü

- to			ý	spare

- 11011111	þ

- 11100000	ü

- to			ý	reserved for national use

- 11111110	þ

- 11111111		spare

- Note – In national networks code 00001001 may be used to indicate that the calling party is a national operator.

- 3.10	xe ""§Cause indicators

- The format of the cause indicators parameter field is shown in Figure 12/Q.763.

- Figure 12/Q.763 - T1110070-88

- The following codes are used in the subfields of the cause indicators parameter field:

- a)	Extension indicator (ext)

- 0	octet continues through the next octet (e.g. octet 1 to 1a)

- 1	last octet

- b)	Coding standard

- 00	CCITT standard, as described below

- 01	reserved for other international standards (Note)

- 10	national standard (Note)

- 11	standard specific to identified location (Note)

- Note – These other coding standards should be used only when the desired cause cannot be represented with the CCITT standard.

- c)	Location

- 0000	user

- 0001	private network serving the local user

- 0010	public network serving the local user

- 0011	transit network

- 0100	public network serving the remote user

- 0101	private network serving the remote user

- 0111	international network

- 1010	beyond an interworking point, all other values are reserved.

- Note – Depending on the location of the users, the public network serving the local user may be the same network serving the remote user. Rules for coding the location field are defined in Recommendation Q.931 Annex J.

- d)	Recommendation

- 0000000	Rec. Q.763

- 0000011	Rec. X.21

- 0000100	Rec. X.25

- 0000101	Public land mobile networks, Q.1000 Series.

- All other values are reserved.

- Note – If octet 1a is omitted, Recommendation Q.763 is assumed.

- e)	Cause value

- The cause value is divided into two fields, a class (bits 5 through 7) and a value within a class (bits 1 through 4). The decimal equivalent of the cause value is shown in brackets beside the cause value.

- Class 000 and 001 – normal event:

- 0000001	(1)	unallocated (unassigned) number

- 0000010	(2)	no route to specified transit network (national use)

- 0000011	(3)	no route to destination

- 0000100	(4)	send special information tone

- 0000101	(5)	misdialled trunk prefix

- 0010000	(16)	normal call clearing

- 0010001	(17)	user busy

- 0010010	(18)	no user responding

- 0010011	(19)	no answer from user (user alerted)

- 0010101	(21)	call rejected

- 0010110	(22)	number changed

- 0011011	(27)	destination out of order

- 0011100	(28)	address incomplete

- 0011101	(29)	facility rejected

- 0011111	(31)	normal – unspecified

- Class 010 – resource unavailable:

- 0100010	(34)	no circuit available

- 0100110	(38)	network out of order

- 0101001	(41)	temporary failure

- 0101010	(42)	switching equipment congestion

- 0101100	(44)	requested channel not available

- 0101111	(47)	resource unavailable – unspecified

- Class 011 – service or option not available:

- 0110010	(50)	requested facility not subscribed

- 0110111	(55)	incoming calls barred within CUG

- 0111001	(57)	bearer capability not authorized

- 0111010	(58)	bearer capability not presently available

- 0111111	(63)	service/option not available – unspecified

- Class 100 – service or option not implemented:

- 1000001	(65)	bearer capability not implemented

- 1000101	(69)	requested facility not implemented

- 1000110	(70)	only restricted digital information bearer capability is available

- 1001111	(79)	service or option not implemented – unspecified

- Class 101 – invalid mesage (e.g. parameter out of range):

- 1010111	(87)	called user not member of CUG

- 1011000	(88)	incompatible destination

- 1011011	(91)	invalid transit network selection (national use)

- 1011111	(95)	invalid message – unspecified

- Class 110 – protocol error (e.g. unknown message):

- 1100001	(97)	message type non–existant or not implemented

- 1100011	(99)	parameter non–existant or not implemented – discarded

- 1100101	(103)	parameter non–existent or not implemented – passed on

- 1101111	(111)	protocol error – unspecified

- Class 111 – interworking:

- 1111111	(127)	interworking unspecified

- f)	Diagnostic

- The format and existence of the diagnostic field is dependant on the cause value and the location of generation. For causes generated by a public network, the following diagnostics may be included:

- Cause

- Diagnostic

- Format

- 1

- Condition

- See below

- 2

- Transit Network identity

- See § 3.34 (Note)

- 3

- Condition

- See below

- 16

- Condition

- See below

- 21

- Condition

- See below

- 22

- Called party number (new)

- See § 3.7 (Note)

- 29

- Rejected parameter (Note)

- 50

- Rejected parameter (Note)

- 57

- Attribute identity

- See below

- 58

- Attribute identity

- See below

- 65

- Attribute identity

- See below

- 69

- Rejected parameter (Note)

- 97

- Message type

- See Table 3/Q.763

- 99

- Parameter name(s)

- See Table 4/Q.763

- 103

- Parameter name(s)

- See Table 4/Q.763

- Note – These diagnostics shall also include the parameter name and length octets.

- 1)	Diagnostic with attribute identity

- The format of the diagnostic field when coded with an attribute identity is shown in Figure 13/Q.763.

- Figure 13/Q.763 - T1110080-88

- The attribute number subfield identifies the rejected attribute as follows:

- 0110001	Information transfer capability

- 0110010	Information transfer mode

- 0110011	Information transfer rate

- 0110100	Structure

- 0110101	Configuration

- 0110110	Etablishment

- 0110111	Symmetry

- 0111000	Information transfer rate (dest to orig)

- 0111001	Layer identification and corresponding user information

- The rejected attribute and available attribute subfields are coded the same as in the equivalent octet of the user service information parameter field (see § 3.36) which contains the relevant attribute. Bits not related to the rejected attribute are coded 0. If more than one bearer capability attribute was rejected, the diagnostic field can be repeated.

- The extension bit (ext), when coded 0, indicates that this diagnostic continues to the next octet (e.g. octet 3a to 3b). The inclusion of the available attribute subfield is optional.

- 2)	Condition diagnostic

- A condition diagnostic is a 1 octet field containing an extension bit (bit 8) and one of the following codes in bits 2–1:

- 00	unknown

- 01	permanent

- 10	transient

- 11	spare

- Bits 3 to 7 of a condition diagnostic are spare.

- 3.11	xe ""§Circuit group supervision message type indicator

- The format of the circuit group supervision message type indicator parameter field is shown in Figure 14/Q.763.

- Figure 14/Q.763 - CCITT 73370

- The following codes are used in the circuit group supervision message type indicator parameter field:

- bits	B	A:	Type indicator

- 0	0	maintenance oriented

- 0	1	hardward failure oriented

- 1	0	reserved for national use (used in 1984 version)

- 1	1	spare

- bits	C	H:	Spare

- 3.12	xe ""§Circuit state indicator

- The format of the circuit state indicator parameter field is shown in Figure 15/Q.763.

- figure 15/Q.763 - T1110090-88

- The number of octets in the circuit state indicator parameter field is equal to the specified range + 1. Each circuit state indicator octet is associated with a circuit identification code such that octet n is associated with circuit identification code m+n–1, where m is the circuit identification code contained in the message.

- The following codes are used in each circuit state indicator octet.

- a)	for bits D C = 0 0

- bits	B	A:	Maintenance blocking state

- 0	0	transient

- 0	1	spare

- 1	0	spare

- 1	1	unequipped

- bits	E–H:	Spare

- b)	for bits D C not equal to 0 0

- bits	B	A:	Maintenance blocking state

- 0	0	no blocking (active)

- 0	1	locally blocked

- 1	0	remotely blocked

- 1	1	locally and remotely blocked

- bits	D	C:	Call processing state

- 0	1	circuit incoming busy

- 1	0	circuit outgoing busy

- 1	1	idle

- bits	F	E:	Hardware blocking state (Note)

- 0	0	no blocking (active)

- 0	1	locally blocked

- 1	0	remotely blocked

- 1	1	locally and remotely blocked

- bits	G–H:	Spare

- Note – If bits F E are not coded 0 0, bits D C must be coded 1 1.

- 3.13	xe ""§Closed user group interlock code

- The format of the closed user group interlock code parameter field is shown in Figure 16/Q.763.

- Figure 16/Q.763 - T1110100-88

- The following codes are used in the subfields of the closed user group interlock code parameter field:

- a)	Network identity (NI) (octets 1 and 2)

- Each digit is coded in the binary coded decimal representation from 0 to 9.

- If the first digit of this field is coded 0 or 9, the TCC (Telephony Country Code) follows in the second to fourth NI digits (the most significant TCC digit is in the 2nd NI digit). If the TCC is one or two digits long, the excess digit(s) is inserted with the code for RPOA or network identification, if necessary. If octet 2 is not required, it is coded all 0.

- Coding of the first digit as 1 or 8 is excluded.

- If the first digit is not 0, 9, 1 or 8 this field contains a DNIC (Data Network Identification Code) as defined in Recommendation X.121.

- b)	Binary code (octets 3 and 4)

- A code allocated to a closed user group administered by a particular ISDN or data network. Bit 8 of octet 3 is the most significant and bit 1 of octet 4 is the least significant.

- 3.14	xe ""§Connected number

- The format of the connected number parameter field corresponds to the format shown in Figure 17/Q.763.

- Figure 17/Q.763 - T1110060-88

- The following codes are used in the subfields of the connected number parameter field:

- a)	Odd/even indicator: see § 3.7 a)

- b)	Nature of address indicator: see § 3.7 b)

- c)	Numbering plan indicator: see § 3.7 d)

- d)	Address presentation restricted indicator: see § 3.8 e)

- e)	Screening indicator: see § 3.8 f)

- f)	Address signal: see § 3.8 g)

- g)	Filler: see § 3.7 h).

- 3.15	xe ""§Connection request

- The format of the connection request parameter field is shown in Figure 18/Q.763.

- The following codes are used in the subfields of the connection request parameter field:

- a)	Local reference

- A code indicating the local reference allocated by the signalling connection control part to the end–to–end connection.

- b)	Point code

- A code identifying the signalling point at which the connection request originated.

- c)	Protocol class

- A code identifying in pure binary representation, the protocol class requested for the end–to–end connection.

- d)	Credit

- A code identifying in pure binary representation the window size requested for the end–to–end connection.

- Figure 18/Q.763 - CCITT 73351

- 3.16	xe ""§Continuity indicators

- The format of the continuity indicators parameter field is shown in Figure 19/Q.763.

- Figure 19/Q.763 - CCITT 73370

- The following codes are used in the continuity indicators parameter field.

- bit	A:		Continuity indicator

- 0		continuity check failed

- 1		continuity check successful

- bits	B–H:	Spare

- 3.17	xe ""§End of optional parameters indicator

- The last optional parameter field of a message is followed by the end of optional parameters indicator, which occupies a one octet field containing all zeros.

- 3.18	xe ""§Event information

- The format of the event information parameter field is shown in Figure 20/Q.763.

- Figure 20/Q.763 - CCITT73370

- The following codes are used in the event indicator parameter field:

- bits	GFEDCBA:	Event indicator

- 0000000		spare

- 0000001		ALERTING

- 0000010		PROGRESS

- 0000011		in–band information of an appropriate pattern is now available

- 0000100		call forwarded on busy

- 0000101		call forwarded on no reply

- 0000110		call forwarded unconditional

- 0000111	ü

- to		ý	spare

- 1111111	þ

- bit	H:	Event presentation restricted indicator

- 0	no indication

- 1	presentation restricted

- 3.19	xe ""§Facility indicator

- The format of the facility indicator parameter field is shown in Figure 21/Q.763.

- Figure 21/Q.763 - CCITT 73380

- The following codes are used in the facility indicator parameter field.

- 00000000		spare

- 00000001		spare

- 00000010		user–to–user service

- 00000011	ü

- to			ý	spare

- 11111111	þ

- 3.20	xe ""§Forward call indicators

- The format of the forward call indicators parameter field is shown in Figure 22/Q.763.

- Figure 22/Q.763 - CCITT 73375

- The following codes are used in the forward call indicators parameter field:

- bit	A:		National/international call indicator

- 0		call to be treated as a national call

- 1		call to be treated as an international call

- This bit can be set to any value in the country of origin. In the international network this bit is not checked. In the destination country, calls from the international network will have this bit set to 1.

- bits	C	B:	End–to–end method indicator (Note)

- 0	0	no end–to–end method available (only link–by–link method available)

- 0	1	pass along method available

- 1	0	SCCP method available

- 1	1	pass along and SCCP methods available

- bit	D:		Interworking indicator (Note)

- 0		no interworking encountered (No. 7 signalling