MTP LEVEL 3

TEST NUMBER:	8.3

PAGE:	1 of 1

REFERENCE:	Q.704   § 11.2.7

TITLE:	Signalling traffic flow control

SUB TITLE:	Reception of a UPU

PURPOSE:	To check the actions of the system on reception of a UPU

PRE–TEST CONDITIONS:	One link available

CONFIGURATION:	A

TYPE OF TEST:	VAT

TYPE OF SP:	see note

MESSAGE SEQUENCE:

SP	A

SP	B

Link

Link

:Star traffic

1 – 1		TRAFFIC
			(DPC=B, SI=X)

------------>

1 – 1		TRAFFIC
			(DPC=C, SI=X)

------------>

<------------

1 – 1		TRAFFIC
	(OPC=C, SI=X)

<------------

1 – 1		UPU
	(OPC=B, SI=X)

1 – 1		TRAFFIC
			(DPC=C, SI=X)

------------>

<------------

1 – 1		TRAFFIC
	(OPC=C, SI=X)

:Wait

:Stop traffic

Note – The impact of the reception of a UPU on the traffic from A to B requires further study. The SPs having user part(s) are concerned.

TEST DESCRIPTION

1.

Start traffic to B and C with SI=X.

2.

Send a UPU from B to C with SI=X.

3.

Check that the UPU message is received correctly without impact on the traffic from to A to C.

4.

Wait and stop traffic.

MTP LEVEL 3

TEST NUMBER:	8.4

PAGE:	1 of 1

REFERENCE:	Q.704   § 11.2.7

TITLE:	Signalling traffic flow control

SUB TITLE:	Sending of a UPU

PURPOSE:	To check the detection of an unavailability of a user part

PRE–TEST CONDITIONS:	One link available

CONFIGURATION:	A

TYPE OF TEST:	VAT

TYPE OF SP:	See note

MESSAGE SEQUENCE:

SP	A

SP	B

Link

Link

:Start traffic

1 – 1		TRAFFIC
			(to B and C, SI=X)

------------------------->

<-------------------------

1 – 1	TRAFFIC
	(from B and C, SI=X)

:Deactivate user part X (see note)

<-------------------------

1 – 1		MESSAGE
	(from B to A, SI=X)

1 – 1		UPU
			(DPC = B, SI=X)

------------------------->

<-------------------------

1 – 1		MESSAGE
	(from C to A, SI=X)

1 – 1		UPU
			(DPC = C, SI=X)

------------------------->

<-------------------------

1 – 1		MESSAGE
	(from B to A, SI=X)

1 – 1			UPU
				(DPC = B, SI=X)
			:Reactivate user part X

------------------------->

<-------------------------

1 – 1		TRAFFIC
	(from B and C to A, SI=X)

1 – 1		TRAFFIC
			(to B and C, SI=X)

------------------------->

:Wait

:Stop traffic

Note – The notion of unavailability of a user part is specific to the implementation, consequently, the ability to deactivate a user part is implementation dependent. The SPs having user part(s) are concerned.

TEST DESCRIPTION

1.

Start traffic to B and C with SI = X.

2.

Deactivate the user part X.

3.

Send a message from B to the user part X in A and check that this message is discarded and that a UPU is sent back.

4.

Send a message from C to the user part X in A and check that this message is discarded and that a UPU is sent back.

5.

Repeat point 3 and reactivate the user part.

6.

Check that the messages sent from B and C are received correctly and that no UPU is sent back. Wait and stop traffic.

MTP LEVEL 3

TEST NUMBER:	9.1.1

PAGE:	1 of 1

REFERENCE:	Q.704   § 13,   Fig. 29, Fig. 44

TITLE:	Signalling route management

SUB TITLE:	Sending of a TFP on an alternative route – failure of normal linkset

PURPOSE:	To check the sending of a TFP on the alternative route when the normal linkset becomes unavailable

PRE–TEST CONDITIONS:	All linksets available

CONFIGURATION:	D

TYPE OF TEST:	VAT, CPT

TYPE OF SP:	STP

MESSAGE SEQUENCE:

SP 	A

SP	B

SP	C

SP	·

Link

Link

Link

Link

:Start traffic

1 – 1

TRAFFIC

5 – 1	 ----------->

---------------------------->

SP

D

(from A and F)

6 – 1

---------------------------->

SP

E

2 – 1

TRAFFIC

(from A and F)

-------------------------------->

7 – 1	 -------->

SP

E

1 – 1

:Deactivate			(MML command or failure)

2 – 1

TFP, PC = B

-------------------------------->

2 – 1

TFP, PC = D

-------------------------------->

2 – 1

TRAFFIC

-------------------------------->

7 – 1	 -------->

SP

E

(from 1 – 1)

8 – 1	 -------->

SP

D

:Wait

:Stop traffic

Note – A changeover procedure is performed after deactivation of link 1 – 1 but it is not described in this transfer prohibited test.

TEST DESCRIPTION

1.

Start traffic to D and E on linkset 1 and 2.

2.

Deactivate link 1 – 1 and check that TFPs concerning B and D are sent from A to C (alternative route to reach B and D). Check that no TFP concerning E is sent from A to C (load sharing between linksets 1 and 2 in A to reach E).

3.

Check that time out T8 is started for each TFP sent.

4.

Check that the traffic to D and E is diverted to C.

5.

Stop traffic and check that it was not disturbed.

MTP LEVEL 3

TEST NUMBER:	9.1.2

PAGE:	1 of 1

REFERENCE:	Q.704   § 13,   Fig. 29, Fig. 44

TITLE:	Signalling route management

SUB TITLE:	Sending of a TFP on an alternative route – on reception of a TFP

PURPOSE:	To check the sending of a TFP on the alternative route when the normal route becomes unavailable on reception of a TFP

PRE–TEST CONDITIONS:	Linkset 4 unavailable

CONFIGURATION:	D

TYPE PF TEST:	VAT, CPT

TYPE OF SP:	STP

MESSAGE SEQUENCE:

SP	A

SP	B

SP	C

SP

·

Link

Link

Link

Link

:Start traffic

1 – 1

TRAFFIC

----------->

5 – 1

----------------------------->

SP

D

(from A and F)

6 – 1

----------------------------->

SP

E

2 – 1

TRAFFIC

---------------------------------------->	7 – 1

---------->

(from A and F)

SP

E

5 – 1

:Deactivate

See note

<-----------

1 – 1	TFP, PC = D

2 – 1

TFP, PC=D

-------------------------------------->

1 – 1

TRAFFIC

----------->

6 – 1

-------------------------------->

SP

E

(from A and F)

2 – 1

TRAFFIC

--------------------------------------->	8 – 1

----------->

SP

D

(from A and F, and from 1 – 1 to D)	7 – 1

----------->

SP

E

:Wait

:Stop traffic

Note – A forced rerouting is performed after the reception of TFP for D in A but it is not described in this transfer prohibited test.

TEST DESCRIPTION

1.

Start traffic to D and E.

2.

Deactivate link 5 – 1 and check that a TFP concerning D is sent to A.

3.

Check that a TFP concerning D is received from A and that traffic to D is diverted via C.

4.

Check that a time out T8 is started.

5.

Stop traffic and check that traffic to E has not been disturbed. Some messages to D may have been lost.

MTP LEVEL 3

TEST NUMBER:	9.2.1

PAGE:	1 of 1

REFERENCE:	Q.704   § 13,   Fig. 29, Fig. 44

TITLE:	Signalling route management

SUB TITLE:	Broadcast of TFPs – on one linkset failure

PURPOSE:	To check the broadcast of TFPs when one point is inaccessible

PRE–TEST CONDITIONS:	All linksets available

CONFIGURATION:	D

TYPE OF TEST:	VAT, CPT

TYPE OF SP:	STP

MESSAGE SEQUENCE:

SP	A

SP	B

SP	C

SP	F

Link

Link

Link

Link

:Start traffic

3 – 1

TRAFFIC

------------------------------------------------------>

(from A, D and E)

3 – 1

:Deactivate   (MML command or failure)

1 – 1

TFP, P C= F

------------>

2 – 1

TFP, PC = F

-------------------------------->

:Wait

:Stop traffic

Note – The propagation of TFPs is not presented to simplify the test description.

TEST DESCRIPTION

1.

Start traffic to F.

2.

Deactivate link 1 – 1 and check that TFPs concerning F are broadcasted.

3.

Check that a timer T8 is started.

4.

Stop traffic.

- MTP LEVEL 3

- TEST NUMBER:	9.2.2

- PAGE:	1 of 2

- REFERENCE:	Q.704 § 13 Fig. 29, Fig. 44

- TITLE:	Signalling route management

- SUB TITLE:	Broadcast of TFPs – On multiple failures

- PURPOSE:	To check the broadcast of TFPs when several point are inaccessible (various reasons)

- PRE–TEST CONDITIONS:	Linkset 1 unavailable

- CONFIGURATION:	D

- TYPE OF TEST:	VAT, CPT

- TYPE OF SP:	STP

- MESSAGE SEQUENCE:

- SP	A

- SP	B

- SP	C

- SP	·

- Link

- Link

- Link

- Link

- :Start traffic

- 2 – 1

- TRAFFIC

- -------------------------------------->

- 7 – 1

- ------------>

- SP

- E

- (from A and F)

- 8 – 1

- ------------>

- SP

- D

- 2 – 1

- :Deactivate (MML command or failure)

- 3 – 1

- TFP, PC = B

- ---------------------------------------------------------->

- SP

- F

- 3 – 1

- TFP, PC = C

- ---------------------------------------------------------->

- 3 – 1

- TFP, PC = D

- ---------------------------------------------------------->

- 3 – 1

- TFP, PC = E

- ---------------------------------------------------------->

- :Wait

- :Stop traffic

- TEST DESCRIPTION

- 1.

- Start traffic to D and E.

- 2.

- Deactivate linkset 2 and check that TFPs concerning B, C, D and E are broadcasted (to F).

- 3.

- Check that for each TFP sent a timer T8 is started.

- 4.

- Repeat test but with linkset 2 unavailable as pre–test condition and then deactivate linkset 1.

- MTP LEVEL 3

- TEST NUMBER:	9.2.2 Continued

- PAGE:	2 of 2

- REFERENCE:	Q.704 § 13 Fig. 29, Fig. 44

- TITLE:	Signalling route management

- SUB TITLE:	Broadcast of TFPs – On multiple failures

- PURPOSE:	See page 1

- PRE–TEST CONDITIONS:	Linksets 1 and 4 unavailable

- CONFIGURATION:	D

- TYPE OF TEST:	VAT, CPT

- TYPE OF SP:	STP

- MESSAGE SEQUENCE:

- SP	A

- SP	C

- SP	D

- SP	·

- Link

- Link

- Link

- Link

- :Start traffic

- 2 – 1

- TRAFFIC

- ------->

- 8 – 1

- ----------->

- (from A and F)

- 7 – 1

- ----------------------------------->

- SP

- E

- 8 – 1

- :Deactivate

- <-------

- 2 – 1

- TFP, PC = D

- 3 – 1

- TFP, PC = D

- -------------------------------------------------------------------------->

- SP

- F

- 2 – 1

- TRAFFIC

- ------->

- 7 – 1

- ----------------------------------->

- SP

- E

- (from A and F)

- :Wait

- :Stop traffic

- TEST DESCRIPTION

- 1.

- Start traffic to D and E.

- 2.

- Deactivate linkset 8 and check that a TFP (PC = D) is sent. Check that TFPs are broadcasted (here to F).

- 3.

- Check that a time out T8 started.

- 4.

- Stop traffic and check that traffic to E has not been disturbed.

- 5.

- Repeat the test with linksets 2 and 4 unavailable as pre–test conditions and then deactivate linkset 5. Repeat the test with linksets 4 and 8 unavailable as pre–test conditions and then deactivate linkset 1.

- 6.

- Repeat the test with linksets 4 and 5 unavailable as pre–test conditions and then deactivate linkset 2.

- MTP LEVEL 3

- TEST NUMBER:	9.3

- PAGE:	1 of 2

- REFERENCE:	Q.704 § 13 Fig. 29, Fig. 44

- TITLE:	Signalling route management

- SUB TITLE:	Reception of a message for an unaccessible destination

- PURPOSE:	To check that a TFP is sent in response to a message received for an unaccessible destination

- PRE–TEST CONDITIONS:	Linksets 1, 4 and 8 unavailable

- CONFIGURATION:	D

- TYPE OF TEST:	VAT

- TYPE OF SP:	STP

- MESSAGE SEQUENCE:

- SP	A

- SP	F

- Link

- Link

- :Sent a message

- to D

- <---------------

- 3 – 1

- MESSAGE TO D

- 3 – 1

- TFP, PC = D

- --------------->

- ½
	½T8
	½

- <---------------

- 3 – 1

- MESSAGE TO D

- TEST DESCRIPTION

- 1.

- Send from F a message with OPC = D to A.

- 2.

- Check that a TFP PC = D is sent in response. Check that a time out T8 is started.

- 3.

- During T8, send a new message with OPC = D to A and check that no TFP is sent.

- MTP LEVEL 3

- TEST NUMBER:	9.3 Continued

- PAGE:	2 of 2

- REFERENCE:	Q.704 § 13 Fig. 29, Fig. 44

- TITLE:	Signalling route management

- SUB TITLE:	Reception of a message for an unaccessible destination

- PURPOSE:	See page 1

- PRE–TEST CONDITIONS:	Linksets 1 and 8 unavailable

- CONFIGURATION:	D

- TYPE OF TEST:	VAT

- TYPE OF SP:	STP

- MESSAGE SEQUENCE:

- SP	A

- SP	B

- SP	C

- SP	·

- Link

- Link

- Link

- Link

- :Start traffic

- 3 – 1

- TRAFFIC

- ---------------------------------------------------------->

- SP

- F

- (from A, D and E)

- 3 – 1

- :Deactivate (MML command or failure)

- 2 – 1

- TFP, PC = F
	½
	½  T8
	½
	½
	½
	½

- ------------------------------->
 -------------------------------½
		<---------------- ½


<-------------------------------

- 4 – 1



2 – 1

- MESSAGE TO F

- TEST DESCRIPTION

- 1.

- Start traffic to F.

- 2.

- Deactivate linkset 3 and check that TFPs are broadcasted.

- 3.

- Within T8, send one message with DPC = F from C to A and check that no TFP is sent in response.

- MTP LEVEL 3

- TEST NUMBER:	9.4.1

- PAGE:	1 of 1

- REFERENCE:	Q.704 § 13 Fig. 29, Fig. 45

- TITLE:	Signalling route management

- SUB TITLE:	Sending of a TFA on an alternative route – Recovery of normal linkset

- PURPOSE:	To check the sending of a TFA on an alternative route when the normal linkset becomes available

- PRE–TEST CONDITIONS:	Linkset 1 unavailable (end of test 9.1.1)

- CONFIGURATION:	D

- TYPE OF TEST:	VAT, CPT

- TYPE OF SP:	STP

- MESSAGE SEQUENCE:

- SP	A

- SP	B

- SP	C

- SP	·

- Link

- Link

- Link

- Link

- :Start traffic

- 2 – 1

- TRAFFIC

- ------------------------------->

- 8 – 1

- ------------>

- SP

- D

- (from A and F)

- 7 – 1

- ------------>

- SP

- E

- 1 – 1

- :Activate (depending of the activation mean previously used)

- 2 – 1

- TFA, PC = B

- ------------------------------->

- 2 – 1

- TFA, PC = D

- ------------------------------->

- 1 – 1

- TFP, PC = D

- ------->

- 1 – 1

- TFP, PC = E

- ------->

- 1 – 1

- TRAFFIC

- ------->

- 5 – 1

- ---------------------------------------------->

- SP

- D

- (from A and F and from 2 – 1)

- 6 – 1

- ---------------------------------------------->

- SP

- E

- 2 – 1

- TRAFFIC

- (from A and F)

- ------------------------------->

- 7 – 1

- ------------>

- SP

- E

- :Wait

- :Stop traffic

- Note – a changeback procedure is performed after activation of link 1–1 but it is not described in this transfer allowed test.

- TEST DESCRIPTION

- 1.

- Start traffic to D and E.

- 2.

- Activate linkset 1 and check that traffic to D and E is diverted on linkset 1 and that a TFA concerning D is sent from A to C. Check that no TFA is sent concerning E (load sharing in A between linksets 1 and 2).

- 3.

- Stop traffic and check that is was rerouted correctly without loss of messages, duplication and missequencing.

- MTP LEVEL 3

- TEST NUMBER:	9.4.2

- PAGE:	1 of 1

- REFERENCE:	Q.704 § 13 Fig. 29, Fig. 45

- TITLE:	Signalling route management

- SUB TITLE:	Sending of a TFA on an alternative route – On reception of a TFA

- PURPOSE:	To check that a TFA is sent on the alternative route when the normal route becomes available on reception of a TFA

- PRE–TEST CONDITIONS:	Linksets 4 and 5 unavailable (end of test 9.1.2)

- CONFIGURATION:	D

- TYPE OF TEST:	VAT, CPT

- TYPE OF SP:	STP

- MESSAGE SEQUENCE:

- SP	A

- SP	B

- SP	C

- SP	·

- Link

- Link

- Link

- Link

- :Start traffic

- 1 – 1

- TRAFFIC

- ------->

- 6 – 1

- ---------------------------------------------->

- SP

- E

- (from A and F)

- 2 – 1

- TRAFFIC

- ------------------------------->

- 7 – 1

- ------------>

- SP

- E

- (from A and F)

- 8 – 1

- ------------>

- SP

- D

- 5 – 1

- :Activate

- See note

- <-------

- 1 – 1

- TFA, PC = D

- 1 – 1

- TFP, PC = D

- ------->

- 2 – 1

- TFA, PC = D

- ------------------------------->

- 1 – 1

- TRAFFIC

- ------->

- 5 – 1

- ---------------------------------------------->

- SP

- D

- (from A and F, from 2 – 1 to D)

- 6 – 1

- ---------------------------------------------->

- SP

- E

- 2 – 1

- TRAFFIC

- ------------------------------->

- 7 – 1

- ------------>

- SP

- E

- (from A and F)

- :Wait

- :Stop traffic

- Note – a controlled rerouting is performed after the activation of linkset 5 it is not described in this transfer allowed test.

- TEST DESCRIPTION

- 1.

- Start traffic to D and E.

- 2.

- Activate link 5 – 1 and check that a TFA concerning D is sent to A.

- 3.

- Check that the traffic to D is diverted via B and check that a TFA concerning D is sent from A to C.

- 4.

- Stop traffic and check that traffic was not disturbed.