MTP LEVEL 3

TEST NUMBER:	1.1

PAGE:	1 of 1

REFERENCE:	Q.704 § 3 Fig. 7, Fig. 36, Fig. 37, Fig. 38

TITLE:	Signalling link management

SUBTITLE:	First signalling link activation

PURPOSE:	To put into service a signalling linkset with 1 signalling link

PRE–TEST CONDITIONS:	Signalling links deactivated

CONFIGURATION:	A

TYPE OF TEST:	VAT, CPT

TYPE OF SP:	ALL

MESSAGE SEQUENCE:

SP	A

SP		B

Link

Link

1 – 1

:Activate

1 – 1

:Activate

<----------------

1 – 1		SLTM

1 – 1

SLTA

----------------->

1 – 1

SLTM

----------------->

<----------------

1 – 1		SLTA

:Start traffic

1 – 1

TRAFFIC

----------------->

<----------------

1 – 1		TRAFFIC

:Wait

:Stop traffic

TEST DESCRIPTION

1.

Check that the signalling link becomes available.

2.

Check the reception and sending of variable length messages on the activated linkset from/to the SP at the other end of this linkset (and, in case of VAT, from/to other SP crossing the SP at the other end of this linkset).

3.

Check that, after the alignment, the level 2 does not send any message received before or during the deactivation.

4.

Check that all messages are correctly received (no loss of messages, no duplication and no missequencing).

5.

Stop traffic.

6.

Repeat the test with different SLC values.

- MTP LEVEL 3

- TEST NUMBER:	1.2

- PAGE:	1 of 1

- REFERENCE:	Q.704 § 3 Fig. 7, Fig. 36, Fig. 37, Fig. 38

- TITLE:	Signalling link management

- SUBTITLE:	Signalling linkset deactivation

- PURPOSE:	To remove from service a signalling linkset with 1 signalling link

- PRE–TEST CONDITIONS:	One signalling link (1–1) activated

- CONFIGURATION:	A

- TYPE OF TEST:	VAT, CPT

- TYPE OF SP:	ALL

- MESSAGE SEQUENCE:

- SP		A

- SP		B

- Link

- Link

- 1 – 1	Deactivate

- TEST DESCRIPTION

- 1.

- Check that the signalling linkset becomes unavailable.

- MTP LEVEL 3

- TEST NUMBER:	1.3

- PAGE:	1 of 1

- REFERENCE:	Q.704 § 3, 12.2.4.1 Fig. 7, Fig. 36, Fig. 37, Fig. 38

- TITLE:	Signalling link management

- SUBTITLE:	Signalling linkset activation

- PURPOSE:	To put into service a signalling linkset with 4 signalling links

- PRE–TEST CONDITIONS:	Signalling links deactivated

- CONFIGURATION:	A

- TYPE OF TEST:	VAT, CPT

- TYPE OF SP:	ALL

- MESSAGE SEQUENCE:

- SP		A

- SP		B

- Link

- Link

- 1 – 1

- :Activate

- 1 – 1

- :Activate

- 1 – 2

- :Activat

- 1 – 2

- :Activate

- 1 – 3

- :Activate

- 1 – 3

- :Activate

- 1 – 4

- :Activate

- 1 – 4

- :Activate

- :Start traffic

- 1 – 1

- TRAFFIC

- ----------------->

- <----------------

- 1 – 1

- TRAFFIC

- 1 – 2

- TRAFFIC

- ----------------->

- <----------------

- 1 – 2

- TRAFFIC

- 1 – 3

- TRAFFIC

- ----------------->

- <----------------

- 1 – 3

- TRAFFIC

- 1 – 4

- TRAFFIC

- ----------------->

- <----------------

- 1 – 4

- TRAFFIC

- :Wait

- :Stop traffic

- Note – This test describes the activation of a linkset. The signalling link activation order is given simultaneously to all signalling links of the signalling linkset (Q.704 § 12.2.4.1). However, depending on in which order the links are getting aligned, changeback procedures will be performed. This test does not describe the transitory states (changeback procedure is checked in other tests).

- TEST DESCRIPTION

- 1.

- Check that the signalling links become available and start traffic between A and B (and A and C in VAT).

- 2.

- Check the reception and sending of variable length messages on the activated linkset from/to the SP at the other end of this linkset (and, in case of VAT, from/to other SP crossing the SP at the other end of this linkset).

- 3.

- Check that, after the alignment, the level 2 does not send any message received before or during the deactivation.

- 4.

- Check that all messages are correctly received (no loss of messages, no duplication and no missequencing).

- 5.

- Stop traffic.

- MTP LEVEL 3

- TEST NUMBER:	2.1

- PAGE:	1 of 1

- REFERENCE:	Q.704 § 3 Fig. 24 § 2.4

- TITLE:	Signalling message handling

- SUBTITLE:	Message received with an invalid SSF (discrimination function)

- PURPOSE:	To check the response to a message with an invalid SSF

- PRE–TEST CONDITIONS:	Signalling linkset activated

- CONFIGURATION:	A

- TYPE OF TEST:	VAT

- TYPE OF SP:	ALL

- MESSAGE SEQUENCE:

- SP		A

- SP 		B

- Link

- Link

- <----------------			1 – 1

- :Invalid SLTM
:(invalid SSF)

- TEST DESCRIPTION

- 1.

- Send an SLTM with an erroneous SSF.

- 2.

- Check that no response is received.

- MTP LEVEL 3

- TEST NUMBER:	2.2

- PAGE:	1 of 1

- REFERENCE:	Q.704 § 2 Fig. 24, Fig. 26

- TITLE:	Signalling message handling

- SUBTITLE:	Message received with an invalid DPC

- PURPOSE:	To check the response to a message with an invalid DPC

- PRE–TEST CONDITIONS:	Signalling linkset activated

- CONFIGURATION:	A

- TYPE OF TEST:	VAT

- TYPE OF SP:	ALL

- MESSAGE SEQUENCE:

- SP	A

- SP	B

- Link

- Link

- <----------------				1 – 1

- :Invalid ECO
:(erronenous DPC)

- 1 – 1	TFP

- ----------------->

- (only if the tested point A has an STP function)

- TEST DESCRIPTION

- 1.

- Send a ECO message with an erroneous DPC.

- 2.

- Check that no response is received if the tested point has not STP function. If the tested point has the STP function, check that a TFP is received.

- MTP LEVEL 3

- TEST NUMBER:	2.3

- PAGE:	1 of 1

- REFERENCE:	Q.704 § 2.4 Fig. 24, Fig. 25

- TITLE:		Signalling message handling

- SUBTITLE:		Message received with an erroneous SI (distribution function)

- PURPOSE:	To check the response to a message received with an erroneous SI

- PRE–TEST CONDITIONS:	Signalling linkset activated

- CONFIGURATION:	A

- TYPE OF TEST:	VAT

- TYPE OF SP:	ALL

- MESSAGE SEQUENCE:

- SP		A

- SP	B

- Link

- Link

- <------------------

- 1 – 1		:invalid SLTM
		:(invalid SI)

- TEST DESCRIPTION

- 1.

- Send an SLTM message with an invalid SI.

- 2.

- Check that no response is received.