TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.2.2

REFERENCE:	Q.724 § 1.8

TITLE:	CGC

SUBTITLE:	CGC sent

PURPOSE:	To verify that SP A is able to generate a circuit group congestion signal

PRE–TEST CONDITIONS:	Arrange the data in signalling point A such that a circuit group congestion signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

CGC

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Attempt to make a call from SP B to SP A.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – An address complete signal (without subscriber free) may be sent in the backward direction before a CGC signal is sent.

Note 2 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.3.1

REFERENCE:	Q. 724 § 1.8

TITLE:	NNC

SUBTITLE:	NNC received

PURPOSE:	To verify that the call will be immediately released by the outgoing signalling point if a national network congestion signal is received and the correct indication is given to the calling party

PRE–TEST CONDITIONS:	Arrange the data in SP B such that a national network congestion signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	AT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM

------------------------->

<-------------------------

NNC

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Attempt to make a call from SP A to SP B.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – An address complete signal (without subscriber free) may be sent in the backward direction before a NNC signal is sent.

Note 2 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.3.2

REFERENCE:	Q.724 § 1.8

TITLE:	NNC

SUBTITLE:	NNC sent

PURPOSE:	To verify that SP A is able to generate a national network congestion signal

PRE–TEST CONDITIONS:	Arrange the data in signalling point A such that a national network congestion signal is returned to the call request, where SP A is now an I/C exchange

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

NNC

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Attempt to make a call from SP B to SP A.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – An address complete signal (without subscriber free) may be sent in the backward direction before a NNC signal is sent.

Note 2 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.4.1

REFERENCE:	Q.724 § 1.7

TITLE:	ADI

SUBTITLE:	ADI received

PURPOSE:	To verify that on receipt of an address incomplete message the call is immediately released and the correct indication given to the calling party

PRE–TEST CONDITIONS:	Signalling point B should be able to determine that the proper number of digits has not been received

CONFIGURATION:	1

TYPE OF TEST:	VAT and CPT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM

------------------------->

<-------------------------

ADI

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Make a call from SP A to SP B, but do not enter the final digit.
Record the message sequence using a signal monitor.

2.

CHECK A:	WAS THE CORRECT TONE OR ANNOUNCEMENT SENT TO THE CALLING SUBSCRIBER? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.4.2

REFERENCE:	Q.724 § 1.7

TITLE:	ADI

SUBTITLE:	ADI sent

PURPOSE:	To verify that signalling point A is able to generate an address incomplete signal

PRE–TEST CONDITIONS:	SP A should be able to determine that the proper number of digits has not been received

CONFIGURATION:	1

TYPE OF TEST:	VAT and CPT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

ADI

------------------------->

<-------------------------

DLF

RLG

------------------------->

TEST DESCRIPTION

1.

Make a call from SP B to SP A, but do not enter the final digit.
Record the message sequence using a signal monitor.

2.

CHECK A:	WAS THE CORRECT TONE OR ANNOUNCEMENT SENT TO THE CALLING SUBSCRIBER? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.5.1

REFERENCE:	Q.724 § 6.3

TITLE:	CFL

SUBTITLE:	CFL received

PURPOSE:	To verify that the call will be immediately released by the outgoing signalling point if a call failure signal is received and the correct indication is given to the calling party

PRE–TEST CONDITIONS:	Arrange the data in signalling point B such that a call failure signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM

------------------------->

<-------------------------

CFL

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Attempt to make a call from SP A to SP B.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CALL IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – An address complete signal may be sent in the backward direction before a CFL signal is sent.

Note 2 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.5.2

REFERENCE:	Q.724 § 6.3

TITLE:	CFL

SUBTITLE:	CFL sent

PURPOSE:	To verify that signalling point A is able to generate a call failure signal

PRE–TEST CONDITIONS:	Arrange the data in SP A such that a call failure signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

CFL

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Attempt to make a call from SP B to SP A.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CALL IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – An address complete signal may be sent in the backward direction before a CFL signal is sent.

Note 2 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.6.1

REFERENCE:	Q.724 § 1.9

TITLE:	SSB

SUBTITLE:	SSB Received

PURPOSE:	To verify that the call will be immediately released by SP A if a Subscriber–busy signal is received and the correct indication is given to the calling party

PRE–TEST CONDITIONS:	Called termination must be busy

CONFIGURATION:	1

TYPE OF TEST:	VAT and CPT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM

------------------------->

<-------------------------

SSB

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Attempt to make a call from SP A to SP B.
Record the message sequence with a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

Note 2 – This sequence may not be possible at International Gateways.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.6.2

REFERENCE:	Q.724 § 1.9

TITLE:	SSB

SUBTITLE:	SSB Sent

PURPOSE:	To verify that signalling point A is able to generate or retransmit a subscriber busy signal

PRE–TEST CONDITIONS:	The called termination must be busy

CONFIGURATION:	1

TYPE OF TEST:	VAT and CPT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

SSB

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Attempt to make a call from SP B to SP A.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

Note 2 – This sequence may not be possible at International Gateways.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.7.1

REFERENCE:	Q.724

TITLE:	UNN

SUBTITLE:	UNN Received

PURPOSE:	To verify that the call will be immediately released by SP A if an Unallocated–number signal is received and the correct indication is given to the calling party

PRE–TEST CONDITIONS:	Arrange the data in signalling point B such that a UNN Signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT and CPT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM

------------------------->

<-------------------------

UNN

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Attempt to make a call from SP A to SP B.
Record the message sequence with a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

Note 2 – This sequence may not be possible at International Gateways.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.7.2

REFERENCE:	Q.724

TITLE:	UNN

SUBTITLE:	UNN Sent

PURPOSE:	To verify that signalling point A is able to generate an Unallocated–number signal

PRE–TEST CONDITIONS:	Arrange the data in signalling point A such that a UNN Signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT and CPT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

UNN

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Attempt to make a call from SP B to SP A.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

Note 2 – This sequence may not be possible at International Gateways.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.8.1

REFERENCE:	Q.724

TITLE:	LOS

SUBTITLE:	LOS Received

PURPOSE:	Verify that the call will be immediately released by SP A if a Line out of service signal is received and the correct indication is given to the calling party

PRE–TEST CONDITIONS:	Arrange the data in signalling point B such that a LOS Signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM

------------------------->

<-------------------------

LOS

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Attempt to make a call from SP A to SP B.
Record the message sequence with a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.8.2

REFERENCE:	Q.724

TITLE:	LOS

SUBTITLE:	LOS Sent

PURPOSE:	To verify that signalling point A is able to retransmit a Line–out–of–service signal

PRE–TEST CONDITIONS:	Arrange the data in signalling point A such that a LOS Signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

LOS

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Attempt to make a call from SP B to SP A.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note 1 – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

Note 2 – This sequence may not be possible at International Gateways.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.9.1

REFERENCE:	Q.724

TITLE:	SST

SUBTITLE:	SST received

PURPOSE:	To verify that a call will be immediately released by the outgoing signalling point if a send–special–information–tone signal is received and the correct indication is given to the calling party

PRE–TEST CONDITIONS:	Arrange the data in signalling point B such that a SST signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM

------------------------->

<-------------------------

SST

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Attempt to make a call from SP A to SP B
Record the message sequence with a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.9.2

REFERENCE:	Q.724

TITLE:	SST

SUBTITLE:	SST sent

PURPOSE:	To verify that signalling point A is able to generate a send–special–information–tone signal

PRE–TEST CONDITIONS:	Arrange the data in signalling point A such that a SST signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

SST

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Attempt to make a call from SP B to SP A
Record the message sequence with a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.10.1

REFERENCE:	Q.724 § 10.2

TITLE:	ACB

SUBTITLE:	ACB received

PURPOSE:	To verify that because of incompatible CUG information the call is rejected and an access barred signal is returned to the call request

PRE–TEST CONDITIONS:	Arrange the signalling point data such that the CUG information contained in the IAI is compatible with the information stored at SP B

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAI

------------------------->

<-------------------------

ACB

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Make a CUG call from SP A to SP B.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.10.2

REFERENCE:	Q.724 § 10.2

TITLE:	ACB

SUBTITLE:	ACB sent

PURPOSE:	To verify that SP A is able to generate or receive an access barred signal

PRE–TEST CONDITIONS:

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAI

ACB

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Make a CUG call from SP B to SP A.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE APPROPRIATE TONE OR ANNOUNCEMENT RETURNED TO THE CALLING PARTY? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.11.1

REFERENCE:	Q.724 § 10.7

TITLE:	DPN

SUBTITLE:	DPN received

PURPOSE:	To verify that the call will be immediately released by the SP A if a digital path not provided signal is received and the correct indicator is given to the calling party

PRE–TEST CONDITIONS:

a)	Ensure the IAM is set to indicate that an all digital path is required.

b)	Ensure the data in signalling point B is configured such that a digital path not provided signal is returned to the call request.

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM

------------------------->

<-------------------------

DPN

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Attempt to make a call from SP A to SP B.
Ensuring that the IAM is set to indicate that an all digital path is required. Record the message sequence using a signal monitor.

2.

CHECK A:	IS THE CIRCUIT IDLE? . . . . . .

3.

CHECK B:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

Note – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	4.11.2

REFERENCE:	Q.724 § 10.7

TITLE:	DPN

SUBTITLE:	DPN sent

PURPOSE:	To verify that signalling point A is able to generate a digital path not provided signal

PRE–TEST CONDITIONS:	Arrange the data in signalling point A such that a DPN signal is returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

DPN

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Attempt to make a call from SP B to SP A.
Record the message sequence with a signal monitor.

2.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

3.

CHECK C:	WAS THE MESSAGE SEQUENCE AS SHOWN ABOVE? . . . . . .

Note – It may not be possible to confirm that the appropriate tone is returned to the calling party. In this case it must be verified that the signalling point under test retransmits the signal received.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	5.1

REFERENCE:	Q.724 § 6.2.1

TITLE:	Inability to release in response to a CLF

SUBTITLE:

PURPOSE:	To verify that if the signalling point is unable to return a circuit to the idle condition in response to a clear forward signal, the circuit will be blocked

PRE–TEST CONDITIONS:	Arrange the data in signalling point A such that it is unable to return the circuit to the idle condition in response to a clear forward signal

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

ACM

------------------------->

Ringing tone

–––––––––––––

ANC

------------------------->

Speech

–––––––––––––

Speech

<-------------------------

CLF

BLO

------------------------->

<-------------------------

BLA

RLG

------------------------->

TEST DESCRIPTION

1.

Make a call from SP A to SP B.
Record the message sequence using a signal monitor.

2.

CHECK A:	IS RINGING TONE HEARD? . . . . . .

3.

The called party should answer the call

4.

CHECK B:	IS SPEECH POSSIBLE? . . . . . .

5.

The calling party should release the call.

6.

CHECK C:	VERIFY THAT A CALL CAN NOT BE ORIGINATED FROM EITHER SP

7.

CHECK D:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

8.

Repeat this test in the reverse direction.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	5.2

REFERENCE:	Q.724 § 6.2.2

TITLE:	Inability to release in response to a backward signal

SUBTITLE:

PURPOSE:	To verify that if signalling point is unable to return the circuit to an idle condition in response to a backward signal, the circuit will be blocked

PRE–TEST CONDITIONS:	Arrange the data in signalling point A such that it is unable to return the circuit to an idle condition in response to a backward signal

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM

------------------------->

<-------------------------

ADI

BLO

------------------------->

<-------------------------

BLA

CLF

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Make a call from SP A to SP B, but do not enter the final digit.

2.

CHECK A:	VERIFY THAT A CALL CAN NOT BE ORIGINATED FROM EITHER EXCHANGE

3.

CHECK B:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

4.

Repeat this test in the reverse direction.

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	5.3.1

REFERENCE:	Q.724 § 6.4.3a

TITLE:	Timers

SUBTITLE:	T2

PURPOSE:	To check the value of timer T2

PRE–TEST CONDITIONS:	Arrange the data in signalling point B such that an address complete message is not returned to the call request

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

IAM		–

------------------------->

½

T2	½	T2  | 20–30 seconds

½

CLF	–

------------------------->

<-------------------------

RLG

TEST DESCRIPTION

1.

Attempt to make a call from SP A to SP B.
Record the message sequence with a signal monitor.

2.

CHECK A:	WAS THE CLEAR FORWARD SIGNAL SENT BEFORE 20–30 SECONDS? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .

TUP LEVEL 4 TEST SPECIFICATION

TEST NUMBER:	5.3.2

REFERENCE:	Q.724 § 6.4.3b

TITLE:	Timers

SUBTITLE:	T3

PURPOSE:	To check the value of timer T3

PRE–TEST CONDITIONS:

a)	Signalling point A should be able to determine that the proper number of digits have not been received.

b)	Arrange the data in signalling point B such that a clear forward signal is not returned in response to an address incomplete message.

CONFIGURATION:	1

TYPE OF TEST:	VAT

TYPE OF SP:	SP

EXPECTED MESSAGE SEQUENCE:

SP	A

SP	B

<-------------------------

IAM

ADI		–

------------------------->

½

T3	½	4–15 seconds

½

CFL	–

------------------------->

<-------------------------

CLF

RLG

------------------------->

TEST DESCRIPTION

1.

Attempt to make a call from SP B to SP A but do not dial the last digit.
Record the message sequence using a signal monitor.

2.

CHECK A:	WAS A CALL FAILURE SIGNAL SENT BETWEEN 4–15 SECONDS AFTER SENDING OF THE ADDRESS INCOMPLETE MESSAGE? . . . . . .

3.

CHECK B:	IS THE CIRCUIT IDLE? . . . . . .

4.

CHECK C:	WAS THE MESSAGE SEQUENCE AS ABOVE? . . . . . .