|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Supplement  2

TELECOMMUNICATION	(Series E)
STANDARDIZATION  SECTOR
OF  ITU	(01/94)

TELEPHONE  NETWORK  AND  ISDN

OPERATION,  NUMBERING,  
ROUTING  AND  MOBILE  SERVICE

VARIOUS  TONES  USED  
IN  NATIONAL  NETWORKS

Supplement 2
(to ITU-T Recommendation  E.180)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

Supplement 2 was revised by the ITU-T Study Group 1 (1993-1996) and was approved under the provisions of WTSC-93 Resolution No. 5 by Study Group 1 at its meeting 18-28 January 1994. This is the first revision of this supplement undertaken since 1980.

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector.

In order not to delay publication of this Recommendation, no change has been made in the text to references containing the acronyms “CCITT, CCIR or IFRB” or their associated entities such as Plenary Assembly, Secretariat, etc. Future editions of this Recommendation will contain the proper terminology related to the new ITU structure.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1994

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Supplement 2

Supplement 2  –  Series E     (01/94)

VARIOUS  TONES  USED
IN  NATIONAL  NETWORKS

| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |
| --- | --- | --- |

ALBANIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.3 – 0.3 |
| CONGESTION TONE | 425 | 0.15 – 0.15 |

ALGERIA

| RINGING TONE | 25//50 | 1.5 – 3.5 |
| --- | --- | --- |
| BUSY TONE | 450 | 0.5 – 0.5 |

ANGOLA

| DIAL TONE | 400 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 25 | 1.0 – 5.0 |
| BUSY TONE | 400 | 0.5 – 0.5 |
| CONGESTION TONE | 400 | 0.5 – 0.5 |
| NUMBER UNOBTAINABLE TONE | 400 | 0.2 – 0.2 |
| WAITING TONE | 400 | 1.0 – 5.0 |

ANGUILLA

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 350+440 | 3×(0.1 – 0.1) – CONTINUOUS |
| RINGING TONE | 440+480 | 2.0 – 4.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 450 | 0.2 – 0.2 – 0.65 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 480+620 | 0.25 – 0.25 |
| WAITING TONE | 440 | 1×(0.5 – 10.0 – 0.5) |
| PAYPHONE RECOGNITION TONE | 1600/1143 | 0.2 – 0.2 – 0.2 – 2.0 |

ANTIGUA AND BARBUDA

| DIAL TONE | 350/440 | CONTINUOUS |  |
| --- | --- | --- | --- |
| SECOND DIAL TONE | 350/440 | CONTINUOUS |  |
| RECALL DIAL TONE | 350/440 | CONTINUOUS |  |
| RINGING TONE | 440/480 | 2.0 – 4.0 |  |
| BUSY TONE | 480/620 | 0.5 – 0.5 |  |
| CONGESTION TONE | 480/620 | 0.25 – 0.25 |  |
| NUMBER UNOBTAINABLE TONE |  | recorded announcement |  |
| WAITING TONE | 480 | 0.5 – 10.0 – 0.5 (once only) |  |
| TEST NUMBER TONE | 1004 | CONTINUOUS |  |

ARGENTINA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.3 – 0.2 |
| CONGESTION TONE | 425 | 0.3 – 0.4 |
| WARNING TONE – OPERATOR INTERVENING | 1400 | 5×0.1 – 1.9 |
| WAITING TONE | 425 | 0.4 – 0.2 – 0.4 – 4.0 |
| CALL WAITING TONE | 425 | 0.3 – 10.0 |
| END OF THREE PARTY SERVICE TONE | 950/1400/1800 | 3×0.33 (single burst) |
| SERVICE ACTIVATED TONE | 425 | 1.0 – 0.25 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

ARUBA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WAITING TONE | 425 | 0.2 – 0.2 – 0.2 – 4.4 |

ASCENSION

| DIAL TONE | 350/440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440/480 | 1.0 – 3.0 |
| BUSY TONE | 480/620 | 0.5 – 0.5 |
| WARNING TONE – OPERATOR INTERVENING | 440 | 0.8 – 6.0 – 0.2 – 6.0 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 480/620 | 0.25 – 0.25 |

AUSTRALIA

| DIAL TONE | 425×25 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE (PABX) | 154 1) | CONTINUOUS |
| RINGING TONE | 400×17 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| CONGESTION TONE | 400 20) | 0.375 – 0.375 |
| WARNING TONE – OPERATOR INTERVENING | 425//525 | 1.0 – 15.0 – 0.36 – 15.0 |
| NUMBER UNOBTAINABLE TONE | 400 | 2.5 – 0.5 |
| WAITING TONE | 425/525 | 0.1 – 0.1 – 0.1 – 4.7 |
| RECORD TONE | 1400 | 0.425 – 14.525 |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.075 – 0.15 – 0.075 – 2.7 |
| FACILITIES TONE | 425 | CONTINUOUS |
| OFFERING TONE | 525 6) 10) | 0.1 – 0.1 |

AUSTRIA

| DIAL TONE | 420//450 31) | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 380+420 | CONTINUOUS |
| RECALL DIAL TONE | 420 | CONTINUOUS |
| RINGING TONE | 420//450 31) | 1.0 – 5.0 |
| BUSY TONE I | 420 | 0.4 – 0.4 |
| BUSY TONE II 27) | 450 | 0.3 – 0.3 |
| CONGESTION TONE I | 420 | 0.2 – 0.2 |
| CONGESTION TONE II 27) | 450 | 0.3 – 0.3 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 420//450 31) | 0.15 – 0.15 – 0.15 – 1.95 |
| NUMBER UNOBTAINABLE TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WAITING TONE | 420 | 0.04 – 1.95 |
| PAYPHONE RECOGNITION TONE | 1336/1633 | 0.2 – 0.2 – 0.2 – 2.0 |
| POSITIVE INDICATION TONE | 380+420 | 1.0 – 5.0 |
| NEGATIVE INDICATION TONE | 380+420 | 0.4 – 0.4 |

BAHAMAS

| RINGING TONE | 440/480//420/20//400 | 2.0 – 4.0 |
| --- | --- | --- |
| BUSY TONE | 480/620/420//400 | 0.5 – 0.5 |

BAHRAIN

| DIAL TONE | 330×440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440//450 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| CONGESTION TONE I | 400 | 0.4 – 0.35 – 0.225 – 0.525 |
| CONGESTION TONE II | 400 | 0.375 – 0.375 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

BAHRAIN (continuation)

| INTERCEPT TONE | 450//425+475 | 0.05 – 0.05 |
| --- | --- | --- |
| ROUTE TONE | 425 | 0.06 – 0.06 |
| PAYPHONE RECOGNITION TONE | 816/1209 | 0.125 – 1.5 |

BARBADOS

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 350+440 | 2×(1.0 – 1.0) – CONTINUOUS |
| RECALL DIAL TONE | 350+440 | 3×(0.1 – 0.1) – CONTINUOUS |
| RINGING TONE | 440+480 | 2.0 – 4.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 | 0.25 – 0.25 |

BELARUS

| RINGING TONE | 425 | 0.8 – 3.2 |
| --- | --- | --- |
| BUSY TONE | 425 | 0.4 – 0.4 |

BELGIUM

| DIAL TONE | 425//450 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE 27) | 900/1020/1140 | 3×0.333 |
| RINGING TONE | 425//450 | 1.0 – 3.0 |
| BUSY TONE | 425//450 | 0.5 – 0.5 |
| CONGESTION TONE | 425//450 | 0.167 – 0.167 |
| SPECIAL INFORMATION TONE | 900/1380/1860 | 3×0.333 – 1.0 |

BENIN

| RINGING TONE | 440 | 1.5 – 3.5 |
| --- | --- | --- |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |

BERMUDA

| DIAL TONE | 350+440 |  | CONTINUOUS |
| --- | --- | --- | --- |
| RINGING TONE | 440+480 |  | 2.0 – 4.0 |
| BUSY TONE | 480+620 |  | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 |  | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 200//400 |  | 0.5 (for six seconds) |
| WAITING TONE | 440 | (two bursts, ten seconds apart) |  |

BHUTAN

| DIAL TONE | 400+25 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400+25 | 0.4 – 0.2 – 0.4 – 2.6 |
| BUSY TONE | 400 | 0.6 – 0.6 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| WAITING TONE | 400 | 0.5 – 0.25 |
| OFFERING TONE | 450 | 0.2 – 0.2 – 0.2 – 0.6 |

BOTSWANA

| DIAL TONE | 350+450 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400//450 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.16 – 2.0 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| WAITING TONE | 425 | 0.2 – 1.0 |
| PAYPHONE RECOGNITION TONE | 1600 | 0.07 – 2.0 |

BRAZIL

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE (PABX) | 425 | 0.975 – 0.05 |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.25 – 0.25 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

BRAZIL (continuation)

| NUMBER UNOBTAINABLE TONE | 425 | 0.75 – 0.25 – 0.25 – 0.25 |
| --- | --- | --- |
| WAITING TONE | 425 | 0.05 – 1.0 |
| EXECUTIVE OVERRIDE TONE | 750 | 0.02 – 1.0 |
| FUNCTION ACKNOWLEDGE TONE | 425 | 0.1 – 0.1 – 0.1 – 2.0 |
| PAY TONE | 300 2) | 0.75 |

BRITISH VIRGIN ISLANDS

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 350+440 | 3×(0.1 – 0.1) – CONTINUOUS |
| RINGING TONE | 440+480 | 2.0 – 4.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 450 | 0.2 – 0.2 – 0.65 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 480+620 | 0.25 – 0.25 |
| WAITING TONE | 440 | 1×(0.5 – 10.0 – 0.5) |
| PAYPHONE RECOGNITION TONE | 1600/1143 | 0.2 – 0.2 – 0.2 – 2.0 |

BRUNEI DARUSSALAM

| DIAL TONE | 400×50 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE – TRANSMISSION | 1000 | CONTINUOUS |
| RINGING TONE | 400+450 | 0.4 – 0.3 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| CONGESTION TONE | 400 | 0.375 – 0.375 |
| WARNING TONE – OPERATOR INTERVENING | 400 | 0.25 – 2.0 |
| WARNING TONE – END OF PERIOD | 400 | 0.624 – 4.376 |
| WARNING TONE – PIP TONE | 900 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| WAITING TONE | 400×24 | 0.5 |
| CONFIRMATION TONE | 400×24 | 0.1 – 0.1 – 0.3 – 0.3 |
| HOLDING TONE | 400×24 | 0.5 – 2.5 |

BULGARIA

| DIAL TONE I | 425 | 0.25 – 0.3 – 0.7 – 0.8 |
| --- | --- | --- |
| DIAL TONE II | 425 | 0.25 – 0.75 – 0.75 – 1.0 |
| RINGING TONE I | 425 | 1.0 – 9.0 |
| RINGING TONE II | 425 | 1.0 – 4.0 |
| RINGING TONE III | 425 | 1.83 – 3.0 |
| BUSY TONE I | 425 | 0.25 – 0.25 |
| BUSY TONE II | 425 | 0.5 – 0.5 |
| BUSY TONE III | 425 | 0.2 – 0.5 |
| CONGESTION TONE I | 425 | 0.25 – 0.25 |
| CONGESTION TONE II | 425 | 0.5 – 0.5 |
| CONGESTION TONE III | 425 | 0.2 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING I | 425 | 0.15 – 0.25 – 0.15 – 1.15 |
| WARNING TONE – OPERATOR INTERVENING II | 425 | 0.2 – 0.3 – 0.2 – 1.5 |
| NUMBER UNOBTAINABLE TONE | 950/1400/1800 | 3×0.33 – 1.0 |

BURKINA FASO

| DIAL TONE I | 425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE II 27) | 350/440 | CONTINUOUS |
| RINGING TONE I | 425 | 1.0 – 4.0 |
| RINGING TONE II 27) | 440/480 | 0.2 – 0.2 |
| BUSY TONE I | 425 | 0.5 – 0.5 |
| BUSY TONE II 27) | 480/620 | 0.5 – 0.5 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

BURKINA FASO (continuation)

| CONGESTION TONE 27) | 480/620 | 2.4 – 2.4 |
| --- | --- | --- |
| ROUTE TONE I | 425 | 0.05 – 0.05 |
| ROUTE TONE II 27) | 425 | 0.04 – 0.04 |

BURUNDI

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.7 – 3.3 |
| BUSY TONE | 450 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 150/450 | 0.5 – 0.5 |
| ROUTE TONE | 425 | 0.5 – 0.5 |

CAMEROON

| DIAL TONE | 440 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 330//440 | CONTINUOUS |
| RINGING TONE | 440 | 1.7 – 3.3 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| ROUTE TONE | 440 | 0.05 – 0.05 |

CANADA

| DIAL TONE | 600×120//350+440 | CONTINUOUS |  |
| --- | --- | --- | --- |
| DIAL TONE (PABX) | 350+440 | CONTINUOUS |  |
| RECALL DIAL TONE (PABX) | 350+440 | 0.1 – 0.1 |  |
| RINGING TONE | 440+480 | 2.0 – 4.0 |  |
| RINGING TONE (PABX) | 440+480 | 1.0 – 3.0 |  |
| BUSY TONE | 480+620 | 0.5 – 0.5 |  |
| CONGESTION TONE | 480+620 | 0.25 – 0.25 |  |
| EXECUTIVE OVERRIDE TONE (PABX) | 440 | 3.0 |  |
| INTERCEPT TONE (PABX) | 440/620 | 0.25 – 0.25 |  |
| CONFIRMATION TONE (PABX) | 350+440 | 3×0.1 – 2×0.1 |  |

CAYMAN ISLANDS

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.25 – 0.25 |

CENTRAL AFRICAN REP.

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425+330 | CONTINUOUS |
| RINGING TONE | 425 | 1.7 – 3.3 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.5 – 0.5 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.5 – 0.5 |
| ROUTE TONE | 425 | 0.05 – 0.05 |
| PAY TONE | 12000 | 0.15 – 0.15 |

CHANNEL ISLANDS: JERSEY

| DIAL TONE | 350+450 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400+450 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| CONGESTION TONE | 400 | 0.4 – 0.35 – 0.225 – 0.525 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| WAITING TONE | 400 | 0.1 – 2.5 – 0.1 |
| PAY TONE | 400 | 0.125 – 0.125 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

CHILE

| DIAL TONE | 400 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 330+440 | CONTINUOUS |
| RINGING TONE | 400 | 1.0 – 3.0 |
| BUSY TONE | 400 | 0.5 – 0.5 |
| CONGESTION TONE | 400 | 0.2 – 0.2 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 1400 | 0.5 – 15.0 |
| WAITING TONE | 900+1300 | 0.5 – 0.5 |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |
| IDENTIFICATION TONE | 800/1200 | 0.6 – 2.0 – 0.6 – 2.0 |
| WARNING TONE – END OF PERIOD | 800 | 2.0 |

CHINA

| DIAL TONE | 450 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 450 | CONTINUOUS |
| RINGING TONE | 450 | 1.0 – 4.0 |
| BUSY TONE | 450 | 0.35 – 0.35 |
| CONGESTION TONE | 450 | 0.7 – 0.7 |
| INTRUSION TONE | 450 6) | 0.2 – 0.2 – 0.2 – 0.6 |
| SPECIAL INFORMATION TONE I | 450 | 0.4 – 0.04 |
| SPECIAL INFORMATION TONE II | 950 | 0.4 – 10.0 |
| NUMBER UNOBTAINABLE TONE | 450 | 3×(0.1 – 0.1) – 0.4 – 0.4 |
| WAITING TONE | 450 | 0.4 – 4.0 |

COMOROS

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 850 | CONTINUOUS |
| RINGING TONE | 425 | 1.71 – 3.31 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| ROUTE TONE | 425 | 0.05 – 0.05 |

COSTA RICA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.2 – 4.6 |
| BUSY TONE | 425 | 0.3 – 0.3 |

CÔTE D'IVOIRE

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 50//425×50 | 1.66 – 3.33 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| ROUTE TONE | 425 | 0.05 – 0.05 |

CROATIA

| DIAL TONE | 425 | 0.2 – 0.3 – 0.7 – 0.8 |
| --- | --- | --- |
| SPECIAL DIAL TONE | 425 | 0.4 – 0.04 |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING 32) | 425 | 0.2 – 0.3 – 0.7 – 0.8 |
| NUMBER UNOBTAINABLE TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WAITING TONE | 425 | 0.3 – 8.0 |
| PAYPHONE RECOGNITION TONE | 1206/850 | 0.2 – 0.2 – 0.2 – 2.0 (four cycles) |
| POSITIVE INDICATION TONE | 425 | CONTINUOUS |
| NEGATIVE INDICATION TONE | 425 | 0.25 – 0.25 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

CUBA

| DIAL TONE I | 600×120//425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE II | 425 | 0.75 – 0.75 – 0.25 – 0.25 |
| SECOND DIAL TONE I | 425+330 3) | CONTINUOUS |
| SECOND DIAL TONE II | 1050 6) | CONTINUOUS |
| RINGING TONE I | 440+480//440+620//480+620 | 2.0 – 4.0 |
| RINGING TONE II | 425 | 1.0 – 4.0 |
| RINGING TONE III | 420×40 | 1.0 – 2.0 |
| BUSY TONE | 600×120//620+480//425 | 0.5 – 0.5 |
| CONGESTION TONE | 600×120 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.33 – 2×0.03) – 1.0 |
| NUMBER UNOBTAINABLE TONE | 520 | 0.4 – 0.12 – 2×(0.12 – 0.12) |

CYPRUS

| DIAL TONE | 350+450 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE I | 425 | 1.5 – 3.0 |
| RINGING TONE II | 400×25 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| WARNING TONE – OPERATOR INTERVENING | 1400 | 0.5 – 14.0 |
| NUMBER UNOBTAINABLE TONE | 425 | 2.5 – 0.5 |
| WAITING TONE | 425 | 0.2 – 0.6 |
| PAYPHONE RECOGNITION TONE | 1638/1206/943 | 3×(0.068 – 0.068) |

CZECH Rep.

| DIAL TONE I | 425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE II | 425 | 0.33 – 0.33 – 0.66 – 0.66 |
| DIAL TONE III | 425 | 0.165 – 0.165 – 0.660 – 0.660 |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE I | 425 | 0.333 – 0.333 |
| BUSY TONE II | 425 | 0.167 – 0.167 |
| BUSY TONE III | 425 | 0.165 – 0.165 – 0.165 – 0.330 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.33 – 2×0.03) – 1.0 |
| WAITING TONE | 425 | 0.33 – 5.0 |
| CONNECTION TONE | 425 | 0.05 – 0.05 |
| OFFERING TONE | 425 6) | 0.33 – 0.33 – 0.33 – 1.5 |

DENMARK

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.25 – 0.25 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 150 | 0.05 – 0.5 |
| NUMBER UNOBTAINABLE TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| PAY TONE | 950/1400/1800 | 3×0.22 – 1.0 |
| PAYPHONE RECOGNITION TONE | 1477+941/1400+950 | 0.2 – 0.2 – 0.2 – 2.0 |

DIEGO GARCIA

| DIAL TONE | 350/440 | CONTINUOUS |
| --- | --- | --- |
| BUSY TONE | 480/620 | interrupted at 60 ipm |
| RINGING TONE | 440/480 | 1.0 – 3.0 |
| NUMBER UNOBTAINABLE TONE | 480/620 | interrupted at 120 ipm |

DJIBOUTI

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.5 – 3.5 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| ROUTE TONE | 425 | 0.05 – 0.05 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

DOMINICA (Commonwealth of)

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 350+440 | 3×(0.1 – 0.1) – CONTINUOUS |
| RINGING TONE | 440+480 | 2.0 – 4.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 450 | 0.2 – 0.2 – 0.65 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 480+620 | 0.25 – 0.25 |
| WAITING TONE | 440 | 1×(0.5 – 10.0 – 0.5) |
| PAYPHONE RECOGNITION TONE | 1600/1143 | 0.2 – 0.2 – 0.2 – 2.0 |

DOMINICAN Rep.

| DIAL TONE I | 50 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE II | 33 | 0.0166 – 0.0166 |
| RINGING TONE | 400//450 | 0.4 – 0.2 – 0.4 – 2.2 |
| BUSY TONE I | 400 | 0.8 – 0.8 |
| BUSY TONE II | 400 | 0.375 – 0.375 |
| CONGESTION TONE I | 400 | 0.8 – 0.8 |
| CONGESTION TONE II | 400 | 0.375 – 0.375 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |

ECUADOR

| DIAL TONE | 425 | 15.0 |
| --- | --- | --- |
| RINGING TONE | 425 | 1.2 – 4.65 |
| BUSY TONE | 425 | 0.33 – 0.33 |
| CONGESTION TONE | 425 | 0.33 – 0.33 |
| WAITING TONE | 425 | 0.2 – 0.6 |

EGYPT

| RINGING TONE | 425×50 | 2.0 – 1.0 |
| --- | --- | --- |
| BUSY TONE | 425×50 | 1.0 – 4.0 |
| CONGESTION TONE | 450 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 0.33 – 0.33 – 0.33 – 0.33 – 0.33 – 0.33 |
| INTRUSION TONE | 450 | 0.5 – 0.5 |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |

EL SALVADOR

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.2 – 5.0 |
| BUSY TONE | 425 | 0.333 – 0.333 |

ESTONIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.3 – 0.3 |
| CONGESTION TONE | 425 | 0.2 – 0.2 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.2 – 0.3 – 0.2 – 1.3 |
| WAITING TONE | 950/1400/1800 | 3×0.33 – 0.3 |

ETHIOPIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.4 – 0.4 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| INTRUSION TONE | 425 | 0.5 – 0.2 |
| WAITING TONE | 425 | 0.2 – 0.6 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

FALKLAND ISLANDS

| DIAL TONE | 350+450 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400+450 | 0.4 – 0.2 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| CONGESTION TONE | 400 | 0.4 – 0.35 – 0.255 – 0.525 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 1000 | 0.1 – 0.1 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |

FAROE ISLANDS

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.25 – 0.25 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 950/1400/1800 | 3×0.33 – 1.0 |

FIJI

| DIAL TONE | 33//50 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE (PABX) | 33 1/3 | CONTINUOUS |
| RINGING TONE | 133×16 2/3//400×25 | 0.4 – 0.2 – 0.4 – 2.0 |
| RINGING TONE (PABX) | 400×16 2/3 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.75 – 0.75 |
| BUSY TONE (PABX) | 400 | 0.375 – 0.375 |
| WARNING TONE – OPERATOR INTERVENING | 400 | 0.5 – 0.17 – 0.17 – 0.17 |
| NUMBER UNOBTAINABLE TONE | 400 | 2.5 – 0.5 |
| NUMBER UNOBTAINABLE TONE (PABX) | 400 | 2.5 – 0.5 |

FINLAND

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE (PABX) | 425 | 2×(0.2 – 0.3) – 0.2 – 0.8 |
| SPECIAL DIAL TONE | 425 | 0.65 – 0.025 |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.3 – 0.3 |
| CONGESTION TONE | 425 | 0.2 – 0.2 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.2 – 0.3 – 0.2 – 1.3 |
| QUEUE TONE | 950/950/1400 | 0.65 – 0.325 – 0.325 – 1.3 – 2.6 |
| WAITING TONE | 425 | 0.15 – 0.15 – 0.15 – 8.0 |

FRANCE

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

FRENCH POLYNESIA

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

GABON

| DIAL TONE I | 440 | 15.0 |
| --- | --- | --- |
| DIAL TONE II | 425 | CONTINUOUS |
| SECOND DIAL TONE | 440+330 | 7.0 |
| RINGING TONE I | 440 | 1.5 – 3.0 |
| RINGING TONE II | 425 | 1.5 – 3.5 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

GABON (continuation)

| BUSY TONE I | 440 | 0.5 – 0.5 |
| --- | --- | --- |
| BUSY TONE II | 425 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 900/1380/1860 | 0.35 – 0.30 – 0.35 – 1.0 |
| ROUTE TONE I | 440 | 0.05 – 0.05 |
| ROUTE TONE II | 425 | 0.05 – 0.05 |

GAMBIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |

GERMANY

| DIAL TONE | 425//450 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 425+400 | CONTINUOUS |
| RINGING TONE I 8) | 425//450 | 0.25 – 4.0 – 1.0 – 4.0 – 1.0 – 4.0 |
| RINGING TONE II 8) | 425//450 | 0.5 – 4.0 – 1.0 – 4.0 – 1.0 – 4.0 |
| RINGING TONE III | 425//450 19) | 1.0 – 5.0 |
| RINGING TONE IV | 450 19) | 1.0 – 9.0 |
| BUSY TONE I | 425 | 0.48 – 0.48 |
| BUSY TONE II | 425//450 | 0.15 – 0.475 |
| CONGESTION TONE I | 425 | 0.24 – 0.24 |
| CONGESTION TONE II | 425//450 | 0.15 – 0.475 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 425//450 | 0.24 – 0.24 – 0.24 – 1.28 |
| WAITING TONE 14) | 425 | 0.2 – 0.2 – 0.2 – 5.0 |
| PAYPHONE RECOGNITION TONE I | 1633/1336 | 0.2 – 0.2 – 0.2 – 2.0 (five cycles) |
| PAYPHONE RECOGNITION TONE II | 1366/1024 | 0.2 – 0.2 – 0.2 – 2.2 (five cycles) |
| PAYPHONE RECOGNITION TONE III | 1336/1024 | 2.2 – 0.2 – 0.2 – 0.2 (six cycles) |
| PAYPHONE RECOGNITION TONE IV | 1645+857/1215+935 | 0.2 – 0.2 – 0.2 – 2.0 (five cycles) |

GHANA

| DIAL TONE | 400 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400 | 1.0 – 4.0 |
| BUSY TONE | 400 | 0.25 – 0.25 |
| CONGESTION TONE | 400 | 0.5 – 0.5 |
| NUMBER UNOBTAINABLE TONE | 400 | 0.075 – 0.1 – 0.075 – 0.1 – 0.075 – 0.4 |
| WAITING TONE | 400 | 0.5 – 0.3 – 0.2 – 0.3 – 0.2 – 3.0 |
| HOLDING TONE | 400 | 0.5 – 0.5 – 0.5 – 0.25 |

GIBRALTAR

| DIAL TONE | 350+400 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400+450 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| CONGESTION TONE | 400 | 0.4 – 0.35 – 0.225 – 0.525 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| WAITING TONE | 400 | 0.1 – 3.0 |

GREECE

| DIAL TONE | 425//450 | 0.2 – 0.3 – 0.7 – 0.8 |
| --- | --- | --- |
| SPECIAL DIAL TONE (SPC) | 400/425//425/450 | 0.2 – 0.3 – 0.7 – 0.8 |
| RINGING TONE | 425//450 | 1.0 – 4.0 |
| BUSY TONE | 425//450 | 0.3 – 0.3 |
| CONGESTION TONE (SPC) | 425 | 0.15 – 0.15 |
| SPECIAL INFORMATION TONE (SPC) | 950/1450/1800 | 3×0.33 – 1.0 |
| WAITING TONE (SPC) | 425 | 0.3 – 10.0 – 0.3 – 10.0 |
| INTRUSION TONE | 425//450 | 0.15 – 0.25 – 0.15 – 1.45 |
| HOLDING TONE (SPC) | 900 | 0.5 – 0.5 |
| VALID TONE (RADIO-PAGING) | 450 | 0.1 – 0.1 – 0.1 – 0.7 |
| NUMBER UNOBTAINABLE TONE (RADIO-PAGING) | 450 | CONTINUOUS |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

GREENLAND

| DIAL TONE | 445//425 33) | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 445//425 33) | CONTINUOUS |
| RINGING TONE | 445//425 33) | 1.0 – 4.0 |
| BUSY TONE | 445//425 33) | 0.25 – 0.25 |
| CONGESTION TONE | 445//425 33) | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE I | 950/1400/1800 | 3×0.33 – 1.0 |
| NUMBER UNOBTAINABLE TONE II | 425 33) | 0.25 – 0.25 |
| PAY TONE | 950/1400/1800 | 3×0.22 – 1.0 |

GRENADA

| RINGING TONE | 440+480 | 2.0 – 4.0 |
| --- | --- | --- |
| BUSY TONE | 460+620 | 0.5 – 0.5 |
| CONGESTION TONE |  | recorded announcement |

GUADELOUPE (French Dep.)

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

GUIANA (French Dep.)

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

GUINEA

| RINGING TONE | 450 | 0.4 – 0.2 |
| --- | --- | --- |
| BUSY TONE | 450 | 0.2 – 0.2 |

GUYANA

| DIAL TONE | 360/400 | 1.3 – 0.3 – 0.2 – 0.5 |
| --- | --- | --- |
| RINGING TONE | 425 | 1.75 – 3.25 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.5 – 0.5 |
| WAITING TONE | 480 | 0.5 – 18.0 |

HONDURAS

| DIAL TONE | 440 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 440 | 0.135 – 0.135 |
| RINGING TONE | 440 | 0.5 – 4.0 – 2.0 – 4.0 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| CONGESTION TONE | 440 | 0.25 – 0.25 |
| WAITING TONE | 440 | 0.5 – 0.5 – 0.2 – 2.7 |

HONGKONG

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 350+440 | 3×(0.1 – 0.1)+CONTINUOUS |
| RINGING TONE | 440+480 | 0.4 – 0.2 – 0.4 – 3.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 480+620 | CONTINUOUS |
| WAITING TONE | 440 | 3×(0.5 – 0.5) – 8.0 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

HUNGARY

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425 | CONTINUOUS |
| SPECIAL DIAL TONE | 350+375+400 | CONTINUOUS |
| RINGING TONE | 425 | 1.2 – 3.7 |
| BUSY TONE | 425 | 0.3 – 0.3 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.3 – 0.3 – 0.3 – 1.5 |
| WAITING TONE | 425+450 | 0.3 – 0.3 |
| PAYPHONE RECOGNITION TONE | 1100/750 | 0.2 – 0.2 – 0.2 – 2.0 |
| POSITIVE INDICATION TONE | 300+420 | 1.0 – 0.2 + announcement |
| NEGATIVE INDICATION TONE | 300+420 | 0.2 – 0.2 + announcement |
| OFFERING TONE | 425 | 0.3 – 0.3 – 0.3 – 1.5 |

ICELAND

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE 18) | 425 | 0.4 – 0.04 |
| RINGING TONE | 425 | 1.2 – 4.7 |
| BUSY TONE | 425 | 0.25 – 0.25 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE |  | recorded announcement |
| WAITING TONE | 425 | 0.2 – 0.6 – 0.2 – 5.0 |
| PAYPHONE RECOGNITION TONE | 1633/1209 | 0.2 – 0.2 – 0.2 – 2.0 |

INDIA

| DIAL TONE | 33//50//400×25 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE I | 400×25 | 0.4 – 0.2 – 0.4 – 2.6 |
| RINGING TONE II | 133//400×25 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE I | 400 | 0.75 – 0.75 |
| BUSY TONE II | 400 | 0.6 – 0.6 |
| NUMBER UNOBTAINABLE TONE I | 400 | CONTINUOUS |
| NUMBER UNOBTAINABLE TONE II | 400 | 2.5 – 0.5 |

INDONESIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.33 – 2×0.03) – 1.0 |
| NUMBER UNOBTAINABLE TONE | 425 | 2.0 – 0.5 |
| PAYPHONE RECOGNITION TONE | 1200/800 | 0.2 – 0.2 – 0.2 – 2.0 |
| OFFERING TONE 6) | 425 | 0.5 – 0.5 – 0.5 – 1.0 |

IRAN

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 400/425 | CONTINUOUS |
| RECALL DIAL TONE | 425 | CONTINUOUS |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WAITING TONE | 425 | 0.2 – 0.2 – 0.2 – 10.0 |
| HOLDING TONE | 950/950/1400 | 0.65 – 0.325 – 0.325 – 1.3 – 2.6 |
| OFFERING TONE 6) | 425 | 0.15 – 0.25 – 0.15 – 1.45 |
| PREEMPTION TONE | 1400 | 0.1 – 0.1 (three bursts) |
| PERMANENT SIGNAL TONE | 1400 | CONTINUOUS |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

IRAQ

| RECALL DIAL TONE | 133 | 0.4 – 0.2 – 0.4 – 1.5 |
| --- | --- | --- |
| RINGING TONE | 400 | CONTINUOUS |
| BUSY TONE | 400 | 1.0 – 1.0 |

IRELAND

| DIAL TONE | 400/425/450 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400//450//425×25 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| NUMBER UNOBTAINABLE TONE | 400//425 | 6.0 – 1.0 |
| ROUTE TONE | 425 | 0.06 – 0.06 |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |

ISRAEL

| DIAL TONE | 400//425//440 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 400//440 | 3×(0.1 – 0.1)+CONTINUOUS |
| RECALL DIAL TONE | 400//440 | 3×(0.1 – 0.1)+CONTINUOUS |
| SPECIAL DIAL TONE | 400 | 1.0 – 0.25 |
| RINGING TONE | 400//425//440 | 1.0 – 3.0 |
| BUSY TONE | 400//425//440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 975/1400/1800 | 3×0.333 – 1.0 + announcement |
| NUMBER UNOBTAINABLE TONE | 975/1400/1800 | 3×0.333 – 1.0 + announcement |
| WAITING TONE | 400//440 | 0.15 – 10.0 – 0.15 (only once) |
| CALLER WAITING TONE | 400//440 | 2×(0.1 – 0.1) – 0.6 – 3.0 |
| CONFIRMATION TONE I | 400//440 | 0.15 – 0.15 – 0.3 (only once) |
| CONFIRMATION TONE II | 400 | 0.04 – 0.04 |
| PAYPHONE RECOGNITION TONE | 1209/452 8) | 0.2 – 0.2 – 0.2 – 2.0 |
| SEARCH TONE 28) | 1004 | 0.01 – 1.0 |
| HOLDING TONE 29) | 400 | 0.05 – 2.0 |

ITALY

| DIAL TONE | 425 | 0.6 – 1.0 – 0.2 – 0.2 |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.2 – 0.2 |

JAMAICA

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE I | 440×480 | 2.0 – 4.0 |
| RINGING TONE II | 440+480 | 1.5 – 4.5 |
| RINGING TONE III | 440+480 | 1.0 – 4.0 |
| RINGING TONE IV | 400//133 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE I | 400 | 0.75 – 0.75 |
| BUSY TONE II | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE I | 480+620 | 0.3 – 0.2 |
| CONGESTION TONE II | 480+620 | 0.2 – 0.3 |
| NUMBER UNOBTAINABLE TONE | 400 | 3.0 – 0.5 |
| RECORD TONE | 480+620 | 0.25 – 0.25 |

JAPAN

| DIAL TONE | 400 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE (PABX) | 400 | 0.25 – 0.25 |
| SECOND DIAL TONE I 17) | 400 | 0.125 – 0.125 |
| SECOND DIAL TONE II | 440 | 0.125 – 0.125 |
| SECOND DIAL TONE III | 400 | 0.15 – 0.15 |
| RECALL DIAL TONE | 350+440 | CONTINUOUS |
| RINGING TONE | 400×16 | 1.0 – 2.0 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

JAPAN (continuation)

| RINGING TONE (PABX) | 400×20 | 1.0 – 2.0 |
| --- | --- | --- |
| BUSY TONE 17) | 400 | 0.5 – 0.5 |
| CONGESTION TONE | 400 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 400 | 0.1 – 0.1 |
| WAITING TONE I | 440 | 0.1 – 0.1 – 0.1 – 1.0 |
| WAITING TONE II | 400×16/400 | 0.5 – 0.5 – 0.5 – 2.5 |
| CALL WAITING TONE | 400×16/400 | 0.5 – 0.0~4.0 – 0.05 – 0.45 – 0.05 – 3.45 – 0.05 – 0.45 – 0.05 – 3.45 |
| PAY TONE I | 250 | 1.0 |
| PAY TONE II | 250 | 0.5 |
| POSITIVE INDICATION TONE | 400 | 0.125 – 0.125 – 0.125 – 0.625 |

JORDAN

| DIAL TONE | 420×40//400+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE I | 420×40//440+400 | 1.2 – 4.8 |
| RINGING TONE II | 400+16 2/3 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE I | 400 | 0.75 – 0.75 |
| BUSY TONE II | 420×40//440+400 | 0.5 – 0.5 |
| CONGESTION TONE I | 420×40 | 0.5 – 0.5 |
| CONGESTION TONE II | 400+440 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 600×120//540+660 | 2.5 – 0.5 |
| WAITING TONE | 420×40//400+440 | 0.5 – 2×(0.3 – 0.2) – 3.0 |
| CONFIRMATION TONE | 400 | 20×(0.125 – 0.125) – CONTINUOUS |
| HOLDING TONE | 420×40//400+440 | 0.5 – 0.5 – 0.5 – 0.25 |

KENYA

| DIAL TONE | 425//50 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425 | 1.0 – 0.2 – 0.75 – 0.75 |
| RINGING TONE | 425 | 1.0 – 4.0 – 1.0 – 2.0 – 1.0 – 5.0 – 0.4 – 0.2 |
| BUSY TONE | 425 | 0.5 – 0.5 – 0.4 – 0.2 |
| CONGESTION TONE | 425 | 0.2 – 0.2 – 0.4 – 0.35 – 0.225 – 0.525 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.2 – 0.2 – 0.2 – 1.4 |
| NUMBER UNOBTAINABLE TONE I | 950/1400/1800 | 3×0.33 – 1.0 |
| NUMBER UNOBTAINABLE TONE II | 400 | CONTINUOUS |
| WAITING TONE | 425 | CONTINUOUS |

KIRIBATI

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.5 – 0.5 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.25 – 0.25 |
| WAITING TONE | 425 | 0.4 – 0.2 – 0.4 – 2.0 |

KOREA (Rep. of)

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE I | 400//450 | 1.0 – 4.0 |
| RECALL DIAL TONE II | 400//450 | 1.0 – 2.0 |
| RINGING TONE | 440+480 | 1.0 – 2.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 | 0.3 – 0.2 |
| WARNING TONE – OPERATOR INTERVENING | 392/494/587 | 2×(0.5 – 0.5 – 1.5) |
| NUMBER UNOBTAINABLE TONE | 450 | 0.2 – 0.1 – 0.2 – 1.5 |
| WAITING TONE | 350+440 | 0.25 – 0.25 – 0.25 – 3.25 |
| INTERCEPT TONE | 350+440 | 0.125 – 0.25 – 0.125 – 1.5 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

KUWAIT

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 425 16) | 0.4 – 0.04 |
| RINGING TONE I | 425 | 1.0 – 4.0 |
| RINGING TONE II | 400 | 1.0 – 3.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.5 – 0.5 |
| WARNING TONE – OPERATOR INTERVENING | 425 15) | 1.0 – 1.5 – 0.4 – 1.5 |
| INTRUSION TONE | 425 6) | 0.5 – 0.2 – 0.2 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| INTERCEPT TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| LINE LOCKOUT TONE | 425 | 0.5 – 0.5 |
| REFUSAL TONE | 425 | 0.6 – 0.6 |
| ACCEPTANCE TONE | 425 | CONTINUOUS |

KYRGYZSTAN

| RINGING TONE | 425 | 0.8 – 3.2 |
| --- | --- | --- |
| BUSY TONE | 425 | 0.4 – 0.4 |
| CONGESTION TONE | 425 | 0.4 – 0.4 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.4 – 0.4 |

LAO P.D.R.

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 425 | 0.4 – 0.04 |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 1.0 – 1.0 |
| CONGESTION TONE | 425 | 0.375 – 0.375 |
| NUMBER UNOBTAINABLE TONE | 425 | 2.5 – 0.5 |
| WAITING TONE | 425 | 0.4 – 0.4 |
| CONFIRMATION TONE | 950 13) | CONTINUOUS |

LEBANON

| RINGING TONE | 435 | 1.2 – 4.4 |
| --- | --- | --- |
| BUSY TONE | 435 | 0.4 – 0.2 |

LIBERIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425//450 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.6 – 0.2 – 3×(0.2 – 0.2) |

LITHUANIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 0.8 – 3.2 |
| BUSY TONE | 425 | 0.4 – 0.4 |
| WAITING TONE | 950/1400/1800 | 3×0.333 – 1.0 |

LUXEMBOURG

| DIAL TONE | 425//450 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425//450 | 0.5 – 0.5 |
| CONGESTION TONE | 425//450 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 425//450 | 0.25 – 0.25 – 0.25 – 1.25 |
| INTERCEPT TONE | 1190/1280 | 0.25 – 0.25 |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |

MACAU

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 425 | 1.8 – 0.2 |
| RINGING TONE | 425 | 1.0 – 4.0 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

MACAU (continuation)

| BUSY TONE | 425 | 0.5 – 0.5 |
| --- | --- | --- |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| WAITING TONE | 425 | 0.2 – 0.6 |
| INTERCEPT TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |

MADAGASCAR

| RINGING TONE | 25 | 1.66 – 3.33 |
| --- | --- | --- |
| BUSY TONE | 450 | 0.5 – 0.5 |
| WAITING TONE | 450 | 0.125 – 0.125 |

MALAWI

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE I | 425 | 1.0 – 4.0 |
| RINGING TONE II | 400//133 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE I | 400 | 0.75 – 0.75 |
| BUSY TONE II | 525 | 0.5 – 0.5 |
| BUSY TONE III | 400 | 0.4 – 0.333 |
| NUMBER UNOBTAINABLE TONE | 400 | 2.5 – 0.5 |

MALAYSIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425 | CONTINUOUS |
| SPECIAL DIAL TONE | 425 | 0.25 – 0.25 – 0.25 – 0.25 – 2.0 |
| RINGING TONE | 425 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 900/1400/1800 | 3×1.0 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.5 – 0.2 – 0.2 – 0.2 |
| WARNING TONE – END OF PERIOD | 900 | 0.1 – 0.1 – 0.1 – 0.1 – 0.1 (only once) |
| NUMBER UNOBTAINABLE TONE | 425 | 2.5 – 0.5 |
| WAITING TONE | 425 | 1.0 – 10.0 – 0.5 – 0.25 – 0.5 – 10.0 – 0.5 – 0.25 |
| LINE LOCKOUT TONE | 425 | 0.5 – 0.5 |

MALDIVES

| DIAL TONE | 400/50 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.5 – 0.5 |
| CONGESTION TONE | 400 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| WAITING TONE | 400 | 1.0 – 10.0 |

MALI

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425+330 | CONTINUOUS |
| SPECIAL DIAL TONE | 425 | 0.40 – 0.04 |
| RINGING TONE | 425 | 1.7 – 3.3 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 900/1400/1800 | 3×0.33 – 0.05 – 0.03) – 1.0 |
| ROUTE TONE | 425 | 0.05 – 0.04 |
| ACCEPTANCE TONE | 950 | CONTINUOUS |

MALTA

| RINGING TONE | 400+450//400×25//400×16 2/3 |  | 0.4 – 0.2 – 0.4 – 2.0 |
| --- | --- | --- | --- |
| BUSY TONE |  | 400 | 0.375 – 0.375 |
| CONGESTION TONE |  | 400 | 0.4 – 0.35 – 0.225 – 0.525 |
| NUMBER UNOBTAINABLE TONE |  | 400 | CONTINUOUS |
| PAY TONE |  | 400 | 0.15 – 0.15 |
| TONE |  | FREQUENCY (Hz) | CADENCE (Seconds) |

MARTINIQUE (French Dep.)

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

MAURITANIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.5 – 3.5 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| ROUTE TONE | 425 | 0.05 – 0.05 |

MAURITIUS

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.75 – 0.75 |
| ROUTE TONE | 425 | 0.05 – 0.05 |

MAYOTTE

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

MEXICO

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.25 – 0.25 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.5 – 0.17 – 0.17 – 0.17 |

MOLDOVA

| RINGING TONE | 425 | 0.8 – 3.2 |
| --- | --- | --- |
| BUSY TONE | 425 | 0.4 – 0.4 |
| CONGESTION TONE | 425 | 0.4 – 0.4 |

MONTSERRAT

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 350+440 | 3×(0.1 – 0.1) – CONTINUOUS |
| RINGING TONE | 440+480 | 2.0 – 4.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 450 | 0.2 – 0.2 – 0.65 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 480+620 | 0.25 – 0.25 |
| WAITING TONE | 440 | 1×(0.5 – 10.0 – 0.5) |
| PAYPHONE RECOGNITION TONE | 1600/1143 | 0.2 – 0.2 – 0.2 – 2.0 |

MOROCCO

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 660/440/590/740 | 0.5 – 0.5 – 0.5 – 0.3 – 0.2 |
| RINGING TONE | 425 | 1.66 – 3.33 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| ROUTE TONE | 425 | 0.05 – 0.05 |

MOZAMBIQUE

| RINGING TONE | 400 | 1.0 – 5.0 |
| --- | --- | --- |
| BUSY TONE | 400 | 0.5 – 0.5 |
| NUMBER UNOBTAINABLE TONE | 400 | 0.2 – 0.2 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

NAMIBIA

| DIAL TONE | 440×33 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400×33 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE I | 400 | 0.75 – 0.75 |
| BUSY TONE II | 400 | 0.5 – 0.5 |
| BUSY TONE III | 400 | 0.25 – 0.25 |
| CONGESTION TONE | 400 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 400 | 2.5 – 0.5 |
| PAYPHONE RECOGNITION TONE | 770/1209 | 0.125 – 1.5 |
| PAY TONE | 400 | 0.125 – 0.125 |

NAURU

| DIAL TONE | 400 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE (PABX) | 33 | CONTINUOUS |
| RECALL DIAL TONE (PABX) | 33 | CONTINUOUS |
| RINGING TONE | 400 | 1.0 – 4.0 |
| RINGING TONE (PABX) | 425×25 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.5 – 0.5 |
| BUSY TONE (PABX) | 425 | 0.375 – 0.375 |
| CONGESTION TONE (PABX) | 425 | 0.375 – 0.375 |
| NUMBER UNOBTAINABLE TONE | 400 | 0.75 – 0.25 – 0.25 – 0.25 |

NEPAL

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.1 – 3.1 |
| BUSY TONE | 425 | 0.25 – 0.25 |
| CONGESTION TONE | 425 | 0.175 – 0.175 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.25 – 0.5 |

NETHERLANDS

| DIAL TONE | 425//450 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 425//450 | 0.5 – 0.05 |
| RINGING TONE | 425//450 | 1.0 – 4.0 |
| BUSY TONE I 9) | 425//450 | 0.5 – 0.5 |
| BUSY TONE II 12) | 425//450 | 0.25 – 0.25 |
| CONGESTION TONE | 425//450 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WAITING TONE | 425//450 | 0.5 – 9.5 |
| PAY TONE | 770//850//950 11) |  |
| POSITIVE INDICATION TONE | 425//450 | CONTINUOUS |
| NEGATIVE INDICATION TONE | 425//450 | 0.0625 – 0.0625 |

NEW CALEDONIA

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

NEW ZEALAND

| DIAL TONE 17) | 400 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400+450 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 17) | 0.5 – 0.5 |
| CONGESTION TONE I | 900 4) | 0.5 – 0.5 |
| CONGESTION TONE II | 400 4) 17) | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 400 17) | 3×(0.75 – 0.1) – 0.75 – 0.4 |
| WAITING TONE I | 400+450 | 0.5 |
| WAITING TONE II | 400 | 0.25 – 0.25 – 0.25 – 3.25 |
| WAITING TONE III | 523/659 | 3×(0.2 – 3.0) – 0.2 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

NEW ZEALAND (continuation)

| FUNCTION ACKNOWLEDGE TONE | 440+450 5) | 2.0 – 0.4 – 0.2 – 0.4 |
| --- | --- | --- |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |
| HOLDING TONE | 400/400+450 | 0.5 – 0.5 – 0.5 – 0.25 |

NIGER

| DIAL TONE | 440 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 330//440 | CONTINUOUS |
| RINGING TONE | 440 | 1.7 – 3.3 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| CONGESTION TONE | 440 | 0.5 – 0.5 |
| ROUTE TONE | 440 | 0.05 – 0.05 |

NIGERIA

| DIAL TONE | 400//425//450 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425 | CONTINUOUS |
| RECALL DIAL TONE | 400//425 | CONTINUOUS |
| RINGING TONE | 400//425//450 | 2.0 – 4.0 |
| BUSY TONE | 400 | 0.5 – 0.5 |
| CONGESTION TONE | 400 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 400//33 | 0.2 – 1.5 |
| WAITING TONE – OPERATOR INTERVENING | 400 | 0.15 – 0.25 – 0.15 – 1.45 |
| NUMBER UNOBTAINABLE TONE | 450 | 5.5 – 0.5 |
| WAITING TONE | 425 | 2.0 – 4.0 |
| HOLDING TONE | 950/950/1400 | 0.65 – 0.325 – 0.325 – 1.3 – 2.6 |

NORWAY

| RINGING TONE I | 425 | 1.0 – 4.0 |
| --- | --- | --- |
| RINGING TONE II | 400//450 | 1.0 – 3.0 |
| BUSY TONE I | 425 | 0.5 – 0.5 |
| BUSY TONE II | 400//450 | 0.2 – 0.4 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| PAYPHONE RECOGNITION TONE | 1477/941 | 0.2 – 0.2 – 0.2 – 2.0 (four cycles) |

OMAN

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425 | CONTINUOUS |
| RINGING TONE | 425 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.5 – 0.5 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.5 – 0.17 – 0.17 – 0.17 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.2 – 0.3 |
| WAITING TONE | 425 | 0.3 – 1.0 |
| LINE LOCKOUT TONE | 425 | 0.48 – 0.48 |

PAKISTAN

| RINGING TONE I | 450 | 1.0 – 4.0 |
| --- | --- | --- |
| RINGING TONE II | 400+16 2/3 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE I | 400 | 0.75 – 0.75 |
| BUSY TONE II | 450 | 0.4 – 0.675 – 0.13 – 0.17 |

PANAMA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.20 – 4.65 |
| BUSY TONE | 425 | 0.32 – 4.65 |
| CONGESTION TONE | 425 | 0.32 – 4.65 |
| SPECIAL INFORMATION TONE | 425 | 0.4 – 0.04 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.18 – 0.18 – 0.5 – 0.18 |
| NUMBER UNOBTAINABLE TONE 30) |  |  |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

PAPUA NEW GUINEA

| DIAL TONE | 425//400×25//350+440 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 425 | 0.4 – 0.04 |
| RINGING TONE | 425//400×25//400×16 2/3// 400+450//410×20 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 425//400 | 0.375 – 0.375 |
| CONGESTION TONE | 425//400 20) | 0.375 – 0.375 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.01 |
| WARNING TONE – OPERATOR INTERVENING | 1400 | 0.5 – 11.5 |
| NUMBER UNOBTAINABLE TONE | 425//400 | 2.5 – 0.5 |
| WAITING TONE | 425 | 0.04 – 10.0 – 0.04 – 20.0 – 0.04 – 20.0 |
| PAYPHONE RECOGNITION TONE I | 1209/852 | 0.2 – 0.2 – 0.2 – 2.0 (four cycles) |
| PAYPHONE RECOGNITION TONE II | 1336/941 | 0.2 – 0.2 – 0.2 – 2.0 (two cycles) |
| PAYPHONE RECOGNITION TONE III | 900 | 0.075 – 0.15 – 0.075 – 2.7 (two cycles) |

PARAGUAY

| RINGING TONE | 425 | 1.0 – 4.0 |
| --- | --- | --- |
| BUSY TONE | 425 | 1.0 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.15 – 0.25 – 0.15 – 1.45 |
| NUMBER UNOBTAINABLE TONE | 425 | 2.0 – 0.5 |
| WAITING TONE | 950/950/1400 | 0.65 – 0.325 – 0.125 – 1.3 – 2.6 |
| POSITIVE INDICATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |

PHILIPPINES

| DIAL TONE I | 600×120//425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE II | 400 | 0.2 – 0.2 |
| SPECIAL DIAL TONE | 400+425 | CONTINUOUS |
| RINGING TONE I | 450 | 1.0 – 10.0 |
| RINGING TONE II | 425+480 | 1.0 – 4.0 |
| BUSY TONE I | 600×120//425 | 0.5 – 0.5 |
| BUSY TONE II | 450 | 0.4 – 0.2 |
| CONGESTION TONE | 600×120//480+620 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 600×120 | 0.25 – 0.25 |

POLAND

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425+350 | CONTINUOUS |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WAITING TONE | 425 | 0.15 – 0.15 – 0.15 – 4.0 |
| ROUTE TONE | 425 | 0.05 – 0.05 |

PORTUGAL 7)

| DIAL TONE | 400//425 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 425 | 1.0 – 0.2 |
| RINGING TONE | 400//425 | 1.0 – 5.0 |
| BUSY TONE | 400//425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.2 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 400//425 27) | 0.2 – 0.2 |
| WAITING TONE | 425 | 0.2 – 0.2 – 0.2 – 5.0 |
| PAYPHONE RECOGNITION TONE | 1477/941 | 0.2 – 0.2 – 0.2 – 2.0 |

PUERTO RICO

| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |
| --- | --- | --- |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

QATAR

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE I | 400+450 | 0.4 – 0.2 – 0.4 – 2.2 |
| RINGING TONE II | 400+450 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE I | 400//450 | 0.8 – 0.8 |
| BUSY TONE II | 400 | 0.375 – 0.375 |
| CONGESTION TONE | 400 | 0.4 – 0.35 – 0.225 – 0.525 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |

REUNION (French Dep.)

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

ROMANIA

| DIAL TONE | 400//450 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400×16//450×25 | 2.0 – 4.0 |
| BUSY TONE | 133//450 | 0.15 – 0.20 |
| NUMBER UNOBTAINABLE TONE | 400//450 | 6×(0.033 – 0.03) – 0.1 – 0.03 |

RUSSIA

| RINGING TONE | 425 | 0.8 – 3.2 |
| --- | --- | --- |
| BUSY TONE | 425 | 0.4 – 0.4 |
| WAITING TONE | 950/1400/1800 | 3×0.333 – 1.0 |

RWANDA

| DIAL TONE | 440 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 340 3) | CONTINUOUS |
| RINGING TONE | 440 | 1.7 – 3.3 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| ROUTE TONE | 440 | 0.05 – 0.05 |

S. HELENA

| DIAL TONE | 350+400 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400+450 | 0.4 – 0.2 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| CONGESTION TONE | 400 | 0.4 – 0.35 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| PAY TONE | 400 | 0.125 – 0.125 |

S.-KITTS-AND-NEVIS

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 350+440 | 3×(0.1 – 0.1) – CONTINUOUS |
| RINGING TONE | 440+480 | 2.0 – 4.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 450 | 0.2 – 0.2 – 0.65 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 480+620 | 0.25 – 0.25 |
| WAITING TONE | 440 | 1×(0.5 – 10.0 – 0.5) |
| PAYPHONE RECOGNITION TONE | 1600/1143 | 0.2 – 0.2 – 0.2 – 2.0 |

S. LUCIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425 | CONTINUOUS |
| RINGING TONE | 425 | 0.375 – 0.25 – 0.375 – 2.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

S. LUCIA (continuation)

| CONGESTION TONE | 425 | 0.25 – 0.25 |
| --- | --- | --- |
| SPECIAL INFORMATION TONE | 425+330 | CONTINUOUS |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.1 – 4.9 |
| NUMBER UNOBTAINABLE TONE |  | recorded announcement |
| WAITING TONE | 425 | 0.2 – 0.2 |
| ROUTE TONE | 425 | 0.05 – 0.05 |

S. PIERRE AND MIQUELON

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

S. VINCENT AND THE GRENADINES

| DIAL TONE | 350+450 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400+450 | 0.2 – 0.4 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| CONGESTION TONE | 400 | 0.4 – 0.35 – 0.255 – 0.525 |

SAN MARINO

| DIAL TONE | 425 | 0.6 – 1.0 – 0.2 – 0.2 |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.2 – 0.2 |

SAO TOME AND PRINCIPE

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 5.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.2 – 0.2 |
| PAY TONE | 12000 | CONTINUOUS |

SAUDI ARABIA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 425 | CONTINUOUS |
| RINGING TONE | 425 | 1.2 – 4.6 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| INTRUSION TONE | 425 6) | 0.5 – 0.2 – 0.2 – 0.2 |
| WAITING TONE | 425 | 0.2 – 0.6 – 0.2 – 4.0 |
| PAYPHONE RECOGNITION TONE | 1600/1200 | 0.2 – 0.2 – 0.2 – 2.0 |
| REFUSAL TONE | 425 | 0.6 – 0.6 |
| ACCEPTANCE TONE | 425 | CONTINUOUS |

SENEGAL

| DIAL TONE | 440 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 440+330 | CONTINUOUS |
| RINGING TONE | 400 | 1.7 – 3.3 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| ROUTE TONE | 440 | 0.05 – 0.05 |

SEYCHELLES

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 425 | 0.375 – 0.375 |
| CONGESTION TONE | 425 | 0.4 – 0.35 – 0.225 – 0.525 |
| NUMBER UNOBTAINABLE TONE | 425 | CONTINUOUS |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

SIERRA LEONE

| DIAL TONE | 425//450 33) | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425//450 33) | 1.0 – 4.0 |
| BUSY TONE | 425//450 33) | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.2 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 950//450/150 33) | 0.33 – 0.03 |
| WAITING TONE | 425 | 1.0 (one burst) |

SINGAPORE

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425×24 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 425 | 0.75 – 0.75 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.125 – 0.125 |
| NUMBER UNOBTAINABLE TONE | 425 | 2.5 – 0.5 |
| WAITING TONE | 425 | 0.3 – 0.2 – 0.3 – 3.2 |
| INTERCEPT TONE | 425 | 0.25 – 2.0 |
| PAYPHONE RECOGNITION TONE | 1206/832 | 0.2 – 0.2 – 0.2 – 2.0 |

SLOVAKIA

| DIAL TONE I | 425 | 0.33 – 0.33 – 0.66 – 0.66 |
| --- | --- | --- |
| DIAL TONE II (centrex) | 425 | CONTINUOUS |
| SPECIAL DIAL TONE | 425 | 3×(0.165 – 0.165) – 0.660 – 0.660 |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.33 – 0.33 |
| CONGESTION TONE | 425 | 0.165 – 0.165 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.33 – 2×0.03) – 1.0 |
| WAITING TONE | 425 | 0.33 – 9.0 |
| CALLER WAITING TONE | 425 | 1.0 – 0.17 – 0.33 – 3.5 |
| CONNECTION TONE | 1400 | 0.05 – 0.05 |
| OFFERING TONE 6) | 425 | 0.33 – 0.33 – 0.33 – 1.5 |
| CONFERENCE TONE | 425 | 0.66 |

SLOVENIA

| DIAL TONE | 425 | 0.7 – 0.8 – 0.2 – 0.3 |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.2 – 0.2 |
| SPECIAL INFORMATION TONE | 950/1400/1600 | 3×0.333 – 1.0 |

SOLOMON

| DIAL TONE | 400 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400+450 | 0.4 – 0.2 |
| BUSY TONE | 400 | 0.5 – 0.5 |
| CONGESTION TONE | 400 | 0.1 – 0.1 |
| NUMBER UNOBTAINABLE TONE | 400 | 0.075 – 0.1 |
| WAITING TONE | 400+450/400 | 0.5 – 0.5 |
| PAY TONE | 852 | 0.2 – 2.0 |

SOUTH AFRICA

| DIAL TONE | 400×33 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 400×33 1/3 | 4×(0.25 – 0.25) – CONTINUOUS |
| RINGING TONE | 400×33 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.5 – 0.5 |
| CONGESTION TONE | 400 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 400 | 0.15 – 0.25 – 0.15 – 1.45 |
| NUMBER UNOBTAINABLE TONE | 400 | 2.5 – 0.5 |
| WAITING TONE | 400×33 1/3 | 3×(0.25 – 0.25) – 0.25 – 7.25 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

SOUTH AFRICA (continuation)

| PAY TONE | 900 | 0.2 – 1.8 |
| --- | --- | --- |
| PAYPHONE RECOGNITION TONE | 770/1209 | 0.125 – 1.5 |
| POSITIVE INDICATION TONE | 700/1100 | 0.25 – 0.25 – 0.25 – 0.25 |
| COMFORT TONE | 950/950/1400 | 0.65 – 0.325 – 0.325 – 1.3 – 2.6 |
| NOTIFY TONE | 900 | 3×0.2 – 2×0.2 |

SPAIN

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE 3) | 600 | CONTINUOUS |
| SPECIAL DIAL TONE | 425 | 1.0 – 0.1 |
| RINGING TONE | 425 | 1.5 – 3.0 |
| BUSY TONE | 425 | 0.2 – 0.2 |
| CONGESTION TONE | 425 | 2×(0.2 – 0.2) – 0.2 – 0.6 |
| SPECIAL INFORMATION TONE I | 950/1400/1800 | 3×0.333 – 1.0 |
| SPECIAL INFORMATION TONE II | 950/1400/1800 | (3×0.33 – 2×0.03) – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 1400 | 0.4 – 5.0 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.2 – 0.2 – 0.2 – 0.6 |
| WAITING TONE I | 425 | 0.2 – 0.6 |
| WAITING TONE II | 425 | 0.175 – 0.175 – 0.175 – 3.5 |
| PAYPHONE RECOGNITION TONE I | 1600 | 0.05 – 1.28 |
| PAYPHONE RECOGNITION TONE II | 1600 | 0.05 – 0.05 – 0.05 – 1.55 |
| PAYPHONE RECOGNITION TONE III | 1600 | 0.05 – 0.05 – 0.05 – 0.05 – 0.05 – 1.5 |

SRI LANKA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425 | 0.65 – 0.03 |
| RINGING TONE | 425 | 0.4 – 0.2 |
| BUSY TONE | 425 | 0.75 – 0.75 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| WAITING TONE | 425 | 0.2 – 0.2 – 0.2 – 4.5 |
| ROUTE TONE | 425 | 0.05 – 0.05 |

SURINAME

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 15.0 – 0.5 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |

SWAZILAND

| DIAL TONE | 50 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400 | 0.4 – 0.2 – 0.4 – 4.0 |
| BUSY TONE I | 400 | 0.5 – 0.5 |
| BUSY TONE II | 400 | 0.4 – 0.4 |
| NUMBER UNOBTAINABLE TONE | 400 | 4.8 – 0.2 |
| PAYPHONE RECOGNITION TONE I | 1430/943 | 2.0 – 2.0 |
| PAYPHONE RECOGNITION TONE II | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |
| PAY TONE | 900 | 0.25 – 0.25 |

SWEDEN

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425 | CONTINUOUS |
| SPECIAL DIAL TONE | 425 | 0.32 – 0.01 |
| RINGING TONE I | 425 | 1.0 – 5.0 |
| RINGING TONE II 27) | 400 | 1.0 – 9.0 |
| BUSY TONE | 425 | 0.25 – 0.25 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

SWEDEN (continuation)

| CONGESTION TONE | 425 | 0.25 – 0.75 |
| --- | --- | --- |
| SPECIAL INFORMATION TONE I | 950/1400/1800 | 3×0.333 – 1.0 + announcement |
| SPECIAL INFORMATION TONE II 27) | 400 | 3×(0.05 – 0.2) – 0.6 |
| WARNING TONE – OPERATOR INTERVENING | 1400 | 0.1 – 1.5 |
| WARNING TONE I | 1400 15) | 0.35 – 15.0 |
| WARNING TONE II | 425 21) | 2.0 – 5.0 – CONTINUOUS |
| NUMBER UNOBTAINABLE TONE | 950/1400/1800 | 3×0.333 – 1.0 + announcement |
| WAITING TONE | 425 | 0.2 – 0.5 – 0.2 (only once) |
| PAYPHONE RECOGNITION TONE | 1633/1336 | 0.2 – 0.2 – 0.2 – 4.0 (seven cycles) |
| PAY TONE 2) | 941 | 0.2 – 0.2 – 0.2 – 3.6 (two cycles) |

SWITZERLAND

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE (PABX) | 500 | CONTINUOUS |
| RINGING TONE | 425//500 | 1.0 – 4.0 |
| BUSY TONE I | 425 | 0.5 – 0.5 |
| BUSY TONE II | 425 | 0.25 – 0.25 |
| BUSY TONE (PABX) | 500 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.2 – 0.2 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| RECORD TONE | 1400 | 0.45 – 15.0 |
| PAYPHONE RECOGNITION TONE I | 800/1200 | 0.2 – 0.2 – 0.2 – 2.0 |
| PAYPHONE RECOGNITION TONE II | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |

SYRIA

| DIAL TONE | 450 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 450 | CONTINUOUS |
| RINGING TONE I | 425//475 | 1.0 – 4.0 |
| RINGING TONE II | 450//50 | 1.0 – 3.0 |
| BUSY TONE I | 450 | 0.5 – 0.5 |
| BUSY TONE II | 450 | 0.44 – 0.49 |
| CONGESTION TONE | 450 | 2×(0.2 – 0.23) – 0.2 – 0.92 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.33 – 2×0.03) – 1.0 |
| NUMBER UNOBTAINABLE TONE | 450 | 0.9 – 0.2 – 2×(0.25 – 0.2) |

TAJIKISTAN

| RINGING TONE | 425 | 0.8 – 3.2 |
| --- | --- | --- |
| BUSY TONE | 425 | 0.4 – 0.4 |
| WAITING TONE | 950/1400/1800 | 3×0.333 – 1.0 |

TANZANIA

| DIAL TONE | 50 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE I | 400 | CONTINUOUS |
| RINGING TONE II | 400 | 1.0 – 2.0 |
| BUSY TONE I | 400 | 0.75 – 0.75 |
| BUSY TONE II | 400 | 0.375 – 0.375 |
| NUMBER UNOBTAINABLE TONE | 400 | 2.5 – 0.5 |

THAILAND

| DIAL TONE | 400×50//400 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400 | 1.0 – 4.0 |
| BUSY TONE I | 400 | 0.5 – 0.5 |
| BUSY TONE II | 450 | 0.333 – 0.333 |
| WARNING TONE – OPERATOR INTERVENING | 800 | 1.0 |
| NUMBER UNOBTAINABLE TONE | 400 | 6×(0.1 – 0.9) – 0.3 – 0.7 |
| OFFERING TONE 6) | 400 | 2×(0.17 – 0.83) – 0.51 – 0.49 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

TRINIDAD AND TOBAGO

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440+480 | 2.0 – 4.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| WAITING TONE | 440 | 0.3 – 10.0 |
| RE-ORDER TONE I | 480+620 | 0.3 – 0.2 |
| RE-ORDER TONE II 6) | 480+620 | 0.2 – 0.3 |

TUNISIA

| DIAL TONE I | 425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE II | 425 | 0.4 – 0.04 |
| RINGING TONE | 425 | 1.2 – 4.6 |
| BUSY TONE I | 425 | 0.25 – 0.25 |
| BUSY TONE II (special) | 950 | CONTINUOUS |
| CONGESTION TONE | 425 | 0.167 – 0.167 |

TURKEY

| DIAL TONE | 450 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 350+450 | CONTINUOUS |
| SPECIAL DIAL TONE | 450 | 1.0 – 0.25 |
| RINGING TONE | 450 | 2.0 – 4.0 |
| BUSY TONE | 450 | 0.5 – 0.5 |
| CONGESTION TONE | 450 | 3×(0.2 – 0.2) – 0.6 – 0.2 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.3 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 450 | 0.2 – 0.2 – 0.6 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 450 | 0.2 – 0.2 |
| WAITING TONE | 350+450/450 | 0.5 – 0.5 – 2.5 |
| PAYPHONE RECOGNITION TONE | 1000/1200//1000/1330 | 0.25 – 0.25 – 0.25 – 2.0 |
| CONFIRMATION TONE | 450 | 0.04 – 0.04 |

TURKMENISTAN

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 425 | CONTINUOUS |
| RINGING TONE | 25 | 0.4 – 4.0 |
| BUSY TONE | 425 | 0.4 – 0.4 |
| CONGESTION TONE | 425 | 0.4 – 0.4 |
| NUMBER UNOBTAINABLE TONE | 425 | 0.8 – 4.0 |
| ROUTE TONE | 25 | 0.4 – 0.4 |

TURKS AND CAICOS ISLANDS

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 350+440 | 3×(0.1 – 0.1) – CONTINUOUS |
| RINGING TONE | 440+480 | 2.0 – 4.0 |
| BUSY TONE | 480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 480+620 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 450 | 0.2 – 0.2 – 0.65 – 0.2 |
| NUMBER UNOBTAINABLE TONE | 480+620 | 0.25 – 0.25 |
| WAITING TONE | 440 | 1×(0.5 – 10.0 – 0.5) |
| PAYPHONE RECOGNITION TONE | 1600/1143 | 0.2 – 0.2 – 0.2 – 2.0 |

UGANDA

| DIAL TONE | 33//50 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 133+17//400+17//400+450 | 0.25 – 0.5 – 0.25 – 2.0 |
| BUSY TONE I | 400 | 0.75 – 0.75 |
| BUSY TONE II | 400 | 0.375 – 0.375 |
| NUMBER UNOBTAINABLE TONE I | 400 | CONTINUOUS |
| NUMBER UNOBTAINABLE TONE II | 400 | 2.5 – 0.5 |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

UNITED ARAB EMIRATES

| DIAL TONE | 350+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 400+450//425 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400//425 | 0.375 – 0.375 |
| CONGESTION TONE | 400//425 | 0.4 – 0.35 – 0.225 – 0.525 |
| NUMBER UNOBTAINABLE TONE | 400//425 | CONTINUOUS |

UNITED KINGDOM

| DIAL TONE |  | 50//350+440 | CONTINUOUS |
| --- | --- | --- | --- |
| RINGING TONE | 400+450//400×25//400×16 2/3 |  | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE |  | 400 | 0.375 – 0.375 |
| CONGESTION TONE |  | 400 | 0.4 – 0.35 – 0.225 – 0.525 |
| NUMBER UNOBTAINABLE TONE |  | 400 | CONTINUOUS |
| PAYPHONE RECOGNITION TONE |  | 1200/800 | 0.2 – 0.2 – 0.2 – 2.0 |
| PAY TONE |  | 400 | 0.125 – 0.125 |

UNITED STATES

| DIAL TONE | 600×120 23)//350+440 | CONTINUOUS |
| --- | --- | --- |
| RECALL DIAL TONE | 350+440 | 3×(0.1 – 0.1) – CONTINUOUS |
| RINGING TONE | 420×40 23)//440+480 | 2.0 – 4.0 |
| RINGING TONE (PABX) | 440+480 | 1.0 – 3.0 |
| BUSY TONE | 600×120 23)//480+620 | 0.5 – 0.5 |
| CONGESTION TONE | 600×120 23)//480+620 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.33 |
| WARNING TONE – OPERATOR INTERVENING | 440 24) | 2.0 – 10.0 – 0.5 – 10.0 |
| WARNING TONE – OPERATOR INTERVENING (PABX) | 440 24) 25) | 1.5 – 8.0 – 0.5 – 8.0 |
| WAITING TONE (PABX) | 440 26) | 0.3 – 10.0 |
| RECORD TONE | 1400 | 0.5 – 15.0 |
| EXECUTIVE OVERRIDE TONE (PABX) | 440 | 3.0 |
| INTERCEPT TONE (PABX) | 440/620 | 0.25 – 0.25 |
| CONFIRMATION TONE (PABX) | 350+440 | 3×0.1 – 2×0.1 |

URUGUAY

| DIAL TONE I | 425 | CONTINUOUS |
| --- | --- | --- |
| DIAL TONE II | 450 | 0.7 – 0.8 – 0.2 – 0.3 |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| WARNING TONE – OPERATOR INTERVENING | 425 | 0.15 – 0.25 – 0.15 – 1.45 |
| WAITING TONE | 425 | 0.2 – 0.2 – 0.2 – 4.4 |
| POSITIVE INDICATION TONE | 425 | 2×(0.1 – 0.2) – 0.1 – 1.3 |
| NEGATIVE INDICATION TONE | 425 | 0.5 – 0.5 |

VANUATU

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| SPECIAL DIAL TONE | 425 | 0.4 – 0.04 |
| RINGING TONE I | 425 | 1.0 – 4.0 |
| RINGING TONE II | 400×450//133×16 2/3 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE I | 400 | 0.75 – 0.75 |
| BUSY TONE II | 425 | 0.5 – 0.5 |
| BUSY TONE III | 400 | 0.375 – 0.375 |
| SPECIAL INFORMATION TONE | 950//1400//1800 | 0.33 – 0.03 |
| NUMBER UNOBTAINABLE TONE | 400 | CONTINUOUS |
| WAITING TONE | 425 | 0.3 – 10.0 |
| ROUTE TONE | 425 | 0.05 – 0.05 |
| ACCEPTANCE TONE | 950 | CONTINUOUS |
| TONE | FREQUENCY (Hz) | CADENCE (Seconds) |

WALLIS AND FUTUNA

| DIAL TONE | 440//330+440 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 440 | 1.5 – 3.5 |
| BUSY TONE | 440 | 0.5 – 0.5 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | (3×0.3 – 2×0.03) – 1.0 |
| ROUTE TONE 22) | 440 | 0.05 – 0.05 |

WESTERN SAMOA

| DIAL TONE | 425 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425×25 | 0.375 – 0.25 – 0.375 – 2.0 |
| CONGESTION TONE |  | recorded announcement |
| NUMBER UNOBTAINABLE TONE |  | recorded announcement |

YUGOSLAVIA

| DIAL TONE | 425 | 0.7 – 0.8 – 0.2 – 0.3 |
| --- | --- | --- |
| RINGING TONE I | 450×25 | 1.0 – 9.0 |
| RINGING TONE II | 425 | 1.0 – 4.0 |
| BUSY TONE I | 425 | 0.5 – 0.5 |
| BUSY TONE II | 425 | 0.2 – 0.4 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |

ZAMBIA

| DIAL TONE | 425//50 | CONTINUOUS |
| --- | --- | --- |
| RINGING TONE | 425 | 1.0 – 4.0 |
| BUSY TONE | 425 | 0.5 – 0.5 |
| CONGESTION TONE | 425 | 0.25 – 0.25 |
| SPECIAL INFORMATION TONE | 950/1400/1800 | 3×0.333 – 1.0 |
| INTRUSION TONE | 425 | 0.1 – 4.9 |
| NUMBER UNOBTAINABLE TONE | 950/1400/1800 | 3×0.33 – 1.0 |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |

ZIMBABWE

| DIAL TONE | 400 | CONTINUOUS |
| --- | --- | --- |
| SECOND DIAL TONE | 400×450//400×33 | CONTINUOUS |
| RINGING TONE | 400×450//400×33 | 0.4 – 0.2 – 0.4 – 2.0 |
| BUSY TONE | 400 | 0.375 – 0.375 |
| NUMBER UNOBTAINABLE TONE | 400 | 2.5 – 0.5 |
| WAITING TONE | 523/659 | 1.5 – 1.5 |
| PAYPHONE RECOGNITION TONE | 1100+1750/750+1450 | 0.2 – 0.2 – 0.2 – 2.0 |

FOOTNOTES  TO  TONE  TABLES

1)	With harmonics applied every 30 ms.

2)	Announces that the call is to terminate in a specified time unless the user inserts another coin in the coin box.

3)	International.

4)	Disconnect tone/Overflow tone.

5)	Switching complete tone.

6)	Trunk.

7)	Analogue exchanges: 400 Hz; digital exchanges: 425 Hz.

8)	The first silent part may be minimum zero.

9)	Standardized version.

10)	For one or two seconds only.

11)	This tone burst is given for 1 to 2 seconds between 15 and 20 seconds before the paid time expires.

12)	In older equipments.

13)	Registration confirmation for subscriber dialling alarm call and do not disturb services.

14)	This tone is used within the supplementary service “call waiting” the duration is 30 seconds.

15)	Warning tone - three party conference.

16)	For subscribers with don’t disturb or absent subscriber service activated.

17)	This tone is also used in private branch exchanges (PABX).

18)	Special dial tone is used when call transfer is active.

19)	Divergence till 530 Hz.

20)	Reduced by 10 dB every other time.

21)	Warning tone - at time limit barring.

22)	This tone is generally not retransmitted outside the French network. It can be retransmitted, however, in the case of calls arriving in Paris, which have to go through certain long-distance connections in the national network and in the case of certain neighbouring international relations. This tone is progressively withdrawn.

23)	Not North American precise tone plan; however it is possible that these tones may be received from certain electromechanical switching systems.

24)	Busy verification.

25)	One application of 1.5 to 2 seconds before PABX user intervenes; followed by repeated applications of 0.5 to 0.8 seconds, at intervals of 8 to 20 seconds.

26)	Modern PABXs may have one of the following arrangements using 400 Hz:

a)	single burst tone:	0.3 seconds,

b)	two burst tone:	0.1 - 0.1 - 0.1 seconds,

c)	three burst tone:	0.1 - 0.1 - 0.1 - 0.1 - 0.1 seconds.

27)	Analogue exchanges.

28)	Radio cellular lines only, prior to ringing tone or “not available” verbal announcement.

29)	To the caller on answered waiting calls.

30)	A prerecorded message is sent in electronic exchanges and a period tone in analogue exchanges.

31)	Frequency of 450 Hz used only in analogue exchanges.

32)	Reduce by 8 dB than dial tone.

33)	Analogue exchanges: Greenland = 425 Hz; Sierra Leone = 450 Hz.

___________________

EXPLANATION  OF  SYMBOLS

Frequency in Hz:	f1×f2	f1 is modulated by f2
f1+f2	the juxtaposition of two frequencies f1 and f2 without modulation
f1/f2	f1 is followed by f2
f1//f2	in some exchanges frequency f1 is used and in others frequency f2 is used.

Cadence in seconds:	ON  –  OFF