|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	E.212

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEPHONE  NETWORK  AND  ISDN

OPERATION,  NUMBERING,  ROUTING  
AND  MOBILE  SERVICE

IDENTIFICATION  PLAN  FOR  LAND  MOBILE  STATIONS

ITU-T  Recommendation  E.212

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation E.212 was published in Fascicle II.2 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation E.212

Fascicle II.2 - Rec. E.212

IDENTIFICATION PLAN FOR LAND MOBILE STATIONS

1	Introduction

The purpose of this Recommendation is to define a future international identification plan for land mobile stations in internationally harmonized public land mobile networks (PLMNs), and to establish the principles for allocation of international mobile station identities (IMSI) to stations in such networks.

Note – The term “mobile station”, as used in this Recommendation, includes both card operated mobile stations and mobile stations which are not card operated. In a card operated station, the IMSI may be contained in the card. In stations which are not card operated, the IMSI is contained in the physical mobile station equipment.

In order to enable land mobile stations to roam among public land mobile networks located in different countries, an international identification plan is required for unique international identification of such stations. It is desirable that the allocation of international mobile station identities should be made independently of the numbering plans used for accessing mobile stations from the different public networks. This will enable Administrations to develop their own national numbering plans for land mobile stations for different services without the need for coordinating them with other countries.

Note – The word “country” in this Recommendation is also used with the meaning of geographical area.

2	Design considerations

The design considerations that form the basis for the international identification plan for land mobile stations are as follows:

2.1	Public land mobile communication services may be provided internationally.

2.2	There could be a number of public land mobile networks (PLMNs) in a country.

2.3	When a number of PLMNs exist in one country, it should not be mandatory to integrate the identification plans of the various networks.

2.4	The identification plan shall permit the identification of the country as well as the PLMN in which the mobile station is registered.

2.5	The number of digits used to identify a PLMN within a country and a specific mobile station of that PLMN is a national matter; however, see § 4.2.2.

2.6	The identification plan should provide for substantial spare capacity to accommodate future requirements.

2.7	The identification plan need not be directly related to the numbering plans in use for different services.

2.8	The identification plan should, if necessary, enable the international mobile station identity to be used for:

a)	determination of the PLMN in which a foreign land mobile station is registered;

b)	mobile station identification when information about a specific land mobile station is to be exchanged between PLMNs;

c)	mobile station identification on the radio control path for registering a mobile station in a foreign PLMN;

d)	mobile station identification for all signalling on the radio control path;

e)	mobile station identification for charging and billing of foreign land mobile stations.

f)	subscription management, e.g., for retrieving, providing, changing and updating subscription data for a specific mobile station.

3	Definitions

The following terms are related to this Recommendation:

3.1	mobile country code (MCC)

The part of the mobile station identification uniquely identifying the country of domicile of the mobile station.

3.2	mobile network code (MNC)

A digit or a combination of digits in the national part of the mobile station identification uniquely identifying the home PLMN of the mobile station.

3.3	mobile station identification number (MSIN)

The part of the mobile station identification following the mobile network code uniquely identifying the mobile station within a PLMN.

3.4	national mobile station identity (NMSI)

The mobile station identification uniquely identifying the mobile station nationally.

The NMSI consists of the MNC followed by the MSIN.

3.5	international mobile station identity (IMSI)

The mobile station identification uniquely identifying the mobile station internationally.

The IMSI consists of the MCC followed by the NMSI.

3.6	A comprehensive list of terms related to land mobile systems is given in Recommendation Q.70.

4	Identification plan principles

4.1	Structure of the mobile station identity

According to the definitions given in § 3, the international mobile station identity is structured as given in Figure 1/E.212.

4.2	Identity allocation principles

4.2.1	Only numerical characters (0-9) shall be used.

4.2.2	Since the international mobile station identity is not used for dialling and routing purposes through the public switched telephone network (PSTN), public switched data networks (PDNs), integrated services digital network (ISDN), etc., its length will not be influenced by any numbering constraints of these networks. However, Administrations should endeavour to keep the International Mobile Station Identity as short as possible; it shall under no circumstances exceed 15 digits.

4.2.3	The mobile country code (MCC) consists of 3 digits.

The allocation of MCCs is to be administered by the CCITT and is given in Annex A. Further MCCs may be allocated, if required. The first digits 0, 1, 8 and 9 are reserved for future use.

4.2.4	The national mobile station identity is to be assigned by each Administration.

4.2.5	The allocation of mobile network codes should be such that not more than 6 digits of the international mobile station identity have to be analysed in a foreign PLMN for information transfer.

4.2.6	Only one international mobile station identity shall be assigned to each mobile station independent of the number and type of services which terminate in the mobile station.

ANNEX A

(to Recommendation E.212)

List of mobile country or geographical area codes

Note – The countries or geographical areas shown in this annex include those that already have code assignments in the case of other public telecommunication networks.

Zone 2

Code	Country or Geographical Area

202	Greece

204	Netherlands (Kingdom of the)

206	Belgium

208	France

212	Monaco

214	Spain

216	Hungarian People's Republic

218	German Democratic Republic

220	Yugoslavia (Socialist Federal Republic of)

222	Italy

226	Romania (Socialist Republic of)

228	Switzerland (Confederation of)

230	Czechoslovak Socialist Republic

232	Austria

234	United Kingdom of Great Britain and Northern Ireland

235	United Kingdom of Great Britain and Northern Ireland

238	Denmark

240	Sweden

242	Norway

244	Finland

250	Union of Soviet Socialist Republics

Zone 2 (cont.)

Code	Country or Geographical Area

260	Poland (People's Republic of)

262	Germany (Federal Republic of)

266	Gibraltar

268	Portugal

270	Luxembourg

272	Ireland

274	Iceland

276	Albania (Socialist People's Republic of)

278	Malta (Republic of)

280	Cyprus (Republic of)

284	Bulgaria (People's Republic of)

286	Turkey

288	Faroe Islands

290	Greenland

292	San Marino (Republic of)

––––––––––

Zone 2, Spare Codes: 64

Zone 3

Code	Country or Geographical Area

302	Canada

308	St. Pierre and Miquelon (French Dept. of)

310	United States of America

311	United States of America

312	United States of America

313	United States of America

314	United States of America

315	United States of America

316	United States of America

330	Puerto Rico

332	Virgin Islands (USA)

334	Mexico

338	Jamaica

340	French Antilles

342	Barbados

344	Antigua and Barbuda

346	Cayman Islands

348	British Virgin Islands

350	Bermuda

352	Grenada

354	Montserrat

356	St. Kitts and Nevis

358	St. Lucia

360	St. Vincent and the Grenadines

362	Netherlands Antilles

364	Bahamas (Commonwealth of the)

366	Dominica (Commonwealth of the)

368	Cuba

370	Dominican Republic

372	Haiti (Republic of)

374	Trinidad and Tobago

376	Turks and Caicos Islands

––––––––––

Zone 3, Spare Codes: 68

Zone 4

Code	Country or Geographical Area

404	India (Republic of)

410	Pakistan (Islamic Republic of)

412	Afghanistan (Democratic Republic of)

413	Sri Lanka (Democratic Socialist Republic of)

414	Burma (Socialist Republic of the Union of)

415	Lebanon

416	Jordan (Hashemite Kingdom of)

417	Syrian Arab Republic

418	Iraq (Republic of)

419	Kuwait (State of)

420	Saudi Arabia (Kingdom of)

421	Yemen Arab Republic

422	Oman (Sultanate of)

423	Yemen (People's Democratic Republic of)

424	United Arab Emirates

425	Israel (State of)

426	Bahrain (State of)

427	Qatar (State of)

428	Mongolian People's Republic

429	Nepal

430	United Arab Emirates (Abu Dhabi)

431	United Arab Emirates (Dubai)

432	Iran (Islamic Republic of)

440	Japan

441	Japan

450	Korea (Republic of)

452	Viet Nam (Socialist Republic of)

454	Hong-Kong

455	Macao

456	Democratic Kampuchea

457	Lao People's Democratic Republic

460	China (People's Republic of)

467	Democratic People's Republic of Korea

470	Bangladesh (People's Republic of)

472	Maldives (Republic of)

––––––––––

Zone 4, Spare Codes: 65

Zone 5

Code	Country or Geographical Area

502	Malaysia

505	Australia

510	Indonesia (Republic of)

515	Philippines (Republic of the)

520	Thailand

525	Singapore (Republic of)

528	Brunei Darussalam

530	New Zealand

535	Guam

536	Nauru (Republic of)

537	Papua New Guinea

539	Tonga (Kingdom of)

Zone 5 (cont.)

Code	Country or Geographical Area

540	Solomon Islands

541	Vanuatu

542	Fiji

543	Wallis and Futuna Islands

544	American Samoa

545	Kiribati (Republic of)

546	New Caledonia and Dependencies

547	French Polynesia

548	Cook Islands

549	Western Samoa

––––––––––

Zone 5, Spare Codes: 78

Zone 6

Code	Country or Geographical area

602	Egypt (Arab Republic of)

603	Algeria (People's Democratic Republic of)

604	Morocco (Kingdom of)

605	Tunisia

606	Libya (Socialist People's Libyan Arab Jamahiriya)

607	Gambia (Republic of the)

608	Senegal (Republic of)

609	Mauritania (Islamic Republic of)

610	Mali (Republic of)

611	Guinea (Republic of)

612	Côte d'lvoire (Republic of)

613	Burkina Faso

614	Niger (Republic of the)

615	Togolese Republic

616 	Benin (People's Republic of)

617	Mauritius

618	Liberia (Republic of)

619	Sierra Leone

620	Ghana

621	Nigeria (Federal Republic of)

622	Chad (Republic of the)

623	Central African Republic

624	Cameroon (Republic of)

625	Cape Verde (Republic of)

626	Sao Tome and Principe (Democratic Republic of)

627	Equatorial Guinea (Republic of)

628	Gabonese Republic

629	Congo (People's Republic of the)

630	Zaire (Republic of)

631	Angola (People's Republic on

632	Guinea-Bissau (Republic of)

633	Seychelles (Republic of)

634	Sudan (Republic of the)

635	Rwandese Republic

Zone 6 (cont.)

Code	Country or Geographical area

636	Ethiopia

637	Somali Democratic Republic

638	Djibouti (Republic of)

639	Kenya (Republic on

640	Tanzania (United Republic of)

641	Uganda (Republic of)

642	Burundi (Republic on

643	Mozambique (People's Republic of)

645	Zambia (Republic of)

646	Madagascar (Democratic Republic of)

647	Reunion (French Department of)

648	Zimbabwe (Republic of)

649	Namibia

650	Malawi

651	Lesotho (Kingdom of)

652	Botswana (Republic of)

653	Swaziland (Kingdom of)

654	Comoros (Islamic Federal Republic of the)

655	South Africa (Republic of)

––––––––––

Zone 6, Spare Codes: 47

Zone 7

Code	Country or Geographical Area

702	Belize

704	Guatemala (Republic of)

706	El Salvador (Republic of)

708	Honduras (Republic of)

710	Nicaragua

712	Costa Rica

714	Panama (Republic of)

716	Peru

722	Argentine Republic

724	Brazil (Federative Republic of)

730	Chile

732	Colombia (Republic of)

734	Venezuela (Republic of)

736	Bolivia (Republic of)

738	Guyana

740	Ecuador

742	Guiana (French Department of)

744	Paraguay (Republic of)

746	Suriname (Republic of)

748	Uruguay (Eastern Republic of)

––––––––––

Zone 7, Spare Codes: 80