|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

CCITT	E.430

THE  INTERNATIONAL
TELEGRAPH  AND  TELEPHONE
CONSULTATIVE  COMMITTEE

TELEPHONE  NETWORK  AND  ISDN

QUALITY  OF  SERVICE,
NETWORK  MANAGEMENT  AND  TRAFFIC
ENGINEERING

QUALITY  OF  SERVICE  FRAMEWORK

Recommendation  E.430

|  | Geneva, 1992 |
| --- | --- |

# FOREWORD

The CCITT (the International Telegraph and Telephone Consultative Committee) is a permanent organ of the International Telecommunication Union (ITU). CCITT is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The Plenary Assembly of CCITT which meets every four years, establishes the topics for study and approves Recommendations prepared by its Study Groups. The approval of Recommendations by the members of CCITT between Plenary Assemblies is covered by the procedure laid down in CCITT Resolution No. 2 (Melbourne, 1988).

Recommendation E.430 was prepared by Study Group II and was approved under the Resolution No. 2 procedure on the 16th of June 1992.

___________________

# CCITT  NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication Administration and a recognized private operating agency.

ã  ITU  1992

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation E.430

Recommendation E.430

QUALITY  OF  SERVICE  FRAMEWORKXE "QUALITY  OF  SERVICE  FRAMEWORK"§

# 1	Introduction

A typical user is not interested in how a particular service is implemented, but he is interested in comparing one service with another in terms of certain universal performance parameters which apply to any service. Quality of Service (QOS) is expressed by parameter which:

—	do not depend on assumptions about the network internal design;

—	are referred to in terms of user-perceivable effects and not by their causes in the network;

—	are described in network independent terms and create a common language understandable by both the user and the provider;

—	take into account all aspects of the service which can be objectively measured at the service access point.

A network provider is concerned with the efficiency and effectiveness of the network. From the network providers point of view, network performance is best expressed by parameters which give information for:

—	system development;

—	network planning;

—	operation and maintenance.

The user orientated QOS parameters provide a valuable framework for network design, but they are not necessarily usable in specifying network performance requirements for particular connections.

Similarly, the network performance (NP) parameters primarily determine the Quality of Service, but they do not, necessarily, describe that quality in a way that is meaningful to users.

Both types of parameters are needed, and their values must be related if a network is to be effective in serving its users. The definition of QOS and NP parameters should make mapping of values clear in cases where there is not a simple one-to-one relationship between them.

# 2	Purpose

It is recognised that NP parameters make a major contribution to the QOS. The purpose of this Recommendation is to identify a relationship between Quality of Service (QOS) and network performance (NP) parameters.

# 3	Scope

This Recommendation covers the technical performance of a connection. For compatibility and completeness, Quality of Service aspects are considered in the form of a high level 3 ´ 3 matrix.

# 4	MatrixXE "Matrix"§

A matrix has been designed to enable cross correlation between QOS parameters and relevant QOS and NP Recommendations covering all aspects of telecommunications.

The matrix forms the prime structure for identifying all QOS sources and relationships, relevant to network performance.

The intention is that, by cross-relating the appropriate general identifiers, it will be possible to source the required detailed parameter.

The performance parameters are classified according to the three basic communication functions; access, user information transferXE "access, user information transfer"§ and disengagementXE "disengagement"§, and are described according to speed, accuracy and dependability criteria.

SpeedXE "Speed"§ is the time-related criterion used to describe the function performance.

AccuracyXE "Accuracy"§ is the criterion that describes the degree of correctness with which the function is performed.

DependabilityXE "Dependability"§ is the performance criterion that describes the degree of certainty (or surety) with which a function is performed regardless of speed or accuracy, but within a given observation interval (from Recommendation I.350).

| Criterion/Function | Speed | Accuracy | Dependability |
| --- | --- | --- | --- |
| Connection set-up (access) | E.431 See List A | E.845 (Rev.1) See List B | E.845, E.846 See List C |
| User information transfer | E.432 See List D | E.432, E.855 (Rev.1) See List E | E.850 (Rev.1) See List F |
| Connection disengagement (release) | E.431 See List G | See List H | See List I |

FIGURE 1/E.430

High-level 3 ´ 3 matrix

TABLE  1/E.430

Recommendations related the entries of the 3 ´ 3 matrix

| List A | List B | List C |
| --- | --- | --- |
| E.431 | E.845 (Rev.1) | E.845 E.846 |
| X.135 | X.136 | X.136 |
| I.352 | I.352 | I.352 |
| E.422 E.427 E.721 | E.422 E.424 E.425 E.426 E.427 |  |
| List D | List E | List F |
| E.432 | G.821 | E.850 (Rev.1) |
| X.135 | G.82x | X.136 |
| I.35B | E.422 E.424 E.432 | I.35C |
| I.35C | E.855 (Rev.1) | I.35P |
| I.35P | X.136 | I.35B |
|  | I.35B | E.428 |
|  | I.35P |  |
| List G | List H | List I |
| E.431 |  |  |
| X.135 | X.136 | X.136 |
| I.352 | I.352 | I.352 |
| E.721 |  |  |