|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	E.510

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEPHONE  NETWORK  AND  ISDN

QUALITY  OF  SERVICE,  NETWORK  MANAGEMENT  AND  TRAFFIC  ENGINEERING

DETERMINATION  OF  THE  NUMBER  OF  CIRCUITS  IN  MANUAL  OPERATION

ITU-T  Recommendation  E.510

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation E.510 was published in Fascicle II.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation E.5101)

Fascicle II.3 - Rec. E.510

DETERMINATION OF THE NUMBER OF CIRCUITS

IN MANUAL OPERATION

1	The quality of an international manual demand service should be defined as the percentage of call requests which, during the average busy hour (as defined later under § 3) cannot be satisfied immediately because no circuit is free in the relation considered.

By call requests satisfied immediately are meant those for which the call is established by the same operator who received the call, and within a period of two minutes from receipt of that call, whether the operator (when she does not immediately find a free circuit) continues observation of the group of circuits, or whether she makes several attempts in the course of this period.

Ultimately, it will be desirable to evolve a corresponding definition based on the average speed of establishing calls in the busy hour, i.e. the average time which elapses between the moment when the operator has completed the recording of the call request and the moment when the called subscriber is on the line, or the caller receives the advice subscriber engaged, no reply, etc. But for the moment, in the absence of information about the operating time in the European international service, such a definition cannot be established.

2	The number of circuits it is necessary to allocate to an international relation, in order to obtain a given grade of service, should be determined as a function of the total holding time of the group in the busy hour.

The total holding time is the product of the number of calls in the busy hour and a factor which is the sum of the average call duration and the average operating time.

These durations will be obtained by means of a large number of observations made during the busy hours, by agreement between the Administrations concerned. If necessary, the particulars entered on the tickets could also serve to determine the average duration of the calls.

The average call duration will be obtained by dividing the total number of minutes of conversation recorded by the recorded number of effective calls.

The average operating time will be obtained by dividing the total number of minutes given to operating (including ineffective calls) by the number of effective calls recorded.

3	The number of calls in the busy hour will be determined from the average of returns taken during the busy hours on a certain number of busy days in the year.

Exceptionally busy days, such as those which occur around certain holidays, etc., will be eliminated from these returns. The Administrations concerned should plan, whenever possible, to put additional circuits into service for these days.

In principle, these returns will be taken during the working days of two consecutive weeks, or during ten consecutive working days. If the monthly traffic curve shows only small variations, they will be repeated twice a year only. They will be taken three or four times a year or more if there are material seasonal variations, so that the average established is in accordance with all the characteristic periods of traffic flow.

4	The total occupied time thus determined should be increased by a certain amount determined by agreement between the Administrations concerned according to the statistics of traffic growth during earlier years, to take account of the probable growth in traffic and the fact that putting new circuits into service takes place some time after they are first found to be necessary.

5	The total holding time of the circuits thus obtained, in conjunction with a suitable table (see Table 1/E.510), will enable the required number of circuits to be ascertained.

6	In the international manual telephone service, the following Tables A and B should be used as a basis of minimum allocation:

Table A corresponds to about 30% of calls failing at the first attempt because of all circuits being engaged and to about 20% of the calls being deferred.

Table B, corresponding to about 7% of calls deferred, will be used whenever possible.

These tables do not take account of the fact that the possibility of using secondary routes permits, particularly for small groups, an increase in the permissible occupation time.

TABLE 1/E.510

Capacity of circuit groups

(See Supplement No. 2 - Fascicle II.3 of the Blue Book)

|  | Table A |  | Table B |  |
| --- | --- | --- | --- | --- |
| Number of circuits | Percentage of circuit usage | Minutes of circuit usage possible in the busy hour | Percentage of circuit usage | Minutes of circuit usage possible in the busy hour |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 | 65.0 76.7 83.3 86.7 88.6 90.0 91.0 91.7 92.2 92.6 93.0 93.4 93.6 93.9 94.1 94.2 94.3 94.4 94.5 94.6 | 39 92 150 208 266 324 382 440 498 556 614 672 730 788 846 904 962 1020 1078 1136 | – 46.6 56.7 63.3 68.3 72.0 74.5 76.5 78.0 79.2 80.1 81.0 81.7 82.3 82.8 83.2 83.6 83.9 84.2 84.6 | – 56 102 152 205 259 313 367 421 475 529 583 637 691 745 799 853 907 961 1015 |

Note – Tables A and B can be extended for groups comprising more than 20 circuits by using the values given for 20 circuits.

1)	This Recommendation dates from the XIIIth Plenary Assembly of the CCIF (London, 1946) and has not been fundamentally revised since. It was studied under Question 13/II in the Study Period 1968-1972 and was found to be still valid.