|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	E.800

TELECOMMUNICATION	(08/94)
STANDARDIZATION  SECTOR
OF  ITU

TELEPHONE  NETWORK  AND  ISDN

QUALITY  OF  SERVICE,  NETWORK  MANAGEMENT
AND  TRAFFIC  ENGINEERING

TERMS  AND  DEFINITIONS  RELATED
TO  QUALITY  OF  SERVICE  AND  NETWORK
PERFORMANCE  INCLUDING  DEPENDABILITY

ITU-T  Recommendation  E.800

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation E.800 was revised by ITU-T Study Group 2 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 12th of August 1994.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation E.800     (08/94)

Page

1	Introduction		1

1.1	Overview		1

1.2	General guide to concepts		1

1.3	General performance concepts		3

2	General terminology		3

2.1	Service related performance		3

2.2	Item related performance		4

2.3	Common concepts		4

3	Quality of service terminology		5

3.1	Service support		5

3.2	Service operability		6

3.3	Serveability		7

3.4	Service security		10

4	Network performance terminology		10

4.1	Trafficability		10

4.2	Dependability		10

4.3	Transmission		17

4.4	Charging		18

5	Evaluation concepts		18

5.1	Common concepts		18

5.2	Measure modifiers		18

5.3	Events and states		19

5.4	Statistical concepts		27

5.5	Maintenance concepts		32

5.6	Time concepts		36

5.7	Test, data, design and analysis concepts		41

Annex A  –  Related QoS/NP Handbooks and E-Series Recommendations		46

Annex B  –  Relations between defect, failure and fault concepts		48

Annex  C  –  List of recommended symbols and abbreviations		49

Index  –  Alphabetical list of definitions contained in this Recommendation		50

Recommendation E.800

Recommendation E.800     (08/94)

TERMS  AND  DEFINITIONS  RELATED  TO  QUALITY  OF  SERVICE
AnD  NETWORK  PERFORMANCE  INCLUDING  DEPENDABILITY

(revised in 1994)

# 1	Introduction

## 1.1	Overview

Terminology standardization is necessary for two main reasons:

–	to avoid confusion to standards users by introducing conflicting terms and definitions;

–	to assist alignment between the various groups involved in telecommunication standards development.

A consistent set of terms and definitions is required, therefore, to develop the important areas of quality of service, network performance and dependability standards pertaining to the planning, provisioning and operation of telecommunication networks.

The intention of this Recommendation is to set out a comprehensive set of terms and definitions relating to these concepts. Associated terminology covering statistical terms, recommended modifiers etc. is also included to ensure the broadest possible coverage in one document. These collective terms and definitions can be universally applied to all telecommunication services and the network arrangements used to provide them.

The terms and definitions start at clause 2 and the general list can be found indexed in Annex D.

## 1.2	General guide to concepts

Figure 1 (Performance Concepts) is a framework intended to provide a general guide to the factors which contribute collectively to the overall quality of service as perceived by the user of a telecommunication service. The terms in the diagram can be thought of as generally applying either to the quality of service levels actually achieved in practice, to objectives which represent quality of service goals, or to requirements which reflect design specifications.

The figure is structured to show that one quality of service factor can depend on a number of others. It is important to note – although it is not explicitly stated in any of the definitions contained in this document – that the value of a characteristic measure of a particular factor may depend directly on corresponding values of other factors which contribute to it. This necessitates, whenever the value of a measure is given, that all of the conditions having an impact on that value be clearly stated.

The essential aspect of the global evaluation of a service is the opinion of the users of the service. The result of this evaluation expresses the users’ degrees of satisfaction. This Recommendation provides a framework for:

1)	the quality of service concept;

2)	relating quality of service and network performance;

3)	a set of performance measures.

It is obvious that a service can be used only if it is provided, and it is desirable that the provider has a detailed knowledge about the quality of the offered service. From the provider’s viewpoint, network performance is a concept by which network characteristics can be defined, measured and controlled to achieve a satisfactory level of service quality. It is up to the Service Provider to combine different network performance parameters in such a way that the economic requirements of the Service Provider as well as the satisfaction of the User are both fulfilled.

FIGURE  1/E.800...[D01] =

In the utilization of a service the User normally identifies the Service Provider only. The User’s degree of satisfaction with the service provided depends on quality of service, that is on the latter’s perception of the following service performance:

–	the support;

–	the operability;

–	the serveability;

–	the security.

All are dependent on network characteristics1). However, the serveability performance is the most generally affected. It is further subdivided into three terms:

–	service accessibility performance;

–	service retainability performance;

–	service integrity performance.

Serveability performance depends on trafficability performance and its influencing factors of resourcing and facility, dependability and transmission performance (of which propagation performance is a subset), as shown in Figure 1. The trafficability performance is described in terms of losses and delay times. Dependability is the combined aspects of availability, reliability, maintainability and maintenance support performance and relates to the ability of an item to be in a state to perform a required function. Propagation performance refers to the ability of the transmitting medium to transmit the signal within intended tolerances.

The resources and facilities box includes planning performance, provisioning performance and the related administrative functions. This spotlights the importance of the network planning and provisioning aspects, etc. to the overall quality of service results. These items are for further study.

## 1.3	General performance conceptsXE "General performance concepts"§

All performance concepts may be related to instant of time (instantaneous, etc.) or expressed as a mean value over a time interval.

While dependability is used only for a general description in non-quantitative terms, the actual quantification is done under the heading of availability performance, reliability performance, maintainability performance and maintenance support performance.

The properties expressed by these measures impact the measures relating to quality of service and network performance and are thus implicitly characterizations of these performance measures.

Measures are connected to events (failure, restoration, etc.), states (fault, up state, down state, outage, etc.) or activities (e.g. maintenance), with their time durations.

# 2	General terminology

## 2.1	Service related performanceXE "Service related performance"§

2101	quality of service

F:	qualité de service

S:	calidad de servicio

The collective effect of service performance which determine the degree of satisfaction of a user of the service.

NOTES

1	The quality of service is characterized by the combined aspects of service support performance, service operability performance, serveability performance, service security performance and other factors specific to each service.

2	The term “quality of service” is not used to express a degree of excellence in a comparative sense nor is it used in a quantitative sense for technical evaluations. In these cases a qualifying adjective (modifier) should be used.

## 2.2	Item related performanceXE "Item related performance"§

2201	network performance

F:	qualité de fonctionnement du réseau

S:	calidad de funcionamiento de la red

The ability of a network or network portion to provide the functions related to communications between users.

NOTES

1	Network performance applies to the Network Provider’s planning, development, operations and maintenance and is the detailed technical part of QOS, excluding service support performance and human factors.

2	Network performance is the main influence on serveability performance.

3	Network performance measures are meaningful to network providers and are quantifiable at the part of the network to which they apply. Quality of service measures are only quantifiable at a service access point.

4	It is up to the Network Provider to combine the Network Performance parameters in such a way that the economic requirements of the Network Provider, as well as the satisfaction of the User, are both fulfilled.

## 2.3	Common concepts

2301	service

F:	service

S:	servicio

A set of functions offered to a user by an organization.

2302	item; entity; element

F:	entité; individu; élément

S:	elemento; entidad; item

Any part, device, subsystem, functional unit, equipment or system that can be individually considered.

NOTES

1	An item may consist of hardware, software or both, and may also include people, e.g. operators in a telephone operator system.

2	In French, the term “entité” replaces the term dispostif previously used in this meaning, because the term dispositif is also the common equivalent for the English term “device”.

3	In French, the term “individu” is used mainly in statistics.

2303	user

F:	usager

S:	usuario

Any entity external to the network which utilizes connections through the network for communication.

2304	connection

F:	connexion

S:	conexión

An association of resources providing means for communication between two or more devices in, or attached to, a telecommunication network.

2305	teletraffic; telecommunications traffic

F:	télétrafic; trafic de télécommunication

S:	teletráfico; tráfico de telecomunicaciones

A process of events related to demands for the utilization of resources in a telecommunication network.

2306	resource

F:	ressource

S:	órgano; recurso

Any set of physically or conceptually identifiable entities within a telecommunications network, the use of which can be unambiguously determined.

2307	capability

F:	capacité; capabilité (d'une entité)

S:	capacidad

The ability of an item to meet a demand of a given size under given internal conditions.

NOTES

1	Internal conditions refer, for example, to any given combination of faulty and not faulty sub-items.

2	Trafficability performance and effectiveness are capabilities.

3	Demands are of two types – service and traffic.

# 3	Quality of service terminologyXE "Quality of service terminology"§

## 3.1	Service supportXE "Service support"§

3100	service support performance

F:	logistique du service

S:	logística del servicio

The ability of an organization to provide a service and assist in its utilization.

NOTE – An example of service support performance is the ability to provide assistance in commissioning a basic service, or a supplementary service such as the call waiting service or directory enquiries service.

For this performance concept the following measures will be used:

3101	mean service provisioning time

F:	délai moyen pour la fourniture d'un service

S:	tiempo medio de espera (para la prestación de un servicio)

The expectation of the duration between the instant of time a potential user requests that an organization provides the necessary means for a service, and the instant of time when these means are furnished.

3102	billing error probability

F:	probabilité d'erreur de facturation

S:	probabilidad de error de facturación

The probability of an error when billing a user of a service.

3103	incorrect charging or accounting probability

F:	probabilité de taxation ou de comptabilisation erronées

S:	probabilidad de tarificación o de contabilidad incorrectas

The probability of a call attempt receiving incorrect charging or accounting treatment.

3104	undercharging probability

F:	probabilité de sous–taxation

S:	probabilidad de subtarificación

The probability that a call attempt will be undercharged for any reason.

3105	overcharging probability

F:	probabilité de surtaxation

S:	probabilidad de sobretarificación

The probability that a call attempt will be overcharged for any reason.

3106	billing integrity (probability)

F:	justesse de la facturation (probabilité de)

S:	integridad de la facturación (probabilidad de)

The probability that the billing information presented to a user correctly reflects the type, destination and duration of the call attempt.

## 3.2	Service operabilityXE "Service operability"§

3200	service operability performance

F:	facilité d'utilisation d'un service

S:	facilidad de utilización (de un servicio)

The ability of a service to be successfully and easily operated by a user.

For this performance concept the following measures will be used:

3201	service user mistake probability

F:	probabilité d'erreur d'un usager en service

S:	probabilidad de error de un usuario (de un servicio)

Probability of a mistake made by a user in his attempt to utilize a service.

3202	dialling mistake probability

F:	probabilité d'erreur de numérotation

S:	probabilidad de error de marcación

The probability that the user of a telecommunication network makes dialling mistakes during his call attempts.

3203	service user abandonment probability

F:	probabilité d'abandon d'un service par un usager

S:	probabilidad de abandono de un servicio por un usuario

The probability that a user abandons the attempt to use a service.

NOTE – Abandonments may be caused by excessive user mistake rates, by excessive service access delays, etc.

3204	call abandonment probability

F:	probabilité d'abandon d'une tentative d'appel

S:	probabilidad de abandono de un intento de llamada

The probability that a user abandons the call attempt through a telecommunication network.

## 3.3	ServeabilityXE "Serveability"§

3300	serveability performance

F:	servibilité d'un service

S:	servibilidad del servicio

The ability of a service to be obtained – within specified tolerances and other given conditions – when requested by the user and continue to be provided without excessive impairment for a requested duration.

NOTE – Serveability performance may be subdivided into the service accessibility performance, service retainability performance and the service integrity performance.

3310	service accessibility performance

F:	accessibilité d'un service

S:	accesibilidad del servicio

The ability of a service to be obtained, within specified tolerances and other given conditions, when requested by the user.

NOTE – This takes into account the transmission tolerance and the combined aspects of propagation performance, trafficability performance and availability performance of the related systems.

For this performance concept the following measures will be used:

3311	service accessibility; service access probability

F:	accessibilité d'un service; probabilité d'accès à un service

S:	accesibilidad del servicio; probabilidad de acceso al servicio

The probability that a service can be obtained within specified tolerances and other given operating conditions when requested by the user.

3312	mean service access delay

F:	durée moyenne d'accès à un service

S:	retardo medio de acceso al servicio; demora media de acceso al servicio

The expectation of the time duration between an initial bid by the user for the acquisition of a service and the instant of time the user has access to the service, the service being obtained within specified tolerances and other given operating conditions.

3313	network accessibility

F:	accessibilité d'un réseau

S:	accesibilidad de la red

The probability that the user of a service after a request receives the proceed-to-select signal within specified conditions.

NOTE – The proceed-to-select signal is that signal inviting the user to select the desired destination.

3314	connection accessibility

F:	accessibilité d'une connexion

S:	accesibilidad de una conexión

The probability that a connection can be established within specified tolerances and other given conditions following receipt by the exchange of a valid code.

3315	mean access delay

F:	durée moyenne d'accès

S:	retardo medio de acceso; demora media de acceso

The expectation of the time duration between the first call attempt made by a user of a telecommunication network to reach another user or a service and the instant of time the user reaches the wanted other user or service, within specified tolerances and under given operational conditions.

3316	p-fractile access delay

F:	quantile-p de la durée d'accès

S:	cuantil-p del retardo de acceso; cuantil-p de la demora de acceso

The p-fractile value of the duration between the first call attempt made by a user of a telecommunication network to reach another user or a service and the instant of time the user reaches the wanted other user or service, within specified tolerances and under given operational conditions.

3317	accessibility of a connection to be established

F:	accessibilité d'une communication à établir

S:	accesibilidad de una conexión a establecer

The probability that a switched connection can be established, within specified transmission tolerances, to the correct destination, within a given time interval, when requested by the user.

NOTES

1	For user–originated calls, it could express the probability of a successful call establishment on the first attempt. For operator-handled calls, it could represent the probability of having a satisfactory connection established within a given time duration.

2	In general, the tolerances should correspond to a level of transmission performance which makes the connection unsatisfactory for service such that, for example, a substantial percentage of users would abandon the connection.

3318	unacceptable transmission probability

F:	probabilité d'une transmission inacceptable

S:	probabilidad de transmisión inaceptable

The probability of a connection being established with an unacceptable speech path transmission quality.

3319	no tone probability

F:	probabilité d'absence de la tonalité

S:	probabilidad de ausencia de tono

The probability of a call attempt encountering no tone following receipt of a valid code by the exchange.

3320	misrouting probability

F:	probabilité d'acheminement erroné

S:	probabilidad de encaminamiento erróneo

The probability of a call attempt being misrouted following receipt by the exchange of a valid code.

3330	service retainability performance

F:	continuabilité d'un service

S:	retenibilidad del servicio

The ability of a service, once obtained, to continue to be provided under given conditions for a requested duration.

NOTE – Generally this depends on the transmission tolerances, the propagation performance and reliability performance of the related systems. For some services, for example packet switching, this also depends on the trafficability performance and the availability performance of the related systems.

For this performance concept the following measures will be used:

3331	service retainability

F:	continuabilité d'un service

S:	retenibilidad del servicio

The probability that a service, once obtained, will continue to be provided under given conditions for a given time duration.

3332	connection retainability

F:	continuabilité d'une connexion

S:	retenibilidad de la conexión

The probability that a connection, once obtained, will continue to be provided for a communication under given conditions for a given time duration.

3333	retainability of an established connection

F:	continuabilité d'une connexion établie

S:	retenibilidad de la conexión establecida

The probability that a switched connection, once established, will operate within specified transmission tolerances without interruption for a given time interval.

3334	premature release probability; cut-off call probability

F:	probabilité de libération prématurée; probabilité de commune d'une communication

S:	probabilidad de liberación prematura; probabilidad de corte de la llamada

The probability that an established connection will be released for a reason other than intentionally by any of the parties involved in the call.

3335	release failure probability

F:	probabilité d'échec de la libération

S:	probabilidad de fallo de liberación

The probability that the required release of a connection will not take place.

3336	probability of successful service completion

F:	probabilité d'exécution correcte d'un service

S:	probabilidad de prestación satisfactoria de un servicio

The probability that a connection can be established, under satisfactory operating conditions, and retained for a given time interval.

3340	service integrity performance

F:	integrité d'un service

S:	integridad del servicio

The degree to which a service is provided without excessive impairments, once obtained.

For this performance concept the following measures will be used:

3341	interruption; break (of service)

F:	interruption; coupure (d'un service)

S:	interrupción; corte (del servicio)

Temporary inability of a service to be provided persisting for more than a given time duration, characterized by a change beyond given limits in at least one parameter essential for the service.

NOTES

1	An interruption of a service may be caused by disabled states of the items used for the service or by external reasons such as high service demand.

2	An interruption of a service is generally an interruption of the transmission, which may be characterized by an abnormal value of power level, noise level, signal distortion, error rate, etc.

3342	time between interruptions

F:	temps entre interruptions

S:	tiempo entre interrupciones

The time duration between the end of one interruption and the beginning of the next.

3343	interruption duration

F:	durée d'interruption

S:	duración de interrupción

The time duration of an interruption.

3344	mean time between interruptions (MTBI)

F:	durée moyenne entre interruptions (MTBI)

S:	tiempo medio entre interrupciones (MTBI)

The expectation of the time between interruptions.

3345	mean interruption duration (MID)

F:	durée moyenne d'une interruption (MID)

S:	duración media de una interrupción (MID)

The expectation of the interruption duration.

## 3.4	Service securityXE "Service security"§

3400	service security performance

F:	Securité d'un service

S:	Seguridad del servicio

The protection provided against unauthorized monitoring, fraudulent use, malicious impairment, misuse, human mistake and natural disaster.

For this performance concept the following measures will be used:

(For further study.)

# 4	Network performance terminologyXE "Network performance terminology"§

## 4.1	TrafficabilityXE "Trafficability"§

4100	trafficability performance

F:	traficabilité; capacité d'écoulement du trafic

S:	aptitud para cursar tráfico

The ability of an item to meet a traffic demand of a given size and other characteristics, under given internal conditions.

NOTE – Given internal conditions refer, for example, to any combination of faulty and not faulty sub-items.

For this performance concept the following measures will be used:

(For further study – see Recommendation E.600.)

## 4.2	Dependability

4200	dependability

F:	sûreté de fonctionnement

S:	seguridad de funcionamiento

The collective term used to describe the availability performance and its influencing factors: reliability performance, maintainability performance and maintenance support performance.

NOTE – Dependability is used only for general descriptions in non-quantitative terms.

4210	availability performance

F:	disponibilité

S:	disponibilidad

The ability of an item to be in a state to perform a required function at a given instant of time or at any instant of time within a given time interval, assuming that the external resources, if required, are provided.

NOTES

1	This ability depends on the combined aspects of the reliability performance, the maintainability performance and the maintenance support performance of an item.

2	In the definition of the item the external resources required must be delineated.

3	The term availability is used as an availability performance measure.

4	Warning: the term availability has occasionally been used in connection with the term item, but with an implied meaning of item being entirely different from that of this Recommendation.

For this performance concept the following measures will be used:

4211	instantaneous availability; pointwise availability; A(t) (symbol)

F:	disponibilité (instantanée), A(t) (symbole)

S:	disponibilidad instantánea, A(t) (símbolo)

The probability that an item is in an up state at a given instant of time, t.

NOTE – In French the term disponibilité is also used to denote the performance quantified by this probability.

4212	instantaneous unavailability; pointwise unavailability; U(t) (symbol)

F:	indisponibilité instantanée, U(t) (symbole)

S:	indisponibilidad instantánea, U(t) (símbolo)

The probability that an item is in a down state at a given instant of time, t.

4213	mean availability, \s\up6(–);A)(t1 (symbol)

F:	disponibilité moyenne, \s\up6( –);A)(t1 (symbole)

S:	disponibilidad media, \s\up6( –);A)(t1 (símbolo)

The normalized integral of the instantaneous availability in a given time interval (t1, t2).

NOTE – The mean availability is related to the instantaneous availability as

\s\up5( –);A)(t1

4214	mean unavailability, \s\up6(–) (symbol)

F:	indisponibilité moyenne, \s\up6( –);U)(t1 (symbole)

S:	indisponibilidad media, \s\up6( –);U)(t1 (símbolo)

The normalized integral of the instantaneous unavailability in a stated time interval (t1, t2).

NOTE – The mean unavailability is related to the instantaneous unavailability as

\s\up5( –

4215	(asymptotic) availability; (steady-state) availability, A (symbol)

F:	disponibilité asymptotique, A (symbole)

S:	disponibilidad (asintótica); disponibilidad (en régimen permanente); A (símbolo)

The limit, if this exists, of the instantaneous availability when the time tends to infinity.

NOTE – Under certain conditions, for instance constant failure rate and constant repair rate, the asymptotic availability may be expressed as:

where

MDT is the mean down time

MUT is the mean up time.

4216	asymptotic unavailability, U (symbol)

F:	indisponibilité asymptotique, U (symbole)

S:	indisponibilidad asintótica, U (símbolo)

The limit, if this exists, of the instantaneous unavailability when the time tends to infinity.

NOTE – Under certain conditions, for instance constant failure rate and constant repair rate, the asymptotic unavailability may be expressed as:

where

MDT is the mean down time

MUT is the mean up time.

4217	asymptotic mean availability, \s\up6(– (symbol)

F:	disponibilité moyenne asymptotique, asymptotic mean availability, \s\up6(–  (symbol) (symbole)

S:	disponibilidad media asintótica, asymptotic mean availability, \s\up6(– (symbol) (símbolo)

The limit, if this exists, of the mean availability over a time interval (t1, t2) when t2 tends to infinity.

NOTES

1	The asymptotic mean availability is related to the mean availability as

2	When such a limit exists it is not dependent on t1.

4218	asymptotic mean unavailability, \s\up6(– (symbol)

F:	indisponibilité moyenne asymptotique, \s\up6( – (symbole)

S:	indisponibilidad media asintótica, \s\up6( – (símbolo)

The limit, if this exists, of the mean unavailability over a time interval (t1, t2) when t2 tends to infinity.

NOTES

1	The asymptotic mean unavailability is related to the mean unavailability as

2	When such a limit exists it is not dependent on t1.

4219	mean up time (MUT)

F:	temps moyen de disponibilité; durée moyenne de disponibilité (MUT)

S:	tiempo medio de disponibilidad (MUT)

The expectation of the up time.

4220	mean accumulated down time (MADT)

F:	durée cumulée moyenne d'indisponibilité (MADT)

S:	tiempo medio acumulado de indisponibilidad (MADT)

The expectation of the accumulated down time.

4221	instantaneous availability of a leased circuit

F:	disponibilité instantanée d'un circuit loué

S:	disponibilidad instantánea de un circuito arrendado

The probability that, under stated operating conditions, a leased circuit can perform a required function when requested by the subscriber.

4230	reliability performance

F:	fiabilité

S:	fiabilidad

The ability of an item to perform a required function under given conditions for a given time interval.

NOTES

1	It is generally assumed that the item is in a state to perform this required function at the beginning of the time interval.

2	The term reliability is used as a measure of reliability performance.

For this performance concept the following measures will be used:

4231	reliability, R (symbol)

F:	fiabilité, R (symbole)

S:	fiabilidad, R (símbolo)

The probability that an item can perform a required function under stated conditions for a given time interval.

NOTES

1	It is generally assumed that the item is in a state to perform this required function at the beginning of the time interval.

2	In French, the term fiabilité is also used to denote the performance quantified by this probability.

4232	(instantaneous) failure rate, l(t) (symbol)

F:	 taux (instantané) de défaillance, l(t) (symbole)

S:	tasa (instantánea) de fallos, l(t) (símbolo)

The limit, if this exists, of the ratio of the conditional probability that the time to failure, T, of an item falls within a given time interval, (t, t+Dt), to the length of this interval, Dt, when Dt tends to zero, given that the item is in a state to perform a required function at the beginning of the time interval.

NOTE – The instantaneous failure rate is expressed by formula as:

where T is the instant of time of failure.

The formula is also applicable if T denotes the time to failure.

4233	mean failure rate, \s\up7(–);l)(t1 (symbol)

F:	taux moyen de défaillance \s\up6(–);l)(t1 (symbole)

S:	tasa media de fallos, \s\up6(–);l)(t1 (símbolo)

The normalized integral of the instantaneous failure rate over a given time interval, (t1, t2).

NOTE – The mean failure rate relates to instantaneous failure rate as

\s\up6(–

4234	(instantaneous) failure intensity, z(t) (symbol)

F:	intensité (instantanée) de défaillance, z(t) (symbole)

S:	intensitad (instantánea) de fallos, z(t) (símbolo)

The limit, if this exists, of the ratio of the mean number of failures of a repaired item in a time interval, (t, t + Dt), to the length of this interval, Dt, when the length of the time interval tends to zero.

NOTE – The instantaneous failure intensity is expressed by formula as:

where N (t) is the number of failures in the time interval (0, t).

4235	mean failure intensity, \s\up4(–);z)(t1 (symbol)

F:	intensité moyenne de défaillance, \s\up4(–);z)(t1(symbole)

S:	intensidad media de fallos\s\up4(–);z)(t1 (símbolo)

The normalized integral of the instantaneous failure intensity over a given time interval (t1, t2).

NOTE – The mean failure intensity is related to instantaneous failure intensity as:

\s\up3(–

4236	mean time to first failure (MTTFF)

F:	durée moyenne de fonctionnement avant la première défaillance (MTTFF)

S:	tiempo medio hasta el primer fallo (MTTFF)

The expectation of the time to first failure.

4237	mean time to failure (MTTF)

F:	durée moyenne de fonctionnement avant défaillance (MTTF)

S:	tiempo medio hasta el fallo (MTTF)

The expectation of the time to failure.

4238	mean time between failures (MTBF)

F:	moyenne des temps entre défaillances (MTBF)

S:	tiempo medio entre fallos (MTBF)

The expectation of the time between failures.

4239	failure rate acceleration factor

F:	 facteur d'accélération du taux de défaillance

S:	factor de aceleración de la tasa de fallos

The ratio of the accelerated testing failure rate to the failure rate under stated reference test conditions.

NOTE – Both failure rates refer to the same time period in the life of the tested items.

4240	failure intensity acceleration factor

F:	facteur d'accélération de l'intensité de défaillance

S:	factor de aceleración de la intensidad de fallos

In a time interval of given duration, whose beginning is specified by a fixed age of a repaired item, the ratio of the number of failures obtained under two different sets of stress conditions.

4250	maintainability performance

F:	maintenabilité

S:	mantenibilidad

The ability of an item under stated conditions of use, to be retained in, or restored to, a state in which it can perform a required function, when maintenance is performed under given conditions and using stated procedures and resources.

NOTE – The term maintainability is used as a measure of maintainability performance.

For this performance concept the following measures will be used:

4251	maintainability

F:	maintenabilité

S:	mantenibilidad

The probability that a given active maintenance action, for an item under given conditions of use can be carried out within a stated time interval, when the maintenance is performed under stated conditions and using stated procedures and resources.

NOTE – In French the term maintenabilité is also to denote the performance quantified by this probability.

4252	(instantaneous) repair rate, m(t) (symbol)

F:	taux (instantané), de réparation, m (t) (symbole)

S:	tasa (instantánea) de reparaciones, m (t) (símbolo)

The limit, if this exists, of the ratio of the conditional probability that the corrective maintenance action terminates in a time interval, (t, t+Dt) to the length of this time interval, when Dt tends to zero, given that the action had not terminated at the beginning of the time interval.

NOTE – The instantaneous repair rate is expressed by formula as:

where T is the instant of time of restoration.

T may also represent the time to restoration.

4253	mean repair rate, \s\up5(–);m)(t1 (symbol)

F:	taux moyen de réparation, \s\up5(–);m)(t1 (symbole)

S:	tasa media de reparaciones, \s\up5(–);m)(t1 (símbolo)

The normalized integral of the instantaneous repair rate over a given time interval (t1, t2).

NOTE – The mean repair rate is related to instantaneous repair rate as:

\s\up4(–

4254	mean maintenance man-hours

F:	durée moyenne équivalente de maintenance

S:	duración media equivalente de mantenimiento; media de horas-hombre de mantenimiento

The expectation of the maintenance man-hours.

4255	mean down time (MDT)

F:	temps moyen d'indisponibilité; durée moyenne d'indisponibilité (MDT)

S:	tiempo medio de indisponibilidad (MDT)

The expectation of the down time.

4256	mean repair time (MRT)

F:	durée moyenne de réparation (MRT)

S:	tiempo medio de reparación (MRT)

The expectation of the repair time.

4257	p-fractile repair time

F:	quantile-p de la durée de réparation

S:	cuantil-p del tiempo de reparación

The p-fractile value of the repair time.

4258	mean active repair time (MART)

F:	durée moyenne de réparation active (MART)

S:	tiempo medio de reparación activa (MART)

The expectation of the active repair time.

4259	p-fractile active repair time

F:	quantile-p de la durée de réparation active

S:	cuantil-p del tiempo de reparación activa

The p-fractile value of the active repair time.

4260	mean time to restoration (MTTR); mean time to recovery; mean time to repair (deprecated)

F:	durée moyenne de panne (MTTR)

S:	tiempo medio hasta el restablecimiento (MTTR)

The expectation of the time to restoration.

4261	fault coverage

F:	couverture des pannes

S:	cobertura de averías

The proportion of faults of an item that can be recognized under given conditions.

4262	repair coverage

F:	couverture des réparations

S:	cobertura de reparaciones

The proportion of faults of an item that can be successfully removed.

4270	maintenance support performance

F:	logistique de maintenance

S:	logística de mantenimiento

The ability of a maintenance organization, under given conditions, to provide upon demand the resources required to maintain an item, under a given maintenance policy.

NOTE – The given conditions are related to the item itself and to the conditions under which the item is used and maintained.

For this performance concept the following measures will be used:

4271	mean administrative delay (MAD)

F:	durée moyenne du délai administratif (MAD)

S:	retardo medio administrativo; demora media administrativa (MAD)

The expectation of the administrative delay.

4272	p-fractile administrative delay

F:	quantile-p du délai administratif

S:	cuantil-p del retardo administrativo; cuantil–p de la demora administrativa

The p-fractile value of the administrative delay.

4273	mean logistic delay (MLD)

F:	durée moyenne du délai logistique (MLD)

S:	retardo medio logístico; demora media logística (MLD)

The expectation of the logistic delay.

4274	p-fractile logistic delay

F:	quantile-p du délai logistique

S:	cuantil-p del retardo logístico; cuantil-p de la demora logística

The p-fractile value of the logistic delay.

## 4.3	TransmissionXE "Transmission"§

4300	transmission performance

F:	qualité de transmission

S:	calidad de transmisión

The level of reproduction of a signal offered to a telecommunications system, under given conditions, when this system is in an up state.

NOTE – The given conditions may include the effect of propagation performance where applicable.

For this performance concept the following measures are applicable. (This section is for further study.)

4301	bit error ratio (BER)

F:	taux d'erreur sur les bits (BER)

S:	tasa de errores de bit; tasa de error en los bits (BER)

The ratio of the number of bit errors to the total number of bits transmitted in a given time interval.

4302	error free seconds ratio

F:	taux de secondes sans erreur

S:	segundos sin error

The ratio of the number of one-second intervals during which no bits are received in error to the total number of one-second intervals in the time interval.

NOTES

1	The length of the time interval needs to be specified.

2	This ratio is usually expressed as a percentage.

4303	propagation performance

F:	caractéristiques de propagation

S:	característica de propagación

The ability of a propagation medium, in which a wave propagates without artificial guide, to transmit a signal within the given tolerances.

NOTES

1	The given tolerances may apply to variations in signal level, noise, interference levels, etc.

2	Propagation performance applies to radiocommunication only.

## 4.4	ChargingXE "Charging"§

4401	charging correctness

F:	exactitude de la taxation

S:	probabilidad de tarificación correcta; precisión de la tasación

The probability that the network correctly charges the communication by type, destination, time location and duration.

# 5	Evaluation conceptsXE "Evaluation concepts"§

## 5.1	Common concepts

5101	effectiveness (performance)

F:	efficacité

S:	efectividad

The ability of an item to meet a service demand of a given size.

NOTE – This ability depends on the combined aspects of the capability and the availability performance of the item.

5102	durability

F:	durabilité

S:	durabilidad

The ability of an item to remain in a condition where it can perform a required function under stated conditions of use and maintenance until a limiting state is reached.

NOTE – A limiting state of an item may be characterized by the end of the useful life, unsuitability for any economic or technological reasons, etc.

5103	modification (of an item)

F:	modification (d'une entité)

S:	modificación (de un elemento)

The combination of all technical and corresponding administrative actions intended to alter the capability of an item by changing, adding or deleting one or more required functions.

## 5.2	Measure modifiersXE "Measure modifiers"§

5201	true ...

F:	... vrai

S:	... verdadero

The ideal value which characterizes a quantity perfectly defined under the conditions which exist at the moment when that quantity is observed, or the subject of a determination.

NOTE – This value could be arrived at only if all causes of measurement error were eliminated.

5202	predicted ...

F:	... prédit; ... prévu

S:	... previsto; ... predicho

The numerical value assigned to a quantity, before the quantity is actually observable, computed on the basis of earlier observed or estimated values of the same quantity or of other quantities using a mathematical model.

5203	extrapolated ...

F:	... extrapolé

S:	... extrapolado

The predicted value based on estimated values for one or a set of conditions, intended to apply to other conditions such as time, maintenance and environmental conditions.

5204	estimated ...

F:	... estimé

S:	... estimado

The value obtained as the result of an estimation.

NOTE – The result may be expressed either as a single numerical value, a point estimate, or as a confidence interval.

5205	intrinsic ...; inherent ...

F:	... intrinsèque; ... inhérent

S:	... intrínseco; ... inherente

Value of a measure determined when maintenance and operational conditions are assumed to be ideal.

5206	operational ...

F:	... opérationnel

S:	... operacional

Value determined under given operational conditions.

5207	mean ...; average ... (deprecated)

F:	... moyen (adjectif)

S:	... medio (adjetivo); promedio (desaconsejado)

1)	The value obtained as the expectation of a random variable.

2)	The normalized integral of a time dependant quantity.

5208	p-fractile ...

F:	... quantile-p

S:	cuantil-p de ...

The value obtained as the p-fractile of the distribution of a random variable.

5209	instantaneous ...

F:	... instantané

S:	... instantáneo

The value of a measure determined for a given instant of time.

## 5.3	Events and statesXE "Events and states"§

### 5.3.1	Defects

5301	defect

F:	défaut

S:	defecto

Any departure of a characteristic of an item from requirements.

NOTES

1	The requirements may or may not be expressed in the form of a specification.

2	A defect may or may not affect the ability of an item to perform a required function.

5302	bug

F:	erreur de programmation; bogue

S:	error de programación

A software defect caused by a mistake.

5303	critical defect

F:	défaut critique

S:	defecto crítico

A defect that is assessed likely to result in injury to persons or significant material damage.

5304	non-critical defect

F:	défaut non critique

S:	defecto no crítico

A defect other than a critical defect.

5305	major defect

F:	défaut majeur

S:	defecto mayor

A defect that is likely to result in a failure or to reduce materially the usability of the item for its intended purpose.

5306	minor defect; imperfection

F:	défaut mineur; imperfection

S:	defecto menor; imperfección

A defect other than a major defect.

5307	defective; defective item

F:	défectueux; entité défectueuse

S:	defectuoso; elemento defectuoso

An item which contains one or more defects.

5308	critical defective item

F:	défectueux critique

S:	elemento defectuoso crítico

An item which contains one or more critical defect.

5309	major defective item

F:	défectueux majeur

S:	elemento defectuoso mayor

An item which contains one or more major defects.

5310	minor defective item

F:	défectueux mineur

S:	elemento defectuoso menor

An item which contains one or more minor defects but no major defects.

5311	design defect

F:	défaut de conception

S:	defecto de diseño

A defect due to an inadequate design of an item.

5312	manufacturing defect

F:	défaut de fabrication

S:	defecto de fabricación

A defect due to non conformance in manufacture to the design of an item or to specified manufacturing processes.

### 5.3.2	Failures

5321	failure

F:	défaillance

S:	fallo

The termination of the ability of an item to perform a required function.

NOTE – After failure the item has a fault.

5322	critical failure

F:	défaillance critique

S:	fallo crítico

A failure which is assessed likely to result in injury to persons or significant material damage.

5323	non-critical failure

F:	défaillance non critique

S:	fallo no crítico

A failure other than a critical failure.

5324	misuse failure

F:	défaillance par mauvaise utilisation

S:	fallo por uso incorrecto

A failure due to induced stresses during use which are beyond the stated capabilities of the item.

5325	mishandling failure

F:	défaillance par fausse manoeuvre

S:	fallo por manejo incorrecto

A failure caused by incorrect handling or lack of care of the item.

5326	(inherent) weakness failure

F:	défaillance par fragilité (inhérente)

S:	fallo por fragilidad (inherente)

A failure due to a weakness inherent in the item itself when subjected to stresses within the stated capabilities of the item.

5327	design failure

F:	défaillance de conception

S:	fallo de diseño

A failure due to a design defect.

5328	manufacturing failure

F:	défaillance de fabrication

S:	fallo de fabricación

A failure due to a manufacturing defect.

5329	ageing failure; wearout failure

F:	défaillance par vieillissement; défaillance par usure

S:	fallo por envejecimiento; fallo por desgaste

A failure whose probability of occurrence increases with the passage of time, as a result of processes inherent in the item.

5330	sudden failure

F:	défaillance soudaine

S:	fallo repentino

A failure that could not be anticipated by prior examination or monitoring.

5331	gradual failure; degradation failure; drift failure

F:	défaillance progressive; dégradation; défaillance par dérive

S:	fallo gradual; fallo por degradación; fallo por deriva

A failure due to a gradual change in time of given characteristics of an item and that could be anticipated by prior examination or monitoring.

NOTE – A gradual failure can sometimes be avoided by preventive maintenance.

5332	cataleptic failure; catastrophic failure (deprecated)

F:	défaillance cataleptique

S:	fallo cataléptico; fallo catastrófico (desaconsejado)

A sudden failure which results in a complete fault.

5333	relevant failure

F:	défaillance pertinente; défaillance à prendre en compte

S:	fallo pertinente; fallo relevante

A failure to be included in interpreting test or operational results or in calculating the value of a reliability performance measure.

NOTE – The criteria for the inclusion should be stated.

5334	non-relevant failure

F:	défaillance non pertinente; défaillance à ne pas prendre en compte

S:	fallo no pertinente; fallo irrelevante

A failure to be excluded in interpreting test or operational results or in calculating the value of a reliability performance measure.

NOTE – The criteria for the exclusion should be stated.

5335	primary failure

F:	défaillance primaire

S:	fallo primario

A failure of an item, not caused either directly or indirectly by the failure or the fault of another item.

5336	secondary failure

F:	défaillance secondaire

S:	fallo secundario

A failure of an item, caused either directly or indirectly by the failure or the fault of another item.

5337	failure cause

F:	cause de défaillance

S:	causa de fallo

The circumstances during design, manufacture or use which have led to a failure.

5338	failure mechanism

F:	mécanisme de défaillance

S:	mecanismo de fallo

The physical, chemical or other process which has led to a failure.

5339	systematic failure; reproducible failure; deterministic failure

F:	défaillance systématique; défaillance reproductible

S:	fallo sistemático; fallo reproducible; fallo determinístico

A failure related in a deterministic way to a certain cause, which can only be eliminated by a modification of the design or manufacturing process, operational procedures, documentation or other relevant factors.

NOTES

1	Corrective maintenance without modification will usually not eliminate the failure cause.

2	A systematic failure can be induced at will by simulating the failure cause.

### 5.3.3	Faults

5341	fault

F:	panne; dérangement

S:	avería

The inability of an item to perform a required function, excluding that inability due to preventive maintenance, lack of external resources or planned actions.

NOTE – A fault is often the result of a failure of the item itself, but may exist without prior failure.

5342	critical fault

F:	panne critique

S:	avería crítica

A fault which is assessed likely to result in injury to persons or significant damage to material.

5343	non-critical fault

F:	panne non critique

S:	avería no crítica

A fault, other than a critical fault.

5344	major fault

F:	panne majeure

S:	avería mayor

A fault which affects a function considered to be of major importance.

5345	minor fault

F:	panne mineure

S:	avería menor

A fault other than a major fault.

5346	misuse fault

F:	panne par mauvaise utilisation

S:	avería por uso incorrecto

A fault due to induced stresses during use which are beyond the stated capabilities of the item.

5347	mishandling fault

F:	panne par fausse manoeuvre

S:	avería por manejo incorrecto

A fault caused by incorrect handling or lack of care of the item.

5348	(inherent) weakness fault

F:	panne par fragilité (inhérente)

S:	avería por fragilidad (inherente)

A fault due to a weakness inherent in the item itself when subjected to stresses within the stated capabilities of the item.

5349	design fault

F:	panne de conception

S:	avería de diseño

A fault due to a design defect.

5350	manufacturing fault

F:	panne de fabrication

S:	avería de fabricación

A fault due to a manufacturing defect.

5351	ageing fault; wearout fault

F:	panne par vieillissement; panne par usure

S:	avería por envejecimiento; avería por desgaste

A fault resulting from an ageing failure.

5352	programme-sensitive fault

F:	panne dépendante du programme

S:	avería dependiente del programa

A fault that is revealed as a result of the execution of some particular sequence of instructions

5353	data-sensitive fault

F:	panne dépendante des données

S:	avería dependiente de los datos

A fault that is revealed as a result of the processing of a particular pattern of data.

5354	complete fault; function preventing fault

F:	panne complète

S:	avería completa

A fault characterized by complete inability to perform all required functions of an item.

NOTE – The criteria for a complete fault have to be stated.

5355	partial fault

F:	panne partielle

S:	avería parcial

A fault of an item other than a complete fault.

5356	persistent fault; permanent fault; solid fault

F:	panne permanente

S:	avería permanente

A fault of an item that persists until an action of corrective maintenance is performed.

5357	intermittent fault; volatile fault; transient fault

F:	panne intermittente; panne temporaire

S:	avería intermitente; avería transitoria

A fault of an item which persists for a limited time duration following which the item recovers the ability to perform a required function without being subjected to any action of corrective maintenance.

NOTE – Such a fault is often recurrent.

5358	determinate fault

F:	panne franche

S:	avería clara; avería determinable

For an item, which produces a response as a result of an action, a fault for which the response is the same for all actions.

5359	indeterminate fault

F:	panne indéterminée

S:	avería indeterminable

For an item, which produces a response as a result of an action, a fault such that the error affecting the response depends on the action applied.

NOTE – An example would be a data-sensitive fault.

5360	latent fault

F:	panne latente

S:	avería latente

An existing fault that has not yet been recognized.

5361	systematic fault

F:	panne systématique

S:	avería sistemática

A fault resulting from a systematic failure.

5362	fault mode; failure mode (deprecated)

F:	mode de panne; mode de défaillance (terme déconseillé)

S:	modo de avería; modo de fallo (desaconsejado)

One of the possible states of a faulty item, for a given required function.

5363	faulty

F:	en panne

S:	averiado

Property of having a fault.

### 5.3.4	Errors and mistakesXE "Errors and mistakes"§

5371	error

F:	erreur

S:	error

A discrepancy between a computed, observed or measured value or condition and the true, specified or theoretically correct value or condition.

NOTE – An error can be caused by a faulty item, e.g. a computing error made by a faulty computer equipment.

5372	execution error; generated error

F:	erreur d'exécution

S:	error de ejecución

Error produced during the operation of a faulty item.

5373	interaction error (man-machine)

F:	erreur d'interaction (homme-machine)

S:	error de interacción (hombre-máquina)

An error in the response of an item caused by a mistake during its use.

5374	propagated error

F:	erreur propagée

S:	error propagado

An error in the response to erroneous data input to a non-faulty item.

5375	mistake; error (deprecated in this sense)

F:	erreur (humaine); faute

S:	equivocación; error (desaconsejado en este sentido)

A human action that produces an unintended result.

### 5.3.5	Item related statesXE "Item related states"§

(See also Figure 2.)

FIGURE  2/E.800...[D02] =

5381	operating state

F:	état de fonctionnement

S:	estado de funcionamiento; estado operacional

The state when an item is performing a required function.

5382	non-operating state

F:	état de non-fonctionnement

S:	estado de no funcionamiento

The state when an item is not performing a required function.

5383	standby state

F:	état d'attente; en réserve

S:	estado de espera (en reserva)

A non-operating up state during the required time.

5384	idle state; free state

F:	état libre; état vacant

S:	estado de reposo; estado libre

A non-operating up state during non-required time.

5385	disabled state; outage

F:	état d'incapicité

S:	estado de incapacidad

A state of an item characterized by its inability to perform a required function, for any reason.

5386	external disabled state

F:	état d'incapacité externe

S:	estado de incapacidad externa

That subset of the disabled state when the item is in an up state, but lacks required external resources.

5387	down state; internal disabled state

F:	état d'indisponibilité; état d'incapacité interne

S:	estado de indisponibilidad; estado de incapacidad interna

A state of an item characterized by a fault or by a possible inability to perform a required function during preventive maintenance.

NOTE – This state relates to availability performance.

5388	up state

F:	état de disponibilité

S:	estado de disponibilidad

A state of an item characterized by the fact that it can perform a required function, assuming that the external resources, if required, are provided.

NOTE – This state relates to availability performance.

5389	busy state

F:	état occupé; occupation

S:	estado de ocupación; estado de ocupado

The state of an item in which it performs a required function for a user and for that reason is not accessible by other users.

5390	critical state

F:	état critique

S:	estado crítico

A state of an item assessed likely to result in injury to persons or significant material damage.

NOTE – A critical state may be the result of a critical fault, but not necessarily.

## 5.4	Statistical conceptsXE "Statistical concepts"§

The quantitative applications of measures for quality of service, network performance, dependability and trafficability performance require a fundamental set of statistical concepts.

This subclause provides the terms and definitions for such applications

5401	characteristic

F:	caractère (statistique)

S:	característica

A property which helps to differentiate between the individuals of a given population.

NOTE – The differentiation may be either quantitative (by variables) or qualitative (by attributes).

5402	probability

F:	probabilité

S:	probabilidad

For practical reasons, it may be considered that, whenever the conditions of a test can be reproduced, the probability Pr(E) of an event E occurring is the value around which the occurrence frequency of the latter oscillates and towards which it tends when the number of tests are indefinitely increased.

NOTE – The concept of probability may be introduced in either of two forms, depending on whether it is intended to designate a degree of belief or whether it is considered as the limit value of a frequency. In both cases, its introduction requires that some precautions be taken which cannot be developed within the context of an International Standard and for which users should refer to specialized literature.

5403	random variable; variate

F:	variable aléatoire

S:	variable aleatoria

A variable which may take any of the values of a specified set of values and with which is associated a probability distribution.

NOTE – A random variable which may take only isolated values is said to be “discrete”. A random variable which may take all the values of a finite or infinite interval is said to be “continuous”.

5404	random process

F:	processus aléatoire; processus stochastique

S:	proceso aleatorio; proceso estocástico

A collection of time-dependent random variables where the values are governed by a given set of multivariate distributions for all combinations of the random variables.

5405	distribution function

F:	fonction de répartition

S:	función de distribución

A function giving, for every value x, the probability that the random variable X is less than or equal to x:

F(x)  =  Pr (X  £  x)

5406	probability density function

F:	densité de probabilité

S:	función densidad de probabilidad

The derivative, if this exists, of the distribution function:

5407	p-fractile; p-quantile (of a probability distribution)

F:	quantile d'ordre p; quantile-p (d'une loi de probabilité)

S:	cuantil-p; cuantil de orden p (de una ley de distribución de probabilidades)

If p is a number between 0 and 1, the p–fractile is the value of the random variable for which the distribution function equals p or “jumps” from a value less than or equal to p to a value greater than p.

NOTE – It is possible that the distribution function equals p throughout the interval between consecutive possible values of the variate. In this case, any value in this interval may be considered as the p-fractile.

5408	expectation (of a random variable); mean (of a random variable)

F:	espérance mathématique (d'une variable aléatoire); moyenne (d'une variable aléatoire)

S:	esperanza matemática (de una variable aleatoria); media (de una variable aleatoria)

a)	For a discrete random variable X taking the values xi with the probabilites pi,

the sum being extended over all the values xi which can be taken by X.

b)	For a continuous random variable X having the probability density function f(x),

the integral being extended over all values of the interval of variation of X.

NOTES

1	No distinction is made between the expectation of a random variable and that of a probability distribution.

2	The term mean is also used with other meanings, for example as the normalized integral over a time interval.

5409	variance (of a random variable)

F:	variance (d'une variable aléatoire)

S:	varianza (de una variable aleatoria)

The expectation of the square of the difference between a random variable and the expectation of this variable.

5410	standard deviation, d (symbol)

F:	écart–type, d (symbole)

S:	desviación típica, d (símbolo)

The positive square root of the variance.

5411	observed value (in statistics)

F:	valeur observée (en statistique)

S:	valor observado (en estadística)

The value of a characteristic determined as the result of an observation or test.

5412	relative frequency

F:	fréquence (statistique)

S:	frecuencia relativa

The ratio of the number of times a particular value, or a value falling within a given class, is observed to the total number of observations.

5413	statistical test

F:	test statistique

S:	prueba estadística

A procedure that is intended to decide whether a hypothesis about the distribution of one or more populations should be rejected or not rejected (accepted).

NOTES

1	The decision taken is a result of the value of an appropriate statistic or statistics, calculated from values observed in samples taken from the populations under consideration. As the value of the statistic is subject to random variations, there is some risk of error when the decision is taken.

2	It is important to note that, generally speaking, a test assumes a priori that certain assumptions are fulfilled (for example, assumption of independence of the observations, assumption of normality, etc.). These assumptions serve as a basis of the test.

5414	one-sided test

F:	test unilatéral

S:	prueba unilateral

A statistical test in which the statistic used is one-dimensional and the critical region is the set of values lower than, or the set of values greater than, a given number.

5415	two-sided test

F:	test bilatéral

S:	prueba bilateral

A statistical test in which the statistic used is one–dimensional and in which the critical region is the set of values lower than a first given number and the set of values greater than a second given number.

5416	null hypothesis, H0 (symbol)

F:	hypothèse nulle, H0 (symbole)

S:	hipótesis nula, H0 (símbolo)

The hypothesis to be rejected or not rejected (accepted) at the outcome of the statistical test.

5417	alternative hypothesis, H1 (symbol)

F:	hypothèse alternative, H1 (symbole)

S:	hipótesis alternativa, H1 (símbolo)

The hypothesis, usually composite, which is opposed to the null hypothesis.

5418	critical region

F:	région critique

S:	región crítica

The set of possible values of the statistic used such that, if the value of the statistic which results from the observed values belongs to the set, the null hypothesis will be rejected, whereas it will not be rejected (accepted) if the opposite is the case.

5419	critical values

F:	valeurs critiques

S:	valores críticos

The given value(s) which limit the critical region.

5420	error of the first kind

F:	erreur de première espèce

S:	error de primera clase

The error committed in rejecting the null hypothesis, because the statistic takes a value which belongs to the critical region, when the null hypothesis is true.

5421	type I risk

F:	risque de première espèce

S:	riesgo de tipo I

The probability of committing the error of the first kind, which varies according to the real situation (within the framework of the null hypothesis). Its maximum value is the significance level of the statistical test.

5422	error of the second kind

F:	erreur de seconde espèce

S:	error de segunda clase

The error committed in failing to reject (accept) the null hypothesis (because the value of the statistic does not belong to the critical region), when the null hypothesis is not true (the alternative hypothesis therefore being true).

5423	type II risk

F:	risque de seconde espèce

S:	riesgo de tipo II

The probability, designated b, of committing the error of the second kind. Its value depends on the real situation and can only be calculated if the alternative hypothesis is adequately specified.

5424	operating characteristic curve; OC curve (for a statistical test plan)

F:	courbe d'efficacité (d'un plan de test)

S:	curva característica de funcionamiento (para un plan de prueba estadística)

A curve showing, for a given statistical test plan, the probability of acceptance as a function of the actual value of a given measure.

5425	producer’s risk (point)

F:	(point du) risque du fournisseur

S:	(punto de) riesgo del proveedor

A point on the operating characteristic curve corresponding to some predetermined and usually low probability of rejection.

5426	consumer's risk (point)

F:	(point du) risque du client

S:	(punto de) riesgo del consumidor

A point on the operating characteristic curve corresponding to a predetermined and usually low probability of acceptance.

5427	power of the test

F:	puissance du test

S:	potencia de la prueba

The probability of not committing the error of the second kind, equal to 1 – a, and thus the probability of rejecting the null hypothesis when this hypothesis is false.

5428	significance level (of a statistical test), a (symbol)

F:	niveau de signification (d'un test statistique); seuil de signification, a (symbole)

S:	nivel de significación (de una prueba estadística); umbral de significación, a (símbolo)

The given value which limits the probability of the null hypothesis being rejected, if the null hypothesis is true.

NOTE – The critical region is determined in such a way that if the null hypothesis is true, the probability of this null hypothesis being rejected should be not more than this given value.

5429	probability of acceptance

F:	probabilité d'acceptation

S:	probabilidad de aceptación

The probability that an item will be accepted by a given statistical test plan.

5430	probability of rejection

F:	probabilité de rejet

S:	probabilidad de rechazo

The probability that an item will not be accepted by a given statistical test plan.

5431	confidence interval

F:	intervalle de confiance

S:	intervalo de confianza

The random interval limited by two statistics or by a single statistic, such that the probability that a parameter to be estimated is covered by this interval is equal to a given value.

5432	statistical tolerance interval

F:	intervalle statistique de dispersion

S:	intervalo estadístico de tolerancia

A random interval limited by two statistics or by a single statistic, such that the probability that a fraction of the population, equal to or greater than a given value between 0 and 1, is covered by this interval is equal to a given value 1 – a, where a is the significance level.

5433	confidence limit

F:	limite de confiance

S:	límite de confianza

Each of the limits of a two-sided confidence interval, or the single limit of a one-sided confidence interval.

5434	estimation

F:	estimation (de paramètres)

S:	estimación (de parámetros)

The operation made for the purpose of assigning, from the observed values in a sample, numerical values to the parameters of the distribution chosen as the statistical model of the population from which this sample is taken.

5435	estimate

F:	estimation

S:	estimación

The result of an estimation.

NOTE – This result may be expressed either as a single numerical value (point estimation) or as a confidence interval.

5436	estimator

F:	estimateur

S:	estimador

A statistic intended to estimate a population parameter.

5437	confidence coefficient; confidence level

F:	niveau de confiance

S:	coeficiente de confianza; nivel de confianza

The value of the probability associated with a confidence interval or a statistical tolerance interval.

5438	statistic

F:	statistique

S:	estadístico

A function of the observed values derived from a sample.

5439	acceptable level (of a measure)

F:	niveau acceptable (d'une caractéristique)

S:	nivel aceptable (de una medida)

A level for a measure of a given performance which in a test plan corresponds to a specified but relatively high probability of acceptance.

## 5.5	Maintenance conceptsXE "Maintenance concepts"§

5501	maintenance philosophy

F:	philosophie de maintenance

S:	filosofía de mantenimiento

A system of underlying principles for the organization and execution of the maintenance.

5502	maintenance policy

F:	politique de maintenance

S:	política de mantenimiento

A description of the interrelationship between the maintenance echelons, the indenture levels and the levels of maintenance to be applied for the maintenance of an item.

5503	maintenance

F:	maintenance

S:	mantenimiento

The combination of all technical and corresponding administrative actions, including supervision actions, intended to retain an item in, or restore it to, a state in which it can perform a required function.

5504	preventive maintenance

F:	maintenance préventive; entretien

S:	mantenimiento preventivo

The maintenance carried out at predetermined intervals or according to prescribed criteria and intended to reduce the probability of failure or the degradation of the functioning of an item.

5505	corrective maintenance; repair

F:	maintenance corrective; réparation; dépannage

S:	mantenimiento correctivo; reparación

The maintenance carried out after fault recognition and intended to restore an item to a state in which it can perform a required function.

5506	deferred maintenance

F:	maintenance différée

S:	mantenimiento diferido

Such corrective maintenance which is not immediately initiated after a fault recognition but is delayed in accordance with given maintenance rules.

5507	scheduled maintenance

F:	maintenance programmée; entretien systématique

S:	mantenimiento programado

The preventive maintenance carried out in accordance with an established time schedule.

5508	unscheduled maintenance

F:	maintenance non programmée

S:	mantenimiento no programado

The maintenance carried out, not in accordance with an established time schedule, but, for example, after reception of an indication regarding the state of an item.

5509	on-site maintenance; in situ maintenance; field maintenance

F:	maintenance in situ

S:	mantenimiento local; mantenimiento sobre el terreno

Maintenance performed at the premises where the item is used.

5510	off-site maintenance

F:	maintenance déportée

S:	mantenimiento no local

Maintenance performed at a place different from where the item is used.

NOTE – An example is the repair of a sub-item at a maintenance centre.

5511	remote maintenance

F:	télémaintenance

S:	mantenimiento remoto; telemantenimiento

Maintenance of an item performed without physical access of the personnel to the item.

5512	automatic maintenance

F:	maintenance automatique

S:	mantenimiento automático

Maintenance accomplished without human intervention.

5513	function-affecting maintenance

F:	maintenance affectant les fonctions

S:	mantenimiento que afecta a la función

A maintenance action that affects one or more of the required functions of a maintained item.

NOTE – Function-affecting maintenance is divided into function-preventing maintenance and function-degrading maintenance.

5514	function-preventing maintenance

F:	maintenance-arrêt; maintenance empêchant l'accomplissement des fonctions

S:	mantenimiento con discontinuidad de funciones

A maintenance action that prevents a maintained item from performing a required function by causing complete loss of all the functions.

5515	function-degrading maintenance

F:	maintenance avec dégradation; maintenance dégradant les fonctions

S:	mantenimiento con degradación de funciones

A maintenance action that affects one or more of the required functions of a maintained item, but not to such extent as to cause complete loss of all the functions.

5516	function-permitting maintenance

F:	maintenance en fonctionnement; maintenance en exploitation

S:	mantenimiento sin discontinuidad de funciones

A maintenance action that does not affect any of the required functions of a maintained item.

5517	level of maintenance

F:	niveau de maintenance

S:	nivel de mantenimiento

The maintenance action to be carried out at a specified indenture level.

NOTE – Examples of a maintenance action are replacing a component, a printed circuit board, a subsystem, etc.

5518	maintenance echelon; line of maintenance

F:	échelon de maintenance

S:	escalón de mantenimiento; línea de mantenimiento

The position in an organization where specified levels of maintenance are to be carried out on an item.

NOTES

1	Examples of maintenance echelons are: field, repair shop, manufacturer.

2	The maintenance echelon is characterized by the skill of the personnel, the facilities available, the location, etc.

5519	indenture level (for maintenance)

F:	niveau d'intervention (pour la maintenance)

S:	nivel de intervención (para el mantenimiento)

A level of subdivision of an item from the point of view of a maintenance action.

NOTES

1	Examples of indenture levels could be a subsystem, a circuit board, a component.

2	The indenture level depends on the complexity of the item’s construction, the accessibility to sub-items, skill level of maintenance personnel, test equipment facilities, safety considerations, etc.

5520	elementary maintenance activity

F:	opération élémentaire de maintenance

S:	acción elemental de mantenimiento

The unit of work into which a maintenance activity may be broken down at a given indenture level.

5521	maintenance action; maintenance task

F:	opération de maintenance; tâche de maintenance

S:	acción de mantenimiento; tarea de mantenimiento

A sequence of elementary maintenance activities carried out for a given purpose.

NOTE – Examples are fault diagnosis, fault localization and function check-out or combinations thereof.

5522	supervision

F:	surveillance; supervision

S:	supervisión

Activity, performed either manually or automatically, intended to observe the state of an item.

NOTE – Automatic supervision may be performed internally or externally to the item.

5523	controlled maintenance

F:	maintenance dirigée

S:	mantenimiento dirigido

A method to sustain a desired quality of service by the systematic application of analysis techniques using centralized supervisory facilities and/or sampling to minimize preventive maintenance and to reduce corrective maintenance.

5524	fault recognition

F:	détection (de panne)

S:	detección (de una avería)

The event when a fault is recognized.

5525	fault diagnosis

F:	diagnostic (de panne)

S:	diagnóstico (de una avería)

Actions taken for fault recognition, fault localization and cause identification.

5526	fault localization; fault location (deprecated in this sense)

F:	localisation de panne

S:	localización (de una avería)

Actions taken to identify the faulty sub-item or sub-items at the appropriate indenture level.

5527	fault correction

F:	correction (de panne)

S:	corrección (de una avería)

Actions taken after fault localization intended to restore the ability of the faulty item to perform a required function.

5528	function check-out

F:	vérification (de fonctionnement)

S:	verificación de funcionamiento

Actions taken after fault correction to verify that the item has recovered its ability to perform the required function.

5529	restoration; recovery

F:	rétablissement

S:	restablecimiento; restauración

That event when the item regains the ability to perform a required function after a fault.

5530	maintenance entity

F:	cellule de maintenance

S:	célula de mantenimiento; entidad de mantenimiento

A sub-item of a given item defined with the intention that an alarm – caused by a fault in that sub-item — will be unambiguously referrable to the sub-item.

## 5.6	Time conceptsXE "Time concepts"§

See also Figure 3.

### 5.6.1	Maintenance related timesXE "Maintenance related times"§

5601	maintenance time

F:	temps de maintenance

S:	tiempo de mantenimiento

The time interval during which a maintenance action is performed on an item either manually or automatically, including technical delays and logistic delays.

NOTE – Maintenance may be carried out while the item is performing a required function.

5602	maintenance man-hours (MMH)

F:	durée équivalente de maintenance (MMH)

S:	duración equivalente de mantenimiento; horas-hombre de mantenimiento (MMH)

The accumulated durations of the maintenance times, expressed in hours, used by all maintenance personnel for a given type of maintenance action or over a given time interval.

5603	active maintenance time

F:	temps de maintenance active

S:	tiempo de mantenimiento activo

That part of the maintenance time during which a maintenance action is performed on an item, either automatically or manually, excluding logistic delays.

NOTE – Active maintenance may be carried out while the item is performing a required function.

5604	preventive maintenance time

F:	temps de maintenance préventive

S:	tiempo de mantenimiento preventivo

That part of the maintenance time during which preventive maintenance is performed on an item, including technical delays and logistic delays inherent in preventive maintenance.

5605	repair time; corrective maintenance time

F:	temps de réparation; temps de maintenance corrective

S:	tiempo de reparación; tiempo de mantenimiento correctivo

That part of the maintenance time during which corrective maintenance is performed on an item, including technical delays and logistic delays inherent in corrective maintenance.

5606	active preventive maintenance time

F:	temps de maintenance préventive active

S:	tiempo de mantenimiento preventivo activo

That part of the active maintenance time during which actions of preventive maintenance are performed on an item.

5607	active repair time; active corrective maintenance time

F:	temps de réparation active; temps de maintenance corrective active

S:	tiempo de reparación activo; tiempo de mantenimiento correctivo activo

That part of the active maintenance time during which actions of corrective maintenance are performed on an item.

5608	undetected fault time

F:	temps de non-détection de panne

S:	tiempo de no detección de una avería

The time interval between a failure and recognition of the resulting fault.

5609	administrative delay (for corrective maintenance)

F:	délai administratif (pour la maintenance corrective)

S:	retardo administrativo (para el mantenimiento correctivo); demora administrativa)

The accumulated time during which an action of corrective maintenance on a faulty item is not performed due to administrative reasons.

5610	logistic delay

F:	délai logistique

S:	retardo logístico; demora logística

That accumulated time during which a maintenance action cannot be performed due to the necessity to acquire maintenance resources, excluding any administrative delay.

NOTE – Logistic delays can be due to, e.g. travelling to unattended installations, awaiting the arrival of spare parts, specialists or test equipment.

5611	fault correction time

F:	temps de correction de panne

S:	tiempo de corrección de una avería

That part of active repair time during which fault correction is performed.

5612	technical delay

F:	délai technique

S:	retardo técnico; demora técnica

The accumulated time necessary to perform auxiliary technical actions associated with the maintenance action itself.

5613	check-out time

F:	temps de vérification du fonctionnement

S:	tiempo de verificación (de funcionamiento)

That part of active repair time during which function check-out is performed.

5614	fault localization time; fault location time (deprecated)

F:	temps de localisation (de panne)

S:	tiempo de localización de una avería

That part of active repair time during which fault localization is performed.

### 5.6.2	Item-state related timesXE "Item-state related times"§

5621	operating time

F:	temps de fonctionnement

S:	tiempo de funcionamiento

The time interval during which an item is an operating state.

5622	non-operating time

F:	temps de non-fonctionnement

S:	tiempo de no funcionamiento

The time interval during which an item is in a non-operating state.

5623	required time

F:	période requise

S:	periodo requerido

The time interval during which the user requires the item to be in a condition to perform a required function.

5624	non-required time

F:	période non requise

S:	periodo no requerido

The time interval during which the user does not require the item to be in a condition to perform a required function.

5625	standby time

F:	période d'attente; période de réserve

S:	tiempo de espera (en reserva)

The time interval during which an item is in a standby state.

5626	idle time; free time

F:	temps libre; période vacante; temps mort

S:	tiempo de reposo; tiempo muerto; tiempo libre

The time interval during which an item is in a free state.

5627	disabled time

F:	temps d'incapacité

S:	tiempo de incapacidad

The time interval during which an item is in a disabled state.

5628	down time

F:	temps d'indisponibilité

S:	tiempo de indisponibilidad

The time interval during which an item is in a down state.

5629	accumulated down time

F:	durée cumulée d'indisponibilité

S:	tiempo de indisponibilidad acumulado

The sum of the duration of down times over a given time interval.

5630	external disabled time; external loss time

F:	temps d'incapacité externe

S:	tiempo de incapacidad externa

The time interval during which an item is in an external disabled state.

5631	up time

F:	temps de disponibilité; temps de bon fonctionnement

S:	tiempo de disponibilidad

The time interval during which an item is in an up state.

### 5.6.3	Time concepts related to reliability performanceXE "Time concepts related to reliability performance"§

5641	time to first failure

F:	durée de fonctionnement avant la première défaillance

S:	tiempo hasta el primer fallo

Total time duration of the operating time of an item from the instant of time it is first put in an up state, until failure.

5642	time to failure

F:	durée de fonctionnement avant défaillance

S:	tiempo hasta el fallo

Total time duration of the operating time of an item, from the instant of time it goes from a down state to an up state, after a corrective maintenance action, until the next failure.

5643	time between failures

F:	temps entre défaillances

S:	tiempo entre fallos

The time duration between two successive failures of a repaired item.

NOTES

1	Those parts of non-operating time which are included must be identified.

2	In some applications only the up time is considered.

5644	time to restoration; time to recovery

F:	temps de panne

S:	tiempo de avería

The time interval during which an item is in a down state due to a failure.

5645	useful life

F:	(durée de) vie utile

S:	vida útil

Under given conditions, the time interval beginning at a given instant of time, and ending when the failure intensity becomes unacceptable or when the item is considered unrepairable as a result of a fault.

5646	early failure period

F:	période initiale de défaillance

S:	periodo de fallos inicial

That possible early period in the life of an item, beginning at a given instant of time and during which the instantaneous failure intensity for a repaired item or the instantaneous failure rate for a non-repaired item decreases rapidly.

NOTE – In any particular case, it is necessary to explain what is meant by “decreases rapidly”.

5647	constant failure intensity period

F:	période d'intensité constante de défaillance

S:	periodo de intensidad de fallos constante

That possible period in the life of a repaired item during which the failure intensity is approximately constant.

NOTE – In any particular case it is necessary to explain what is meant by “approximately constant”.

5648	constant failure rate period

F:	période de densité constante de défaillance; période de taux constant de défaillance

S:	periodo de tasa de fallos constante

That possible period in the life of a non–repaired item during which the failure rate is approximately constant.

NOTE – In any particular case it is necessary to explain what is meant by “approximately constant”.

5649	wear-out failure period

F:	période de défaillance par vieillissement; période de défaillance par usure

S:	periodo de fallos por envejecimiento

That possible later period in the life of an item during which the instantaneous failure intensity for a repaired item or the instantaneous failure rate for a non–repaired item increases rapidly.

NOTE – In any particular case it is necessary to explain what is meant by “increases rapidly”.

## 5.7	Test, data, design and analysis conceptsXE "Test, data, design and analysis concepts"§

### 5.7.1	Test conceptsXE "Test concepts"§

5701	test

F:	essai

S:	prueba

An experiment made in order to measure or classify a characteristic.

5702	compliance test

F:	essai de conformité

S:	prueba de conformidad

A test used to show whether or not a characteristic of an item complies with the stated requirements.

5703	determination test

F:	essai de détermination

S:	prueba de determinación

A test used to establish the value of a characteristic.

5704	laboratory test

F:	essai en laboratoire

S:	prueba de laboratorio

A compliance test or a determination test made under prescribed and controlled conditions which may or may not simulate field conditions.

5705	field test

F:	essai en exploitation

S:	prueba en condiciones de explotación; prueba en condiciones reales

A compliance test or determination test made in the field where operating, environmental, maintenance and measurement conditions are recorded.

5706	endurance test

F:	essai d'endurance

S:	prueba de resistencia

A test carried out over a time interval to investigate how the properties of an item are affected by the application of stated stresses and by their time duration.

5707	accelerated test

F:	essai accéléré

S:	prueba acelerada

A test in which the applied stress level is chosen to exceed that stated in the reference conditions in order to shorten the time duration required to observe the stress response of the item, or to magnify the responses in a given time duration.

NOTE – To be valid, an accelerated test shall not alter the basic fault modes and failure mechanisms, or their relative prevalence.

5708	step stress test

F:	essai sous contrainte échelonnée

S:	prueba de esfuerzo escalonado

A test consisting of several stress levels applied sequentially for periods of equal time duration to an item, in such a way that during each time interval a stated stress level is applied and the stress level is increased from one time interval to the next.

5709	screening test

F:	essai de sélection

S:	prueba de selección

A test, or combination of tests, intended to remove or detect unsatisfactory items or those likely to exhibit early failures.

5710	time acceleration factor

F:	facteur d'accélération temporelle

S:	factor de aceleración temporal

The ratio between the time durations necessary to obtain the same stated number of failures or degradations in two equal size samples under two different sets of stress conditions involving the same failure mechanisms and fault modes and their relative prevalence.

NOTE – One of the two sets of stress conditions should be a reference set.

5711	maintainability verification

F:	vérification de la maintenabilité

S:	verificación de la mantenibilidad

A procedure applied for the purpose of determining whether the requirements for maintainability performance measures for an item has been achieved or not.

NOTE – The procedures may range from analysis of appropriate data to a maintainability demonstration.

5712	maintainability demonstration

F:	vérification expérimentale de maintenabilité

S:	demostración de la mantenibilidad

A maintainability verification performed as a compliance test.

### 5.7.2	Data conceptsXE "Data concepts"§

5721	observed data

F:	valeur observée; donnée observée

S:	datos observados; valores observados

Values related to an item or a process obtained by direct observation.

NOTE – Values referred to could be events, time instants, time intervals, etc.

5722	test data

F:	données d'essai

S:	datos de prueba

Observed data obtained during tests.

5723	field data

F:	donnée d'exploitation

S:	datos de explotación

Observed data obtained during field operation.

5724	reference data

F:	valeur de référence; données de référence

S:	datos de referencia; valores de referencia

Data, which by general agreement may be used for prediction and/or comparison with observed data.

### 5.7.3	Design conceptsXE "Design concepts"§

5731	redundancy

F:	redondance

S:	redundancia

In an item, the existence of more than one means for performing a required function.

5732	active redundancy

F:	redondance active

S:	redundancia activa

That redundancy wherein all means for performing a required function are intended to operate simultaneously.

5733	standby redundancy

F:	redondance en attente; redondance passive; redondance en secours; redondance en réserve

S:	redundancia pasiva; redundancia de reserva

That redundancy wherein one means for performing a required function is intended to operate, while the alternative means are inoperative until needed.

5734	fail safe

F:	protégé contre défaillances critiques; à sûreté intégrée

S:	prevención de fallos

A designed property of an item which prevents its failures from resulting in critical faults.

5735	fault tolerance

F:	tolérance aux pannes

S:	tolerancia a las averías

The attribute of an item that makes it able to perform a required function in the presence of certain given sub-item faults.

5736	fault masking

F:	masquage de panne

S:	enmascaramiento de avería

The condition in which a fault exists in a sub-item of an item but cannot be recognized because of a feature of the item or because of another fault of the sub-item or of another sub-item.

### 5.7.4	Analysis conceptsXE "Analysis concepts"§

5741	prediction

F:	prévision; prédiction

S:	previsión; predicción

1)	The process of computation used to obtain (a) predicted value(s) of a quantity.

2)	The predicted value(s) of a quantity.

5742	reliability model

F:	modèle de fiabilité

S:	modelo de fiabilidad

A mathematical model used for prediction or estimation of reliability measures of an item or for similar purposes.

5743	fault modes and effects analysis (FMEA)

F:	analyse des modes de panne et de leurs effets (FMEA)

S:	análisis de los modos de avería y de sus efectos (FMEA)

A qualitative method of reliability analysis which involves the study of the fault modes which can exist in every sub-item of the item and the determination of the effects of each fault mode on other sub-items of the item and on the required functions of the item.

5744	fault modes, effects and criticality analysis (FMECA)

F:	analyse des modes de panne, de leurs effets et de leur criticité (FMECA)

S:	análisis de los modos de avería, sus efectos y su criticidad (FMECA)

Fault modes and effect analysis together with a consideration of the probability of occurrence and a ranking of the seriousness of the fault.

5745	fault tree analysis (FTA)

F:	analyse par arbre de panne (FTA)

S:	análisis arborescente de averías (FTA)

An analysis to determine which fault modes of the sub-items or external events, or combinations thereof, may result in a stated fault mode of the item, resulting in a fault tree.

5746	stress analysis

F:	analyse de contraintes

S:	análisis de esfuerzos

A quantitative or qualitative determination of the physical, chemical or other stresses an item is subjected to under given use conditions.

5747	reliability block diagram

F:	diagramme de fiabilité

S:	diagrama de bloques de fiabilidad

Block diagram showing, for one or more functional modes of a complex item, how faults of the sub-items represented by the blocks, or combinations thereof, result in a fault of the item.

5748	fault tree

F:	arbre de panne

S:	árbol de averías

A logic diagram showing which fault modes of sub-items or external events, or combinations thereof, result in a given fault mode of the item.

5749	state-transition diagram

F:	diagramme de transition d'états

S:	diagrama de transición de estados

A diagram showing the set of possible states of an item and the possible one step transitions between these states.

5750	stress model

F:	modèle de contraintes

S:	modelo de esfuerzos

A mathematical model which describes how a reliability performance measure of an item varies as a function of the applied stresses.

5751	fault analysis

F:	analyse des pannes

S:	análisis de averías

The logical, systematic examination of an item or its diagram(s) to identify and analyse the probability, causes and consequences of potential and real faults.

5752	maintainability model

F:	modèle de maintenabilité

S:	modelo de mantenibilidad

A mathematical model used for prediction or estimation of maintainability performance measures of an item or for similar purposes.

NOTE – An example is the maintenance tree.

5753	maintainability prediction

F:	prévision de maintenabilité; prédiction de maintenabilité

S:	previsión de la mantenibilidad; predicción de la mantenibilidad

An activity performed with the intention to forecast the numerical values of a maintainability performance measure of an item, taking into account the maintainability performance and reliability performance measures of its sub-items, under given operational and maintenance conditions.

5754	maintenance tree

F:	arbre de maintenance

S:	árbol de mantenimiento

A logic diagram showing the pertinent alternative sequences of elementary maintenance activities to be performed on an item and the conditions for their choice.

5755	maintainability allocation; maintainability apportionment

F:	répartition de la maintenabilité

S:	distribución de la mantenibilidad; asignación de la mantenibilidad

A procedure applied during the design of an item intended to apportion the requirements for maintainability performance measures for an item to its sub-items according to given criteria.

### 5.7.5	Improvement processesXE "Improvement processes"§

5761	learning process

F:	apprentissage

S:	aprendizaje

Growth in experience and familiarity by personnel with design or constructional techniques, which reduces the risk of future mistakes.

5762	burn-in

F:	rodage

S:	rodaje

A process of reliability improvement of hardware, employing operation of every item in a prescribed environment, with successive fault correction, replacement or removal at every failure, during the steeply falling failure intensity period within the early failure period.

5763	reliability growth

F:	croissance de la fiabilité

S:	crecimiento de la fiabilidad; incremento de la fiabilidad

A condition characterized by a progressive improvement of a reliability performance measure of an item, or population of similar items, with time.

NOTE – A growth can result either from active improvement or from burn-in.

5764	reliability improvement

F:	amélioration de fiabilité

S:	mejora de la fiabilidad

A process undertaken with the deliberate intention of promoting reliability growth by the elimination of systematic faults.

5765	maintainability programme

F:	programme de maintenabilité

S:	programa de mantenibilidad

A detailed plan, including the human and material resources, procedures, tasks and responsibilities during the life of an item, intended to determine the fulfilment of the requirements for maintainability performance measures for an item and facilitate the planning of the maintenance.

Annex  A

Related QoS/NP Handbooks and E-Series Recommendations

(This annex forms an integral part of this Recommendation)

# Handbooks

CCITT Manual on Quality of Service, Network Management and Maintenance, ITU, Geneva, 1984.

CCITT Handbook on Quality of Service and Network Performance, ITU, Geneva, 1993.

# Recommendations

E.420	Checking the quality of the international telephone service - General considerations.

E.421	Service quality observations on a statistical basis.

E.422	Observations on international outgoing telephone calls for quality of service.

E.423	Observations on traffic set up by operators.

E.424	Test calls.

E.425	International automatic observations.

E.426	General guidance to the percentage of effective attempts which should be observed for international telephone calls.

E.427	Collection and statistical analysis of special quality of service observation data for measurements of customer difficulties in the international automatic service.

E.428	Connection retention.

E.430	Quality of service framework.

E.431	Service quality assessment for connection set up and release delays.

E.432	Connection quality.

E.433	Billing integrity.

E.434	Subscriber to subscriber measurement of the public switched telephone network.

E.450	Facsimile quality of service on PSTN – General aspects.

E.451	Facsimile call cut-off performance.

E.452	Facsimile modem speed reductions and transaction time.

E.505	Measurements of the performance of common channel signalling networks.

E.525	Designing networks to control grade of service.

E.540	Overall grade of service of the international part of an international connection.

E.541	Overall grade of service for international connections (subscriber to subscriber).

E.543	Grades of service in digital international telephone exchanges.

E.550	Grade of service and new performance criteria under failure.

E.600	Terms and definitions of traffic engineering.

E.720	ISDN grade of service concept.

E.721	Network grade of service parameters in ISDN.

E.723	GOS parameters for Signalling System No. 7 networks.

E.733	Methods for dimensioning resources in Signalling System No. 7 networks.

E.770	Land mobile and fixed network interconnection traffic grade of service concept.

E.771	Network grade of service parameters and target values for circuit-switched quality of service and dependability vocabulary.

E.810	Framework of the Recommendations on the serveability performance and integrity for telecommunication services.

E.820	Call models for serveability and service integrity performance.

E.830	Models for the specification, evaluation and allocation of serveability and service integrity.

E.845	Connection accessibility objective for the international telephone service.

E.846	Accessibility for 64 kbit/s switched international end-to-end ISDN connection types.

E.850	Connections retainability objective for the international telephone service.

E.855	Connection integrity objective for the international telephone service.

E.862	Dependability planning of telecommunication networks.

E.880	Field data collection and evaluation on the performance of equipment, networks and services.

Annex  B

Relations between defect, failure and fault concepts

(This annex forms an integral part of this Recommendation)

TABLE  B.1/E.800

| Defect | Failure | Fault |
| --- | --- | --- |
| Critical defect | Critical failure | Critical fault |
| Non-critical defect | Non-critical failure | Non-critical fault |
| Major defect | – | Major fault |
| Minor defect | – | Minor fault |
| – | Misuse failure | Misuse fault |
| – | Mishandling failure | Mishandling fault |
| – | Inherent weakness failure | Inherent weakness fault |
| Design defect | Design failure | Design fault |
| Manufacturing defect | Manufacturing failure | Manufacturing fault |
| – | Ageing failure | Ageing fault |
| – | Sudden failure | – |
| – | Gradual failure | – |
| – | Cataleptic failure | – |
| – | Relevant failure | – |
| – | Non-relevant failure | – |
| – | Primary failure | – |
| – | Secondary failure | – |
| – | Failure cause | – |
| – | Failure mechanism | – |
| – | – | Programme-sensitive fault |
| – | – | Data-sensitive fault |
| – | – | Complete fault |
| – | – | Partial fault |
| – | – | Persistent fault |
| – | – | Intermittent fault |
| – | – | Fault mode |
| – | – | Determinate fault |
| – | – | Indeterminate fault |
| – | – | Latent fault |
| – | Systematic failure | Systematic fault |
| Bug | – | – |

Annex  C

List of recommended symbols and abbreviations

(This annex forms an integral part of this Recommendation)

a	Significance level

b	Type II risk

l(t)	Instantaneous failure rate

\s\up6(–);l) (t1	Mean failure rate [in time interval (t1, t2)]

m(t)	Instantaneous repair rate

\s\up5(– );m) (t1	Mean repair rate [in time interval (t1, t2)]

d	Standard deviation

A	Asymptotic availability

A(t)	Instantaneous availability

\s\up6( –	Asymptotic mean availability

\s\up6( –);A)(t1	Mean availability [in time interval (t1, t2)]

ASR	Answer seizure ratio

E(X)	Mean (of X)

f(x)	Probability density function

F(x)	Distribution function

FMEA	Fault modes and effect analysis

FMECA	Fault modes, effects and criticality analysis

FTA	Fault tree analysis

H0	Null hypothesis

H1	Alternative hypothesis

MAD	Mean administrative delay

MADT	Mean accumulated down time

MART	Mean active repair time

MDT	Mean down time

MID	Mean interruption duration

MLD	Mean logistic delay

MMH	Maintenance man-hours

MRT	Mean repair time

MTBF	Mean time between failures

MTBI	Mean time between interruptions

MTTF	Mean time to failure

MTTFF	Mean time to first failure

MTTR	Mean time to restoration

MUT	Mean up time

N(t1, t2)	Number of failures [in time interval (t1, t2)]

R	Reliability

U	Asymptotic unavailability

U(t)	Instantaneous unavailability

\s\up6( –	Asymptotic mean unavailability

\s\up6( –);U)(t1 	Mean unavailability [in time interval (t1, t2)]

z(t)	Instantaneous failure intensity

\s\up4(–);z)(t1	Mean failure intensity [in time interval (t1, t2)]

INDEX

Alphabetical list of definitions contained
in this Recommendation

Accelerated test	5707

Acceptable level (of a mesure)	5439

Accessibility of a connection to be established	3317

Accumulated down time	5629

Active corrective maintenance time	5607

Active maintenance time	5603

Active preventive maintenance time	5606

Active redundancy	5732

Active repair time	5607

Administrative delay
(for corrective maintenance)	5609

Ageing failure	5329

Ageing fault	5351

Alternative hypothesis	5417

Asymptotic availability	4215

Asymptotic mean availability	4217

Asymptotic mean unavailability	4218

Asymptotic unavailability	4216

Automatic maintenance	5512

Availability performance	4210

Average ...	5207

Billing error probability	3102

Billing integrity (probability)	3106

Bit error ratio (BER)	4301

Break (of service)	3341

Bug	5302

Burn-in	5762

Busy state	5389

Call abandonment probability	3204

Capability	2307

Cataleptic failure	5332

Catastrophic failure	5332

Characteristic	5401

Charging correctness	4401

Check-out time	5613

Complete fault	5354

Compliance test	5702

Confidence coefficient	5437

Confidence interval	5431

Confidence level	5437

Confidence limit	5433

Connection	2304

Connection accessibility	3314

Connection retainability	3332

Constant failure intensity period	5647

Constant failure rate period	5648

Consumer’s risk (point)	5426

Controlled maintenance	5523

Corrective maintenance	5505

Corrective maintenance time	5605

Critical defect	5303

Critical defective item	5308

Critical failure	5322

Critical fault	5342

Critical region	5418

Critical state	5390

Critical values	5419

Cut-off probability	3334

Data-sensitive fault	5353

Defect	5301

Defective	5307

Defective item	5307

Deferred maintenance	5506

Degradation failure	5331

Dependability	4200

Design defect	5311

Design failure	5327

Design fault	5349

Determinate fault	5358

Determination test	5703

Deterministic failure	5339

Dialling mistake probability	3202

Disabled state	5385

Disabled time	5627

Distribution function	5405

Down state	5387

Down time	5628

Drift failure	5331

Durability	5102

Early failure period	5646

Effectiveness (performance)	5101

Element	2302

Elementary maintenance activity	5520

Endurance test	5706

Entity	2302

Error	5371

Error free seconds (EFS)	4302

Error of the first kind	5420

Error of the second kind	5422

Estimate	5435

Estimated ...	5204

Estimation	5434

Estimator	5436

Execution error	5372

Expectation (of a random variable)	5408

External disabled state	5386

External disabled time	5630

External loss time	5630

Extrapolated ...	5203

Fail safe	5734

Failure	5321

Failure cause	5337

Failure intensity acceleration factor	4240

Failure mechanism	5338

Failure mode	5362

Failure rate acceleration factor	4239

Fault	5341

Fault analysis	5751

Fault correction	5527

Fault correction time	5611

Fault coverage	4261

Fault diagnosis	5525

Fault localization	5526

Fault localization time	5614

Fault location	5526

Fault location time	5614

Fault masking	5736

Fault mode	5362

Fault modes and effects analysis (FMEA)	5743

Fault modes, effects and critically
analysis (FMECA)	5744

Fault recognition	5524

Fault tolerance	5735

Fault tree	5748

Fault tree analysis (FTA)	5745

Faulty	5363

Field data	5723

Field maintenance	5509

Field test	5705

Free state	5384

Free time	5626

Function check-out	5528

Function preventing fault	5354

Function-affecting maintenance	5513

Function-degrading maintenance	5515

Function-permitting maintenance	5516

Function-preventing maintenance	5514

Generated error	5372

Gradual failure	5331

Idle state	5384

Idle time	5626

Imperfection	5306

In situ maintenance	5509

Incorrect charging or accounting probability	3103

Indenture level (for maintenance)	5519

Indeterminate fault	5359

Inherent ...	5205

Inherent weakness failure	5326

Inherent weakness fault	5348

Instantaneous ...	5209

Instantaneous availability	4211

Instantaneous availability of a leased circuit	4221

Instantaneous failure intensity	4234

Instantaneous failure rate	4232

Instantaneous repair rate	4252

Instantaneous unavailability	4212

Interaction error (man-machine)	5373

Intermittent fault	5357

Internal disabled state	5387

Interruption	3341

Interruption duration	3343

Intrinsic ...	5205

Item	2302

Laboratory test	5704

Latent fault	5360

Learning process	5761

Level of maintenance	5517

Line of maintenance	5518

Logistic delay	5610

Maintainability	4251

Maintainability allocation	5755

Maintainability apportionment	5755

Maintainability demonstration	5712

Maintainability model	5752

Maintainability performance	4250

Maintainability prediction	5753

Maintainability programm	5765

Maintainability verification	5711

Maintenance	5503

Maintenance action	5521

Maintenance echelon	5518

Maintenance entity (ME)	5530

Maintenance man-hours (MMH)	5602

Maintenance philosophy	5501

Maintenance policy	5502

Maintenance support performance	4270

Maintenance task	5521

Maintenance time	5601

Maintenance tree	5754

Major defect	5305

Major defective item	5309

Major fault	5344

Manufacturing defect	5312

Manufacturing failure	5328

Manufacturing fault	5350

Mean (of a random variable)	5408

Mean ...	5207

Mean access delay	3315

Mean accumulated down time (MADT)	4220

Mean active repair time (MART)	4258

Mean administrative delay (MAD)	4271

Mean availability	4213

Mean down time (MDT)	4255

Mean failure intensity	4235

Mean failure rate	4233

Mean interruption duration (MID)	3345

Mean logistic delay (MLD)	4273

Mean maintenance man-hours	4254

Mean repair rate	4253

Mean repair time (MRT)	4256

Mean service access delay	3312

Mean service provisioning time	3101

Mean time between failures (MTBF)	4238

Mean time between interruptions (MTBI)	3344

Mean time to failure (MTTF)	4237

Mean time to first failure (MTTFF)	4236

Mean time to recovery	4260

Mean time to repair	4260

Mean time to restoration (MTTR)	4260

Mean unavailability	4214

Mean up time (MUT)	4219

Minor defect	5306

Minor defective item	5310

Minor fault	5345

Mishandling failure	5325

Mishandling fault	5347

Misrouting probability	3320

Mistake	5375

Misuse failure	5324

Misuse fault	5346

Modification (of an item)	5103

Network accessibility	3313

Network performance	2201

No tone probability	3319

Non-critical defect	5304

Non-critical failure	5323

Non-critical fault	5343

Non-operating state	5382

Non-operating time	5622

Non-relevant failure	5334

Non-required time	5624

Null hypothesis	5416

Observed data	5721

Observed value (in statistics)	5411

OC curve (for a statistical test plan)	5424

Off-site maintenance	5510

On-site maintenance	5509

One-sided test	5414

Operating characteristic curve	5424

Operating state	5381

Operating time	5621

Operational ...	5206

Outage	5385

Overcharging probability	3105

p-fractile	5407

p-fractile ...	5208

p-fractile access delay	3316

p-fractile active repair time	4259

p-fractile administrative delay	4272

p-fractile logistic delay	4274

p-fractile repair time	4257

p-quantile (of a probability distribution)	5407

Partial fault	5355

Permanent fault	5356

Persistent fault	5356

Pointwise availability	4211

Pointwise unavailability	4212

Power of the test	5427

Predicted ...	5202

Prediction	5741

Premature release probability	3334

Preventive maintenance	5504

Preventive maintenance time	5604

Primary failure	5335

Probability	5402

Probability density function (PDF)	5406

Probability of acceptance	5429

Probability of rejection	5430

Probability of successful service completion	3336

Producer’s risk (point)	5425

Programme-sensitive fault	5352

Propagated error	5374

Propagation performance	4303

Quality of service	2101

Random process	5404

Random variable	5403

Recovery	5529

Redundancy	5731

Reference data	5724

Relative frequency	5412

Release failure probability	3335

Relevant failure	5333

Reliability	4231

Reliability block diagram	5747

Reliability growth	5763

Reliability improvement	5764

Reliability model	5742

Reliability performance	4230

Remote maintenance	5511

Repair	5505

Repair coverage	4262

Repair time	5605

Reproducible failure	5339

Required time	5623

Resource	2306

Restoration	5529

Retainability of an established connection	3333

Scheduled maintenance	5507

Screening test	5709

Secondary failure	5336

Serveability performance	3300

Service	2301

Service access probability	3311

Service accessibility	3311

Service accessibility performance	3310

Service integrity performance	3340

Service operability performance	3200

Service retainability	3331

Service retainability performance	3330

Service security performance	3400

Service support performance	3100

Service user abandonment probability	3203

Service user mistake probability	3201

Significance level (of a statistical test)	5428

Solid fault	5356

Standby time	5625

Standard deviation	5410

Standby redundancy	5733

Standby state	5383

State-transition diagram	5749

Statistic	5438

Statistical test	5413

Statistical tolerance interval	5432

Steady-state availability	4215

Step stress test	5708

Stress analysis	5746

Stress model	5750

Sudden failure	5330

Supervision	5522

Systematic failure	5339

Systematic fault	5361

Technical delay	5612

Telecommunications traffic	2305

Teletraffic	2305

Test	5701

Test data	5722

Time acceleration factor	5710

Time between failures	5643

Time between interruptions	3342

Time to failure	5642

Time to first failure	5641

Time to recovery	5644

Time to restoration	5644

Trafficability performance	4100

Transient fault	5357

Transmission performance	4300

True ...	5201

Two-sided test	5415

Type I risk	5421

Type II risk	5423

Unacceptable transmission probability	3318

Undercharging probability	3104

Undetected fault time	5608

Unscheduled maintenance	5508

Up state	5388

up time	5631

Useful life	5645

User	2303

Variance (of a random variable)	5409

Variate	5403

Volatile fault	5357

Wear-out failure period	5649

Wearout failure	5329

Wearout fault	5351

1	)	The service support performance may depend on certain aspects of the network performance, for example through the charging correctness performance.