|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	F.64

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

OPERATIONS  AND  QUALITY  OF  SERVICE

TELEGRAPH  SERVICES

DETERMINATION  OF  THE  NUMBER  
OF  INTERNATIONAL  TELEX  CIRCUITS  REQUIRED  TO  CARRY  A  GIVEN  VOLUME  
OF  TRAFFIC

ITU-T  Recommendation  F.64

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation F.64 was published in Fascicle II.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation F.64

Fascicle II.4 - Rec. F.64

DETERMINATION OF THE NUMBER OF INTERNATIONAL TELEX CIRCUITS

REQUIRED TO CARRY A GIVEN VOLUME OF TRAFFIC

The CCITT,

considering

(a)	that it is essential to provide an adequate number of circuits between two telex networks in order to provide the rapid service stipulated in Recommendation F.60;

(b)	that the use of tables for the determination of the number of circuits as a function of the traffic to be dealt with during the busy hour is an established practice in all Administrations, and is a convenient means of indicating a standard;

(c)	that international telex circuits may be selected either at manual positions, or via automatic switching equipment, particularly where subscriber-to-subscriber selection is employed between two networks;

unanimously declares

(1)	that Administrations should use Tables 1/F.64 or 2/F.64 below, according to the system of selection employed (i.e. manual selection or automatic selection) in the international service;

(2)	Administrations should aim for full availability of circuits on intercontinental and ARQ radio routes operated with signalling in accordance with Recommendations U.1, U.11, U.12 and U.20. Where an Administration is unable to provide the full availability, it should provide an availability to achieve not less than 90% of the full availability capacity relative to the number of circuits on the route at a grade of service of one in 50.

1	Introduction

1.1	Table 1/F.64 shows values for manual traffic carried. If for the purpose of design (as distinct from the maintenance of rapid service) it is desired to obtain values for offered traffic in erlangs, these may be determined by adding the respective values of lost traffic to the figures for carried traffic in Table 1/F.64.

1.2	Table 1/F.64 is directly applicable only to full-availability groups of circuits that are operated either wholly as bothway circuits, or wholly as undirectional circuits.

1.3	Tables 2/F.64 shows values for traffic offered in the automatic service and is directly applicable to full-availability groups and groups with availabilities between 10 and 50.

Given the traffic offered (A) in erlangs and the availability (K), the number of circuits required to provide a loss probability (B) of 0.02 may be determined from. Table 2/F.64. Groups of up to 200 circuits and availabilities of 10, 20, 30, 50 and N circuits (N circuits corresponding to full availability) are covered. The method of applying the table is shown in Figure 1/F.64.

1.4	Where groups of circuits are divided into bothway and unidirectional components, the division and number of circuits in each component will be agreed between Administrations.

TABLE 1/F.64

Traffic capacity table for manually selected telex circuits (Note 1)

| Number of circuits | Average intensity for traffic carried in the busy hour, expressed in erlangs, for a grade of service (probability of loss) of: |  |  |
| --- | --- | --- | --- |
|  | 1 in 10 (Note 2) | 1 in 30 (Note 3) | 1 in 50 (Note 3) |
| a | b | c | d |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 | 0.2 0.9 1.5 2.3 3.2 | 0.066 0.43 0.89 1.49 2.17 2.92 3.77 4.66 5.56 6.47 7.39 8.31 9.24 10.2 11.1 12.1 13.0 13.9 14.9 15.9 | 0.034 0.33 0.76 1.29 1.92 2.67 3.44 4.25 5.09 5.93 6.79 7.67 8.57 9.48 10.4 11.3 12.3 13.2 14.1 15.0 |

Note 1 – Table 1/F.64 makes allowance for the manual operator to continue the search for a free line over the group of circuits concerned for a period of 30 seconds if all are engaged, after which the search is abandoned and the call suspended.

Note 2 – Column b of Table 1/F.64 will, in general, only be used in respect of small groups of circuits of considerable length, having due regard to the desire to provide a rapid service, as well as to economic considerations.

Note 3 – In all other cases the figures of column c shall be used in preference to those of column d.

TABLE 2/F.64

Traffic capacity table for automatic circuits

| K |  | 10 | 20 | 30 | 50 | ... N |
| --- | --- | --- | --- | --- | --- | --- |
| N |  |  |  |  |  |  |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 | A | 5.08 5.68 6.29 6.90 7.52 8.15 8.77 9.40 10.0 10.7 11.3 12.0 12.6 13.3 13.9 14.6 15.2 15.9 16.5 17.2 17.8 18.5 19.2 19.8 20.5 21.1 21.8 22.5 23.1 23.8 24.4 25.1 25.8 26.4 27.1 27.8 28.4 29.1 29.8 30.4 31.1 | 13.2 13.9 14.7 15.4 16.2 16.9 17.7 18.4 19.2 19.9 20.7 21.5 22.2 23.0 23.8 24.6 25.3 26.1 26.9 27.7 28.4 29.2 30.0 30.8 31.6 32.3 33.1 33.9 34.7 35.5 36.3 | 21.9 22.7 23.5 24.3 25.1 26.0 26.8 27.6 28.4 29.2 30.0 30.8 31.7 32.5 33.3 34.1 34.9 35.8 36.6 37.4 38.2 | 40.3 | 0.02 0.22 0.60 1.09 1.66 2.28 2.94 3.63 4.34 5.08 5.84 6.61 7.40 8.20 9.01 9.83 10.7 11.5 12.3 13.2 14.0 14.9 15.8 16.6 17.5 18.4 19.3 20.2 21.0 21.9 22.8 23.7 24.6 25.5 26.4 27.3 28.3 29.2 30.1 31.0 31.9 32.8 33.8 34.7 35.6 36.5 37.5 38.4 39.3 40.3 |

TABLE 2/F.64

(cont’d)

| K |  | 10 | 20 | 30 | 50 | ... N |
| --- | --- | --- | --- | --- | --- | --- |
| N |  |  |  |  |  |  |
| 50 | A | 31.1 | 36.3 | 38.2 | 40.3 | 40.3 |
| 51 52 53 54 55 |  | 31.8 32.4 33.1 33.8 34.4 | 37.1 37.9 38.7 39.4 40.2 | 39.1 39.9 40.7 41.6 42.4 | 41.1 42.0 42.8 43.7 44.6 | 41.2 42.1 43.1 44.0 44.9 |
| 56 57 58 59 60 |  | 35.1 35.7 36.4 37.1 37.7 | 41.0 41.8 42.6 43.4 44.2 | 43.2 44.1 44.9 45.7 46.6 | 45.4 46.3 47.2 48.0 48.9 | 45.9 46.8 47.8 48.7 49.6 |
| 61 62 63 64 65 |  | 38.4 39.1 39.7 40.4 41.0 | 45.0 45.8 46.6 47.4 48.2 | 47.4 48.2 49.1 49.9 50.8 | 49.8 50.6 51.5 52.4 53.3 | 50.6 51.5 52.5 53.4 54.4 |
| 66 67 68 69 70 |  | 41.7 42.4 43.0 43.7 44.4 | 49.0 49.8 50.6 51.4 52.2 | 51.6 52.4 53.3 54.1 55.0 | 54.1 55.0 55.9 56.7 57.6 | 55.3 56.3 57.2 58.2 59.1 |
| 71 72 73 74 75 |  | 45.0 45.7 46.3 47.0 47.6 | 53.0 53.8 54.6 55.4 56.2 | 55.8 56.6 57.5 58.3 59.2 | 58.5 59.4 60.2 61.1 62.0 | 60.1 61.0 62.0 62.9 63.9 |
| 76 77 78 79 80 |  | 48.3 49.0 49.6 50.3 50.9 | 57.0 57.8 58.6 59.4 60.2 | 60.0 60.9 61.7 62.6 63.4 | 62.9 63.8 64.6 65.5 66.4 | 64.9 65.8 66.8 67.7 68.7 |
| 81 82 83 84 85 |  | 51.6 52.2 52.9 53.6 54.2 | 61.0 61.8 62.6 63.4 64.2 | 64.3 65.1 66.0 66.8 67.6 | 67.3 68.2 69.0 69.9 70.8 | 69.6 70.6 71.6 72.5 73.5 |
| 86 87 88 89 90 |  | 54.9 55.5 56.2 56.8 57.5 | 65.0 65.9 66.7 67.5 68.3 | 68.5 69.3 70.2 71.0 71.9 | 71.7 72.6 73.5 74.3 75.2 | 74.5 75.4 76.4 77.3 78.3 |
| 91 92 93 94 95 |  | 58.1 58.8 59.4 60.1 60.7 | 69.1 69.9 70.7 71.5 72.3 | 72.7 73.6 74.4 75.3 76.2 | 76.1 77.0 77.9 78.8 79.7 | 79.3 80.2 81.2 82.2 83.1 |
| 96 97 98 99 100 |  | 61.4 62.0 62.7 63.3 64.0 | 73.1 73.9 74.7 75.5 76.3 | 77.0 77.9 78.7 79.6 80.4 | 80.5 81.4 82.3 83.2 84.1 | 84.1 85.1 86.0 87.0 88.0 |

TABLE 2/F.64

(concluded)

| K |  | 10 | 20 | 30 | 50 | ... N |
| --- | --- | --- | --- | --- | --- | --- |
| N |  |  |  |  |  |  |
| 100 | A | 64.0 | 76.3 | 80.4 | 84.1 | 88.0 |
| 102 104 106 108 110 |  | 65.3 66.5 67.8 69.1 70.4 | 77.9 79.6 81.2 82.8 84.4 | 82.1 83.8 85.5 87.3 89.0 | 85.9 87.6 89.4 91.2 93.0 | 89.9 91.9 93.8 95.7 97.7 |
| 112 114 116 118 120 |  | 71.7 72.9 74.2 75.5 76.8 | 86.0 87.6 89.3 90.9 92.5 | 90.7 92.4 94.1 95.8 97.5 | 94.8 96.6 98.3 100.1 101.9 | 99.6 101.6 103.5 105.5 107.4 |
| 122 124 126 128 130 |  | 78.1 79.3 80.6 81.9 83.2 | 94.1 85.7 97.3 99.0 100.6 | 99.3 101.0 102.7 104.4 106.1 | 103.7 105.5 107.3 109.1 110.9 | 109.4 111.3 113.3 115.2 117.2 |
| 132 134 136 138 140 |  | 84.5 85.7 87.0 88.3 89.6 | 102.2 103.8 105.4 107.0 108.7 | 107.9 109.6 111.3 113.0 114.7 | 112.7 114.5 116.3 118.1 119.9 | 119.1 121.1 123.1 125.0 127.0 |
| 142 144 146 148 150 |  | 90.8 92.1 93.4 94.7 96.0 | 110.3 111.9 113.5 115.1 116.7 | 116.5 118.2 119.9 121.6 123.4 | 121.7 123.5 125.3 127.1 128.9 | 128.9 130.9 132.9 134.8 136.8 |
| 152 154 156 158 160 |  | 97.2 98.5 99.8 101.1 102.4 | 118.3 120.0 121.6 123.2 124.8 | 125.1 126.8 128.5 130.3 132.0 | 130.7 132.5 134.3 136.1 137.9 | 138.8 140.7 142.7 144.7 146.6 |
| 162 164 166 168 170 |  | 103.6 104.9 106.2 107.5 108.8 | 126.4 128.0 129.6 131.2 132.9 | 133.7 135.4 137.2 138.9 140.6 | 139.7 141.5 143.3 145.1 146.9 | 148.6 150.6 152.6 154.5 156.5 |
| 172 174 176 178 180 |  | 110.0 111.3 112.6 113.9 115.2 | 134.5 136.1 137.7 139.3 140.9 | 142.3 144.1 145.8 147.5 149.2 | 148.7 150.5 152.3 154.1 155.9 | 158.5 160.4 162.4 164.4 166.4 |
| 182 184 186 188 190 |  | 116.4 117.7 119.0 120.3 121.6 | 142.5 144.1 145.7 147.3 148.9 | 151.0 152.7 154.4 156.2 157.9 | 157.7 159.6 161.4 163.2 165.0 | 168.3 170.3 172.3 174.3 176.3 |
| 192 194 196 198 200 |  | 122.8 124.1 125.4 126.7 128.0 | 150.6 152.2 153.8 155.4 157.0 | 159.6 161.3 163.1 164.8 166.5 | 166.8 168.6 170.4 172.2 174.1 | 178.2 180.2 182.2 184.2 186.2 |