|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	F.125

TELECOMMUNICATION	(08/93)
STANDARDIZATION  SECTOR
OF  ITU

OPERATIONS  AND  QUALITY  OF  SERVICE

MOBILE  SERVICE

NUMBERING  PLAN  FOR  ACCESS  TO  THE
MOBILE-SATELLITE  SERVICES  OF  INMARSAT
FROM  THE  INTERNATIONAL  TELEX  SERVICE

ITU-T  Recommendation  F.125

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations  by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation F.125 was revised by ITU-T Study Group 1 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 31st of August 1993.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1994

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation F.125     (08/93)

Page

1	Introduction		1

1.1	Purpose		1

1.2	Terminology		1

1.3	Basic considerations		1

2	Format of INMARSAT mobile international number		2

3	Telex destination codes for INMARSAT		2

4	Format of INMARSAT mobile number		2

4.1	General format		2

4.2	Formats of the INMARSAT mobile number for INMARSAT-A system		3

4.3	Formats of the INMARSAT mobile number for INMARSAT-B system		3

4.4	Format of the INMARSAT mobile number for INMARSAT-C system		4

4.5	Format of the INMARSAT mobile number for INMARSAT aeronautical system		4

4.6	Future INMARSAT systems		4

5	Digit analysis		5

6	Presentation of INMARSAT mobile numbers in directories		5

6.1	General		5

Annex A – Use of ship station identification for maritime applications of systems operated by INMARSAT		5

A.1	General		5

A.2	Constraints on ship station identification and numbering		6

A.3	Applications of ship station identity		6

Annex B – Group call numbering scheme for the INMARSAT system		7

B.1	Categories for group call services		7

B.2	Group call formats		8

Reference		8

Recommendation F.125

Recommendation F.125     (08/93)

NUMBERING  PLAN  FOR  ACCESS  TO  THE  MOBILE-SATELLITE  SERVICES
OF  INMARSAT  FROM  THE  INTERNATIONAL  TELEX  SERVICEXE "NUMBERING  PLAN  FOR  ACCESS  TO  THE  MOBILE-SATELLITE  SERVICES
OF  INMARSAT  FROM  THE  INTERNATIONAL  TELEX  SERVICE"§

(revised at Geneva, 1993)

# 1	Introduction

## 1.1	Purpose

The purpose of this Recommendation is to specify a telex numbering plan for mobile earth stations in systems operated by INMARSAT. Such systems may include maritime land and aeronautical satellite systems. In the future the range of mobile satellite systems may also include satellite systems for other applications.

## 1.2	Terminology

The telephone/ISDN numbering plan for INMARSAT is contained in Recommendation E.215. Recommendations E.215 and F.125 are designed to be as similar as possible.

For the purpose of this Recommendation, the following definitions apply:

1.2.1	ship station identity:  As defined in the Radio Regulations, Appendix 43. See also Recommendation F.120.

1.2.2	INMARSAT mobile international number:  The addressing information, excluding any prefix, comprising a telex destination code and INMARSAT mobile number, used to access a terminal equipment connected to an INMARSAT mobile earth station from the international telex service.

1.2.3	INMARSAT mobile number:  The part of the INMARSAT mobile international number which follows the telex destination code allocated to an INMARSAT satellite region.

1.2.4	INMARSAT mobile terminal number:  That part of the INMARSAT mobile number which identifies a specific terminal equipment connected to the mobile earth station.

### 1.2.5	Other definitions

For definition of terms such as maritime mobile-satellite service, aeronautical mobile-satellite service, ship earth station, etc., see the Radio Regulations.

## 1.3	Basic considerations

The considerations which form the basis for the numbering plan are:

1.3.1	It should be possible to identify a mobile earth station, uniquely from the INMARSAT mobile number.

1.3.2	The INMARSAT mobile number should have a format where the same number could be used for access from all types of public network.

1.3.3	The number of telex destination codes listed in Recommendation F.69 required for supporting future INMARSAT requirements should be as few as possible.

1.3.4	Different routings may be used for calls to mobile earth stations designed to different INMARSAT system standards.

1.3.5	Recognized operating agencies (ROA) and INMARSAT may apply different charging and accounting rates to different INMARSAT system standards.

1.3.6	The numbering plan should provide capacity for the identification of terminal equipment connected to a mobile earth station.

1.3.7	The numbering plan should support access to multichannel mobile earth stations.

1.3.8	Any new mobile earth station numbering plan should incorporate numbering plan(s) already in use for the INMARSAT-A system.

1.3.9	The length of the INMARSAT mobile international number should not exceed 12 digits to comply with Recommendations U.11 and U.12.

1.3.10	For maritime-satellite applications the ship station numbering plan should support access to several ship earth stations in the same ship within one ship station identity.

1.3.11	The Radio Regulations should make provision for the allocation of additional maritime identification digits (MIDs) for a specific country, when necessary.

# 2	Format of INMARSAT mobile international numberXE "Format of INMARSAT mobile international number"§

The format of the INMARSAT mobile international number is:

CCCT X1 ... Xk

where CCC is the telex destination code, in accordance with Recommendation F.69, allocated to INMARSAT and 
T X1 ... Xk is the INMARSAT mobile number. The format of the INMARSAT mobile number is given in 4.

NOTE – The INMARSAT mobile international number will vary depending on the satellite region selected by the caller in which connection with the MES is to be attempted.

# 3	Telex destination codes for INMARSAT

The telex destination codes (see Recommendation F.69) allocated for INMARSAT are shown in Table 1.

TABLE  1/F.125

Telex destination codes for INMARSAT

| Telex destination code | Geographical destination |
| --- | --- |
| 581 | Atlantic-east satellite region, INMARSAT |
| 582 | Pacific satellite region, INMARSAT |
| 583 | Indian satellite region, INMARSAT |
| 584 | Atlantic-west satellite region, INMARSAT |

# 4	Format of INMARSAT mobile numberXE "Format of INMARSAT mobile number"§

## 4.1	General format

4.1.1	The general format of the INMARSAT mobile number is:

T X1X2 ... Xk

where the digit T is used for discrimination between different INMARSAT systems.

The formats used for the various INMARSAT systems are defined below. The values of the T digits are summarized in Table 2.

The T digits represent a limited resource and a new T digit should therefore only be allocated when necessary for technical or operational reasons.

The TSB is responsible for coordinating the allocation of new T (or U) (see 4.6) digits with the competent Study Groups.

TABLE  2/F.125

Value of T digit for various applications

| T digit | Applications |
| --- | --- |
| 0 | Group call in INMARSAT-A, see 4.2.2 |
| 1 | Ordinary call in INMARSAT-A, see 4.2.1 |
| 2 | Reserved for future use |
| 3 | Ordinary call in INMARSAT-B, see 4.3 |
| 4 | Ordinary call in INMARSAT-C, see 4.4 |
| 5 | Ordinary call in INMARSAT aeronautical system, see 4.5 |
| 6 | INMARSAT-M (excluding telex) |
| 7 | Reserved for future use |
| 8 | Expedient access to special service terminations in INMARSAT-A, see Recommendation E.215 |
| 9 | Reserved for future expansion, see 4.6 |

4.1.2	To meet the requirements of the international telex service, and, in particular, the signalling conditions specified in Recommendations U.11 and U.12, the INMARSAT mobile number should not contain more than 9 digits.

## 4.2	Formats of the INMARSAT mobile number for INMARSAT-A system

### 4.2.1	Ordinary callsXE "Ordinary calls"§

The format of the INMARSAT mobile number used for ordinary calls to mobile earth stations in INMARSAT-A system is as follows:

1 X1X2X3X4X5X6 (7 digits)

where 1 corresponds to the T digit and the digits X1X2X3X4X5X6 are allocated by INMARSAT.

### 4.2.2	Group callsXE "Group calls"§

For group calls, the INMARSAT mobile number takes the following format:

0 X1X2X3X4X5X6X7X8 (9 digits)

where 0 corresponds to the T digit and X1 through X8 are allocated by INMARSAT.

The group call numbering scheme is shown in B.2.2.

## 4.3	Formats of the INMARSAT mobile number for INMARSAT-B systemXE "Formats of the INMARSAT mobile number for INMARSAT-B system"§

### 4.3.1	Ordinary calls

The format of the INMARSAT mobile number used for ordinary calls to mobile earth stations in INMARSAT-B system, shall be as follows:

3 M1I2D3X4X5X6X7X8 (9 digits)

where 3 corresponds to the T digit and the M1I2D3X4X5X6 are the first 6 digits of the ship station identity MIDXXX000 (see Annex A). The INMARSAT mobile terminal number digits X7X8 may be used for identifying terminal equipment connected to a ship earth station, for discriminating between channels for multichannel ship earth stations and for discriminating between several ship earth stations on the same ship.

The number format is:

3 X1X2X3X4X5X6X7X8 (9 digits)

where the digit X1 may take the values 8 or 9, which are reserved for future INMARSAT applications.

### 4.3.2	Group calls

For further study.

## 4.4	Format of the INMARSAT mobile number for INMARSAT-C systemXE "Format of the INMARSAT mobile number for INMARSAT-C system"§

### 4.4.1	Ordinary calls – Maritime mobile

The format of the INMARSAT mobile number used for ordinary calls to mobile earth stations in INMARSAT-C system, shall be as follows:

4 M1I2D3X4X5X6X7X8 (9 digits)

where 4 corresponds to the T digit and where at least the digits M1I2D3X4X5X6 are part of the ship station identity. The digits X7X8 may also be part of the ship station identity or be used for discrimination between several ship earth stations on the same ship.

The number format is:

4 X1X2X3X4X5X6X7X8 (9 digits)

### 4.4.2	Ordinary calls – Land mobile

The format of the INMARSAT mobile number used for ordinary calls to land-based mobile earth stations in the INMARSAT-C system, shall be as follows:

4 8 M2C3C4X5X6X7X8 (9 digits)

where 4 corresponds to the T digit and the digit 8 signifies a land-based mobile earth station and the digits M2C3C4 correspond with the mobile country codes listed in Annex A/E.212.

### 4.4.3	Group calls

Group call selection in the INMARSAT-C system is achieved using two stage access procedures which do not conform with the scheme outlined in Annex B.

## 4.5	Format of the INMARSAT mobile number for INMARSAT aeronautical systemXE "Format of the INMARSAT mobile number for INMARSAT aeronautical system"§

The general format of INMARSAT mobile numbers in the INMARSAT aeronautical system is as follows:

5 X1X2X3X4X5X6X7X8 (9 digits)

where 5 corresponds to the T digit.

The format of the digits X1 through X8 is still to be determined.

## 4.6	Future INMARSAT systemsXE "Future INMARSAT systems"§

T digits will be allocated for each new INMARSAT system in the future. If an earlier system is taken out of service, T digits allocated for that system may be reallocated to new systems.

If the capacity provided by the T digits of Table 2 is not sufficient, further capacity may be made available by using T = 9 followed by an additional digit (U) as follows:

9 U X1X2 ... Xk

where the digits X1 ... Xk identifies the mobile earth station and any extension connected to it. The digit U is used to identify new INMARSAT systems or for technical and operational reasons (see 6 below).

# 5	Digit analysisXE "Digit analysis"§

If different routing and/or accounting arrangements are applied to different INMARSAT systems, the digits CCCT will need to be analysed at international exchanges.

If the routing capacity is increased by using T = 9 (see 4.6), the digits CCC9U need to be analysed and this is for further study.

# 6	Presentation of INMARSAT mobile numbers in directoriesXE "Presentation of INMARSAT mobile numbers in directories"§

## 6.1	General

INMARSAT mobile numbers may be published in separate directories or in separate sections of general directories.

In directories, only the INMARSAT mobile numbers, as specified in 4.1, shall be listed. The telex destination code to be used and instruction for the subscribers should be contained in general parts of the directories.

The subject of directories for mobile-satellite services requires further studies.

Annex A

Use of ship station identification for maritime applications 
of systems operated by INMARSATXE "Use of ship station identification for maritime applications 
of systems operated by INMARSAT"§

(This annex forms an integral part of this Recommendation)

Note on Reservation on the use of this annex

The INMARSAT-B and C systems depend on analysis of blocks 2 and 3 unlike INMARSAT-A and the following text is therefore the subject of further study.

## A.1	General

Appendix 43 of the Radio Regulations defines an international identification plan for ships participating in the maritime mobile services. The ship station identity consists of 9 digits and is composed as follows:

M1I2D3X4X5X6X7X8X9

where the digits M1I2D3 determine the ship's nationality.

For ships participating in systems operated by INMARSAT, the main part of this Recommendation specifies a format of the INMARSAT mobile number as follows:

T X1X2 ... Xk

The purpose of the digit T is explained in 4.

For maritime applications, the number can be regarded as being composed of three blocks as follows:

| T | X1X2 ... Xn | Xn+1 ... Xk |
| --- | --- | --- |
| Block 1 | Block 2 | Block 3 |

where the digit in block 1 is the digit T, the digits in block 2 are related to the ship station identity as explained below, and block 3 contains digits which are used for other purposes (e.g. INMARSAT mobile terminal number). In some INMARSAT systems, block 3 may be empty.

NOTES

1	For the INMARSAT-A system, INMARSAT applies a ship numbering plan which is not related to the ship station identification plan of the Radio Regulations. In this numbering plan the digit T takes the fixed value T = 1.

2	For INMARSAT-B and -C systems, the digit X1 may take the values 8 or 9 for non-maritime applications. In this case, the digits in block 2 are not related to the ship station identification plan.

## A.2	Constraints on ship station identification and numberingXE "Constraints on ship station identification and numbering"§

A.2.1	To meet the requirements of the international telex service, and in particular the signalling conditions specified in Recommendations U.11 and U.12, the INMARSAT mobile number should not contain more than 9 digits.

A.2.2	The new numbering plan must cater for the following:

–	identification for calls to ship-board terminal equipment connected to the ship earth station;

–	the possibility of several ship earth stations on the same ship where all ship earth stations have a number associated with the unique ship station identity of the ship;

–	the capability of supporting multichannel ship earth stations.

These capabilities may require digits in block 3 of the INMARSAT mobile number, thus reducing the available space for block 2.

## A.3	Applications of ship station identityXE "Applications of ship station identity"§

### A.3.1	Digit capacity in block 2XE "Digit capacity in block 2"§

The INMARSAT-A system can only support 6 digits in block 2 because of the addressing capacity on the radio path.

The addressing capacity of INMARSAT-B and -C systems on the radio path can cater for up to 7 digits in block 2. However, the limited digit capacity of the terrestrial networks puts the following initial constraints to the number of digits in block 2:

–	for the INMARSAT-B system, the initial digit capacity in block 2 is 6 digits in order to allow sufficient capacity in block 3 for supporting the capabilities listed in A.2.2 above;

–	for the INMARSAT-C system, the initial digital capacity in block 2 is 6 digits to allow sufficient capacity in block 3 for supporting the possibility of identifying several terminal equipments connected to a ship earth station and of several ship earth stations on the same ship.

### A.3.2	Mapping between ship station identity and digits in block XE "Mapping between ship station identity and digits in block "§2

The mapping between ship station identity and digits in block 2 is shown in Table A.1.

TABLE  A.1/F.125

Mapping between ship station identity and digits in block 2 
of the INMARSAT mobile station number

| Ship station identity |  |  | XXX XXX 000 |  | XXX XXX 0X0 |  | XXX XXX 0XX |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block 2 mapping | Size of block 2 | 6 digits |  | XXX XXX |  | Mapping not possible |  | Mapping not possible |
| X Any digit between zero (0) and nine (9) 0 Zero (0) |  |  |  |  |  |  |  |  |

For ship earth stations, the ship station identity is thus derived from the digits in block 2 by adding 0s at the end until the identity consists of 9 digits.

The digit T in block 1 determines the type of ship earth station and, implicitly, the number of digits in block 2. The relationship is shown in Table A.2. Further details of the number structure are given in the main part of the Recommendation.

TABLE  A.2/F.125

Relationship between the digit T and the format of the ship station 
identity in 12 digit INMARSAT mobile international numbers

| Value of digit T | INMARSAT standard system | Number of digits in block 2 | Format of ship station identity |
| --- | --- | --- | --- |
| 0 | A | (Note 1) | (Note 1) |
| 1 | A | 6 | (Note 2) |
| 2 | Reserved | – | – |
| 3 | B | 6 | XXX XXX 000 |
| 4 | C | 6 | XXX XXX 000 |
| 5 | Aeronautical | (Note 3) | (Note 3) |
| 6 | Reserved (INMARSAT-M) | 6 | (Note 5) |
| 7 | Reserved | – | – |
| 8 | A | (Note 4) | (Note 4) |
| 9 | Future expansion | Further study | Further study |
| NOTES 1 Group call address (see Annex B for format of group call addresses). 2 The INMARSAT mobile number is not related to the ship station identification plan of Appendix 43, Radio Regulations. 3 The numbering plan for the aeronautical-satellite service is not related to the ship station identification plan of Appendix 43, Radio Regulations. 4 See clause 4 for the use of this T digit. 5 INMARSAT-M (excluding telex). |  |  |  |

### A.3.3	Ships equipped with several INMARSAT systemsXE "Ships equipped with several INMARSAT systems"§

The ship station identity for such ships is the one derived from the ship earth station standard having the smallest size of block 2. This applies only if the numbering systems for the ship earth station standards are related to the ship station identification plan.

Annex B

Group call numbering scheme for the INMARSAT systemXE "Group call numbering scheme for the INMARSAT system"§

(This annex forms an integral part of this Recommendation)

## B.1	Categories for group call servicesXE "Categories for group call services"§

At present, four different categories of group call service have been envisaged within the maritime mobile-satellite service.

### B.1.1	National group callsXE "National group calls"§

The category is defined to address all ships of the same nationality.

### B.1.2	Fleet group callsXE "Fleet group calls"§

This category is defined to address all ships within one fleet.

### B.1.3	Selected group callsXE "Selected group calls"§

This category is defined to address a number of ships having a community of interest irrespective of nationalities or fleets, and forming a predefined group.

### B.1.4	Area group callsXE "Area group calls"§

This category is defined to address all ships of any nationality located within a predetermined geographical area.

## B.2	Group call formatsXE "Group call formats"§

B.2.1	The general group call format is T X1X2X3X4X5X6X7X8, where the digits T X1X2X3X4X5X6X7X8 take the values in B.2.2 for INMARSAT.

B.2.2	The group call numbering schemes for the INMARSAT-A system will use eight decimal digits X1 ... X8 following the T digit, with T = 0, allocated as follows:

M2I3D40506070809	National group call

M2I3D4F5F6F7F8F9	Fleet group call

0203S4S5S6S7S8S9	Selected group call

020304A5A6A7A8A9	Area group call

where M2 ¹ 0     M2 ¹ 1     F5 ¹ 0     S4 ¹ 0.

For T = 1 or 8, the group call number is not valid.

B.2.3	For INMARSAT-B and INMARSAT-Aeronautical, the format of the digits X1 ... X8 is for further study.

B.2.4	The MIDs in national and fleet group numbers are those allocated in Table 1 of Appendix 43, Radio Regulations [1].

B.2.5	In accordance with category 4 of the above-mentioned appendix, the particular MID reflects only the country allocating the group call identity and, therefore, does not prevent group calls to fleets containing more than one ship nationality. Allocation of selected group numbers should be avoided when the same group could equally well be assigned a fleet group number.

B.2.6	National group numbers and fleet group numbers should be allocated by countries. Selected group numbers and area group numbers as applicable to the INMARSAT system should be allocated by INMARSAT: allocation of such numbers may require cooperation with other organizations.

B.2.7	A country having assigned a national group or fleet group number should notify the Director-General of INMARSAT if those numbers are going to be used within the INMARSAT system.

## Reference

[1]	Radio Regulations, Appendix 43, ITU, Geneva, 1982, revised in 1985, 1986 and 1988.