|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	F.202

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

OPERATIONS  AND  QUALITY  OF  SERVICE

TELEMATIC SERVICE

INTERWORKING  BETWEEN  THE  TELEX  SERVICE  AND  THE  TELETEX  SERVICE -GENERAL  PROCEDURES  AND  OPERATIONAL  REQUIREMENTS  FOR  THE  INTERNATIONAL  INTERCONNECTION  OF  TELEX/TELETEX  CONVERSION  FACILITIES

ITU-T  Recommendation  F.202

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation F.202 was published in Fascicle II.5 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation F.202

Fascicle II.5 - Rec. F.202

INTERWORKING BETWEEN THE TELEX SERVICE AND THE TELETEX SERVICE  –

GENERAL PROCEDURES AND OPERATIONAL REQUIREMENTS FOR THE INTERNATIONAL

INTERCONNECTION OF TELEX/TELETEX CONVERSION FACILITIES

CONTENTS

1	Scope

2	Introduction

3	Service Outline

4	International Interconnection

5	Delivery/Non-Delivery Notification

6	Message Transfer

7	Elements of inter-CF message transfer procedures

8	Error Recovery

9	Message Input and Message Delivery

1	Scope

1.1	Many Administrations have implemented, or are about to implement teletex/telex conversion facilities (CF). In many cases the telex selection and message input procedures, for the CF, differ for each country of destination.

1.2	This Recommendation proposes operational procedures for the international interconnection of CFs allowing access from the telex service to the international teletex service to be in the country of origin.

2	Introduction

2.1	The introduction of the international interconnection of CFs will, in many cases, lead to single selection and message input procedures, for telex access to the international teletex service, in the country of origin. Thus ensuring that the message input procedure may be tailored to meet national needs.

2.2	Use of error detecting transmission protocols between interconnected CFs will extend the performance characteristics of the teletex service across the international boundary when interworking between the telex and teletex services.

2.3	Telex customers interworking with the international teletex service may be identified in the country of origin.

2.4	The return of non-delivery and delivery notifications across international boundaries may be simplified.

2.5	Greater transmission efficiency may be achieved with multi-address messages and messages addressed to both telex and teletex addresses may be accommodated.

2.6	International accounting principles may be established for the international inter-CF traffic.

3	Service Outline

3.1	The international interconnection of CFs should be established between Administrations on a bilateral basis.

3.2	The general service requirements for telex to teletex and teletex to telex directions of interworking respectively should be in accordance with § 2 of Recommendation F.201.

3.3	The interconnection of CFs allows a telex or teletex subscriber to deposit a message with a CF in the originating country for subsequent delivery, via a CF in the destination country, to a teletex or telex subscriber respectively.

3.4	In the international interconnection of CFs, using store-and-forward principles, the message is deposited with a CF in the originating country and transferred to a CF in the destination country for delivery.

3.5	Provision may be made for both single and multi-address messages, where the multi-address message may contain both telex and teletex addresses.

3.6	In the event of failure to deliver to any address or addresses a non-delivery notification shall be issued to the originating CF by the destination CF. The requirement to send a non-delivery notification is mandatory.

3.7	A delivery notification may be issued to the originating CF, to indicate successful delivery to any address or addresses, by the destination CF by bilateral agreement. However it should be a function of the CF that delivery notification is provided if requested.

3.8	Any request for delivery notification should be provided on a service basis and not on an individual message basis.

3.9	The need for a status enquiry for outstanding messages is for further study.

4	International Interconnection

4.1	The term “network management boundary” refers to the boundary within which the CF service is provided under the control of one Administration.

4.2	The extension of CF facilities beyond the management network boundary of an Administration requires co-operation between CFs across international connections.

4.3	Administrations should agree bilaterally on the network(s) to be used to interconnect CFs.

4.4	Five possibilities may be considered:

a)	Circuit Switched Public Data Network (CSPDN);

b)	Packet Switched Public Data Network (PSPDN);

c)	Public Switched Telephone Network (PSTN);

d)	Private Leased Circuits;

e)	ISDN.

4.5	International routes shall be capable of supporting a minimum data rate of 2400 bit/s; data transmission rates should be agreed bilaterally.

4.6	In the international interconnection of CFs the responsibility to deliver messages is transferred from the originating Administration to the destination Administration.

4.7	In the basic service messages addressed to more than one destination CF management network should be separated at the originating CF management network.

4.8	The possibility of forwarding messages via transit CF management networks is for further study.

5	Delivery/Non-Delivery Notification

5.1	In the international interconnection of CFs it is necessary to return delivery/non-delivery status information to the originating CF. This information should be compiled at the destination CF when either the message has been delivered or no further attempts to deliver will be made to that address.

5.2	The return of delivery and non-delivery advice information to the originating CF may be on a per message address or per message basis.

5.3	Delivery and non-delivery information provided on a per message address basis requires explicit notification to the originating CF.

5.4	Delivery and non-delivery information provided on a per message basis may only require explicit notification  of non-deliveries and implicit notification of deliveries.

5.5	Where bilateral agreement exists to return delivery notifications, all notifications should be explicit.

5.6	The method employed on an international connection between two CFs to transfer delivery/non-delivery information should be the subject of bilateral agreement. Account should be taken of the means by which the interconnection is established and the possible effects on service.

5.7	If technically possible the attempt to deliver the message, in the telex to teletex direction, by the destination CF should allow the provision of an on-line delivery acknowledgement facility.

5.8	The interconnection procedures with CFs employing single-stage selection in telex to teletex direction are for further study.

6	Message Transfer

6.1	The co-operation of two or more CFs may be required to complete the function of message transfer.

6.2	The transfer of the message between CFs should be as soon as practically possible after the deposit of the message in the originating CF.

6.3	Where an originating CF is unable to transfer the message to the destination CF a non-delivery notification should be returned to the originating customer.

6.4	In assuming responsibility for the delivery of the message the destination CF should attempt to deliver the message immediately after receipt of the message from the originating CF. The action to be taken where the message cannot immediately be delivered is for further study.

6.5	The delivery time targets for both the transmission of messages between CFs and the delivery of the message from the destination CF should be agreed bilaterally.

6.6	The destination CF should return the delivery information, to the originating CF, immediately after delivery of the message or the final attempt to deliver the message.

6.7	In the event of non-delivery the action to be taken with regard to the message text is for further study. It may be required to return the message text with the non-delivery advice.

7	Elements of inter-CF message transfer procedure

7.1	The basic element of the message transfer procedure is the message transfer unit. This unit is classified as either a text message transfer unit or a service message transfer unit allowing easy identification of the function(s) for which co-operation is required.

7.2	Text message transfer units carry messages submitted by a telex or teletex customer for delivery to a specified address or addresses.

7.3	Service message transfer units do not contain customer messages but are used to convey service information about messages.

7.4	There are four types of message transfer unit which may be used to provide a CF interworking capability:

i)	Text transfer – used to transfer destination address information and the customer message;

ii)	Delivery notification – used to provide information on an address (or addresses) to which the message has been delivered;

iii)	Non-delivery notification – used to provide information on an address (or addresses) to which the message has not been delivered;

iv)	Combined delivery/non-delivery notification – used to provide information on whether a message has or has not been delivered to a number of addresses.

Other types of message transfer units are for further study.

7.5	The provision of a status report covering one or more messages is for further study.

7.6	To achieve reliable message transfer the control procedures for the transfer of messages between CFs should be in accordance with the relevant CCITT technical Recommendations.

8	Error Recovery

8.1	For error conditions arising during the CF to CF transfer normal error recovery procedures should be in accordance with the relevant CCITT technical Recommendations.

8.2	For error conditions arising during the input to or delivery from a CF the procedures detailed in Recommendations F.201 and T.90 should apply.

9	Message Input and Message Delivery

9.1	Message input and delivery procedures should be generally in accordance with Recommendations F.201 and T.90 for telex and teletex respectively.