|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	F.421

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

OPERATIONS  AND  QUALITY  OF  SERVICE

MESSAGE  HANDLING  SERVICES

MESSAGE  HANDLING  SERVICES:  INTERCOMMUNICATION  BETWEEN THE  IPM  SERVICE  AND  THE  TELEX  SERVICE

ITU-T  Recommendation  F.421

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation F.421 was published in Fascicle II.6 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation F.4211)

Fascicle II.6 - Rec. F.421

MESSAGE HANDLING SERVICES:

INTERCOMMUNICATION BETWEEN THE IPM SERVICE

AND THE TELEX SERVICE

The establishment in various countries of message handling service in association with public networks creates the need to produce Recommendations covering the aspects of public message handling services.

The CCITT,

## considering

(a)	the need for public message handling services;

(b)	the strategic and commercial importance of standardization of message handling services;

(c)	the urgent need for intercommunication arrangements for existing telematic services, and other services with public message handling services;

(d)	the need for a clear distinction between the responsibilities to be allocated to service providers and those of subscribers and/or users;

(e)	the need for establishing international compatibility between different messaging systems;

(f)	the growth of the installed base of terminals and personal computers with the ability to access message handling systems;

(g)	that several F-series Recommendations describe public message handling services;

(h)	that certain X, T and U-series Recommendations cover relevant aspects of systems used for the provision of messaging services;

(i)	that Recommendations F.60 and F.69 define the service requirements for the telex service;

(j)	that Recommendation F.72 defines international telex store-and-forward;

(k)	that the U-series Recommendations define the technical requirements for the telex service;

(l)	that Recommendation U.204 defines the technical requirements for the intercommunication between the IPM service and the telex service;

unanimously declares

that operational procedures for intercommunication between the public interpersonal messaging service and the telex service shall be in accordance with this Recommendation.

CONTENTS

1	Scope

2	Introduction

3	Service outline

4	Operational procedures

4.1	IPM service to telex service direction

4.2	Telex service to IPM service direction

4.3	Construction of the IP-message

Annex A – Abbreviations

Annex B – Actions to be taken by the PTLXAU/examples

Annex C – IPM message to telex

Annex D – Telex message to IPM

1	Scope

1.1	This Recommendation describes the general, operational and service procedures for the provision of intercommunication between the public interpersonal messaging service and the telex service.

1.2	The intercommunication is based on store-and-forward principles which allow users of one service to exchange messages with the users of the other service.

2	Introduction

The IPM service is a messaging service which may be provided on a variety of networks and allows several forms of addresses, whereas the telex service provides direct connection between subscribers in the telex network.

Therefore, to match dissimilar characteristics of the two services, it is necessary to provide intercommunication via a public telex access unit (PTLXAU). In both IPM service to telex service, and telex service to IPM service directions, the complete message is deposited in the PTLXAU for onward transmission.

In general the selection procedures for the telex subscriber will be two-stage; however, where the destination IPM service user is assigned a numeric address that is part of the national telex numbering plan of the destination country, one-stage selection procedures may be used.

3	Service outline

3.1	Communication between subscribers of the telex service and the IPM service is on store-and-forward basis; thus conversational mode interworking between users is not applicable.

3.2	Public access to the IPM service for telex subscribers and delivery of messages to telex subscribers from IPM service users is provided by means of a PTLXAU.

3.3	The PTLXAU belongs to the IPM service.

3.4	In the IPM service to telex service direction, the IPM service retains the responsibility for the message until delivery to the telex subscriber has been completed.

3.5	In the telex service to IPM service direction, the IPM service is responsible for the delivery of the message to the IPM service user, once the input is completed under normal conditions.

3.6	In both IPM service to telex service direction and telex service to IPM service direction, the international connection should be via the international telex network, as shown in Figure 1/F.421.

3.7	Where two Administrations offer an IPM service, the international boundary may be placed within the IPM service by bilateral agreement. In this configuration, however, international telex connections should continue to be established via the international telex network.

4	Operational procedures

4.1	IPM service to telex service direction

4.1.1	Messages from an IPM service user to a telex subscriber are sent as normal IP-message with the appropriate IPM elements of service, in accordance with Recommendation F.420.

4.1.2	When a message is received by the PTLXAU, the message content will be converted into the format and character repertoire defined for the telex service. This may result in loss of information if the IPM service user does not conform to these defined rules.

Note – The conversion process may take place in the message transfer system (MTS) associated with the PTLXAU.

4.1.3	The PTLXAU shall be responsible for the action to be taken for IPM elements of service received in accordance with Recommendation F.420. Annex B shows examples of IPM elements of service together with proposed actions to be taken by the PTLXAU.

4.1.4	Call establishment by and delivery of the message to the telex subscriber should be in accordance with Recommendations F.72 and U.204.

4.1.5	The IP-message sent to the called telex subscriber shall be preceded by a PTLXAU identification. The content of this identification is a national matter but should include the service code “CI”, the code expression “IPM”, and the telex network identification code in accordance with Recommendation F.69, e.g. “CI ® IPM ® CH”.

4.1.6	A general layout of an IP-message delivered to the telex service is shown in Annex C.

4.1.7	The elements of service related to the IP-message heading shall be converted into printable text. The language of this text is a national matter. The PTLXAU shall transmit the originator O/R address to the called telex subscriber in the form necessary for recall, in accordance with the indications of Table 2/F.421 (see example in Annex C).

4.1.8	In order to help the telex recipients to recall the originator, the PTLXAU could transmit, as the first element of the message heading, some information as guidance. The contents of this field is a national matter, but when used should be called “FOR RECALL” (see Annex C).

4.1.9	Upon delivery of the message to the telex subscriber, a delivery notification should be sent back to the originating IPM service user if requested. In the event of non-delivery of the message to the telex subscriber, a non-delivery notification shall be sent back to the IPM service user (unless the IPM user has requested prevention of non-delivery notification).

4.2	Telex service to IPM service direction

In this direction, Administrations may implement either or both one-stage and two-stage call set-up procedures.

4.2.1	One-stage selection

4.2.1.1		Where one-stage call set-up procedures are used, the number assigned to a user in the IPM service must appear to be part of the national telex numbering plan.

4.2.1.2		The length of the number assigned to the IPM service user shall be in accordance with the relevant U-series signalling Recommendations.

4.2.1.3		The procedures for message transfer within the IPM service, e.g. mapping of the assigned number to an O/R address, are a national matter and not covered by this Recommendation.

4.2.1.4		The call shall be established using normal telex call set-up procedures.

4.2.1.5		The telex number received by the PTLXAU from the telex network shall be verified by the IPM service as being proper to a registered IPM service user. If the verification fails:

a)     where the PTLXAU is provided by the Administration which also provides all or part of the telex network,                                                                                                                                                         the service signal NP may be returned;

b)     where the PTLXAU is not provided by the Administration which also provides all or part of the telex                                                                                                                                                              network, the procedures to be applied shall be in accordance with Recommendation F.74.

4.2.1.6		The answerback returned to the calling telex subscriber at call establishment and also during the text input stage shall contain the national telex number assigned to the IPM service user.

4.2.1.7		The call shall be cleared using normal telex call clearing procedures.

4.2.1.8		When the message cannot be delivered to the IPM service user, a non-delivery notification shall be returned to the telex subscriber. The procedures for establishing the calling telex address are specified in Recommendation U.204.

4.2.1.9		The non-delivery notification returned to the originating telex subscriber should contain a reference consisting of the telex address of the IPM service user and time and date of submission to the PTLXAU.

4.2.1.10	The action to be taken when a non-delivery notification cannot be returned to the calling telex subscriber is for further study.

4.2.1.11	The format of notifications and the procedures for their delivery should be in accordance with Recommendation U.204.

4.2.1.12	The use of IPM elements of service by the telex subscriber is for further study.

4.2.2	Two-stage selection

4.2.2.1		The telex subscribers shall use normal telex call procedures to access the PTLXAU which is allocated a telex number that is part of the national telex numbering plan of the country in which the PTLXAU is located.

4.2.2.2		Procedures for access to the PTLXAU shall follow Recommendation U.204.

4.2.2.3		A service identifier may be input before the O/R address(es) of the first message. It may allow the Administrations to provide intercommunication with several services through only one PTLXAU (see Tables 1/F.421, 3/F.421 and Annex D).

4.2.2.4		The PTLXAU shall be able to accommodate the following O/R address forms:

-	Mnemonic O/R address;

-	Terminal O/R address;

-	Numeric O/R address;

as specified in Recommendation F.401. The O/R address should be input in accordance with the requirements of Recommendation U.204.

It is the responsibility of the originating telex subscriber to be aware of the required attributes specific to the domain of the called IPM service user. Each attribute of the O/R address shall be identified and delimited. The complete O/R address shall be terminated with an end-of-address (EOA) indicator.

The structure of the service identifier and the address input is shown in Table 1/F.421.

Each attribute of the address structure shall be contained in one line.

Each address attribute and the service shall be identified by a code expression according to Tables 2/F.421 and 3/F.421.

4.2.2.5		Under normal conditions, the message input will be terminated by an end of message (EOM) or and end of transmission (EOT) signal. In case where no EOM or EOT signal is received, the PTLXAU shall forward any input received prior to call disconnect with the added text “THIS MESSAGE MAY BE INCOMPLETE”. Annex D shows a general layout applicable in case of submission of message(s) to the PTLXAU by the telex subscriber.

4.2.2.6		Except as defined in 4.2.2.5 above, the action to be taken when abnormal conditions are encountered during message input shall be in accordance with Recommendation U.204.

TABLE 1/F.421

Telex service to IPM service address structure

Service identifier Address attribute identifier <value> . . . Address attribute identifier <value> End of single O/R address (+) [Next O/R address(es)] [Request for IPM elements of service] End of address(es) (BT) [Request for delivery notification]

Note – [ ] indicates optional attributes.

TABLE 2/F.421

Code expressions for address attribute identifiers

| Address attribute | Format |
| --- | --- |
| Country name Administration domain name Private domain name Organization name Organisation unit name(s) Personal name – Surname – Given name – Initials – Generation qualifier – Common name Numeric user identifier Terminal type and network address for telex teletex fascimile videotex Domain defined attribute(s) – Type – Value | CTN ® <value> ADM ® <value> PRI ® <value> ORG ® <value> OUN ® <value> SUR ® <value> GIV ® <value> INI ® <value> GEN ® <value> COM ® <value> NUS ® <value> TLX ® <value> TTX ® <value> FAX ® <value> VTX ® <value> DDT ® <value> DDV ® <value> |

Note 1 – The symbol ® equals a space .

Note 2  – Allowed attribute values are specified in Recommendation F.401.

TABLE 3/F.421

Code expression for the service identifier

| Service | Format |
| --- | --- |
| Interpersonal messaging service | IPM |

4.2.2.7	During the input stage of the address, the PTLXAU shall validate, as a minimum, the following O/R address formats, as specified by the domain:

-	The existence of mandatory attributes.

-	The existence of non-allowed attributes.

-	The minimum and maximum allowed number of characters in each attribute.

-	The existence of non-allowed characters in an attribute.

Where applicable, the existence/non-existence of non-significant character(s) preceding or following the attribute values shall not prevent validation.

Despite the acceptance by the PTLXAU of the submitted O/R address, there is no guarantee that the message will be subsequently delivered and, in this case, the originating telex subscriber will be charged for a message which was not delivered. It is therefore desirable that a means of verifying the existence of the O/R adress be provided and the method of achieving this is left for further study.

4.2.2.8		The service principles for delivery and non-delivery notifications should be in accordance with Recommendation F.72. The format of the notification messages is defined in Recommendation U.204. Delivery notification may be requested as a code expression following the end of address signal.

4.3	Construction of the IP-message

The message received by the PTLXAU shall be delivered to the IPM user(s) in accordance with following rules.

4.3.1	P2 body part

The received message, excluding the recipient address(es), shall form the body of the IP-message. All the received characters shall be delivered except the WRU signals.

4.3.2	Recipient O/R address

All recognized O/R addresses shall be assumed as primary recipients. By default, these primary recipients will not be disclosed to each other.

4.3.3	Originator indication

The calling telex subscriber address shall be converted by the PTLXAU into the format of a terminal O/R address and shall be placed in the originator indication element of service field.

4.3.4	Subject indication

The PTLXAU shall generate the element of service which will cause TELEX to appear in the subject indication element of service field.

4.3.5	IP-message identification

The content of the message reference information returned to the calling telex subscriber shall also be used as the unique identifier in the IP-message identification element of service field.

4.3.6	Grade of delivery selection

The PTLXAU shall set the grade of delivery selection element of service to the value URGENT.

4.3.7	Conversion prohibition in case of loss of information

The use of the element of service – conversion prohibition in case of loss of information – is for further study.

4.3.8	Disclosure of other recipients

This element of service shall be set by the PTLXAU when the originating telex subscriber requests the disclosure of other recipients. The procedures for requesting this disclosure are defined in Recommendation U.204.

4.3.9	Deferred delivery

This element of service shall be set by the PTLXAU when the originating telex subscriber requests deferred delivery of his message. The procedures for requesting deferred delivery are defined in Recommendation U.204.

Note – The code expressions to be used for the selection of the elements of service described in §§ 4.3.8 and 4.3.9 by the telex subscriber, are shown in Table 4/F.421.

TABLE 4/F.421

Code expressions for the use of IPM elements of service

| IPM element of service | Format |
| --- | --- |
| Disclosure of other recipients Deferred delivery Delivery notification | DUR DEF ® <value> BT, ACKa) |

a) The request for delivery notification may be given together with the

code for end of adresse (es) (BT) if delivery notification is required.

Note – The symbol ® equals a space.

4.3.10		Other elements of service

Elements of service of the basic IPM service other than those specified above shall be set by the PTLXAU in accordance with the requirements of the domain to which it belongs.

ANNEX A

(to Recommendation F.421)

Abbreviations

| A/B ACK ADM BT CI COM CTN DDT DDV DEF DUR EOA EOM EOT FAX GEN GIV I IP IPM MT MTS NP NUS O/R ORG OUN P2 PRI PTLXAU SUR TID TLX TTX UTC VTX WRU + ® | Answerback Request for Delivery Notification Signal Administration Management Domain End of Address(es) Signal Conversation Impossible Common Name Country Name Domain Defined Attribute Type Domain Defined Attribute Value Deferred Delivery Disclosure of other Recipients End of Address End of Message End of Transaction Facsimile Generation Qualifier Given Name Initials Interpersonal Interpersonal Messaging Message Transfer Message Transfer System The called party is not, or no longer, a subscriber Numeric User Identifier Originator/Recipient Organization Name Organization Unit Name(s) IPM Protocol Private Domain Name Public Telex Access Unit Surname Terminal Identifier Telex Teletex Universal Coordinated Time Videotex Who Are You End of Single O/R Address signal Space |
| --- | --- |

Note 1 –  For a glossary of terms see Annex A to Recommendation F.400.

Note 2 –  For references see Recommendation  F.400.

ANNEX B

(to Recommendation F.421)

Actions to be taken by the PTLXAU/examples

Basic IPM elements of service and essential optional IPM user facilities which have to be processed by the PTLXAU in the case where a message is sent from the IPM service to the telex service direction (Table B-1/F.421).

TABLE B-1/F.421

| Reference Rec. F.400 Annex B | Elements of service | Action to be taken | Examples |
| --- | --- | --- | --- |
| B.5 | Authorizing users indication | Display in message heading | Authority : ® <value> |
| B.6 | Auto-forwarded indication | Ignore |  |
| B.8 | Blind copy recipient indication | Display the O/R descriptor information of the blind copy recipient(s) | BCC ® <value> |
| B.9 | Body part encryption indication | The PTLXAU shall send a non-delivery notification to the originator |  |
| B.12 | Content type indication | National matter for content types different than P2 |  |
| B.13 | Conversion prohibition | If ITA2, ignore. Otherwise the PTLXAU generates a non-delivery notification |  |
| B.15 | Converted indication | Ignore |  |
| B.18 | Cross referencing indication | Display in message heading | Reference ® <value> |
| B.21 | Delivery notification | The PTLXAU shall send a delivery notification to the originator |  |
| B.22 | Delivery time stamp indication | Ignore |  |
| B.25 | Disclosure of other recipients | Disclose all recipients |  |
| B.26 | DL expansion history indication | Ignore |  |
| B.29 | Expiry date indication | Display in message heading | Message invalid after : ® <value> |
| B.31 | Forwarded IP-message indication | the PTLXAU shall build a message heading for each IP-message contained in the body part |  |
| B.32 | Grade of delivery selection | National matter |  |
| B.34 | Implicit conversion | Convert to telex according to Rec. X.408 |  |
| B.35 | Importance indication | Display in message heading | Message importance: ® <value> |
| B.37 | IP-message identification | display in message heading | Message reference: ® <value> |
| B.38 | Language indication | Ignore |  |
| B.39 | Latest delivery designation | National matter |  |

TABLE B-1/F.421 (cont.)

| References Rec. F.400 Annex B | Elements of message | Action to be taken | Examples |
| --- | --- | --- | --- |
| B.41 | Message identification | Ignore |  |
| B.45 | Multi-destination delivery | Deliver the message to all recipients |  |
| B.46 | Multi-part body | Messages containing not supported body parts are not delivered. Send back a non-delivery notification to the originator |  |
| B.47 | Non-delivery notification | The PTLXAU shall generate a delivery report |  |
| B.48 | Non-receipt notification request | Ignore |  |
| B.52 | Obsoleting indication |  | Obsoletes: ® <value> |
| B.54 | Original encoded information types indication | Ignore |  |
| B.55 | Originator indication | Ignore |  |
| B.56 | Originator request alternate recipient | National matter |  |
| B.62 | Primary and copy recipients indication | Display the O/R descriptor information of the recipient(s) in the message heading | TO: ® <value> TO: ® <value> CC: ® <value> CC: ® <value> |
| B.63 | Probe | National matter |  |
| B.67 | Receipt notification request indication | Ignore |  |
| B.72 | Reply request indication | Display in message heading | Reply ® requested ® by ® sender |
| B.73 | Replying IP – message indication | Display in message heading | Reply to message: ® <value> |
| B.80 | Sensibility indication | Display in message heading just above text of body |  |
| B.88 | Subject indication | Display in message heading just above text of body | Subject: ® <value> |
| B.89 | Submission time stamp | Display in message heading | Submitted: ® <value> ® UTC |
| B.90 | Typed body | Ignore |  |

Note – The symbol ® equals a space.

ANNEX C

(to Recommendation F.421)

IPM message to telex

General layout of a message originated by an IPM service user and delivered by the PTLXAU to a telex subscriber.

Display of the originator O/R address related information to the telex user in the message heading:

a)	Two-stage selection:

FROM:® GIV®francois

SUR®maurer

ORG®swiss®ptt

ADM®arcom400

CTN®ch

b)	One-stage selection:

FROM:®(F.74 A/B)

ANNEX D

(to Recommendation F.421)

Telex message to IPM

(Two-stage selection)

General layout of a message originated by a telex subscriber using the two-stage selection, submitted to the PTLXAU for delivery to the IPM service.

1)	This Recommendation is the same as Recommendation F.75 of which only the title appears in Fascicle II.4.