|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.117

TELECOMMUNICATION	(02/96)
STANDARDIZATION  SECTOR
OF  ITU

TRANSMISSION  SYSTEMS  AND  MEDIA

GENERAL  CHARACTERISTICS  OF  INTERNATIONAL
TELEPHONE  CONNECTIONS  AND  INTERNATIONAL
TELEPHONE  CIRCUITS

TRANSMISSION  ASPECTS  OF  
UNBALANCE  ABOUT  EARTH

ITU-T  Recommendation  G.117

(Previously “CCITT  Recommendation”)

FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation G.117 was revised by ITU-T Study Group 12 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 6th of February 1996.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation G.117     (02/96)

Page

1	Objective		1

2	Principles of the scheme of nomenclature		1

3	Summary of the descriptive terms used		2

3.1	One-port networks		2

3.2	Two-port networks		2

3.3	Signal generating devices		2

3.4	Signal receiving devices		2

4	Definitions and measuring techniques based on idealized measuring arrangements		3

4.1	One-port networks		4

4.2	Two-port networks		7

4.3	Signal generating devices		10

4.4	Signal receiving devices		11

5	Other measurement definitions		12

5.1	Common-mode rejection ratio		13

Annex A – Aspects of conversion from longitudinal to transversal signals at analogue ports in some practical cases		14

A.1	Introduction		14

A.2	Calculation of the longitudinal conversion loss		14

A.3	The difference between LCLc and LCLm in a practical case – two examples		16

A.4	Additional Insight into Longitudinal Conversion Loss		16

References		18

SUMMARY

In many applications, the balance performance of equipment is crucial in order to ensure a satisfactory suppression of unwanted signals from the normal transmission path. This Recommendation gives guidance of suitable measurement methods for evaluating different effects of unbalance.

Recommendation G.117

Recommendation G.117     (02/96)

TRANSMISSION  ASPECTS  OF  UNBALANCE  ABOUT  EARTH

(Geneva, 1980; amended at Malaga-Torremolinos, 1984 and Melbourne, 1988;
revised in 1996)

# 1	Objective

This Recommendation gives a comprehensive set of prescriptive measurements of various balance parameters for one-port and two-port networks. These are intended for use either in the field or in the factory with relatively simple test apparatus (e.g. standard transmission oscillators, level measuring sets), and a special test bridge. Measuring arrangements for assessing the degree of unbalance are covered in Recommendation O.9 [1], which are consistent with this Recommendation.

The definitions and methods are so devised that the results obtained from separately-measured (or specified) items of equipment (e.g. feeding-bridges, cable pairs, audio inputs to channel translating equipment, etc.) can be meaningfully combined though not necessarily by simple decibel addition. This allows the performance of a tandem connection of such items to be predicted or at least, bounds determined for that performance. Performance in this sense means those features affected by unbalanced conditions, e.g. level of impulsive noise, sensitivity to longitudinal exposure, crosstalk ratios, etc.

# 2	Principles of the scheme of nomenclature

Many different terms have been used throughout the literature concerning unbalance about earth, some conflicting, or in other respects inadequate. The descriptive titles of the quantities given in this Recommendation are based on the following principles which have been adopted:

a)	Mode conversion, e.g. a poor (unbalanced) termination will develop an unwanted transverse signal when excited by a longitudinal signal. The measure of this effect is here termed longitudinal conversion ratio, and when expressed in transmission units Longitudinal Conversion Loss, or LCL.

b)	When a two-port is involved where for example an excitation at one port produces a signal at the other port, then the designation will include the word transfer, for example longitudinal conversion transfer ratio and the corresponding loss, LCTL.

c)	The impedance of the longitudinal path presented by a test object is a key parameter. The term longitudinal impedance ratio and the corresponding decibel expression, longitudinal impedance loss, are used to characterize the particular measurement defined.

d)	Active devices which are sources of signals (e.g. an oscillator, the output port of an amplifier) are additionally characterized by the amount of unwanted longitudinal signal that is present in the output. The key word output is now included, to give longitudinal output voltage, and the corresponding longitudinal output level. When such unwanted signals are expressed as a proportion of the wanted (transverse) signal the key phrase is output signal balance ratio, the decibel expression of which is output signal balance.

e)	Devices which continuously respond to signals (e.g. level-measuring sets, the input port of an amplifier) and which can in principle respond to unwanted longitudinal signals by reason of internal mechanisms (i.e. even if their input impedances were perfectly balanced) are characterized by measures containing the words input interference. These measures are input longitudinal interference ratio and the corresponding decibel expression input longitudinal interference loss. The long-established and well-defined common-mode rejection ratio is maintained. The term sensitivity coefficient is avoided, since this is widely used in the Directives [2] and the work of Study Group 5 with a rather specialized meaning.

f)	When a two-port network is involved, the input and output signals may not be the same, for example, they may have different levels, frequencies (FDM modems) or structure (PCM multiplex equipments). These aspects should be taken into account when formulating proposals for the item under test.

g)	In the case of receiving devices in which the operation is not a linear continuous function of the level of the input signal (e.g. a group-delay measuring set or a data modem) the key principle is the threshold level of the interference; this is the level at or above which an unacceptable amount of degradation of performance or misoperation occurs. Thus longitudinal interference threshold voltage and the corresponding levels are obtained.

# 3	Summary of the descriptive terms used

## 3.1	One-port networksXE "One-port networks"§

a)	transverse reflexion factor (transverse return loss: TRL);

b)	transverse conversion ratio (loss: TCL);

c)	longitudinal conversion ratio (loss: LCL);

d)	longitudinal impedance ratio (loss: LIL);

e)	transverse output voltage (level: TOL);

f)	longitudinal output voltage (level: LOL).

Voltages e) and f) are unwanted signals uncorrelated to the wanted signals.

## 3.2	Two-port networksXE "Two-port networks"§

### 3.2.1	Separate measurement

For each port taken separately the one-port measures:

a)	transverse reflexion factors (transverse return losses: TRL);

b)	transverse conversion ratio (loss: TCL);

c)	longitudinal conversion ratios (losses: LCL);

d)	longitudinal impedance ratios (losses: LIL);

e)	transverse output voltage (levels: TOL);

f)	longitudinal output voltage (levels: LOL).

### 3.2.2	Measurement combined

In addition, the following transfer parameters are for each of the two directions of transmission:

a)	transverse transfer ratios (losses: TTL);

b)	 transverse conversion transfer ratios (losses: TCTL);

c)	longitudinal transfer ratios (losses: LTL);

d)	longitudinal conversion transfer ratios (losses: LCTL).

## 3.3	Signal generating devicesXE "Signal generating devices"§

–	Output signal balance ratio (losses: OSB).

This is in addition to the six one-port measures listed in 3.1.

## 3.4	Signal receiving devicesXE "Signal receiving devices"§

a)	Input longitudinal interference ratio (loss: ILIL).

b)	Longitudinal interference threshold voltage (level).

These are in addition to the six one-port measures listed in 3.1. If the wanted signal is longitudinal (e.g. as in a signalling system) and the interfering voltage transverse, replace the word longitudinal with transverse in the descriptive terms.

# 4	Definitions and measuring techniques based on idealized measuring arrangements

The illustrated definitions in this clause assume ideal test bridges (with lossless infinite-inductance centre-tapped coils), zero impedance voltage generators and infinite-impedance voltmeters.

An important aspect of this set of mutually consistent measurements is that the test bridge provides simultaneously defined reference terminations of Z ohms for the transverse paths, and Z /4 ohms for the longitudinal paths. From this starting point, the performance of cascaded items, each measured in the prescribed fashion, can be calculated. This takes account of the fact that the cascaded items do not, in general, exhibit the reference impedances provided by the test conditions.

It simplifies the mathematical treatment if the reference impedance is non-reactive and this also accords with the important objective of being able to use readily-available transmission test-apparatus to obtain field and factory measurement results.

The ideal test bridge configurationXE "ideal test bridge configuration"§ used in the following pages is shown in Figure 1.

Note, however, that this test bridge configuration, with transversal impedance Z and longitudinal impedance Z /4, in general does not represent conditions met in practice. Thus, some care is needed when translating measured unbalance parameters into useful unbalance information for practical cases. This applies in particular for the commonly used parameter LCL, Longitudinal Conversion Loss. This is discussed in more detail in Annex A.

The transverse and longitudinal sources ET and ELXE "transverse and longitudinal sources ET and EL"§ are activated as required by the particular measurement being made. In Figure 6, neither source is active, and the bridge then provides only passive terminations of Z and Z /4.

NOTE – It would have been in keeping with traditional transmission theory for the parameters to be defined in terms of half the open-circuit e.m.f. However, to harmonize with Recommendation O.9, this Recommendation defines some parameters in terms of VT1. If the input impedance of the device under test is nominally equal to the driving device, then the two methods are equivalent.

FIGURE  1/G.117...[D01] = 5.5

## 4.1	One-port networks

### 4.1.1	Transverse reflexion factor (return loss)XE "Transverse reflexion factor (return loss)"§ (see Figure 2)

FIGURE  2/G.117...[D02] = 17

### 4.1.2	Transverse conversion ratio (loss)XE "Transverse conversion ratio (loss)"§ (see Figure 3)

FIGURE  3/G.117...[D03] = 11.5

### 4.1.3	Longitudinal conversion ratio (loss)XE "Longitudinal conversion ratio (loss)"§ (see Figure 4)

FIGURE  4/G.117...[D04] = 14.5

### 4.1.4	Longitudinal impedance ratio (loss)XE "Longitudinal impedance ratio (loss)"§ (see Figure 5)

FIGURE  5/G.117...[D05] = 11.5

### 4.1.5	Transverse and longitudinal output voltages (levels)XE "Transverse and longitudinal output voltages (levels)"§ (see Figure 6)

FIGURE  6/G.117...[D06] = 11.5

## 4.2	Two-port networks

These follow similar principles to those defined for one-port networks but now signals can be transferred from one port to the other. The two ports are distinguished by the subscripts 1/1¢ for one end and 2/2¢ for the other. There are two types of measurements:

–	those in which the excitation and response are at the same side of the network; these are as already defined for a one-port but will carry a single subscript 1/1¢ or 2/2¢ as appropriate;

–	those in which the excitation and response are at opposite sides of the network. The designation will contain the word transfer and the symbol two subscripts, the order of which indicates the direction of transmission.

### 4.2.1	Transverse reflexion factors (return losses)XE "Transverse reflexion factors (return losses)"§ (see Figure 7)

FIGURE  7/G.117...[D07] = 11

### 4.2.2	Transverse transfer ratios (losses) and conversion transfer ratios (losses)XE "Transverse transfer ratios (losses) and conversion transfer ratios (losses)"§ (see Figure 8)

FIGURE  8/G.117...[D08] = 14

### 4.2.3	Longitudinal transfer ratios (losses) and conversion transfer ratios (losses)XE "Longitudinal transfer ratios (losses) and conversion transfer ratios (losses)"§ (see Figure 9)

FIGURE  9/G.117...[D09] = 16.5

## 4.3	Signal generating devices

In addition to the six one-port measures already defined, an additional measure is required to control the amount of unwanted signal correlated with the wanted signal delivered by the device to the circuit it is connected to. This special measure is the output signal balance ratio (loss).

### 4.3.1	Output signal balance ratio (loss)XE "Output signal balance ratio (loss)"§ (see Figure 10)

FIGURE  10/G.117...[D10] = 14.5

## 4.4	Signal receiving devices

In addition to the six one-port measures already defined, additional measures are required for signal receiving devices to control their sensitivity to unwanted signals. Two cases are important. Firstly, there are receiving devices in which the response is a linear, continuous function of the wanted signal level, e.g. the indication of a level-measuring set. In this case unwanted signals give rise to inaccuracy.

In the other kind of receiver such as data modems, group-delay distortion measuring sets, signalling receivers, unwanted signals cause errors or misoperation. Two additional measures are defined.

### 4.4.1	Input longitudinal interference ratio (loss)XE "Input longitudinal interference ratio (loss)"§ (see Figure 11)

FIGURE  11/G.117...[D11] = 14.5

### 4.4.2	Longitudinal interference threshold voltage (level)XE "Longitudinal interference threshold voltage (level)"§ (see Figure 12)

FIGURE  12/G.117...[D12] = 13.5

# 5	Other measurement definitions

## 5.1	Common-mode rejection ratioXE "Common-mode rejection ratio"§

This is another quantity that is appropriate to signal receivers and is measured in accordance with the principle shown in Figure 13, the input terminals being short-circuited and then energized together.

It is clear that this measure is similar to the input longitudinal interference ratio but since there is no transverse signal (by reason of the short circuit) no longitudinal/transverse conversion mechanism within the test-object is excited. In general, there is no simple relationship between the two measures, as can be seen from the generalized measuring instrument illustrated in Figure 14, in which the input impedance is unbalanced and the gain ratios of the two halves of the differential amplifier are also slightly different. Provided the value for e is as in Figure 14 and D << 1, the various balance parameters are as indicated. This assumes the common mode rejection ratio is not twice the input longitudinal interference ratio, i.e. there is not a 6-dB difference between their decibel values.

FIGURE  13/G.117...[D13] = 9

FIGURE  14/G.117...[D14] = 9.5

Annex  A

Aspects of conversion from longitudinal to transversal signals at analogue ports 
in some practical casesXE "conversion from longitudinal to transversal signals at analogue ports 
in some practical cases"§

(This annex forms an integral part of this Recommendation)

## A.1	Introduction

Cables for telecommunication purposes sometimes have to pass areas that have strong, disturbing electromagnetic fields which may cause significant longitudinal signals along the cables. For voiceband telephone signals, the established technique is to use well-balanced cables and equipment so that the conversion from longitudinal to transversal signals is sufficiently suppressed. In general, the cables have a much higher degree of balance than what is obtainable for terminal equipment. The governing factor is then the properties of the terminal equipment, i.e. the balance to Earth of that port which is connected to the cable.

The balancing properties of an equipment port can be expressed in many ways. The most commonly used parameter appears to be the “Longitudinal Conversion Loss”, LCL. The general definition of LCL is given in clause 2 which in principle applies to all cases. However, in equipment specifications, LCL most often is understood to be a value which is obtained in a specific measuring setup according to Figure 4, where the transverse terminating impedance Z is made 600 ohms resistive and the longitudinal source impedance Z /4 thus is 150 ohms resistive.

The advantage of this latter definition is that LCL is obtained in a well-defined manner and that this LCL value gives a general indication of how well the port is balanced to Earth. However, the configuration in Figure 4 does not correspond too well with a realistic representation of actual, practical cases:

1)	The transverse terminating impedance nowadays is not always 600 ohms resistive but can be a complex, nominal impedance as given for instance in Recommendation Q.552.

2)	The longitudinal source impedance is resistive only in exceptional cases but rather like a high capacitance.

Thus, when a more accurate analysis of the actual longitudinal-to-transversal conversion is needed, either a special test setup, simulating actual conditions, has to be used and/or a more detailed circuit analysis has to be performed. In what follows, such an analysis will be made in order to illustrate the difference between a measured LCL and the actual conversion LCL.

To distinguish the cases, the Longitudinal Conversion Loss measured in the 600/150 ohms test bridge will be designated LCLm, and the Longitudinal Conversion Loss in the actual circuit LCLc.

## A.2	Calculation of the longitudinal conversion lossXE "Calculation of the longitudinal conversion loss"§

The unbalance properties of an equipment port or a network port can be described in many ways by equivalent circuits. The one shown in Figure A.1, employing an ideal choke with a tap, will be used here because it leads to simple expressions. (The factor k is a measure of the degree of unbalance.)

This equivalent circuit type can be used both for the circuit (cable or the test bridge), that introduces the longitudinal voltage, and for the terminating equipment, as Figure A.2 depicts. (Note that the cable and the test bridge are supposed to be perfectly balanced.)

FIGURE  A.1/G.117...[D15] = 7
XE "Equivalent circuit for the unbalance-to-Earth at a port"§

FIGURE  A.2/G.117...[D16] = 7
Equivalent circuit for conversion of the longitudinal voltage E1
to the tranversal voltage VtXE "Equivalent circuit for conversion of the longitudinal voltage E1
to the tranversal voltage Vt"§

Using the equivalent circuit, the following expression for the Longitudinal Conversion Loss LCLc in the general case is derived:

dB	(A-1)

Note that this equation is valid for all values of k, i.e. even if the port would be totally unbalanced. However, in general, the terminating equipment is reasonably well-balanced, i.e. k is small, so that equation A-1 can be simplified to:

dB	(A-2)

In the measuring setup for the longitudinal conversion loss we have:

ohms;	  ohms;	(A-3)

so that:

dB	(A-4)

## A.3	The difference between LCLc and LCLm in a practical case – two examples

The disturbing circuit is assumed to be a subscriber cable, perfectly balanced, and the terminating equipment the input analogue port of a digital exchange. The terminal equipment has a slight unbalance to Earth.

The equipment transversal impedance Zte is complex, and can be represented by a resistance of 275 ohms and a parallel combination of a resistance of 780 ohms and a capacitance of 150 nF. This impedance is a sufficiently close match to the input transversal impedance of the cable circuit so that Ztc = Zte.

The equipment longitudinal impedance Zle is assumed to be 300 ohms resistive.

In the first example the cable is terminated by a telephone set with a very high longitudinal impedance to Earth. Then the circuit, i.e. the cable, input longitudinal impedance Zlc can be represented by a capacitance Cs in series with a small resistance. Here Cs = 500 nF and Rs = 75 ohms are chosen values as being typical for a subscriber line.

In the second example the subscriber line is terminated by an equipment, perfectly balanced, but with a very low longitudinal impedance to Earth. Then Zlc can be considered as a pure resistance, three times higher than in the first case, i.e. Zlc = 225 ohms.

The difference (LCLc – LCLm) for the two cases is shown in Figures A.3 and A.4 respectively.

The first example: high longitudinal impedance to Earth at the far end.

The second example: low longitudinal impedance to Earth at the far end.

## A.4	Additional Insight into Longitudinal Conversion Loss

A more complete analysis of LCL differences between actual longitudinal conversion loss circuits and longitudinal conversion loss test sets has been done. Two conclusions are noteworthy:

1)	The real difference between the test set and the actual longitudinal conversion loss is rather independent of the terminal input impedance and therefore this analysis applies to virtually all reasonable exchange input impedances including 600 ohms and 900 ohms + 2.16 mF.

2)	The difference between the actual longitudinal conversion loss and the longitudinal conversion loss as measured by the test set is compensated for by the lower LCL limits at lower frequencies as noted in Recommendation Q.553. Recommendation Q.553 requires a minimum of 40 dB LCL from 300 Hz to 600 Hz and 46 dB of LCL from 600 Hz to 3400 Hz.

FIGURE  A.3/G.117...[D17] = 11.5
XE "Difference between actual longitudinal conversion loss LCLc and LCLm
measured in the standard test bridge"§

FIGURE  A.4/G.117...[D18] = 11.5

## References

[1]	CCITT Recommendation O.9 (1988), Measuring arrangements to assess the degree of unbalance about earth.

[2]	CCITT Directives concerning the protection of telecommunication lines against harmful effects from electric power and electrified ralways lines, ITU, Geneva, 1990.

[3]	ITU-R Recommendation V.574 (1990), Use of the decibel and the neper in telecommunications.

[4]	ITU-T Recommendation O.41 (1994), Psophometer for use on telephone-type circuits.