|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.215

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

INTERNATIONAL  ANALOGUE  CARRIER  SYSTEMS

GENERAL  CHARACTERISTICS  COMMON  TO  ALL ANALOGUE  CARRIER-TRANSMISSION  SYSTEMS

HYPOTHETICAL  REFERENCE  CIRCUIT  
OF  5000  KM  FOR  ANALOGUE  SYSTEMS

ITU-T  Recommendation  G.215

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.215 was published in Fascicle III.2 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.215

ascicle III.2 - Rec. G.215

HYPOTHETICAL REFERENCE CIRCUIT OF 5000 km
FOR ANALOGUE SYSTEMS

(Geneva, 1980)

1	Composition of the hypothetical reference circuit

This hypothetical reference circuit is 5000 km long and applies to various types of carrier systems on coaxial cable and radio-relay systems, specially designed for very long international circuits. It has, for each direction of transmission, a total of:

-	one pair of channel modulators which includes translation from the audio-frequency band to the basic group   and vice versa;

-	three pairs of group modulators, each pair including translation from the basic group to the basic supergroup and vice versa;

-	six pairs of supergroup modulators, each pair including translation from the basic supergroup to a higher order modem and vice versa;

-	twelve pairs of higher order modulators, each pair providing the necessary modulation stages to and from the line frequency.

Figure 1/G.215 shows the principle of the hypothetical reference circuit.

This hypothetical reference circuit consists of 12 homogeneous sections of equal length (see Recommendation G.212). Two homogeneous sections may be connected in tandem without translating equipment at the junction if the transmission system has suitable line regulating capability and does not introduce undesirable noise and crosstalk into any telephone channel.

Note - Each homogenous section has a length of approximately 420 km.

FIGURE 1/G.215

Diagram of a hypothetical reference circuit of 5000 km

# 2	Design objectives for circuit noise1)

The same noise values as for the 2500 km HRC apply (Recommendation G.222, § 1).

Note 1 - This design objective is in line with Recommendation G.123, "Circuit noise in national networks", which in § 2.1.1 recommends that the line noise in channels used to provide very long-distance circuits (over 2500 km) should not exceed 2 pW0p/km.

Note 2 - Designers are expected to fit their noise distribution curves fall below all §§ 1.1 and 1.2 of Recommendation G.222.

Note 3 - In applying these design objectives, §§ 2.4 through 2.7 of Recommendation G.222 should be taken into

account.

The subdivision of the total noise between the various sources of noise is left entirely to the designer of the system, within the limits of 2500 pWOp for the terminal equipment and 7500 pWOp for the line. This allocation is intended to permit the use of modulating equipment meeting the maximum values recommended in Table 1/G.222 of Recommendation G.222 as indicated in Table 1/G.215.

TABLE 1/G.215

| Equipment | Maximum noise contributed by the send and receive side together | Number of modulator pairs | Total noise |
| --- | --- | --- | --- |
| Channel modulators Group modulators Supergroup modulators Higher order modulators Through connecting equipment | 200 pW0p 80 pW0p 60 pW0p 120 pW0p - | 1 3 6 12 - | 200pW0p 240 pW0p 360 pW0p 1440 pW0p 260 pW0p |
|  |  | Total: | 2500 pW0p |

Note - This Table assumes two stages of modulation in the higher modulator.

1)	Although the noise objective for the 5000 km HRC is in principle agreed, some countries will not be soon in the position to install equipment of the desired performance, and will continue to use existing systems on the very long national and international circuits, according to established planning and design practices.