|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.431

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

INTERNATIONAL  ANALOGUE  CARRIER  SYSTEMS

GENERAL  CHARACTERISTICS  OF  INTERNATIONAL  CARRIER  TELEPHONE  SYSTEMS  ON  
RADIO - RELAY  OR  SATELLITE  LINKS  AND  INTERCONNECTION  WITH  METALLIC  LINES

HYPOTHETICAL  REFERENCE  CIRCUITS  FOR  FREQUENCY - DIVISION  MULTIPLEX  
RADIO - RELAY  SYSTEMS

ITU-T  Recommendation  G.431

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.431 was published in Fascicle III.2 the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.431

Fascicle III.2 - Rec. G.431

HYPOTHETICAL REFERENCE CIRCUITS FOR FREQUENCY-DIVISION

MULTIPLEX RADIO-RELAY SYSTEMS1)

(modified at Geneva, 1964)

# 1	Hypothetical reference circuit for radio-relay systems providing 12 to 60 telephone channels

The hypothetical reference circuit defined in CCIR Recommendation 391 [1], for frequency-division multiplex radio-relay systems with a capacity of 12 to 60 telephone channels per radio channel, has a length of 2500 km.

This circuit has for each direction of transmission:

-	three pairs of channel modulators,

-	six pairs of group modulators,

-	six pairs of supergroup modulators,

it being understood that a "pair of modulators" comprises a modulator and a demodulator (see Figure 1/G.431).

Note - The meaning of the symbols used in this Figure is given in the Preliminary notes to this fascicle.

FIGURE 1/G.431

Hypothetical reference circuit for frequency-division multiplex radio-relay systems

with a capacity of 12 to 60 telephone channels per radio channel

This circuit also has six sets of radio modulators and demodulators, for each direction of transmission, so that they divide the circuit into six homogeneous sections of equal length (see Recommendation G.322).

# 2	Hypothetical reference circuit for radio-relay systems providing more than 60 telephone channels

The hypothetical reference circuit defined in CCIR Recommendation 392 [2], for frequency-division multiplex radio-relay systems with a capacity of more than 60 telephone channels per radio channel, has a length of 2500 km.

This circuit has for each direction of transmission:

-	three pairs of channel modulators,

-	six pairs of group modulators,

-	nine pairs of supergroup modulators,

it being understood that a "pair of modulators" comprises a modulator and a demodulator (see Figure 2/G.431).

Note - The meaning of the symbols used in this Figure is given in the Preliminary notes to this fascicle.

FIGURE 2/G.431

Hypothetical reference circuit for frequency-division multiplex radio-relay systems

with more than 60 telephone channels per radio channel

This circuit also has nine sets of radio modulators and demodulators for each direction of transmission so that they divide the circuit into nine homogeneous sections of equal length (see Recommendation G.322).

References

[1]	CCIR Recommendation Hypothetical reference circuit for radio-relay systems for telephony using frequency-division multiplex with a capacity of 12 to 60 telephone channels, Vol. IX, Rec. 391, Dubrovnik, 1986.

[2]	CCIR Recommendation Hypothetical reference circuit for radio-relay systems for telephony using frequency-division multiplex with a capacity of more than 60 telephone channels, Vol. IX, Rec. 392, Dubrovnik, 1986.

1)	This Recommendation applies only to line-of-sight or near line-of-sight radio-relay systems.