|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

CCITT	G.774

THE  INTERNATIONAL	(09/92)
TELEGRAPH  AND  TELEPHONE
CONSULTATIVE  COMMITTEE

GENERAL  ASPECTS  OF  DIGITAL
TRANSMISSION  SYSTEMS;

TERMINAL  EQUIPMENTS

SYNCHRONOUS  DIGITAL  HIERARCHY  (SDH)
MANAGEMENT  INFORMATION  MODEL
FOR  THE  NETWORK  ELEMENT  VIEW

|  | Recommendation G.774 |
| --- | --- |

# FOREWORD

The CCITT (the International Telegraph and Telephone Consultative Committee) is a permanent organ of the International Telecommunication Union (ITU). CCITT is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The Plenary Assembly of CCITT which meets every four years, establishes the topics for study and approves Recommendations prepared by its Study Groups. The approval of Recommendations by the members of CCITT between Plenary Assemblies is covered by the procedure laid down in CCITT Resolution No. 2 (Melbourne, 1988).

Recommendation G.774 was prepared by Study Group XV and was approved under the Resolution No. 2 procedure on the 1st of September 1992.

___________________

# CCITT  NOTES

1)	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized private operating agency.

2)	A list of abbreviations used in this Recommendation can be found in Annex B.

ã  ITU  1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

PAGE  BLANCHE

Recommendation G.774

Recommendation G.774     (09/92)

SYNCHRONOUS  DIGITAL  HIERARCHY  (SDH)  MANAGEMENT  INFORMATION 
MODEL  FOR  THE  NETWORK  ELEMENT  VIEW

(1992)

The CCITT,

considering

(a) that Recommendations G.707, G.708 and G.709 form a coherent set of specifications for the synchronous digital hierarchy (SDH) and the network node interface (NNI);

(b) that Recommendations G.781, G,782, G.783, and G.784 form a coherent set of specifications for SDH multiplex equipment functions and management;

(c) that Recommendation G.958 specifies the characteristics of digital line systems based on SDH for use on optical fibre cables;

(d) that Recommendation M.3010 defines the principles for a telecommunications management network (TMN);

(e) that Recommendation G.773 defines the protocol suites for Q-interfaces for management of transmission systems;

(f) that Recommendation M.3100 defines a Generic Network Information Model,

recommends

that the management of SDH equipment be carried out by using the information model defined in accordance with the details contained within this Recommendation.

# Abstract

This Recommendation provides an information model for the synchronous digital hierarchy (SDH).  This model describes the managed object classes and their properties that are useful to describe information exchanged across interfaces defined in M.3010 telecommunications management network (TMN) architecture.  This Recommendation specializes the generic object classes of Recommendation M.3100 to provide management information specifically for the SDH.

Keywords: synchronous digital hierarchy, information model, managed object class, attributes, notification, action, GDMO, ASN.1.

# 1	Introduction

## 1.1	Scope

This Recommendation provides an information model for the synchronous digital hierarchy (SDH) [1-3]. It identifies the telecommunications management network (TMN) object classes required for the management of SDH network elements. These objects are relevant to information exchanged across standardized interfaces defined in the Recommendation M.3010 TMN architecture [4]. The managed object classes in this Recommendation are specialized from the generic managed object classes defined in Recommendation M.3100 generic network information model [5].

This Recommendation applies to SDH network elements and those systems in the TMN that manage SDH network elements. Functional capabilities of SDH multiplex equipment are given in Recommendation G.783 [6], and aspects of the management of SDH equipment are provided in Recommendation G.784 [7]. This Recommendation provides the management information required for use with the protocols specified in Recommendation G.784.

## 1.2	Structure of this Recommendation

Section 2 provides an overview of the SDH information model. Sections 3 to 6 describe the information model using the notation mechanisms defined in Recommendation X.722 Guidelines for the definition of managed Objects [8]. Section 7 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in Recommendation X.208 [9]. The relationships between the SDH managed object classes contained in this Recommendation are defined in § 8. Diagrams illustrating the construction of the SDH model are provided in Annex A.

A text version of § 3 to § 8 is available in diskette form from the ITU.

# 2.	SDH information model

## 2.1	Overview

The SDH information model is based on the Generic Network Information Model of Recommendation M.3100. The Generic Network Information Model includes a Termination Point fragment which serves as a structure for specialization of those object classes specific to the SDH network. It is these SDH specific object classes, along with the generic object classes in other fragments of the Generic Network Information Model (e.g. the cross-connection fragment and the equipment fragment), that are used to manage SDH network elements. The services used to manage the SDH resources represented by these object classes are provided in M.3100 and other Recommendations.

The information exchanged at a management interface is modelled using design principles outlined in Recommendation X.720 Management Information Model [10]. Resources are modelled as objects, and the management view of a resource is a managed object. Objects with similar attributes may be grouped into object classes. An object is characterized by its object class and object instance, and may possess multiple attribute types and associated values. The terms “managed object class” and “managed object instance” apply specifically to objects that are being managed. This Recommendation specifies the properties of the resource visible for management.

An object class may be a subclass of another class. A subclass inherits attribute types, packages and behaviours of its superclass, in addition to possessing its own specific attributes and properties. The SDH specific object classes are all derived from superclasses in the Generic Network Information Model Recommendation M.3100.

Object classes and attribute types are defined only for the purpose of communicating network management messages between systems, and need not be related to the structure of data within those systems. The object classes defined in this issue of the SDH information model can apply to multiple management functional areas (e.g. fault management and configuration management).

There are several different viewpoints of management information that may be defined for management purposes. The network element viewpoint is concerned with the information that is required to manage a network element. This refers to information required to manage the network element function and the physical aspects of the network element.  This Recommendation addresses only the network element viewpoint of SDH management.

## 2.2	Requirements

To allow SDH equipment to be represented in a consistent manner across the interface some of the conditional packages in M.3100 are made mandatory in this Recommendation, the following conditional packages inherited from M.3100 shall not be used when the SDH object classes defined in this Recommendation are   instantiated: ttpInstancePackage, ctpInstancePackage, networkLevelPackage, characteristicInformationPackage, channelNumberPackage.

The SDH specific subclasses specified in this Recommendation shall be used to manage the specific transport resources of SDH network elements. Implementations shall conform to both the management information defined in §§ 3 to 7 and the requirements identified in §§ 2.2 and 8.

# 3	Object classes

Recommendation G.774     (09/92)

## Administrative Unit 3 Object Classes

xe "Object Classes:au3CTPBidirectional" \b ""§au3CTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

au3CTPSink,

au3CTPSource;

REGISTERED AS { g774ObjectClass 1 };

xe "Object Classes:au3CTPSink" \b ""§au3CTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

au3CTPSinkPkg PACKAGE

BEHAVIOUR

au3CTPSinkPkgBehaviour BEHAVIOUR

DEFINED AS

This object class represents a termination point where an AU-3 Connection is terminated.

The AU-3 consists of a VC-3 plus an AU pointer which indicates the phase alignment of the VC-3 with respect to the STM-N frame.

A communicationsAlarm notification shall be issued if a loss of AU pointer is detected. The probableCause parameter of the notification shall indicate LOP (Loss of Pointer).

A communicationsAlarm notification shall be issued if an AU path alarm indication signal is detected. The probableCause parameter of the notification shall indicate AIS (Alarm Indication Signal).

;;

ATTRIBUTES

xe "Attributes:au3CTPId"§au3CTPId		GET,

xe "Attributes:pointerSinkType"§pointerSinkType	GET;

;;

REGISTERED AS { g774ObjectClass 2 };

xe "Object Classes:au3CTPSource" \b ""§au3CTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

au3CTPSourcePkg PACKAGE

BEHAVIOUR

au3CTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

This object class represents a termination point where an AU-3 Connection is originated.

The AU-3 consists of a VC-3 plus an AU pointer which indicates the phase alignment of the VC-3 with respect to the STM-N frame.

;;

ATTRIBUTES

xe "Attributes:au3CTPId"§au3CTPId				GET,

xe "Attributes:pointerSourceType"§pointerSourceType	GET;

;;

REGISTERED AS { g774ObjectClass 3 };

## Aministrative Unit 4 Object Classes

xe "Object Classes:au4CTPBidirectional" \b ""§au4CTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

au4CTPSink,

au4CTPSource;

REGISTERED AS { g774ObjectClass 4 };

xe "Object Classes:au4CTPSink" \b ""§au4CTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

au4CTPSinkPkg PACKAGE

BEHAVIOUR

au4CTPSinkPkgBehaviour BEHAVIOUR

DEFINED AS

This object class represents a termination point where an AU-4 Connection is terminated.

The AU-4 consists of a VC-4 plus an AU pointer which indicates the phase alignment of the VC-4 with respect to the STM-N frame.

A communicationsAlarm notification shall be issued if a loss of AU pointer is detected. The probableCause parameter of the notification shall indicate LOP (Loss of Pointer).

A communicationsAlarm notification shall be issued if an AU path alarm indication signal is detected. The probableCause parameter of the notification shall indicate AIS (Alarm Indication Signal).

;;

ATTRIBUTES

xe "Attributes:au4CTPId"§au4CTPId		GET,

xe "Attributes:pointerSinkType"§pointerSinkType	GET;

;;

REGISTERED AS { g774ObjectClass 5 };

xe "Object Classes:au4CTPSource" \b ""§au4CTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

au4CTPSourcePkg PACKAGE

BEHAVIOUR

au4CTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

This object class represents a termination point where an AU-4 Connection is originated.

The AU-4 consists of a VC-4 plus an AU pointer which indicates the phase alignment of the VC-4 with respect to the STM-N frame.

;;

ATTRIBUTES

xe "Attributes:au4CTPId"§au4CTPId		GET,

xe "Attributes:pointerSourceType"§pointerSourceType	GET;

;;

REGISTERED AS { g774ObjectClass 6 };

## Administrative Unit Group Object Classes

xe "Object Classes:augBidirectional" \b ""§augBidirectional MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorBidirectional,

augSink,

augSource;

REGISTERED AS { g774ObjectClass 7 };xe "Object Classes:augSink" \b ""§

augSink MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorSink;

CHARACTERIZED BY

augSinkPkg PACKAGE

BEHAVIOUR

augSinkPkgBehaviour BEHAVIOUR

DEFINED AS

This object class is instantiated if AU-n Connection(s) are being terminated.

An AUG consists of a homogeneous, byte interleaved, assembly of either three AU-3s or one AU-4.

This object class represents the point at which the AU-3/4 pointer is derived, based on the phase of the VC-3/4 POH relative to the STM-N SOH. Also, the STM-N payload is byte-demultiplexed into it’s component AU Groups (AUGs).

;;

ATTRIBUTES

xe "Attributes:augId"§augId				GET,

xe "Attributes:supportableClientList"§"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 8 };

xe "Object Classes:augSource" \b ""§augSource MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorSource;

CHARACTERIZED BY

augSourcePkg PACKAGE

BEHAVIOUR

augSourcePkgBehaviour BEHAVIOUR

DEFINED AS

This object class is instantiated if AU-n Connection(s) are being originated.

An AUG consists of a homogeneous, byte interleaved, assembly of either three AU-3s or one AU-4.

This object class represents the point at which the AU-3/4 pointer is generated to indicate the phase of the VC-3/4 POH relative to the STM-N SOH. Also, the AU Groups (AUGs) are byte-multiplexed to construct the complete STM-N frame.

;;

ATTRIBUTES

xe "Attributes:augId"§augId			GET,

xe "Attributes:supportableClientList"§"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 9 };

## Electrical SPI trail termination point object classes

xe "Object Classes:electricalSPITTPBidirectional" \b ""§electricalSPITTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointBidirectional,

electricalSPITTPSink,

electricalSPITTPSource;

REGISTERED AS { g774ObjectClass 10 };

xe "Object Classes:electricalSPITTPSink" \b ""§electricalSPITTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

electricalSPIPackage,

electricalSPITTPSinkPkg PACKAGE

BEHAVIOUR

electricalSPITTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents the point where the incoming electrical interface signal is converted into an internal logic level and the timing is recovered from the line signal.

The upstream connectivity pointer is NULL for an instance of this class.

A communicationsAlarm notification shall be issued if a loss of signal is detected. The probableCause parameter of the notification shall indicate LOS (Loss of signal).

;;;;

REGISTERED AS { g774ObjectClass 11 };

xe "Object Classes:electricalSPITTPSource" \b ""§electricalSPITTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

electricalSPIPackage,

electricalSPITTPSourcePkg PACKAGE

BEHAVIOUR

electricalSPITTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents the point at which an outgoing internal logic level STM-N signal is converted into a STM-N in-station electrical interface signal.

The downstream connectivity pointer is NULL for an instance of this class.

;;;;

REGISTERED AS { g774ObjectClass 12 };

## Indirect Adaptor Object Classes

xe "Object Classes:indirectAdaptorBidirectional" \b ""§indirectAdaptorBidirectional MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorSink,

indirectAdaptorSource;

REGISTERED AS { g774ObjectClass 13 };

xe "Object Classes:indirectAdaptorSink" \b ""§indirectAdaptorSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721":top;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

indirectAdaptorSinkPkg PACKAGE

BEHAVIOUR

indirectAdaptorSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class provides a naming mechanism which describes the multiplexing hierarchy of the SDH signal.

;;;;

REGISTERED AS { g774ObjectClass 14 };

xe "Object Classes:indirectAdaptorSource" \b ""§indirectAdaptorSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721":top;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

indirectAdaptorSourcePkg PACKAGE

BEHAVIOUR

indirectAdaptorSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class provides a naming mechanism which describes the multiplexing hierarchy of the SDH signal.

;;;;

REGISTERED AS { g774ObjectClass 15 };

## Multiplex Section Connection Termination Point Object Classes

xe "Object Classes:msCTPBidirectional" \b ""§msCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

msCTPSink,

msCTPSource;

REGISTERED AS { g774ObjectClass 16 };

xe "Object Classes:msCTPSink" \b ""§msCTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msCTPPackage,

msCTPSinkPkg PACKAGE

BEHAVIOUR

msCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates a multiplex section connection.

;;;;

REGISTERED AS { g774ObjectClass 17 };

xe "Object Classes:msCTPSource" \b ""§msCTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msCTPPackage,

msCTPSourcePkg PACKAGE

BEHAVIOUR

msCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates a multiplex section connection.

;;;;

REGISTERED AS { g774ObjectClass 18 };

## Multiplex Section Data Communications Channel Object Classes

xe "Object Classes:msDatacomCTPBidirectional" \b ""§msDatacomCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

msDatacomCTPSink,

msDatacomCTPSource;

REGISTERED AS { g774ObjectClass 19 };

xe "Object Classes:msDatacomCTPSink" \b ""§msDatacomCTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msDatacomCTPSinkPkg PACKAGE

BEHAVIOUR

msDatacomCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates the D4-D12 bytes in the MSOH.

;;

ATTRIBUTES

xe "Attributes:msDatacomCTPId"§msDatacomCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 20 };

xe "Object Classes:msDatacomCTPSource" \b ""§msDatacomCTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msDatacomCTPSourcePkg PACKAGE

BEHAVIOUR

msDatacomCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates the D4-D12 bytes in the MSOH.

;;

ATTRIBUTES

xe "Attributes:msDatacomCTPId"§msDatacomCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 21 };

## Multiplex Section Orderwire Object Classes

xe "Object Classes:msOrderwireCTPBidirectional" \b ""§msOrderwireCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

msOrderwireCTPSink,

msOrderwireCTPSource;

REGISTERED AS { g774ObjectClass 22 };

xe "Object Classes:msOrderwireCTPSink" \b ""§msOrderwireCTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msOrderwireCTPSinkPkg PACKAGE

BEHAVIOUR

msOrderwireCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents a termination point where the E2 byte orderwire channel is terminated.

;;

ATTRIBUTES

xe "Attributes:msOrderwireCTPId"§msOrderwireCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 23 };

xe "Object Classes:msOrderwireCTPSource" \b ""§msOrderwireCTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msOrderwireCTPSourcePkg PACKAGE

BEHAVIOUR

msOrderwireCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents a termination point where the E2 byte orderwire channel is originated.

;;

ATTRIBUTES

xe "Attributes:msOrderwireCTPId"§msOrderwireCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 24 };

## Multiplex Section Trail Termination Point Object Classes

xe "Object Classes:msTTPBidirectional" \b ""§msTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointBidirectional,

msTTPSink,

msTTPSource;

CHARACTERIZED BY

msTTPBidirectionalPkg PACKAGE

BEHAVIOUR

msTTPBidirectionalBehaviourPkg BEHAVIOUR

DEFINED AS

When the excessiveBERMtceInhibit attribute is set to TRUE, MS-FERF is not inserted upstream upon detection of excessive BER.

A communicationsAlarm notification shall be issued if a far end receive failure is detected. The probableCause parameter of the notification shall indicate FERF (Far End Receive Failure).

;;;;

REGISTERED AS { g774ObjectClass 25 };

xe "Object Classes:msTTPSink" \b ""§msTTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

msTTPPackage,

msTTPSinkPkg PACKAGE

BEHAVIOUR

msTTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates a multiplex section trail, i.e. the processing and removal of the multiplex section overhead from the incoming signal.

When the excessiveBERMtceInhibit attribute is set to TRUE, AIS is not inserted downstream upon detection of excessive BER.

A communicationsAlarm notification shall be issued if an excessive bit error rate is detected. The probableCause parameter of the notification shall indicate excessive BER.

A communicationsAlarm notification shall be issued if a degraded signal is detected. The probableCause parameter of the notification shall indicate signal degrade.

A communicationsAlarm notification shall be issued if an MS alarm indication signal is detected. The probableCause parameter of the notification shall indicate AIS (Alarm Indication Signal).

;;

ATTRIBUTES

xe "Attributes:excessiveBERMtceInhibit"§excessiveBERMtceInhibit	GET-REPLACE,

xe "Attributes:signalDegradeThreshold"§signalDegradeThreshold	GET-REPLACE;

;;

REGISTERED AS { g774ObjectClass 26 };

xe "Object Classes:msTTPSource" \b ""§msTTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

msTTPPackage,

msTTPSourcePkg PACKAGE

BEHAVIOUR

msTTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates a multiplex section trail, i.e. the generation and addition of the multiplex section overhead to the outgoing signal.

;;;;

REGISTERED AS { g774ObjectClass 27 };

## Optical SDH Physical Interface Trail Termination Point Object Classes

xe "Object Classes:opticalSPITTPBidirectional" \b ""§opticalSPITTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointBidirectional,

opticalSPITTPSink,

opticalSPITTPSource;

REGISTERED AS { g774ObjectClass 28 };

xe "Object Classes:opticalSPITTPSink" \b ""§opticalSPITTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

opticalSPIPackage,

opticalSPITTPSinkPkg PACKAGE

BEHAVIOUR

opticalSPITTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents the point where the incoming optical interface signal is converted into an internal logic level and the timing is recovered from the line signal.

A communicationsAlarm notification shall be issued if a loss of signal is detected. The probableCause parameter of the notification shall indicate LOS (Loss of signal).

;;;;

REGISTERED AS { g774ObjectClass 29 };

xe "Object Classes:opticalSPITTPSource" \b ""§opticalSPITTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

opticalSPIPackage,

opticalSPITTPSourcePkg PACKAGE

BEHAVIOUR

opticalSPITTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents the point at which an outgoing internal logic level STM-N signal is converted into a STM-N in-station or inter-station optical interface signal.

A communicationsAlarm notification shall be issued if the transmit laser fails. The probableCause parameter of the notification shall indicate TransmitFail.

;;;;

REGISTERED AS { g774ObjectClass 30 };

## Regenerator Section Connection Termination Point Object Classes

xe "Object Classes:rsCTPBidirectional" \b ""§rsCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

rsCTPSink,

rsCTPSource;

REGISTERED AS { g774ObjectClass 31 };

xe "Object Classes:rsCTPSink" \b ""§rsCTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsCTPPackage,

rsCTPSinkPkg PACKAGE

BEHAVIOUR

rsCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates an regenerator section connection.

;;;;

REGISTERED AS { g774ObjectClass 32 };

xe "Object Classes:rsCTPSource" \b ""§rsCTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsCTPPackage,

rsCTPSourcePkg PACKAGE

BEHAVIOUR

rsCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates a regenerator section connection.

;;;;

REGISTERED AS { g774ObjectClass 33 };

## Regenerator Section Data Communications Channel Object Classes

xe "Object Classes:rsDatacomCTPBidirectional" \b ""§rsDatacomCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

rsDatacomCTPSink,

rsDatacomCTPSource;

REGISTERED AS { g774ObjectClass 34 };

xe "Object Classes:rsDatacomCTPSink" \b ""§rsDatacomCTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsDatacomCTPSinkPkg PACKAGE

BEHAVIOUR

rsDatacomCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates the D1-D3 bytes in the RSOH.

;;

ATTRIBUTES

xe "Attributes:rsDatacomCTPId"§rsDatacomCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 35 };

xe "Object Classes:rsDatacomCTPSource" \b ""§rsDatacomCTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsDatacomCTPSourcePkg PACKAGE

BEHAVIOUR

rsDatacomCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates the D1-D3 bytes in the RSOH.

;;

ATTRIBUTES

xe "Attributes:rsDatacomCTPId"§rsDatacomCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 36 };

## Regenerator Section Orderwire Object Classes

xe "Object Classes:rsOrderwireCTPBidirectional" \b ""§rsOrderwireCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

rsOrderwireCTPSink,

rsOrderwireCTPSource;

REGISTERED AS { g774ObjectClass 37 };

xe "Object Classes:rsOrderwireCTPSink" \b ""§rsOrderwireCTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsOrderwireCTPSinkPkg PACKAGE

BEHAVIOUR

rsOrderwireCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents the point where the E1 byte orderwire channel is terminated.

;;

ATTRIBUTES

xe "Attributes:rsOrderwireCTPId"§rsOrderwireCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 38 };

xe "Object Classes:rsOrderwireCTPSource" \b ""§rsOrderwireCTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsOrderwireCTPSourcePkg PACKAGE

BEHAVIOUR

rsOrderwireCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents the point where the E1 byte orderwire channel is originated.

;;

ATTRIBUTES

xe "Attributes:rsOrderwireCTPId"§rsOrderwireCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 39 };

## Regenerator Section Trail Termination Point Object Classes

xe "Object Classes:rsTTPBidirectional" \b ""§rsTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointBidirectional,

rsTTPSink,

rsTTPSource;

REGISTERED AS { g774ObjectClass 40 };

xe "Object Classes:rsTTPSink" \b ""§rsTTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

rsTTPPackage,

rsTTPSinkPkg PACKAGE

BEHAVIOUR

rsTTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents the termination of the regenerator section trail, i.e. the processing and removal of the regenerator section overhead from the incoming signal and the descrambling of that signal.

A communicationsAlarm notification shall be issued if a loss of frame is detected. The probableCause parameter of the notification shall indicate LOF (Loss of Frame).

;;;;

REGISTERED AS { g774ObjectClass 41 };

xe "Object Classes:rsTTPSource" \b ""§rsTTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

rsTTPPackage,

rsTTPSourcePkg PACKAGE

BEHAVIOUR

rsTTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represent the origination of the regenerator section trail, i.e. generates the regenerator section overhead for the outgoing signal, and scrambles that signal.

;;;;

REGISTERED AS { g774ObjectClass 42 };

## Regenerator Section User Channel Object Classes

xe "Object Classes:rsUserChannelCTPBidirectional" \b ""§rsUserChannelCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

rsUserChannelCTPSink,

rsUserChannelCTPSource;

REGISTERED AS { g774ObjectClass 43 };

xe "Object Classes:rsUserChannelCTPSink" \b ""§rsUserChannelCTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsUserChannelCTPSinkPkg PACKAGE

BEHAVIOUR

rsUserChannelCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents a class of objects that terminates the F1 byte user channel.

;;

ATTRIBUTES

xe "Attributes:rsUserChannelCTPId"§rsUserChannelCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 44 };

xe "Object Classes:rsUserChannelCTPSource" \b ""§rsUserChannelCTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsUserChannelCTPSourcePkg PACKAGE

BEHAVIOUR

rsUserChannelCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class represents a class of objects that originates the F1 byte user channel.

;;

ATTRIBUTES

xe "Attributes:rsUserChannelCTPId"§rsUserChannelCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 45 };

## SDH Network Element Object Class

xe "Object Classes:sdhNE" \b ""§sdhNE MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":managedElement;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

sdhNEPackage PACKAGE

BEHAVIOUR

sdhNEBehaviour BEHAVIOUR

DEFINED AS

An SDH network element is a system which has no internal open-system interfaces.

;;

;;

REGISTERED AS { g774ObjectClass 46 };

## Tributary Unit 11 Object Classes

xe "Object Classes:tu11CTPBidirectional" \b ""§tu11CTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

tu11CTPSink,

tu11CTPSource;

REGISTERED AS { g774ObjectClass 47 };

xe "Object Classes:tu11CTPSink" \b ""§tu11CTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

tu-nSinkPackage,

tu11CTPSinkPkg PACKAGE

BEHAVIOUR

tu11CTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates a tu-11 connection.

;;

ATTRIBUTES

xe "Attributes:tu11CTPId"§tu11CTPId		GET;

;;

REGISTERED AS { g774ObjectClass 48 };

xe "Object Classes:tu11CTPSource" \b ""§tu11CTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

tu11CTPSourcePkg PACKAGE

BEHAVIOUR

tu11CTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates a tu-11 connection.

;;

ATTRIBUTES

xe "Attributes:tu11CTPId"§tu11CTPId		GET,

xe "Attributes:pointerSourceType"§pointerSourceType	GET;

;;

REGISTERED AS { g774ObjectClass 49 };

## Tributary Unit 12 Object Classes

xe "Object Classes:tu12CTPBidirectional" \b ""§tu12CTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

tu12CTPSink,

tu12CTPSource;

REGISTERED AS { g774ObjectClass 50 };

xe "Object Classes:tu12CTPSink" \b ""§tu12CTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

tu-nSinkPackage,

tu12CTPSinkPkg PACKAGE

BEHAVIOUR

tu12CTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates a tu-12 connection.

;;

ATTRIBUTES

xe "Attributes:tu12CTPId"§tu12CTPId		GET;

;;

REGISTERED AS { g774ObjectClass 51 };

xe "Object Classes:tu12CTPSource" \b ""§tu12CTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

tu12CTPSourcePkg PACKAGE

BEHAVIOUR

tu12CTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates a tu-12 connection.

;;

ATTRIBUTES

xe "Attributes:tu12CTPId"§tu12CTPId		GET,

xe "Attributes:pointerSourceType"§pointerSourceType	GET;

;;

REGISTERED AS { g774ObjectClass 52 };

## Tributary Unit 2 Object Classes

xe "Object Classes:tu2CTPBidirectional" \b ""§tu2CTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

tu2CTPSink,

tu2CTPSource;

REGISTERED AS { g774ObjectClass 53 };

xe "Object Classes:tu2CTPSink" \b ""§tu2CTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

tu-nSinkPackage,

tu2CTPSinkPkg PACKAGE

BEHAVIOUR

tu2CTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates a tu-2 connection.

;;

ATTRIBUTES

xe "Attributes:tu2CTPId"§tu2CTPId		GET;

;;

REGISTERED AS { g774ObjectClass 54 };

xe "Object Classes:tu2CTPSource" \b ""§tu2CTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

tu2CTPSourcePkg PACKAGE

BEHAVIOUR

tu2CTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates a tu-2 connection.

;;

ATTRIBUTES

xe "Attributes:tu2CTPId"§tu2CTPId		GET,

xe "Attributes:pointerSourceType"§pointerSourceType	GET;

;;

REGISTERED AS { g774ObjectClass 55 };

## Tributary Unit 3 Object Classes

xe "Object Classes:tu3CTPBidirectional" \b ""§tu3CTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

tu3CTPSink,

tu3CTPSource;

REGISTERED AS { g774ObjectClass 56 };

xe "Object Classes:tu3CTPSink" \b ""§tu3CTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

tu-nSinkPackage,

tu3CTPSinkPkg PACKAGE

BEHAVIOUR

tu3CTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates a tu-3 connection.

;;

ATTRIBUTES

xe "Attributes:tu3CTPId"§tu3CTPId		GET;

;;

REGISTERED AS { g774ObjectClass 57 };

xe "Object Classes:tu3CTPSource" \b ""§tu3CTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

tu3CTPSourcePkg PACKAGE

BEHAVIOUR

tu3CTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates a tu-3 connection.

;;

ATTRIBUTES

xe "Attributes:tu3CTPId"§tu3CTPId		GET,

xe "Attributes:pointerSourceType"§pointerSourceType	GET;

;;

REGISTERED AS { g774ObjectClass 58 };

## Tributary Unit Group 2 Object Classes

xe "Object Classes:tug2Bidirectional" \b ""§tug2Bidirectional MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorBidirectional,

tug2Sink,

tug2Source;

REGISTERED AS { g774ObjectClass 59 };

xe "Object Classes:tug2Sink" \b ""§tug2Sink MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorSink;

CHARACTERIZED BY

tug2SinkPkg PACKAGE

BEHAVIOUR

tug2SinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class is instantiated if TU-11, TU-12, TU-2 connection(s) are being terminated or originated.

A TUG-2 consists of a homogeneous or heterogeneous, assembly of four TU-11s, three TU-12s or one TU-2.

This object class represents the point at which the TU pointer is derived, based on the phase of the VC 11/12/2 POH relative to the VC-3/4 POH and the VC-3/4 signal is disassembled.

;;

ATTRIBUTES

xe "Attributes:tug2Id"§tug2Id			GET,

xe "Attributes:supportableClientList"§"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 60 };

xe "Object Classes:tug2Source" \b ""§tug2Source MANAGED OBJECT CLASS

DERIVED FROM	 indirectAdaptorSource;

CHARACTERIZED BY

tug2SourcePkg PACKAGE

BEHAVIOUR

tug2SourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class is instantiated if TU-11, TU-12, TU-2 connection(s) are being terminated or originated.

A TUG-2 consists of a homogeneous or heterogeneous, assembly of four TU-11s, three TU-12s or one TU-2.

This object class represents the point at which the TU pointer is processed to indicate the phase of the VC 11/12/2 POH relative to the VC-3/4 POH and assembles the complete VC-3/4.

;;

ATTRIBUTES

xe "Attributes:tug2Id"§tug2Id			GET,

xe "Attributes:supportableClientList"§"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 61 };

## Tributary Unit Group 3 Object Classes

xe "Object Classes:tug3Bidirectional" \b ""§tug3Bidirectional MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorBidirectional,

tug3Sink,

tug3Source;

REGISTERED AS { g774ObjectClass 62 };

xe "Object Classes:tug3Sink" \b ""§tug3Sink MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorSink;

CHARACTERIZED BY

tug3SinkPkg PACKAGE

BEHAVIOUR

tug3SinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class is instantiated if TU-3 connection(s) are being terminated. Or if TU-11s, TU-12s, or TU-2s are being terminated from a VC-4.

A TUG-3 consists of a homogeneous assembly of seven TUG-2s or one TU-3.

;;

ATTRIBUTES

xe "Attributes:tug3Id"§tug3Id			GET,

xe "Attributes:supportableClientList"§"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 63 };

xe "Object Classes:tug3Source" \b ""§tug3Source MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorSource;

CHARACTERIZED BY

tug3SourcePkg PACKAGE

BEHAVIOUR

tug3SourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class is instantiated if TU-3 connection(s) are being originated. Or TU-11s, TU-12s, or TU-2s are being combined to form a VC-4.

A TUG-3 consists of a homogeneous assembly of seven TUG-2s or one TU-3.

;;

ATTRIBUTES

xe "Attributes:tug3Id"§tug3Id			GET,

xe "Attributes:supportableClientList"§"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 64 };

## Virtual Container 11 Object Classes

xe "Object Classes:vc11TTPBidirectional" \b ""§vc11TTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointBidirectional,

vc11TTPSink,

vc11TTPSource;

CHARACTERIZED BY

vc11-2BidirectionalPackage;

REGISTERED AS { g774ObjectClass 65 };

xe "Object Classes:vc11TTPSink" \b ""§vc11TTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc11-2SinkPackage,

vc11TTPSinkPkg PACKAGE

BEHAVIOUR

vc11TTPSinkPkgBehaviour BEHAVIOUR

DEFINED AS

This object class terminates a vc11 trail, i.e. the point at which the VC11-POH is extracted from the STM-N frame.

;;

ATTRIBUTES

xe "Attributes:vc11TTPId"§vc11TTPId		GET;

;;

REGISTERED AS { g774ObjectClass 66 };

xe "Object Classes:vc11TTPSource" \b ""§vc11TTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc11TTPSourcePkg PACKAGE

BEHAVIOUR

vc11TTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

This object class originates a vc11 trail, i.e. the point at which the VC11-POH is added to the STM-N frame.

;;

ATTRIBUTES

xe "Attributes:vc11TTPId"§vc11TTPId		GET,

xe "Attributes:v5SignalLabelSend"§v5SignalLabelSend	GET;

;;

REGISTERED AS { g774ObjectClass 67 };

## Virtual Container 12 Object Classes

xe "Object Classes:vc12TTPBidirectional" \b ""§vc12TTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointBidirectional,

vc12TTPSink,

vc12TTPSource;

CHARACTERIZED BY

vc11-2BidirectionalPackage;

REGISTERED AS { g774ObjectClass 68 };

xe "Object Classes:vc12TTPSink" \b ""§vc12TTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc11-2SinkPackage,

vc12TTPSinkPkg PACKAGE

BEHAVIOUR

vc12TTPSinkPkgBehaviour BEHAVIOUR

DEFINED AS

This object class terminates a vc12 trail, i.e. the point at which the VC12-POH is extracted from the STM-N frame.

;;

ATTRIBUTES

xe "Attributes:vc12TTPId"§vc12TTPId		GET;

;;

REGISTERED AS { g774ObjectClass 69 };

xe "Object Classes:vc12TTPSource" \b ""§vc12TTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc12TTPSourcePkg PACKAGE

BEHAVIOUR

vc12TTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

This object class originates a vc12 trail, i.e. the point at which the VC12-POH is added to the STM-N frame.

;;

ATTRIBUTES

xe "Attributes:vc12TTPId"§vc12TTPId		GET,

xe "Attributes:v5SignalLabelSend"§v5SignalLabelSend	GET;

;;

REGISTERED AS { g774ObjectClass 70 };

## Virtual Container 2 Object Classes

xe "Object Classes:vc2TTPBidirectional" \b ""§vc2TTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointBidirectional,

vc2TTPSink,

vc2TTPSource;

CHARACTERIZED BY

vc11-2BidirectionalPackage;

REGISTERED AS { g774ObjectClass 71 };

xe "Object Classes:vc2TTPSink" \b ""§vc2TTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc11-2SinkPackage,

vc2TTPSinkPkg PACKAGE

BEHAVIOUR

vc2TTPSinkPkgBehaviour BEHAVIOUR

DEFINED AS

This object class terminates a vc2 trail, i.e. the point at which the VC2-POH is extracted from the STM-N frame.

;;

ATTRIBUTES

xe "Attributes:vc2TTPId"§vc2TTPId		GET;

;;

REGISTERED AS { g774ObjectClass 72 };

xe "Object Classes:vc2TTPSource" \b ""§vc2TTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc2TTPSourcePkg PACKAGE

BEHAVIOUR

vc2TTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

This object class originates a vc2 trail, i.e. the point at which the VC2-POH is added to the STM-N frame.

;;

ATTRIBUTES

xe "Attributes:vc2TTPId"§vc2TTPId		GET,

xe "Attributes:v5SignalLabelSend"§v5SignalLabelSend	GET;

;;

REGISTERED AS { g774ObjectClass 73 };

## Virtual Container 3 Object Classes

xe "Object Classes:vc3TTPBidirectional" \b ""§vc3TTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointBidirectional,

vc3TTPSink,

vc3TTPSource;

CHARACTERIZED BY

vc3-4BidirectionalPackage;

REGISTERED AS { g774ObjectClass 74 };

xe "Object Classes:vc3TTPSink" \b ""§vc3TTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc3-4SinkPackage,

vc3TTPSinkPkg PACKAGE

BEHAVIOUR

vc3TTPSinkPkgBehaviour BEHAVIOUR

DEFINED AS

This object class terminates a vc3 trail, i.e. the point at which the SDH VC-3 is terminated.

;;

ATTRIBUTES

xe "Attributes:vc3TTPId"§vc3TTPId		GET;

;;

REGISTERED AS { g774ObjectClass 75 };

xe "Object Classes:vc3TTPSource" \b ""§vc3TTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc3-4SourcePackage,

vc3TTPSourcePkg PACKAGE

BEHAVIOUR

vc3TTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

This object class originates a vc3 trail, i.e. the point at which the SDH VC-3 is originated.

;;

ATTRIBUTES

xe "Attributes:vc3TTPId"§vc3TTPId		GET;

;;

REGISTERED AS { g774ObjectClass 76 };

## Virtual Container 4 Object Classes

xe "Object Classes:vc4TTPBidirectional" \b ""§vc4TTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointBidirectional,

vc4TTPSink,

vc4TTPSource;

CHARACTERIZED BY

vc3-4BidirectionalPackage;

REGISTERED AS { g774ObjectClass 77 };

xe "Object Classes:vc4TTPSink" \b ""§vc4TTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc3-4SinkPackage,

vc4TTPSinkPkg PACKAGE

BEHAVIOUR

vc4TTPSinkPkgBehaviour BEHAVIOUR

DEFINED AS

This object class terminates a vc4 trail, i.e. the point at which the SDH VC-4 is terminated.

;;

ATTRIBUTES

xe "Attributes:vc4TTPId"§vc4TTPId		GET;

;;

REGISTERED AS { g774ObjectClass 78 };

xe "Object Classes:vc4TTPSource" \b ""§vc4TTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc3-4SourcePackage,

vc4TTPSourcePkg PACKAGE

BEHAVIOUR

vc3-4TTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

This object class originates a vc4 trail, i.e. the point at which the SDH VC-4 is originated.

;;

ATTRIBUTES

xe "Attributes:vc4TTPId"§vc4TTPId		GET;

;;

REGISTERED AS { g774ObjectClass 79 };

## VC-n User Channel Object Classes

xe "Object Classes:vcnUserChannelCTPBidirectional" \b ""§vcnUserChannelCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointBidirectional,

vcnUserChannelCTPSink,

vcnUserChannelCTPSource;

REGISTERED AS { g774ObjectClass 80 };

xe "Object Classes:vcnUserChannelCTPSink" \b ""§vcnUserChannelCTPSink MANAGED OBJECT CLASS

DERIVED FROM	 "Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

vcnUserChannelCTPSinkPkg PACKAGE

BEHAVIOUR

vcnUserChannelCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

This object class terminates the F2 byte user channel.

;;

ATTRIBUTES

xe "Attributes:vcnUserChannelCTPId"§vcnUserChannelCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 81 };

xe "Object Classes:vcnUserChannelCTPSource" \b ""§vcnUserChannelCTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

vcnUserChannelCTPSourcePkg PACKAGE

BEHAVIOUR

vcnUserChannelCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

This object class originates the F2 byte user channel.

;;

ATTRIBUTES

xe "Attributes:vcnUserChannelCTPId"§vcnUserChannelCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 82 };

# 4	Packages

xe "Packages:electricalSPIPackage" \b ""§electricalSPIPackage PACKAGE

ATTRIBUTES

xe "Attributes:electricalSPITTPId"§electricalSPITTPId	GET,

xe "Attributes:stmLevel"§stmLevel		GET;

;

xe "Packages:msCTPPackage" \b ""§msCTPPackage PACKAGE

ATTRIBUTES

xe "Attributes:msCTPId"§msCTPId		GET,

xe "Attributes:stmLevel"§stmLevel		GET;

;

xe "Packages:msTTPPackage" \b ""§msTTPPackage PACKAGE

ATTRIBUTES

xe "Attributes:msTTPId"§msTTPId			GET,

xe "Attributes:stmLevel"§stmLevel			GET;

;

xe "Packages:opticalSPIPackage" \b ""§opticalSPIPackage PACKAGE

ATTRIBUTES

xe "Attributes:opticalSPITTPId"§opticalSPITTPId	GET,

xe "Attributes:opticalReach"§opticalReach	GET,

xe "Attributes:opticalWavelength"§opticalWavelength	GET,

xe "Attributes:stmLevel"§stmLevel			GET;

;

xe "Packages:rsCTPPackage rsCTPPackage" \b ""§rsCTPPackage PACKAGE

ATTRIBUTES

xe "Attributes:rsCTPId"§rsCTPId			GET,

xe "Attributes:stmLevel"§stmLevel			GET;

;

xe "Packages:rsTTPPackage" \b ""§rsTTPPackage PACKAGE

ATTRIBUTES

xe "Attributes:rsTTPId"§rsTTPId			GET,

xe "Attributes:stmLevel"§stmLevel			GET;

;

xe "Packages:tu-nSinkPackage" \b ""§tu-nSinkPackage PACKAGE

BEHAVIOUR

tu-nSinkPackageBehaviour BEHAVIOUR

DEFINED AS

A communicationsAlarm notification shall be issued if a loss of TU pointer is detected. The probableCause parameter of the notification shall indicate LOP (Loss of Pointer).

A communicationsAlarm notification shall be issued if an TU path alarm indication signal is detected. The probableCause parameter of the notification shall indicate AIS (Alarm Indication Signal).

;;

ATTRIBUTES

xe "Attributes:pointerSinkType"§pointerSinkType	GET;

;

xe "Packages:vc11-2BidirectionalPackage" \b ""§vc11-2BidirectionalPackage PACKAGE

BEHAVIOUR

vc11-2BidirectionalPackageBehaviour BEHAVIOUR

DEFINED AS

A communicationsAlarm notification shall be issued if a far end receive failure (V5 byte) is detected. The probableCause parameter of the notification shall indicate FERF (Far End Receive Failure).

;;

ATTRIBUTES

xe "Attributes:ferfState"§ferfState			GET-REPLACE;

;

xe "Packages:vc11-2SinkPackage" \b ""§vc11-2SinkPackage PACKAGE

BEHAVIOUR

vc11-2SinkPackageBehaviour BEHAVIOUR

DEFINED AS

A communicationsAlarm notification shall be issued if the signal label received (V5 Byte) does not match the signal label expected. The probableCause parameter of the notification shall indicate signal label mismatch.

;;

ATTRIBUTES

xe "Attributes:v5SignalLabelExpected"§v5SignalLabelExpected	GET-REPLACE,

xe "Attributes:v5SignalLabelReceive"§v5SignalLabelReceive	GET;

;

xe "Packages:vc3-4BidirectionalPackage" \b ""§vc3-4BidirectionalPackage PACKAGE

BEHAVIOUR

vc3-4BidirectionalPackageBehaviour BEHAVIOUR

DEFINED AS

A communicationsAlarm notification shall be issued if a far end receive failure (G1 byte) is detected. The probableCause parameter of the notification shall indicate FERF (Far End Receive Failure).

;;

ATTRIBUTES

xe "Attributes:ferfState"§ferfState			GET-REPLACE;

;

xe "Packages:vc3-4SinkPackage" \b ""§vc3-4SinkPackage PACKAGE

BEHAVIOUR

vc3-4SinkPackageBehaviour BEHAVIOUR

DEFINED AS

A communicationsAlarm notification shall be issued if the signal label received (C2 Byte) does not match the signal label expected. The probableCause parameter of the notification shall indicate signal label mismatch.

A communicationsAlarm notification shall be issued if the path trace received (J1 Byte) does not match the path trace expected. The probableCause parameter of the notification shall indicate path trace mismatch.

A communicationsAlarm notification shall be issued if a loss of TU multiframe indicator (H4 Byte) is detected. The probableCause parameter of the notification shall indicate loss of TU multiframe. This communicationsAlarm notification is only required for high order paths with payloads that require use of the multiframe indicator.

;;

ATTRIBUTES

xe "Attributes:j1PathTraceExpected"§j1PathTraceExpected	GET-REPLACE,

xe "Attributes:j1PathTraceReceive"§j1PathTraceReceive	GET,

xe "Attributes:c2SignalLabelExpected"§c2SignalLabelExpected	GET-REPLACE,

xe "Attributes:c2SignalLabelReceive"§c2SignalLabelReceive	GET;

;

xe "Packages:vc3-4SourcePackage" \b ""§vc3-4SourcePackage PACKAGE

ATTRIBUTES

xe "Attributes:j1PathTraceSend"§j1PathTraceSend	GET-REPLACE,

xe "Attributes:c2SignalLabelSend"§c2SignalLabelSend	GET;

;

# 5	Attributes

## AU-3 Identification

xe "Attributes:au3CTPId" \b ""§au3CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

au3CTPIdBehaviour BEHAVIOUR

DEFINED AS

The au3CTPId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the AU3CTPBidirectional, AU3CTPSink, and AU3CTPSource managed object classes. This attribute specifies the timeslot of the au3CTP within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.

;;

REGISTERED AS { g774Attribute 1 };

## AU-4 Identification

xe "Attributes:au4CTPId" \b ""§au4CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

au4CTPIdBehaviour BEHAVIOUR

DEFINED AS

The au4CTPId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the AU4CTPBidirectional, AU4CTPSink, and AU4CTPSource managed object classes. This attribute specifies the timeslot of the au4CTP within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.

;;

REGISTERED AS { g774Attribute 2 };

## AUG Identification

xe "Attributes:augId" \b ""§augId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

augIdBehaviour BEHAVIOUR

DEFINED AS

The augId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the AUG managed object class. This attribute specifies the timeslot of the aug within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.

;;

REGISTERED AS { g774Attribute 3 };

## C2 Signal Label Expected

xe "Attributes:c2SignalLabelExpected" \b ""§c2SignalLabelExpected ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.C2SignalLabel;

MATCHES FOR	EQUALITY;

BEHAVIOUR

c2SignalLabelExpectedBehaviour BEHAVIOUR

DEFINED AS

This attribute specifies the expected C2 VC Signal Label for an incoming VC-n TTP. See Recommendation G.709 for a list of valid values.

;;

REGISTERED AS { g774Attribute 4 };

## C2 Signal Label Receive

xe "Attributes:c2SignalLabelReceive" \b ""§c2SignalLabelReceive ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.C2SignalLabel;

MATCHES FOR	EQUALITY;

BEHAVIOUR

c2SignalLabelReceiveBehaviour BEHAVIOUR

DEFINED AS

This attribute specifies the C2 VC Signal Label for an incoming VC-n TTP. See Recommendation G.709 for a list of valid values.

;;

REGISTERED AS { g774Attribute 5 };

## C2 Signal Label Send

xe "Attributes:c2SignalLabelSend" \b ""§c2SignalLabelSend ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.C2SignalLabel;

MATCHES FOR	EQUALITY;

BEHAVIOUR

c2SignalLabelSendBehaviour BEHAVIOUR

DEFINED AS

This attribute specifies the C2 VC Signal Label for an outgoing VC-n TTP. See Recommendation G.709 for a list of valid values.

;;

REGISTERED AS { g774Attribute 6 };

## Electrical SDH Physical Interface Trail Termination Point Identification

xe "Attributes:electricalSPITTPId" \b ""§electricalSPITTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

electricalSPITTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the electricalSPITTP object classes.

;;

REGISTERED AS { g774Attribute 7 };

## Excessive Bit Error Ratio Maintenance Inhibit

xe "Attributes:excessiveBERMtceInhibit" \b ""§excessiveBERMtceInhibit ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.Boolean;

MATCHES FOR	EQUALITY;

BEHAVIOUR

excessiveBERMtceInhibitBehaviour BEHAVIOUR

DEFINED AS

This attribute is set to TRUE to cause the inhibition of consequent maintenance signalling upon the detection of excessive bit error ratio.

;;

REGISTERED AS { g774Attribute 8 };

## Ferf State

xe "Attributes:ferfState" \b ""§ferfState ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.FerfState;

MATCHES FOR	EQUALITY;

BEHAVIOUR

ferfStateBehaviour BEHAVIOUR

DEFINED AS

The ferfState is used to control the operation of far end receive failure indication when bidirectional path termination points are used. It normally is set to automatic when a TTP is used for bidirectional transmission. The FERF bit can also be set to forced on or off thus inhibiting automatic operation.

;;

REGISTERED AS { g774Attribute 9 };

## J1 Path Trace Expected

xe "Attributes:j1PathTraceExpected" \b ""§j1PathTraceExpected ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.PathTrace;

MATCHES FOR	EQUALITY;

BEHAVIOUR

j1PathTraceExpectedBehaviour BEHAVIOUR

DEFINED AS

This attribute is used to specify the value of the expected J1 Byte VC Path Trace byte message for instances of the VC-n TTP object class.

;;

REGISTERED AS { g774Attribute 10 };

## J1 Path Trace Receive

xe "Attributes:j1PathTraceReceive" \b ""§j1PathTraceReceive ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.PathTrace;

MATCHES FOR	EQUALITY;

BEHAVIOUR

j1PathTraceReceiveBehaviour BEHAVIOUR

DEFINED AS

This attribute is used to indicate the value of the incoming J1 Byte VC Path Trace byte message for  instances of the VC-n TTP object class.

;;

REGISTERED AS { g774Attribute 11 };

## J1 Path Trace Send

xe "Attributes:j1PathTraceSend" \b ""§j1PathTraceSend ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.PathTrace;

MATCHES FOR	EQUALITY;

BEHAVIOUR

j1PathTraceSendBehaviour BEHAVIOUR

DEFINED AS

This attribute is used to indicate the value of the outgoing J1 VC Path Trace byte message for instances of the VC-n TTP object class.

;;

REGISTERED AS { g774Attribute 12 };

## Multiplex Section Connection Termination Point Identification

xe "Attributes:msCTPId" \b ""§msCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

msCTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the msCTP object class. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.

;;

REGISTERED AS { g774Attribute 13 };

## Multiplex Section Data Communications Channel Connection Termination Point Identification

xe "Attributes:msDatacomCTPId" \b ""§msDatacomCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

msDatacomCTPIdBehaviour BEHAVIOUR

DEFINED AS

The msDatacomCTPId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the msDatacomCTPBidirectional, msDatacomCTPSink, and msDatacomCTPSource managed object classes. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.

;;

REGISTERED AS { g774Attribute 14 };

## Multiplex Section Orderwire Identification

xe "Attributes:msOrderwireCTPId" \b ""§msOrderwireCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

msOrderwireCTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the Multiplex Section Orderwire Channel object class. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.

;;

REGISTERED AS { g774Attribute 15 };

## Multiplex Section Trail Termination Point Identification

xe "Attributes:msTTPId" \b ""§msTTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

msTTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the msTTP object class.

;;

REGISTERED AS { g774Attribute 16 };

## Optical Reach

xe "Attributes:opticalReach" \b ""§opticalReach ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.OpticalReach;

MATCHES FOR	EQUALITY;

BEHAVIOUR

opticalReachBehaviour BEHAVIOUR

DEFINED AS

This attribute indicates the length the optical signal may travel before requiring termination or regeneration.

;;

REGISTERED AS { g774Attribute 17 };

## Optical SDH Physical Interface Trail Termination Point Identification

xe "Attributes:opticalSPITTPId" \b ""§opticalSPITTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

opticalSPITTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the opticalSPITTP object class.

;;

REGISTERED AS { g774Attribute 18 };

## Optical WaveLength

xe "Attributes:opticalWavelength" \b ""§opticalWavelength ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.OpticalWavelength;

MATCHES FOR	EQUALITY;

BEHAVIOUR

opticalWavelengthBehaviour BEHAVIOUR

DEFINED AS

This attribute specifies the optical wavelength used by an opticalSPITTP object instance.

;;

REGISTERED AS { g774Attribute 19 };

## Pointer Sink Type

xe "Attributes:pointerSinkType" \b ""§pointerSinkType ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.PointerSinkType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

pointerSinkTypeBehaviour BEHAVIOUR

DEFINED AS

This attribute indicates the status of the incoming Pointer of a sink or bidirectional CTP.

;;

REGISTERED AS { g774Attribute 20 };

## Pointer Source Type

xe "Attributes:pointerSourceType" \b ""§pointerSourceType ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.PointerSourceType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

pointerSourceTypeBehaviour BEHAVIOUR

DEFINED AS

This attribute indicates the status of the outgoing Pointer of a source or bidirectional CTP.

;;

REGISTERED AS { g774Attribute 21 };

## Regenerator Section Connection Termination Point Identification

xe "Attributes:rsCTPId" \b ""§rsCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

rsCTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the rsCTP object classes. This attribute represents the first and only time slot of this type. The value of this attribute shall be integer one.

;;

REGISTERED AS { g774Attribute 22 };

## Regenerator Section Data Communications Channel Connection Termination Point Identification

xe "Attributes:rsDatacomCTPId" \b ""§rsDatacomCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

rsDatacomCTPIdBehaviour BEHAVIOUR

DEFINED AS

The rsDatacomCTPId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the rsDatacomCTPBidirectional, rsDatacomCTPSink, and rsDatacomCTPSource managed object classes. This attribute represents the first and only time slot of this type. The value of this attribute shall be integer one.

;;

REGISTERED AS { g774Attribute 23 };

## Regenerator Section Orderwire Identification

xe "Attributes:rsOrderwireCTPId" \b ""§rsOrderwireCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

rsOrderwireCTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the Regenerator Section Orderwire Channel object classes. This attribute represents the first and only time slot of this type. The value of this attribute shall be integer one.

;;

REGISTERED AS { g774Attribute 24 };

## Regenerator Section Trail Termination Point Identification

xe "Attributes:rsTTPId" \b ""§rsTTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

rsTTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the rsTTP object classes.

;;

REGISTERED AS { g774Attribute 25 };

## Regenerator Section User Channel Identification

xe "Attributes:rsUserChannelCTPId" \b ""§rsUserChannelCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

rsUserChannelCTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the rsUserChannel object classes. This attribute represents the first and only time slot of this type. The value of this attribute shall be integer one.

;;

REGISTERED AS { g774Attribute 26 };

## Signal Degrade Threshold

xe "Attributes:signalDegradeThreshold" \b ""§signalDegradeThreshold ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.Integer;

MATCHES FOR	EQUALITY;

BEHAVIOUR

signalDegradeThresholdBehaviour BEHAVIOUR

DEFINED AS

This attribute specifies the specific BER used to define the signal degrade threshold. The specific BER used is an equipment issue. This attribute represents the negative power of 10. So, for instance, if this attribute had a value of 5, then the BER threshold is 10 raised to the power of –5.

;;

REGISTERED AS { g774Attribute 27 };

## STM Level

xe "Attributes:stmLevel" \b ""§stmLevel ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.Integer;

MATCHES FOR	EQUALITY;

BEHAVIOUR

stmLevelBehaviour BEHAVIOUR

DEFINED AS

This attribute specifies the level, n, of the STM-n signal being received, transmitted, or received and transmitted for termination point object instance.

;;

REGISTERED AS { g774Attribute 28 };

## Tributary Unit 11 Connection Termination Point Identification

xe "Attributes:tu11CTPId" \b ""§tu11CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

tu11CTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the tu11CTP object classes. This attribute specifies the time slot of the TU-11 CTP within its server TTP or IA. The value shall be the integer which represents the position of the time slot in temporal order. The first time slot shall be numbered one.

;;

REGISTERED AS { g774Attribute 29 };

## Tributary Unit 12 Connection Termination Point Identification

xe "Attributes:tu12CTPId" \b ""§tu12CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

tu12CTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the tu12CTP object classes. This attribute specifies the time slot of the TU-12 CTP within its server TTP or IA. The value shall be the integer which represents the position of the time slot in temporal order. The first time slot shall be numbered one.

;;

REGISTERED AS { g774Attribute 30 };

## Tributary Unit 2 Connection Termination Point Identification

xe "Attributes:tu2CTPId" \b ""§tu2CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

tu2CTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the tu2CTP object classes. This attribute specifies the time slot of the TU-2 CTP within its server TTP or IA. The value shall be the integer which represents the position of the time slot in temporal order. The first time slot shall be numbered one.

;;

REGISTERED AS { g774Attribute 31 };

## Tributary Unit 3 Connection Termination Point Identification

xe "Attributes:tu3CTPId" \b ""§tu3CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

tu3CTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the tu3CTP object classes. This attribute specifies the time slot of the TU-3 CTP within its server TTP or IA. The value shall be the integer which represents the position of the time slot in temporal order. The first time slot shall be numbered one.

;;

REGISTERED AS { g774Attribute 32 };

## TUG-2 Identification

xe "Attributes:tug2Id" \b ""§tug2Id ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

tug2IdBehaviour BEHAVIOUR

DEFINED AS

The tug2Id attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the TUG-2 managed object class. This attribute specifies the time slot of the TUG2 within its server TTP or IA. The value shall be the integer which represents the position of the time slot in temporal order. The first time slot shall be numbered one.

;;

REGISTERED AS { g774Attribute 33 };

## TUG-3 Identification

xe "Attributes:tug3Id" \b ""§tug3Id ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

tug3IdBehaviour BEHAVIOUR

DEFINED AS

The tug3Id attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the TUG-3 managed object class. This attribute specifies the time slot of the TUG3 within its server TTP or IA. The value shall be the integer which represents the position of the time slot in temporal order. The first time slot shall be numbered one.

;;

REGISTERED AS { g774Attribute 34 };

## V5 Signal Label Expected

xe "Attributes:v5SignalLabelExpected" \b ""§v5SignalLabelExpected ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.V5SignalLabel;

MATCHES FOR	EQUALITY;

BEHAVIOUR

v5SignalLabelExpectedBehaviour BEHAVIOUR

DEFINED AS

This attribute specifies the expected V5 VC Signal Label for an incoming VC-n TTP. See Recommendation G.709 for a list of valid values.

;;

REGISTERED AS { g774Attribute 35 };

## V5 Signal Label Receive

xe "Attributes:v5SignalLabelReceive" \b ""§v5SignalLabelReceive ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.V5SignalLabel;

MATCHES FOR	EQUALITY;

BEHAVIOUR

v5SignalLabelReceiveBehaviour BEHAVIOUR

DEFINED AS

This attribute specifies the V5 VC Signal Label for an incoming  VC-n TTP. See Recommendation G.709 for a list of valid values.

;;

REGISTERED AS { g774Attribute 36 };

## V5 Signal Label Send

xe "Attributes:v5SignalLabelSend" \b ""§v5SignalLabelSend ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.V5SignalLabel;

MATCHES FOR	EQUALITY;

BEHAVIOUR

v5SignalLabelSendBehaviour BEHAVIOUR

DEFINED AS

This attribute specifies the V5 VC Signal Label for an outgoing VC-n TTP. See Recommendation G.709 for a list of valid values.

;;

REGISTERED AS { g774Attribute 37 };

## Virtual Container 11 Trail Termination Point Identification

xe "Attributes:vc11TTPId" \b ""§vc11TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

vc11TTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the vc11TTP object classes.

;;

REGISTERED AS { g774Attribute 38 };

## Virtual Container 12 Trail Termination Point Identification

xe "Attributes:vc12TTPId" \b ""§vc12TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

vc12TTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the vc12TTP object classes.

;;

REGISTERED AS { g774Attribute 39 };

## Virtual Container 2 Trail Termination Point Identification

xe "Attributes:vc2TTPId" \b ""§vc2TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

vc2TTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the vc2TTP object classes.

;;

REGISTERED AS { g774Attribute 40 };

## Virtual Container 3 Trail Termination Point Identification

xe "Attributes:vc3TTPId" \b ""§vc3TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

vc3TTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the vc3TTP object classes.

;;

REGISTERED AS { g774Attribute 41 };

## Virtual Container 4 Trail Termination Point Identification

xe "Attributes:vc4TTPId" \b ""§vc4TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

vc4TTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the vc4TTP object classes.

;;

REGISTERED AS { g774Attribute 42 };

## VC-n User Channel Identification

xe "Attributes:vcnUserChannelCTPId" \b ""§vcnUserChannelCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY;

BEHAVIOUR

vcnUserChannelCTPIdBehaviour BEHAVIOUR

DEFINED AS

This attribute is used as an RDN for naming instances of the vcnUserChannel object classes. This attribute represents the first and only time slot of this type. The value of this attribute shall be integer one.

;;

REGISTERED AS { g774Attribute 43 };

# 6	Name Bindings

Recommendation G.774     (09/92)

xe "Name Bindings:au3CTPBidirectional-augBidirectional" \b ""§au3CTPBidirectional-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE	au3CTPId;

BEHAVIOUR

au3CTPBidirectional-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 1 };

xe "Name Bindings:au3CTPSink-augBidirectional" \b ""§au3CTPSink-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE	au3CTPId;

BEHAVIOUR

au3CTPSink-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 2 };

xe "Name Bindings:au3CTPSink-augSink" \b ""§au3CTPSink-augSink NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	augSink;

WITH ATTRIBUTE	au3CTPId;

BEHAVIOUR

au3CTPSink-augSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 3 };

xe "Name Bindings:au3CTPSource-augBidirectional" \b ""§au3CTPSource-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE	au3CTPId;

BEHAVIOUR

au3CTPSource-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 4 };

xe "Name Bindings:au3CTPSource-augSource" \b ""§au3CTPSource-augSource NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	augSource;

WITH ATTRIBUTE	au3CTPId;

BEHAVIOUR

au3CTPSource-augSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 5 };

xe "Name Bindings:au4CTPBidirectional-augBidirectional" \b ""§au4CTPBidirectional-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE	au4CTPId;

BEHAVIOUR

au4CTPBidirectional-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 6 };

xe "Name Bindings:au4CTPSink-augBidirectional" \b ""§au4CTPSink-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE	au4CTPId;

BEHAVIOUR

au4CTPSink-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 7 };

xe "Name Bindings:au4CTPSink-augSink" \b ""§au4CTPSink-augSink NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	augSink;

WITH ATTRIBUTE	au4CTPId;

BEHAVIOUR

au4CTPSink-augSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 8 };

xe "Name Bindings:au4CTPSource-augBidirectional" \b ""§au4CTPSource-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE	au4CTPId;

BEHAVIOUR

au4CTPSource-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 9 };

xe "Name Bindings:au4CTPSource-augSource" \b ""§au4CTPSource-augSource NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	augSource;

WITH ATTRIBUTE	au4CTPId;

BEHAVIOUR

au4CTPSource-augSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 10 };

xe "Name Bindings:augBidirectional-msTTPBidirectional" \b ""§augBidirectional-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	augBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	augId;

BEHAVIOUR

augBidirectional-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 11 };

xe "Name Bindings:augSink-msTTPSink" \b ""§augSink-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	augSink;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPSink;

WITH ATTRIBUTE	augId;

BEHAVIOUR

augSink-msTTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 12 };

xe "Name Bindings:augSource-msTTPSource" \b ""§augSource-msTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	augSource;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPSource;

WITH ATTRIBUTE	augId;

BEHAVIOUR

augSource-msTTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 13 };

xe "Name Bindings:electricalSPITTPBidirectional-sdhNE" \b ""§electricalSPITTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	electricalSPITTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	electricalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 14 };

xe "Name Bindings:electricalSPITTPSink-sdhNE" \b ""§electricalSPITTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	electricalSPITTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	electricalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 15 };

xe "Name Bindings:electricalSPITTPSource-sdhNE" \b ""§electricalSPITTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	electricalSPITTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	electricalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 16 };

xe "Name Bindings:msCTPBidirectional-rsTTPBidirectional" \b ""§msCTPBidirectional-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPBidirectional-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 17 };

xe "Name Bindings:msCTPSink-rsTTPBidirectional" \b ""§msCTPSink-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPSink-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 18 };

xe "Name Bindings:msCTPSink-rsTTPSink" \b ""§msCTPSink-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPSink;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPSink-rsTTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 19 };

xe "Name Bindings:msCTPSource-rsTTPBidirectional" \b ""§msCTPSource-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPSource-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 20 };

xe "Name Bindings:msCTPSource-rsTTPSource" \b ""§msCTPSource-rsTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPSource;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPSource-rsTTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 21 };

xe "Name Bindings:msDatacomCTPBidirectional-msTTPBidirectional" \b ""§msDatacomCTPBidirectional-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPBidirectional-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 22 };

xe "Name Bindings:msDatacomCTPSink-msTTPBidirectional" \b ""§msDatacomCTPSink-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSink-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 23 };

xe "Name Bindings:msDatacomCTPSink-msTTPSink" \b ""§msDatacomCTPSink-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPSink;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSink-msTTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 24 };

xe "Name Bindings:msDatacomCTPSource-msTTPBidirectional" \b ""§msDatacomCTPSource-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSource-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 25 };

xe "Name Bindings:msDatacomCTPSource-msTTPSource" \b ""§msDatacomCTPSource-msTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPSource;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSource-msTTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 26 };

xe "Name Bindings:msOrderwireCTPBidirectional-msTTPBidirectional" \b ""§msOrderwireCTPBidirectional-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPBidirectional-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 27 };

xe "Name Bindings:msOrderwireCTPSink-msTTPBidirectional" \b ""§msOrderwireCTPSink-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSink-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 28 };

xe "Name Bindings:msOrderwireCTPSink-msTTPSink" \b ""§msOrderwireCTPSink-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPSink;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSink-msTTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 29 };

xe "Name Bindings:msOrderwireCTPSource-msTTPBidirectional" \b ""§msOrderwireCTPSource-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSource-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 30 };

xe "Name Bindings:msOrderwireCTPSource-msTTPSource" \b ""§msOrderwireCTPSource-msTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	msTTPSource;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSource-msTTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 31 };

xe "Name Bindings:msTTPBidirectional-sdhNE" \b ""§msTTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	msTTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	msTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 32 };

xe "Name Bindings:msTTPSink-sdhNE" \b ""§msTTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	msTTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	msTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 33 };

xe "Name Bindings:msTTPSource-sdhNE" \b ""§msTTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	msTTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	msTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 34 };

xe "Name Bindings:opticalSPITTPBidirectional-sdhNE" \b ""§opticalSPITTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	opticalSPITTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	opticalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 35 };

xe "Name Bindings:opticalSPITTPSink-sdhNE" \b ""§opticalSPITTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	opticalSPITTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	opticalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 36 };

xe "Name Bindings:opticalSPITTPSource-sdhNE" \b ""§opticalSPITTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	opticalSPITTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	opticalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 37 };

xe "Name Bindings:rsCTPBidirectional-electricalSPITTPBidirectional" \b ""§rsCTPBidirectional-electricalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	electricalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPBidirectional-electricalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 38 };

xe "Name Bindings:rsCTPSink-electricalSPITTPBidirectional" \b ""§rsCTPSink-electricalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	electricalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSink-electricalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 39 };

xe "Name Bindings:rsCTPSink-electricalSPITTPSink" \b ""§rsCTPSink-electricalSPITTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	electricalSPITTPSink;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSink-electricalSPITTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 40 };

xe "Name Bindings:rsCTPSource-electricalSPITTPBidirectional" \b ""§rsCTPSource-electricalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	electricalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSource-electricalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 41 };

xe "Name Bindings:rsCTPSource-electricalSPITTPSource" \b ""§rsCTPSource-electricalSPITTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	electricalSPITTPSource;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSource-electricalSPITTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 42 };

xe "Name Bindings:rsCTPBidirectional-opticalSPITTPBidirectional" \b ""§rsCTPBidirectional-opticalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	opticalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPBidirectional-opticalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 43 };

xe "Name Bindings:rsCTPSink-opticalSPITTPBidirectional" \b ""§rsCTPSink-opticalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	opticalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSink-opticalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 44 };

xe "Name Bindings:rsCTPSink-opticalSPITTPSink" \b ""§rsCTPSink-opticalSPITTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	opticalSPITTPSink;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSink-opticalSPITTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 45 };

xe "Name Bindings:rsCTPSource-opticalSPITTPBidirectional" \b ""§rsCTPSource-opticalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	opticalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSource-opticalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 46 };

xe "Name Bindings:rsCTPSource-opticalSPITTPSource" \b ""§rsCTPSource-opticalSPITTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	opticalSPITTPSource;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSource-opticalSPITTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 47 };

xe "Name Bindings:rsDatacomCTPBidirectional-rsTTPBidirectional" \b ""§rsDatacomCTPBidirectional-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPBidirectional-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 48 };

xe "Name Bindings:rsDatacomCTPSink-rsTTPBidirectional" \b ""§rsDatacomCTPSink-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSink-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 49 };

xe "Name Bindings:rsDatacomCTPSink-rsTTPSink" \b ""§rsDatacomCTPSink-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPSink;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSink-rsTTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 50 };

xe "Name Bindings:rsDatacomCTPSource-rsTTPBidirectional" \b ""§rsDatacomCTPSource-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSource-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 51 };

xe "Name Bindings:rsDatacomCTPSource-rsTTPSource" \b ""§rsDatacomCTPSource-rsTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPSource;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSource-rsTTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 52 };

xe "Name Bindings:rsOrderwireCTPBidirectional-rsTTPBidirectional" \b ""§rsOrderwireCTPBidirectional-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPBidirectional-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 53 };

xe "Name Bindings:rsOrderwireCTPSink-rsTTPBidirectional" \b ""§rsOrderwireCTPSink-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSink-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 54 };

xe "Name Bindings:rsOrderwireCTPSink-rsTTPSink" \b ""§rsOrderwireCTPSink-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPSink;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSink-rsTTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 55 };

xe "Name Bindings:rsOrderwireCTPSource-rsTTPBidirectional" \b ""§rsOrderwireCTPSource-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSource-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 56 };

xe "Name Bindings:rsOrderwireCTPSource-rsTTPSource" \b ""§rsOrderwireCTPSource-rsTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPSource;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSource-rsTTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 57 };

xe "Name Bindings:rsTTPBidirectional-sdhNE" \b ""§rsTTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	rsTTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	rsTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 58 };

xe "Name Bindings:rsTTPSink-sdhNE" \b ""§rsTTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	rsTTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	rsTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 59 };

xe "Name Bindings:rsTTPSource-sdhNE" \b ""§rsTTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	rsTTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	rsTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 60 };

xe "Name Bindings:rsUserChannelCTPBidirectional-rsTTPBidirectional" \b ""§rsUserChannelCTPBidirectional-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPBidirectional-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 61 };

xe "Name Bindings:rsUserChannelCTPSink-rsTTPBidirectional" \b ""§rsUserChannelCTPSink-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSink-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 62 };

xe "Name Bindings:rsUserChannelCTPSink-rsTTPSink" \b ""§rsUserChannelCTPSink-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPSink;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSink-rsTTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 63 };

xe "Name Bindings:rsUserChannelCTPSource-rsTTPBidirectional" \b ""§rsUserChannelCTPSource-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPBidirectional;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSource-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 64 };

xe "Name Bindings:rsUserChannelCTPSource-rsTTPSource" \b ""§rsUserChannelCTPSource-rsTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	rsTTPSource;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSource-rsTTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 65 };

xe "Name Bindings:tu11CTPBidirectional-tug2Bidirectional" \b ""§tu11CTPBidirectional-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu11CTPId;

BEHAVIOUR

tu11CTPBidirectional-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 66 };

xe "Name Bindings:tu11CTPSink-tug2Bidirectional" \b ""§tu11CTPSink-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu11CTPId;

BEHAVIOUR

tu11CTPSink-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 67 };

xe "Name Bindings:tu11CTPSink-tug2Sink" \b ""§tu11CTPSink-tug2Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Sink;

WITH ATTRIBUTE	tu11CTPId;

BEHAVIOUR

tu11CTPSink-tug2SinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 68 };

xe "Name Bindings:tu11CTPSource-tug2Bidirectional" \b ""§tu11CTPSource-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu11CTPId;

BEHAVIOUR

tu11CTPSource-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 69 };

xe "Name Bindings:tu11CTPSource-tug2Source" \b ""§tu11CTPSource-tug2Source NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Source;

WITH ATTRIBUTE	tu11CTPId;

BEHAVIOUR

tu11CTPSource-tug2SourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 70 };

xe "Name Bindings:tu12CTPBidirectional-tug2Bidirectional" \b ""§tu12CTPBidirectional-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu12CTPId;

BEHAVIOUR

tu12CTPBidirectional-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 71 };

xe "Name Bindings:tu12CTPSink-tug2Bidirectional" \b ""§tu12CTPSink-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu12CTPId;

BEHAVIOUR

tu12CTPSink-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 72 };

xe "Name Bindings:tu12CTPSink-tug2Sink" \b ""§tu12CTPSink-tug2Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Sink;

WITH ATTRIBUTE	tu12CTPId;

BEHAVIOUR

tu12CTPSink-tug2SinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 73 };

xe "Name Bindings:tu12CTPSource-tug2Bidirectional" \b ""§tu12CTPSource-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu12CTPId;

BEHAVIOUR

tu12CTPSource-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 74 };

xe "Name Bindings:tu12CTPSource-tug2Source" \b ""§tu12CTPSource-tug2Source NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Source;

WITH ATTRIBUTE	tu12CTPId;

BEHAVIOUR

tu12CTPSource-tug2SourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 75 };

xe "Name Bindings:tu2CTPBidirectional-tug2Bidirectional" \b ""§tu2CTPBidirectional-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu2CTPId;

BEHAVIOUR

tu2CTPBidirectional-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 76 };

xe "Name Bindings:tu2CTPSink-tug2Bidirectional" \b ""§tu2CTPSink-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu2CTPId;

BEHAVIOUR

tu2CTPSink-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 77 };

xe "Name Bindings:tu2CTPSink-tug2Sink" \b ""§tu2CTPSink-tug2Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Sink;

WITH ATTRIBUTE	tu2CTPId;

BEHAVIOUR

tu2CTPSink-tug2SinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 78 };

xe "Name Bindings:tu2CTPSource-tug2Bidirectional" \b ""§tu2CTPSource-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu2CTPId;

BEHAVIOUR

tu2CTPSource-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 79 };

xe "Name Bindings:tu2CTPSource-tug2Source" \b ""§tu2CTPSource-tug2Source NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Source;

WITH ATTRIBUTE	tu2CTPId;

BEHAVIOUR

tu2CTPSource-tug2SourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 80 };

xe "Name Bindings:tu3CTPBidirectional-tug3Bidirectional" \b ""§tu3CTPBidirectional-tug3Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	tug3Bidirectional;

WITH ATTRIBUTE	tu3CTPId;

BEHAVIOUR

tu3CTPBidirectional-tug3BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 81 };

xe "Name Bindings:tu3CTPSink-tug3Bidirectional" \b ""§tu3CTPSink-tug3Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	tug3Bidirectional;

WITH ATTRIBUTE	tu3CTPId;

BEHAVIOUR

tu3CTPSink-tug3BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 82 };

xe "Name Bindings:tu3CTPSink-tug3Sink" \b ""§tu3CTPSink-tug3Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	tug3Sink;

WITH ATTRIBUTE	tu3CTPId;

BEHAVIOUR

tu3CTPSink-tug3SinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 83 };

xe "Name Bindings:tu3CTPSource-tug3Bidirectional" \b ""§tu3CTPSource-tug3Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	tug3Bidirectional;

WITH ATTRIBUTE	tu3CTPId;

BEHAVIOUR

tu3CTPSource-tug3BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 84 };

xe "Name Bindings:tu3CTPSource-tug3Source" \b ""§tu3CTPSource-tug3Source NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	tug3Source;

WITH ATTRIBUTE	tu3CTPId;

BEHAVIOUR

tu3CTPSource-tug3SourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 85 };

xe "Name Bindings:tug2Bidirectional-tug3Bidirectional" \b ""§tug2Bidirectional-tug3Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Bidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	tug3Bidirectional;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Bidirectional-tug3BidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 86 };

xe "Name Bindings:tug2Sink-tug3Sink" \b ""§tug2Sink-tug3Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Sink;

NAMED BY

SUPERIOR OBJECT CLASS	tug3Sink;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Sink-tug3SinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 87 };

xe "Name Bindings:tug2Source-tug3Source" \b ""§tug2Source-tug3Source NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Source;

NAMED BY

SUPERIOR OBJECT CLASS	tug3Source;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Source-tug3SourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 88 };

xe "Name Bindings:tug2Bidirectional-vc3TTPBidirectional" \b ""§tug2Bidirectional-vc3TTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Bidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	vc3TTPBidirectional;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Bidirectional-vc3TTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 89 };

xe "Name Bindings:tug2Sink-vc3TTPSink" \b ""§tug2Sink-vc3TTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Sink;

NAMED BY

SUPERIOR OBJECT CLASS	vc3TTPSink;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Sink-vc3TTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 90 };

xe "Name Bindings:tug2Source-vc3TTPSource" \b ""§tug2Source-vc3TTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Source;

NAMED BY

SUPERIOR OBJECT CLASS	vc3TTPSource;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Source-vc3TTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 91 };

xe "Name Bindings:tug3Bidirectional-vc4TTPBidirectional" \b ""§tug3Bidirectional-vc4TTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tug3Bidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	vc4TTPBidirectional;

WITH ATTRIBUTE	tug3Id;

BEHAVIOUR

tug3Bidirectional-vc4TTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 92 };

xe "Name Bindings:tug3Sink-vc4TTPSink" \b ""§tug3Sink-vc4TTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	tug3Sink;

NAMED BY

SUPERIOR OBJECT CLASS	vc4TTPSink;

WITH ATTRIBUTE	tug3Id;

BEHAVIOUR

tug3Sink-vc4TTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 93 };

xe "Name Bindings:tug3Source-vc4TTPSource" \b ""§tug3Source-vc4TTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	tug3Source;

NAMED BY

SUPERIOR OBJECT CLASS	vc4TTPSource;

WITH ATTRIBUTE	tug3Id;

BEHAVIOUR

tug3Source-vc4TTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 94 };

xe "Name Bindings:vc11TTPBidirectional-sdhNE" \b ""§vc11TTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc11TTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc11TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 95 };

xe "Name Bindings:vc11TTPSink-sdhNE" \b ""§vc11TTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc11TTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc11TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 96 };

xe "Name Bindings:vc11TTPSource-sdhNE" \b ""§vc11TTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc11TTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc11TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 97 };

xe "Name Bindings:vc12TTPBidirectional-sdhNE" \b ""§vc12TTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc12TTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc12TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 98 };

xe "Name Bindings:vc12TTPSink-sdhNE" \b ""§vc12TTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc12TTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc12TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 99 };

xe "Name Bindings:vc12TTPSource-sdhNE" \b ""§vc12TTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc12TTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc12TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 100 };

xe "Name Bindings:vc2TTPBidirectional-sdhNE" \b ""§vc2TTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc2TTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc2TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 101 };

xe "Name Bindings:vc2TTPSink-sdhNE" \b ""§vc2TTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc2TTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc2TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 102 };

xe "Name Bindings:vc2TTPSource-sdhNE" \b ""§vc2TTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc2TTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc2TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 103 };

xe "Name Bindings:vc3TTPBidirectional-sdhNE" \b ""§vc3TTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc3TTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc3TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 104 };

xe "Name Bindings:vc3TTPSink-sdhNE" \b ""§vc3TTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc3TTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc3TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 105 };

xe "Name Bindings:vc3TTPSource-sdhNE" \b ""§vc3TTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc3TTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc3TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 106 };

xe "Name Bindings:vc4TTPBidirectional-sdhNE" \b ""§vc4TTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc4TTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc4TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 107 };

xe "Name Bindings:vc4TTPSink-sdhNE" \b ""§vc4TTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc4TTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc4TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 108 };

xe "Name Bindings:vc4TTPSource-sdhNE" \b ""§vc4TTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc4TTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc4TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 109 };

xe "Name Bindings:vcnUserChannelCTPBidirectional-vc3TTPBidirectional" \b ""§vcnUserChannelCTPBidirectional-vc3TTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	vc3TTPBidirectional;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPBidirectional-vc3TTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 110 };

xe "Name Bindings:vcnUserChannelCTPSink-vc3TTPBidirectional" \b ""§vcnUserChannelCTPSink-vc3TTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	vc3TTPBidirectional;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc3TTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 111 };

xe "Name Bindings:vcnUserChannelCTPSink-vc3TTPSink" \b ""§vcnUserChannelCTPSink-vc3TTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	vc3TTPSink;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc3TTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 112 };

xe "Name Bindings:vcnUserChannelCTPSource-vc3TTPBidirectional" \b ""§vcnUserChannelCTPSource-vc3TTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	vc3TTPBidirectional;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc3TTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 113 };

xe "Name Bindings:vcnUserChannelCTPSource-vc3TTPSource" \b ""§vcnUserChannelCTPSource-vc3TTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	vc3TTPSource;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc3TTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 114 };

xe "Name Bindings:vcnUserChannelCTPBidirectional-vc4TTPBidirectional" \b ""§vcnUserChannelCTPBidirectional-vc4TTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPBidirectional;

NAMED BY

SUPERIOR OBJECT CLASS	vc4TTPBidirectional;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPBidirectional-vc4TTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 115 };

xe "Name Bindings:vcnUserChannelCTPSink-vc4TTPBidirectional" \b ""§vcnUserChannelCTPSink-vc4TTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	vc4TTPBidirectional;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc4TTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 116 };

xe "Name Bindings:vcnUserChannelCTPSink-vc4TTPSink" \b ""§vcnUserChannelCTPSink-vc4TTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSink;

NAMED BY

SUPERIOR OBJECT CLASS	vc4TTPSink;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc4TTPSinkBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 117 };

xe "Name Bindings:vcnUserChannelCTPSource-vc4TTPBidirectional" \b ""§vcnUserChannelCTPSource-vc4TTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	vc4TTPBidirectional;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc4TTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 118 };

xe "Name Bindings:vcnUserChannelCTPSource-vc4TTPSource" \b ""§vcnUserChannelCTPSource-vc4TTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	vc4TTPSource;

WITH ATTRIBUTE	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc4TTPSourceBehaviour BEHAVIOUR

DEFINED AS

The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.

;;

REGISTERED AS { g774NameBinding 119 };

# 7	Supporting ASN.1

Recommendation G.774     (09/92)

SDH {ccitt(0) recommendation(0) g(7) sdhm(774) informationModel(0) asn1Module(2) sdh(0)}

DEFINITIONS IMPLICIT TAGS ::= BEGIN

IMPORTS

xe "SDH ASN.1 Module:NameType" \b ""§NameType   --  M.3100

FROM ASN1DefinedTypesModule {ccitt(0) recommendation(0) m(13) gnm(3100) informationModel(0) asn1Modules(2) asn1DefinedTypeModule(0)}

;

g774 OBJECT IDENTIFIER ::= {ccitt(0) recommendation(0) g(7) sdhm(774) informationModel(0)}

g774ObjectClass OBJECT IDENTIFIER ::= {g774 managedObjectClass(3)}

g774Attribute OBJECT IDENTIFIER ::= {g774 attribute(7)}

g774NameBinding OBJECT IDENTIFIER ::= {g774 nameBinding(6)}

xe "SDH ASN.1 Module:Boolean" \b ""§Boolean ::= BOOLEAN

xe "SDH ASN.1 Module:C2SignalLabel" \b ""§C2SignalLabel ::= INTEGER (0..255)

xe "SDH ASN.1 Module:FerfState" \b ""§FerfState ::= ENUMERATED {

automatic(0),

forceOn(1),

forceOff(2)

}

xe "SDH ASN.1 Module:Integer" \b ""§Integer ::= INTEGER

xe "SDH ASN.1 Module:OpticalReach" \b ""§OpticalReach ::= ENUMERATED {

intraOffice(0),

shortHaul(1),

longHaul(2)

}

xe "SDH ASN.1 Module:OpticalWavelength" \b ""§OpticalWavelength ::= ENUMERATED {

wl1310(0),

wl1550(1)

}

xe "SDH ASN.1 Module:PathTrace" \b ""§PathTrace ::= CHOICE {

null		NULL,

pathtrace	[1]	GRAPHICSTRING

}

xe "SDH ASN.1 Module:PointerSinkType" \b ""§PointerSinkType ::= ENUMERATED {

normalPointer(0),

concatenationIndication(1),

invalidPointer(2)

}

xe "SDH ASN.1 Module:PointerSourceType" \b ""§PointerSourceType ::= ENUMERATED {

normalPointer(0),

concatenationIndication(1)

}

xe "SDH ASN.1 Module:V5SignalLabel" \b ""§V5SignalLabel ::= INTEGER (0..7)

END

# 8	Object relations

Note — The SUBORDINATION RULE and CONSTRAINT RULE templates are used in this Recommendation as an interim specification tool.  A RELATIONSHIP template is currently under study; when it has been standardized, the rules specified in the SUBORDINATION RULE and CONSTRAINT RULE templates will be re-specified using the RELATIONSHIP template.

## 8.1	Syntax

<subordination-rule-label> SUBORDINATION RULE

SUPERIOR OBJECT CLASS <class-label> ;

NAMES SUBORDINATES <class-list> ;

ACCORDING TO RULE <subordination-rule> ;

;

<constraint-rule-label> CONSTRAINT RULE

OBJECT CLASS <class-label> [AND SUBCLASSES] ;

IS RELATED TO <class-list> ;

USING ATTRIBUTE <attribute-label> ;

<constraint-rule-set> ;

;

<constraint-rule-set> ::= <single-constraint-rule> | <named-type-constraint-rules>

<single-constraint-rule> ::= ACCORDING TO RULE <constraint-rule>

<named-type-constraint-rule> ::= CASE { <named-type-constraint-rule-list> }

<named-type-constraint-rule-list> ::= <named-type-constraint-rule-item> |

<named-type-constraint-rule-item> , <named-type-constraint-rule-list>

<named-type-constraint-rule-item> ::=

<named-type> ACCORDING TO RULE <constraint-rule>

<class-label> ::= label string as defined in ISO/IEC IS 10165-4

<attribute-label> ::= label string as defined in ISO/IEC IS 10165-4

<class-list>  ::= <class-label> | <class-list> , <class-label>

<subordination-rule> ::= SET { <subordination-members> }

| CHOICE { <subordination-members> }

| SET SIZE ( <ordinality> ) OF <subordination-term>

<constraint-rule> ::= SET { <constraint-members> }

| SEQUENCE { <constraint-members> }

| CHOICE { <constraint-members> }

| SET SIZE ( <ordinality> ) OF <constraint-term>

| SEQUENCE SIZE ( <ordinality> ) OF <constraint-term>

<subordination-members> ::= <subordination-term>

| <subordination-term> , <subordination-members>

<constraint-members> ::=    <constraint-term>

| <constraint-term> , <constraint-members>

<subordination-term> ::= <class-label> | <subordination-rule>

<constraint-term> ::= <class-label> | <constraint-rule>

<ordinality>  ::= <valueRange> | <valueList>

<valueRange>  ::= <lowerValue> .. <upperValue>

<valueList>   ::= <itemValue> | <itemValue> , <valueList>

<itemValue>   ::= INTEGER

<lowerValue>  ::= INTEGER

<upperValue>  ::= INTEGER | N

### 8.1.1	Subordination rule templates

SUPERIOR OBJECT CLASS <class-label>;

indicates the class that is governing this subordination-rule. A class may govern through more than one subordination-rule several non-overlapping sets of subordinate classes in the NAMES SUBORDINATES clause.

NAMES SUBORDINATES <class-list>;

indicates the set of classes of subordinates that are governed by this subordination-rule.  Any classes that are not in this list are not governed by this subordination-rule.

ACCORDING TO RULE <subordination-rule>

provides the rule

SET { <subordination-members> }

indicates that *all* of the subordination-members must be present.

CHOICE { <subordination-members> }

indicates that *any one* of the subordination-members must be present.

SET SIZE <ordinality> OF <subordination-term>

indicates the number of <subordination-term> that must be present.

### 8.1.2	Constraint rule templates

OBJECT CLASS <class-label>;

indicates the class with this attribute that is governed by this constraint-rule.  A class may be governed by more than one constraint-rule with non-overlapping sets of related classes in the RELATES TO OBJECT CLASSES clause.

IS RELATED TO OBJECT CLASSES <class-list>;

indicates the set of classes of related instances that are governed by this constraint-rule.  Any classes that are not in this list are not governed by this constraint-rule.

USING ATTRIBUTE <attribute-label>;

indicates the attribute that represents a relationship by means of a pointer (DN) to the related object instances.

<constraint-rule-set>;

there can be either a single rule, or a set of rules one for each of a set of named choices.  In the latter case the CASE { ... } structure is used.

CASE { ... };

provides a distinct constraint-rule for each of the set of named choices in the attribute syntax.

ACCORDING TO RULE <constraint-rule>

provides the rule

SET { <constraint-members> }

indicates that *all* of the constraint-members must be present in any order.

SEQUENCE { <constraint-members> }

indicates that *all* of the constraint-members must be present in sequence.

CHOICE { <constraint-members> }

indicates that *any one* of the constraint-members must be present.

SET SIZE <ordinality> OF <constraint-term>

indicates the number of <constraint-term> that must be present in any order.

SEQUENCE SIZE <ordinality> OF <constraint-term>

indicates the number of <constraint-term> that must be present in sequence.

## 8.2	Connectivity pointer constraints

This subsection defines the allowable values for the downstreamConnectivityPointer and upstreamConnectivityPointer attributes using the object classes defined in this Recommendation.

downstreamConnectivityPointer-au3CTPSink CONSTRAINT RULE

OBJECT CLASS

au3CTPSink AND SUBCLASSES;

IS RELATED TO

vc3TTPSink, vc3TTPBidirectional,

au3CTPSource, au3CTPBidirectional,

tu3CTPSource, tu3CTPBidirectional,

vc4TTPSink, vc4TTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSink,vc3TTPBidirectional,

au3CTPSource,au3CTPBidirectional,

tu3CTPSource, tu3CTPBidirectional,

vc4TTPSink,vc4TTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

SET SIZE(1..N) OF CHOICE {

vc3TTPSink, vc3TTPBidirectional,

tu3CTPSource, tu3CTPBidirectional,

au3CTPSource, au3CTPBidirectional },

SET SIZE(1..N) OF CHOICE {

vc4TTPSink, vc4TTPBidirectional }

}

};

;

upstreamConnectivityPointer-au3CTPSource CONSTRAINT RULE

OBJECT CLASS

au3CTPSource AND SUBCLASSES;

IS RELATED TO

vc3TTPSource, vc3TTPBidirectional,

au3CTPSink, au3CTPBidirectional,

tu3CTPSink, tu3CTPBidirectional,

vc4TTPSource, vc4TTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE { vc3TTPSource,vc3TTPBidirectional,

au3CTPSink,au3CTPBidirectional,

tu3CTPSink,tu3CTPBidirectional,

vc4TTPSource,vc4TTPBidirectional }

};

;

downstreamConnectivityPointer-au4CTPSink CONSTRAINT RULE

OBJECT CLASS

au4CTPSink AND SUBCLASSES;

IS RELATED TO

au4CTPSource, au4CTPBidirectional,

vc4TTPSink, vc4TTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc4TTPSink,vc4TTPBidirectional,

au4CTPSource,au4CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc4TTPSink, vc4TTPBidirectional,

au4CTPSource, au4CTPBidirectional }

};

;

upstreamConnectivityPointer-au4CTPSource CONSTRAINT RULE

OBJECT CLASS

au4CTPSource AND SUBCLASSES;

IS RELATED TO

au4CTPSink, au4CTPBidirectional,

vc4TTPSource, vc4TTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc4TTPSource, vc4TTPBidirectional,

au4CTPSink, au4CTPBidirectional }

};

;

downstreamConnectivityPointer-msCTPSink CONSTRAINT RULE

OBJECT CLASS

msCTPSink AND SUBCLASSES;

IS RELATED TO

msTTPSink, msTTPBidirectional,

msCTPSource, msCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

msTTPSink, msTTPBidirectional,

msCTPSource, msCTPBidirectional }

};

;

upstreamConnectivityPointer-msCTPSource CONSTRAINT RULE

OBJECT CLASS

msCTPSource AND SUBCLASSES;

IS RELATED TO

msTTPSource, msTTPBidirectional,

msCTPSink, msCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

msTTPSource, msTTPBidirectional,

msCTPSink, msCTPBidirectional }

};

;

upstreamConnectivityPointer-msTTPSink CONSTRAINT RULE

OBJECT CLASS

msTTPSink AND SUBCLASSES;

IS RELATED TO

msCTPSink, msCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

msCTPSink, msCTPBidirectional }

};

;

downstreamConnectivityPointer-msTTPSource CONSTRAINT RULE

OBJECT CLASS

msTTPSource AND SUBCLASSES;

IS RELATED TO

msCTPSource, msCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

msCTPSource, msCTPBidirectional }

};

;

downstreamConnectivityPointer-rsCTPSink CONSTRAINT RULE

OBJECT CLASS

rsCTPSink AND SUBCLASSES;

IS RELATED TO

rsTTPSink, rsTTPBidirectional,

rsCTPSource, rsCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsTTPSink, rsTTPBidirectional,

rsCTPSource, rsCTPBidirectional }

};

;

upstreamConnectivityPointer-rsCTPSource CONSTRAINT RULE

OBJECT CLASS

rsCTPSource AND SUBCLASSES;

IS RELATED TO

rsTTPSource, rsTTPBidirectional,

rsCTPSink, rsCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsTTPSource, rsTTPBidirectional,

rsCTPSink, rsCTPBidirectional }

};

;

upstreamConnectivityPointer-rsTTPSink CONSTRAINT RULE

OBJECT CLASS

rsCTPSink AND SUBCLASSES;

IS RELATED TO

rsCTPSink, rsCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsCTPSink, rsCTPBidirectional }

};

;

downstreamConnectivityPointer-rsTTPSource CONSTRAINT RULE

OBJECT CLASS

rsCTPSource AND SUBCLASSES;

IS RELATED TO

rsCTPSource, rsCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsCTPSource, rsCTPBidirectional }

};

;

downstreamConnectivityPointer-tu11CTPSink CONSTRAINT RULE

OBJECT CLASS

tu11CTPSink AND SUBCLASSES;

IS RELATED TO

vc11TTPSink, vc11TTPBidirectional,

tu11CTPSource, tu11CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc11TTPSink, vc11TTPBidirectional,

tu11CTPSource, tu11CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc11TTPSink, vc11TTPBidirectional,

tu11CTPSource, tu11CTPBidirectional }

};

;

upstreamConnectivityPointer-tu11CTPSource CONSTRAINT RULE

OBJECT CLASS

tu11CTPSource AND SUBCLASSES;

IS RELATED TO

vc11TTPSource, vc11TTPBidirectional,

tu11CTPSink, tu11CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc11TTPSource, vc11TTPBidirectional,

tu11CTPSink, tu11CTPBidirectional }

};

;

downstreamConnectivityPointer-tu12CTPSink CONSTRAINT RULE

OBJECT CLASS

tu12CTPSink AND SUBCLASSES;

IS RELATED TO

vc12TTPSink, vc12TTPBidirectional,

tu12CTPSource, tu12CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc12TTPSink, vc12TTPBidirectional,

tu12CTPSource, tu12CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc12TTPSink, vc12TTPBidirectional,

tu12CTPSource, tu12CTPBidirectional }

};

;

upstreamConnectivityPointer-tu12CTPSource CONSTRAINT RULE

OBJECT CLASS

tu12CTPSource AND SUBCLASSES;

IS RELATED TO

vc12TTPSource, vc12TTPBidirectional,

tu12CTPSink, tu12CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc12TTPSource, vc12TTPBidirectional,

tu12CTPSink, tu12CTPBidirectional }

};

;

downstreamConnectivityPointer-tu2CTPSink CONSTRAINT RULE

OBJECT CLASS

tu2CTPSink AND SUBCLASSES;

IS RELATED TO

vc2TTPSink, vc2TTPBidirectional,

tu2CTPSource, tu2CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc2TTPSink, vc2TTPBidirectional,

tu2CTPSource, tu2CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc2TTPSink, vc2TTPBidirectional,

tu2CTPSource, tu2CTPBidirectional }

};

;

upstreamConnectivityPointer-tu2CTPSource CONSTRAINT RULE

OBJECT CLASS

tu2CTPSource AND SUBCLASSES;

IS RELATED TO

vc2TTPSource, vc2TTPBidirectional,

tu2CTPSink, tu2CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc2TTPSource, vc2TTPBidirectional,

tu2CTPSink, tu2CTPBidirectional }

};

;

downstreamConnectivityPointer-tu3CTPSink CONSTRAINT RULE

OBJECT CLASS

tu3CTPSink AND SUBCLASSES;

IS RELATED TO

vc3TTPSink, vc3TTPBidirectional,

au3CTPSource, au3CTPBidirectional,

tu3CTPSource, tu3CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSink, vc3TTPBidirectional,

au3CTPSource, au3CTPBidirectional,

tu3CTPSource, tu3CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc3TTPSink, vc3TTPBidirectional,

au3CTPSource, au3CTPBidirectional,

tu3CTPSource, tu3CTPBidirectional }

};

;

upstreamConnectivityPointer-tu3CTPSource CONSTRAINT RULE

OBJECT CLASS

tu3CTPSource AND SUBCLASSES;

IS RELATED TO

vc3TTPSource, vc3TTPBidirectional,

au3CTPSink, au3CTPBidirectional,

tu3CTPSink, tu3CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSource, vc3TTPBidirectional,

au3CTPSink, au3CTPBidirectional,

tu3CTPSink, tu3CTPBidirectional }

};

;

upstreamConnectivityPointer-vc11TTPSink CONSTRAINT RULE

OBJECT CLASS

vc11TTPSink AND SUBCLASSES;

IS RELATED TO

vc11TTPSource, vc11TTPBidirectional,

tu11CTPSink, tu11CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc11TTPSource, vc11TTPBidirectional,

tu11CTPSink, tu11CTPBidirectional }

};

;

downstreamConnectivityPointer-vc11TTPSource CONSTRAINT RULE

OBJECT CLASS

vc11TTPSource AND SUBCLASSES;

IS RELATED TO

vc11TTPSink, vc11TTPBidirectional,

tu11CTPSource, tu11CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc11TTPSink, vc11TTPBidirectional,

tu11CTPSource, tu11CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc11TTPSink, vc11TTPBidirectional,

tu11CTPSource, tu11CTPBidirectional }

};

;

upstreamConnectivityPointer-vc12TTPSink CONSTRAINT RULE

OBJECT CLASS

vc12TTPSink AND SUBCLASSES;

IS RELATED TO

vc12TTPSource, vc12TTPBidirectional,

tu12CTPSink, tu12CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc12TTPSource, vc12TTPBidirectional,

tu12CTPSink, tu12CTPBidirectional }

};

;

downstreamConnectivityPointer-vc12TTPSource CONSTRAINT RULE

OBJECT CLASS

vc12TTPSource AND SUBCLASSES;

IS RELATED TO

vc12TTPSink, vc12TTPBidirectional,

tu12CTPSource, tu12CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc12TTPSink, vc12TTPBidirectional,

tu12CTPSource, tu12CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc12TTPSink, vc12TTPBidirectional,

tu12CTPSource, tu12CTPBidirectional }

};

;

upstreamConnectivityPointer-vc2TTPSink CONSTRAINT RULE

OBJECT CLASS

vc2TTPSink AND SUBCLASSES;

IS RELATED TO

vc2TTPSource, vc2TTPBidirectional,

tu2CTPSink, tu2CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc2TTPSource, vc2TTPBidirectional,

tu2CTPSink, tu2CTPBidirectional }

};

;

downstreamConnectivityPointer-vc2TTPSource CONSTRAINT RULE

OBJECT CLASS

vc2TTPSource AND SUBCLASSES;

IS RELATED TO

vc2TTPSink, vc2TTPBidirectional,

tu2CTPSource, tu2CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc2TTPSink, vc2TTPBidirectional,

tu2CTPSource, tu2CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc2TTPSink, vc2TTPBidirectional,

tu2CTPSource, tu2CTPBidirectional }

};

;

upstreamConnectivityPointer-vc3TTPSink CONSTRAINT RULE

OBJECT CLASS

vc3TTPSink AND SUBCLASSES;

IS RELATED TO

vc3TTPSource, vc3TTPBidirectional,

au3CTPSink, au3CTPBidirectional,

tu3CTPSink, tu3CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSource, vc3TTPBidirectional,

au3CTPSink, au3CTPBidirectional,

tu3CTPSink, tu3CTPBidirectional }

};

;

downstreamConnectivityPointer-vc3TTPSource CONSTRAINT RULE

OBJECT CLASS

vc3TTPSource AND SUBCLASSES;

IS RELATED TO

vc3TTPSink, vc3TTPBidirectional,

au3CTPSource, au3CTPBidirectional,

tu3CTPSource, tu3CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSink, vc3TTPBidirectional,

au3CTPSource, au3CTPBidirectional,

tu3CTPSource, tu3CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc3TTPSink, vc3TTPBidirectional,

au3CTPSource, au3CTPBidirectional,

tu3CTPSource, tu3CTPBidirectional }

};

;

upstreamConnectivityPointer-vc4TTPSink CONSTRAINT RULE

OBJECT CLASS

vc4TTPSink AND SUBCLASSES;

IS RELATED TO

vc4TTPSource, vc4TTPBidirectional,

au4CTPSink, au4CTPBidirectional,

au3CTPSink, au3CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc4TTPSource, vc4TTPBidirectional,

au4CTPSink, au4CTPBidirectional },

concatenated ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

SEQUENCE SIZE(3) OF au3CTPSink,

SEQUENCE SIZE(3) OF au3CTPBidirectional }

};

;

downstreamConnectivityPointer-vc4TTPSource CONSTRAINT RULE

OBJECT CLASS

vc4TTPSource AND SUBCLASSES;

IS RELATED TO

vc4TTPSink, vc4TTPBidirectional,

au4CTPSource, au4CTPBidirectional,

au3CTPSource, au3CTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc4TTPSink, vc4TTPBidirectional,

au4CTPSource, au4CTPBidirectional },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc4TTPSink, vc4TTPBidirectional,

au4CTPSource, au4CTPBidirectional },

concatenated ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

SEQUENCE SIZE(3) OF au3CTPSource,

SEQUENCE SIZE(3) OF au3CTPBidirectional },

broadcastConcatenated ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

SEQUENCE SIZE(3) OF au3CTPSource,

SEQUENCE SIZE(3) OF au3CTPBidirectional }

};

;

## 8.3	Naming constraints

This subsection defines the allowable combinations of subordinate object class instances that made be named by a superior object class instance, using the object classes contained in this Recommendation.

augSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

augSink;

NAMES SUBORDINATES

au3CTPSink,

au4CTPSink;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF au4CTPSink,

SET SIZE(3) OF au3CTPSink

};

;

augSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

augSource;

NAMES SUBORDINATES

au3CTPSource,

au4CTPSource;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF au4CTPSource,

SET SIZE(3) OF au3CTPSource

};

;

augBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

augBidirectional;

NAMES SUBORDINATES

au3CTPSink, au3CTPSource, au3CTPBidirectional,

au4CTPSink, au4CTPSource, au4CTPBidirectional;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF CHOICE {

au4CTPSink, au4CTPSource, au4CTPBidirectional },

SET SIZE(3) OF CHOICE {

au3CTPSink, au3CTPSource, au3CTPBidirectional }

};

;

electricalSPITTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

electricalSPITTPSink;

NAMES SUBORDINATES

rsCTPSink;

ACCORDING TO RULE

SET SIZE(1) OF rsCTPSink;

;

electricalSPITTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

electricalSPITTPSource;

NAMES SUBORDINATES

rsCTPSource;

ACCORDING TO RULE

SET SIZE(1) OF rsCTPSource;

;

electricalSPITTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

electricalSPITTPBidirectional;

NAMES SUBORDINATES

rsCTPSink, rsCTPSource, rsCTPBidirectional;

ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsCTPSink, rsCTPSource, rsCTPBidirectional };

;

opticalSPITTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

opticalSPITTPSink;

NAMES SUBORDINATES

rsCTPSink;

ACCORDING TO RULE

SET SIZE(1) OF rsCTPSink;

;

opticalSPITTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

opticalSPITTPSource;

NAMES SUBORDINATES

rsCTPSource;

ACCORDING TO RULE

SET SIZE(1) OF rsCTPSource;

;

opticalSPITTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

opticalSPITTPBidirectional;

NAMES SUBORDINATES

rsCTPSink, rsCTPSource, rsCTPBidirectional;

ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsCTPSink, rsCTPSource, rsCTPBidirectional };

;

msTTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

msTTPSink;

NAMES SUBORDINATES

augSink,

msDatacomCTPSink,

msOrderwireCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(1,4,16) OF augSink,

SET SIZE(0..1) OF msDatacomCTPSink,

SET SIZE(0..1) OF msOrderwireCTPSink

};

;

msTTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

msTTPSource;

NAMES SUBORDINATES

augSource,

msDatacomCTPSource,

msOrderwireCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(1,4,16) OF augSource,

SET SIZE(0..1) OF msDatacomCTPSource,

SET SIZE(0..1) OF msOrderwireCTPSource

};

;

msTTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

msTTPBidirectional;

NAMES SUBORDINATES

augBidirectional,

msDatacomCTPSink, msDatacomCTPSource,

msDatacomCTPBidirectional,

msOrderwireCTPSink, msOrderwireCTPSource,

msOrderwireCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(1,4,16) OF augBidirectional,

SET SIZE(0..1) OF CHOICE {

msDatacomCTPSink, msDatacomCTPSource,

msDatacomCTPBidirectional },

SET SIZE(0..1) OF CHOICE {

msOrderwireCTPSink, msOrderwireCTPSource,

msOrderwireCTPBidirectional }

};

;

rsTTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

rsTTPSink;

NAMES SUBORDINATES

msCTPSink,

rsDatacomCTPSink,

rsOrderwireCTPSink,

rsUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(1) OF msCTPSink,

SET SIZE(0..1) OF rsDatacomCTPSink,

SET SIZE(0..1) OF rsOrderwireCTPSink,

SET SIZE(0..1) OF rsUserChannelCTPSink

};

;

rsTTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

rsTTPSource;

NAMES SUBORDINATES

msCTPSource,

rsDatacomCTPSource,

rsOrderwireCTPSource,

rsUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(1) OF msCTPSource,

SET SIZE(0..1) OF rsDatacomCTPSource,

SET SIZE(0..1) OF rsOrderwireCTPSource,

SET SIZE(0..1) OF rsUserChannelCTPSource

};

;

rsTTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

rsTTPBidirectional;

NAMES SUBORDINATES

msCTPSink, msCTPSource, msCTPBidirectional,

rsDatacomCTPSink, rsDatacomCTPSource,rsDatacomCTPBidirectional,

rsOrderwireCTPSink, rsOrderwireCTPSource,

rsOrderwireCTPBidirectional,

rsUserChannelCTPSink, rsUserChannelCTPSource,

rsUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(1) OF CHOICE {

msCTPSink, msCTPSource, msCTPBidirectional },

SET SIZE(0..1) OF CHOICE {

rsDatacomCTPSink, rsDatacomCTPSource,

rsDatacomCTPBidirectional },

SET SIZE(0..1) OF CHOICE {

rsOrderwireCTPSink, rsOrderwireCTPSource,

rsOrderwireCTPBidirectional },

SET SIZE(0..1) OF CHOICE {

rsUserChannelCTPSink, rsUserChannelCTPSource,

rsUserChannelCTPBidirectional }

};

;

sdhNESubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

sdhNE;

NAMES SUBORDINATES

electricalSPITTPSink, electricalSPITTPSource,

electricalSPITTPBidirectional,

msTTPSink, msTTPSource, msTTPBidirectional,

opticalSPITTPSink, opticalSPITTPSource, opticalSPITTPBidirectional,

rsTTPSink, rsTTPSource, rsTTPBidirectional,

vc11TTPSink, vc11TTPSource, vc11TTPBidirectional,

vc12TTPSink, vc12TTPSource, vc12TTPBidirectional,

vc2TTPSink, vc2TTPSource, vc2TTPBidirectional,

vc3TTPSink, vc3TTPSource, vc3TTPBidirectional,

vc4TTPSink, vc4TTPSource, vc4TTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(0..N) OF electricalSPITTPSink,

SET SIZE(0..N) OF electricalSPITTPSource,

SET SIZE(0..N) OF electricalSPITTPBidirectional,

SET SIZE(0..N) OF msTTPSink,

SET SIZE(0..N) OF msTTPSource,

SET SIZE(0..N) OF msTTPBidirectional,

SET SIZE(0..N) OF opticalSPITTPSink,

SET SIZE(0..N) OF opticalSPITTPSource,

SET SIZE(0..N) OF opticalSPITTPBidirectional,

SET SIZE(0..N) OF rsTTPSink,

SET SIZE(0..N) OF rsTTPSource,

SET SIZE(0..N) OF rsTTPBidirectional,

SET SIZE(0..N) OF vc11TTPSink,

SET SIZE(0..N) OF vc11TTPSource,

SET SIZE(0..N) OF vc11TTPBidirectional,

SET SIZE(0..N) OF vc12TTPSink,

SET SIZE(0..N) OF vc12TTPSource,

SET SIZE(0..N) OF vc12TTPBidirectional,

SET SIZE(0..N) OF vc2TTPSink,

SET SIZE(0..N) OF vc2TTPSource,

SET SIZE(0..N) OF vc2TTPBidirectional,

SET SIZE(0..N) OF vc3TTPSink,

SET SIZE(0..N) OF vc3TTPSource,

SET SIZE(0..N) OF vc3TTPBidirectional,

SET SIZE(0..N) OF vc4TTPSink,

SET SIZE(0..N) OF vc4TTPSource,

SET SIZE(0..N) OF vc4TTPBidirectional

};

;

tug2SinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug2Sink;

NAMES SUBORDINATES

tu11CTPSink,

tu12CTPSink,

tu2CTPSink;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF tu2CTPSink,

SET SIZE(3) OF tu12CTPSink,

SET SIZE(4) OF tu11CTPSink

};

;

tug2SourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug2Source;

NAMES SUBORDINATES

tu11CTPSource,

tu12CTPSource,

tu2CTPSource;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF tu2CTPSource,

SET SIZE(3) OF tu12CTPSource,

SET SIZE(4) OF tu11CTPSource

};

;

tug2BidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug2Bidirectional;

NAMES SUBORDINATES

tu11CTPSink, tu11CTPSource, tu11CTPBidirectional,

tu12CTPSink, tu12CTPSource, tu12CTPBidirectional,

tu2CTPSink, tu2CTPSource, tu2CTPBidirectional;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF CHOICE {

tu2CTPSink, tu2CTPSource, tu2CTPBidirectional },

SET SIZE(3) OF CHOICE {

tu12CTPSink, tu12CTPSource, tu12CTPBidirectional },

SET SIZE(4) OF CHOICE {

tu11CTPSink, tu11CTPSource, tu11CTPBidirectional }

};

;

tug3SinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug3Sink;

NAMES SUBORDINATES

tug2Sink,

tu3CTPSink;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF tu3CTPSink,

SET SIZE(7) OF tug2Sink

};

;

tug3SourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug3Source;

NAMES SUBORDINATES

tug2Source,

tu3CTPSource;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF tu3CTPSource,

SET SIZE(7) OF tug2Source

};

;

tug3BidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug3Bidirectional;

NAMES SUBORDINATES

tug2Bidirectional,

tu3CTPSink, tu3CTPSource, tu3CTPBidirectional;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF tu3CTPBidirectional,

SET SIZE(7) OF CHOICE {

tug2Sink, tug2Source, tug2Bidirectional }

};

;

vc3TTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc3TTPSink;

NAMES SUBORDINATES

tug2Sink,

vcnUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(7) OF tug2Sink,

SET SIZE(1) OF vcnUserChannelCTPSink

};

;

vc3TTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc3TTPSource;

NAMES SUBORDINATES

tug2Source,

vcnUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(7) OF tug2Source,

SET SIZE(1) OF vcnUserChannelCTPSource

};

;

vc3TTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc3TTPBidirectional;

NAMES SUBORDINATES

tug2Bidirectional,

vcnUserChannelCTPSink, vcnUserChannelCTPSource, vcnUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(7) OF tug2Bidirectional,

SET SIZE(1) OF CHOICE {

vcnUserChannelCTPSink, vcnUserChannelCTPSource,

vcnUserChannelCTPBidirectional }

};

;

vc4TTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc4TTPSink;

NAMES SUBORDINATES

tug3Sink,

vcnUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(3) OF tug3Sink,

SET SIZE(1) OF vcnUserChannelCTPSink

};

;

vc4TTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc4TTPSource;

NAMES SUBORDINATES

tug3Source,

vcnUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(3) OF tug3Source,

SET SIZE(1) OF vcnUserChannelCTPSource

};

;

vc4TTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc4TTPBidirectional;

NAMES SUBORDINATES

tug3Bidirectional,

vcnUserChannelCTPSink, vcnUserChannelCTPSource, vcnUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(3) OF tug3Bidirectional,

SET SIZE(1) OF CHOICE {

vcnUserChannelCTPSink, vcnUserChannelCTPSource,

vcnUserChannelCTPBidirectional }

};

;

ANNEX  A

(to Recommendation G.774)

Entity relationship diagrams

Figure A-1/G.774 — shows the inheritance hierarchy for the termination points, indirect adaptor and network element object classes of the SDH information model.

Figure A-2/G.774 — shows the naming tree for the SDH information model.

Figure A-3/G.774 — illustrates the naming, connectivity pointer and crossconnect relationships for the SDH information model.

Figure A-4/G.774 — is an example of how the managed objects are used to represent a SDH multiplex and regenerator.

ANNEX  B

(to Recommendation G.774)

Alphabetical list of abbreviations used in this Recommendation

AIS	Alarm indication signal

ASN.1	Abstract syntax notation one

AU	Administrative unit

AUG	AU group

BER	Bit error ratio

CP	Connection point

CTP	Connection termination point

DS	Digital section

FERF	Far end receive failure

FO	Optical fibre

GDMO	Guidelines for the definition of managed objects

HPA	Higher order path adaptation

HPC	Higher order path connection

HPT	Higher order path termination

IA	Indirect adaptor

LOF	Loss of frame

LOP	Loss of pointer

LOS	Loss of signal

LPA	Lower order path adaptation

LPC	Lower order path connection

LPT	Lower order path termination

MS	Multiplex section

MSP	Multiplex section protection

MST	Multiplex section termination

NE	Network element

NNI	Network node interface

PDH	Plesiochronous digital hierarchy

POH	Path overhead

RDN	Relative distinguished name

RS	Regenerator section

RST	Regenerator section termination

SA	Section adaptation

SDH	Synchronous digital hierarchy

SOH	Section overhead

SPI	SDH physical interface

STM	Synchronous transfer mode

TMN	Telecommunications management network

TTP	Trail termination point

TU	Tributary unit

TUG	Tributary unit group

VC	Virtual container

References

[1]	CCITT Recommendation G.707 — Synchronous digital hierarchy bit rates.

[2]	CCITT Recommendation G.708 — Network node interface for the synchronous digital hierarchy.

[3]	CCITT Recommendation G.709 — Synchronous multiplexing structure.

[4]	CCITT Recommendation M.3010 — Principles for a telecommunication management network.

[5]	CCITT Recommendation M.3100 — Generic network information model.

[6]	CCITT Recommendation G.783 — Characteristics of synchronous digital hierarchy (SDH) multiplexing equipment functional blocks.

[7]	CCITT Recommendation G.784 — Synchronous digital hierarchy (SDH) management.

[8]	CCITT Recommendation X.722 — Information technology — Open Systems Interconnection — Structure of management information:  Guidelines for the definition of managed objects.

[9]	CCITT Recommendation X.208 — Specification of Abstract Syntax Notation One (ASN.1).

[10]	CCITT Recommendation X.720 — Information technology — Open Systems Interconnection — Structure of management information:  Management information model.