|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.774.03

TELECOMMUNICATION	(11/94)
STANDARDIZATION  SECTOR
OF  ITU

GENERAL  ASPECTS  OF  DIGITAL
TRANSMISSION  SYSTEMS

SYNCHRONOUS  DIGITAL  HIERARCHY  (SDH)  MANAGEMENT  OF  MULTIPLEX-SECTION  PROTECTION  FOR  THE  NETWORK  
ELEMENT  VIEW

ITU-T  Recommendation  G.774.03

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation G.774.03 was prepared by ITU-T Study Group 15 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 1st of November 1994.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation G.774.03     (11/94)

Page

1	Scope		1

1.1	Structure of this Recommendation		1

2	References		2

3	Definitions		2

4	Abbreviations		2

5	Multiplex-Section Protection Management Model		3

5.1	Overview		3

5.2	SDH Multiplex Section Protection Requirements		3

5.2.1	Generic SDH Transmission Protection Functional Requirements		4

5.2.2	SDH Multiplex Section Specific Functional Requirements		4

6	Managed Object Class Definitions		5

6.1	ApsReportRecord		5

6.2	Protected Trail Termination Point		6

6.3	Protection Group		7

6.4	Protection Unit		8

6.5	SDH Multiplex Section Protection Group		8

6.6	SDH Multiplex Section Protection Unit		9

6.7	Unprotected Connection Termination Point		9

7	Packages		10

7.1	Extra Traffic Control		10

7.2	Last Attempt Result		10

7.3	Protection Switch Exercise		10

7.4	Protection Mismatch Status		11

7.5	Priority		11

7.6	Sdh Priority		11

8	Attributes		11

8.1	Channel Number		11

8.2	Last Attempt Result		11

8.3	Priority		11

8.4	Protected Trail Termination Point Identifier		12

8.5	Reported Protection Unit		12

8.6	Protection Group Identifier		12

8.7	Protection Group Type		12

8.8	Protection Mismatch Status		13

8.9	Protection Status		13

8.10	Protection Switch Mode		16

8.11	Protection Unit Identifier		16

8.12	Protecting		16

8.13	Reliable Resource Pointer		17

8.14	Revertive		17

8.15	Sdh Priority		17

8.16	Unprotected Connection Termination Point Identifier		17

8.17	Unreliable Resource Pointer		18

8.18	Wait to Restore Time		18

Page

9	Actions		18

9.1	Invoke Exercise		18

9.2	Invoke Protection		19

9.3	Release Protection		19

10	Notifications		20

10.1	Protection Switch Reporting		20

11	Parameters		20

11.1	Invoke Protection Error		20

11.2	Release Protection Error		20

11.3	Protection Status Parameter		21

12	Name Bindings		21

12.1	Protected Trail Termination Point		21

12.2	Protection Group		22

12.3	Aug		22

12.4	Protection Unit		23

12.5	Unprotected Connection Termination Point		23

13	Subordination Rules		24

14	Pointer Constraints		24

15	Supporting ASN.1 Productions		24

Annex A  –  Naming and Inheritance Diagrams		27

Annex B  –  Examples of MS Protection		29

# SUMMARY

This Recommendation provides an information model for the Synchronous Digital Hierarchy (SDH). This model describes the managed object classes and their properties for the protection switching function, as defined in ITU-T Recommendation G.803 and as related to SDH transmission resources. These objects are useful to describe information exchanged across interfaces defined in ITU-T Recommendation M.3010 Telecommunications Management Network (TMN) architecture for the management of the protection function.

##### Keywords

Action, ASN.1, Attribute, GDMO, Information Model, Managed Object Class, Notification, Protection, Synchronous Digital Hierarchy.

Recommendation G.774.03

Recommendation G.774.03     (11/94)

SYNCHRONOUS  DIGITAL  HIERARCHY  (SDH)
MANAGEMENT  OF  MULTIPLEX-SECTION  PROTECTION
FOR  THE  NETWORK  ELEMENT  VIEW

(Geneva, 1994)

The ITU,

considering

(a)	that Recommendations G.707, G.708, and G.709 form a coherent set of specifications for the Synchronous Digital Hierarchy (SDH) and the Network Node Interface (NNI);

(b)	that Recommendations G.781, G,782, G.783, and G.784 form a coherent set of specifications for SDH multiplex equipment functions and management;

(c)	that Recommendation G.958 specifies the characteristics of digital line systems based on SDH for use on optical fibre cables;

(d)	that Recommendation M.3010 defines the principles for a Telecommunications Management Network (TMN);

(e)	that Recommendation G.773 defines the protocol suites for Q-interfaces;

(f)	that Recommendation M.3100 defines a Generic Network Information Model for the exchange of management information;

(g)	that Recommendation G.774 defines an SDH Management information Model for the Network Element view;

(h)	that Recommendation G.774.01 defines an SDH Management information Model for the Network Element view for performance Monitoring,

recommends

that the management of SDH equipment be carried out by using the information model defined in accordance with the details contained within this Recommendation.

# 1	Scope

This Recommendation provides an information model, as related to the protection function for the Synchronous Digital Hierarchy (SDH). It identifies the Telecommunications Management Network (TMN) object classes required for the management of the protection function for SDH network elements. These objects are relevant to information exchanged across standardized interfaces defined in the Recommendation M.3010 TMN architecture.

This Recommendation applies to SDH network elements which perform the Multiplex Section protection function and those systems in the TMN that manage SDH network elements. Functional capabilities of SDH multiplex equipment, particularly the Multiplex Section protection switching function, are given in Recommendation G.783. Performance monitoring requirements for Multiplex Section protection (for the management of SDH equipment with this capability) are provided in Recommendation G.784, however, the information model which supports these can be found in Recommendation G.774.01.

## 1.1	Structure of this Recommendation

Subclause 5.1 provides an overview of the SDH protection information model in this Recommendation. Clauses 3 to 15 describe the information model using the notation mechanisms defined in X.722 Guidelines for the Definition of Managed Objects. Clause 15 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in Recommendation X.208. Naming and Inheritance are diagrammatically shown

in informative Annex A. Diagrams illustrating the application of the SDH protection model are provided in informative Annex B.

A text version of clauses 5 to 15 is available in diskette from the ITU.

# 2	References

–	ITU-T Recommendation G.707 (1993), Synchronous digital hierarchy bit rates.

–	ITU-T Recommendation G.708 (1993), Network-node interface for the synchronous digital hierarchy.

–	ITU-T Recommendation G.709 (1993), Synchronous multiplexing structure.

–	CCITT Recommendation M.3010 (1992), Principles for a telecommunications management network.

–	CCITT Recommendation M.3100 (1992), Generic network information model.

–	ITU-T Recommendation G.783 (1994), Characteristics of Synchronous Digital Hierarchy (SDH) equipment functional blocks.

–	ITU-T Recommendation G.784 (1994), Synchronous Digital Hierarchy (SDH) management.

–	CCITT Recommendation X.722 (1992), Information technology – OSI – Structure of management information: Guidelines for the definition of managed objects.

–	CCITT Recommendation X.208 (1988), Specification of Abstract Syntax Notation One (ASN.1).

–	CCITT Recommendation X.720 (1992), Information technology – OSI – Structure of management information: Management information model.

–	CCITT Recommendation G.774 (1992), SDH Management information model for the network element view.

–	ITU-T Recommendation G.774.01 (1994), SDH Performance monitoring for the network element view.

–	ITU-T Recommendation G.803 (1993), Architectures of transport networks based on the Synchronous Digital Hierarchy (SDH).

–	CCITT Recommendation X.721 (1992), Definition of management information.

# 3	Definitions

None.

# 4	Abbreviations

For the purposes of this Recommendation, the following abbreviations apply:

APDU	Application Protocol Data Unit

APS	Automatic Protection Switching

CMIP	Common Management Information Protocol

CMIS	Common Management Information Service

CTP	Connection Termination Point

ISO	International Organization for Standardization

ITU	International Telecommunications Union

LOS	Loss of Signal

MS	Multiplex Section

MSP	Multiplex Section Protection

NE	Network Element

OS	Operation System

OSI	Open System Interconnection

Pkg	Packages

RDN	Relative Distinguished Name

SD	Signal Degrade

SDH	Synchronous Digital Hierarchy

SF	Signal Fail

STM-N	Synchronous Transport Module N

TMN	Telecommunication Management Network

TP	Termination Point

TSS	Telecommunication Standardization Sector

TTP	Trail Termination Point

WTR	Wait-to-Restore

# 5	Multiplex-Section Protection Management ModelXE "Multiplex-Section Protection Management Model"§

## 5.1	Overview

The SDH information model for the Multiplex Section protection switching function is based on the requirements to provide the protection function for transmission in SDH networks. Resources which must be protected have been modelled and are described in Recommendation G.774. They are based on the Generic Network Information Model of Recommendation M.3100. The Generic Network Information Model includes a Termination Point fragment which serves as a structure for specialization for specific technologies such as the SDH.

The information exchanged at a management interface is modelled using design principles outlined in Recommendation X.720 Management Information Model. Resources are modelled as objects, and the management view of a resource is a managed object. Objects with similar attributes may be grouped into object classes. An object is characterized by its object class and object instance, and may possess multiple attribute types and associated values. The terms managed object class and managed object instance apply specifically to objects that are being managed. This Recommendation specifies the properties of the resource visible for management.

An object class may be a subclass of another class. A subclass inherits attribute types, packages and behaviours of its super class, in addition to possessing its own specific attributes and properties. The SDH specific object classes are all derived from super classes in the Generic Network Information Model Recommendation M.3100.

Object classes and attribute types are defined only for the purpose of communicating network management messages between systems, and need not be related to the structure of data within those systems. The object classes defined in this issue of the SDH information model can apply to multiple management functional areas (e.g. fault management and configuration management).

There are several different viewpoints of management information that may be defined for management purposes. The network element viewpoint is concerned with the information that is required to manage a network element. This refers to information required to manage the protection function and the physical aspects of the network element. This Recommendation addresses only the network element viewpoint.

## 5.2	SDH Multiplex Section Protection RequirementsXE "SDH Multiplex Section Protection Requirements"§

The SDH specific objects specified in this Recommendation shall be used to manage the specific transport resources of SDH network elements, as related to the Multiplex Section protection function. Implementations shall conform to both the management information defined in clauses 3 to 15 and the requirements identified in 5.2.

This Recommendation addresses the management of the automatic protection switching within the network element at the Multiplex Section. The management for Multiplex Section protection is based on the protection function described in Recommendation G.803.

### 5.2.1	Generic SDH Transmission Protection Functional RequirementsXE "Generic SDH Transmission Protection Functional Requirements"§

The application specific management models for SDH systems are based on a generic protection model for transmission which satisfies the following common protection requirements:

1)	Manage a group of protected resources together with their protecting resources as one protection group, where all members of the protection group in a network element may be configured for:

a)	m:n or 1 + 1 type protection;

b)	revertive or non-revertive type switching;

c)	a specific wait-to-restore time (in the case of revertive systems).

2)	Indicate the ability of the group of protection units as a whole to provide the protection switching function properly and to send a notification when a change in this operational state occurs.

3)	Indicate the status of each individual protection unit such as automatic switch completed.

4)	Send a notification, identifying the protection unit and protection group, when a protection switch event occurs. Specific notification requirements are application specific.

5)	Send a notification when protection resources are added or removed on the NE.

6)	For each protected or protecting resource, the ability to perform the following management operations:

a)	invoke a manual protection request;

b)	invoke a forced protection switch;

c)	lockout a protection or working channel;

d)	determine the operational state of the protection group;

e)	indicate a resource as protecting or protected (although most but not all 1 + 1 systems are symmetrical with respect to their protection switching or management functionality). The equipment in the NE determines this operation and provides this indication;

f)	ability to set the switch priority for protected resources in 1:n systems.

The generic information model for SDH protection management is provided by the protectionGroup and protectionUnit objects.

### 5.2.2	SDH Multiplex Section Specific Functional RequirementsXE "SDH Multiplex Section Specific Functional Requirements"§

The management information model for linear system Multiplex Section protection provided in this Recommendation is based on the generic model and approach to protection, which satisfies the requirements described in 5.2.1. Additionally, the following SDH MS specific requirements exist:

1)	Ability to configure the protection switch mode for all protection resources within a protection group as uni or bidirectional.

2)	Ability to send a notification of the protection status of each protected or protecting resource. The protection status value shall indicate the current switch request active or pending (in the case of automatic switch requests only) on any given protection unit (resource).

3)	Manage extra traffic as follows:

a)	ability to explicitly suspend and resume extra traffic in a protection unit;

b)	prevent extra traffic from being preempted by other switch requests;

c)	allow Extra Traffic to be preempted by higher priority requests and allow Extra Traffic to be automatically re-established when the higher priority request is removed;

d)	ability to change between modes b) and c).

4)	Send the following protection switch related notifications:

a)	when a protected resource, having been switched to a protecting resource, is preempted by a higher priority request from the same or another protected resource (applicable in 1:n systems);

b)	when a protected resource (unit) is switched onto a protecting resource;

c)	when any switch is released;

d)	when an auto-switch condition exists but cannot be served due to a higher priority request already being served.

5)	Ability to detect a mismatch between near and far-end protection group type configured values (1 + 1 or 1:n).

6)	Ability to invoke an exercise on a protected channel.

7)	Allow only one protecting resource.

8)	Store the most recent protection switch or exercise result, and provide a notification – against the associated group of protection resources as a whole – when a failure to perform the protection function is detected.

With respect to Recommendation G.774 termination points which represent the protected (working channels) resources, the following requirements exist:

9)	Provide a relationship between the termination points which represent the protected (working channels) and the termination point which represents the protecting (protection channel), and the termination point which reflects the reliable resource.

10)	In performing a protection switch, update the connectivity pointers of the affected termination points to indicate the new traffic flow.

The objects relevant to SDH MS Protection Switching are the following sub-classes:

sdhMSProtectionGroup and sdhMSProtectionUnit. Additionally, sub-classes of the termination point model are required. These are protectedTTPSink, protectedTTPSource, protectedTTPBidirectional, unprotectedCTPSink, unprotectedCTPSource, and unprotectedCTPBidirectional.

With respect to the creation/deletion of the protection management objects:

11)	It is the responsibility of the NE to create and delete the objects related to protection management according to the make up and mode of the NE.

# 6	Managed Object Class DefinitionsXE "Managed Object Class Definitions"§

To allow SDH equipment to be represented in a consistent manner across the interface, the conditional package crossConnectionObjectPointerPackage in Recommendation M.3100 is made mandatory in this Recommendation. The following conditional packages inherited from Recommendation M.3100 shall not be used when the SDH object classes defined in this Recommendation are instantiated: ttpInstancePackage, ctpInstancePackage, networkLevelPackage, characteristicInformationPackage, channelNumberPackage.

## 6.1	ApsReportRecordXE "ApsReportRecord"§

apsReportRecord MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation X.721 | ISO/IEC 10165-2 : 1992": eventLogRecord;

CHARACTERIZED BY

apsReportRecordPkg PACKAGE

BEHAVIOUR apsReportRecordBeh;

ATTRIBUTES

reportedProtectionUnit GET;;;

REGISTERED AS { g774-03ObjectClass 1 };

apsReportRecordBeh BEHAVIOUR

DEFINED AS

"An instance of the apsReportRecord object is created when a protectionSwitchReporting Notification is generated. The protectionStatusParameter must be included in the management extension.

";

## 6.2	Protected Trail Termination PointXE "Protected Trail Termination Point"§

protectedTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation M.3100 : 1992": trailTerminationPointBidirectional;

protectedTTPSource,

protectedTTPSink;

CHARACTERIZED BY

protectedTTPBidirectionalPkg PACKAGE

BEHAVIOUR protectedTTPBidirectionalBeh;;;

REGISTERED AS { g774-03MObjectClass 2 };

protectedTTPBidirectionalBeh BEHAVIOUR

DEFINED AS

"The protectedTTPBidirectional object class is a class of objects that represents the bidirectional protected resources in a protection system. If the SDH bidirectional multiplex section protection switching function is present, this object class shall be supported.

";

protectedTTPSink MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation M.3100 : 1992": trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100 : 1992": crossConnectionPointerPackage,

protectedTTPSinkPkg PACKAGE

BEHAVIOUR protectedTTPSinkBeh;

ATTRIBUTES

protectedTTPId          GET;;;

REGISTERED AS { g774-03MObjectClass 3 };

protectedTTPSinkBeh BEHAVIOUR

DEFINED AS

"The protectedTTPSink object class is a class of objects that represents the protected resources in a protection system. An instance of this object class is pointed to by the reliableResourcePointer attribute in an instance of the protectionUnit object class. The crossConnectionObjectPointer attribute in an instance of this object class points to its associated protectionUnit instance which has the reliableResourcePointer attribute pointing back to the instance of this object class. The upstreamConnectivityPointer in an instance of this object class points to either null or its associated unprotected CTP object instance; it indicates the actual signal flow, and when a signal is switched to another unit, the value of the upstreamConnectivityPointer is updated. If the attributeValueChangeNotification package is present, then a change in the value of the supportedByObjectList shall cause an attributeValueChange notification.

";

protectedTTPSource MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation M.3100 : 1992": trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100 : 1992": crossConnectionPointerPackage,

protectedTTPSourcePkg PACKAGE

BEHAVIOUR protectedTTPSourceBeh;

ATTRIBUTES

protectedTTPId          GET;;;

REGISTERED AS { g774-03MObjectClass 4 };

protectedTTPSourceBeh BEHAVIOUR

DEFINED AS

"The protectedTTPSource object class is a class of objects that represents the protected resources in a protection system. An instance of this object class is pointed to by the reliableResourcePointer attribute in an instance of the protectionUnit object class. The crossConnectionObjectPointer attribute in an instance of this object class points to its associated protectionUnit object instance which has the reliableResourcePointer attribute pointing back to the instance of this object class. The downstreamConnectivityPointer in an instance of this object class points to either null or its associated unprotected CTP object instance(s); it indicates the actual signal flow, and when a signal is switched to another unit, the pointer is updated. If the attributeValueChangeNotification package is present, then a change in the value of the supportedByObjectList shall cause an attributeValueChange notification.

";

## 6.3	Protection GroupXE "Protection Group"§

protectionGroup MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation X.721 | ISO/IEC 10165-2 : 1992": top;

CHARACTERIZED BY

protectionGroupPkg PACKAGE

BEHAVIOUR protectionGroupBeh;

ATTRIBUTES

"Recommendation X.721 | ISO/IEC 10165-2 : 1992": operationalState        GET,

"Recommendation X.721 | ISO/IEC 10165-2 : 1992": availabilityStatus  GET,

protectionGroupId GET,

protectionGroupType   GET-REPLACE,

revertive       REPLACE-WITH-DEFAULT

DEFAULT VALUE SDHProtASN1.booleanTrueDefault      GET-REPLACE,

"Recommendation M.3100 : 1992": supportedByObjectList        GET,

waitToRestoreTime GET-REPLACE;

ACTIONS

invokeProtection,

releaseProtection;

NOTIFICATIONS

protectionSwitchReporting,

"Recommendation X.721 | ISO/IEC 10165-2 : 1992": stateChange;;;

CONDITIONAL PACKAGES

"Recommendation M.3100 : 1992": createDeleteNotificationsPackage

PRESENT IF   "an instance supports it",

"Recommendation M.3100 : 1992": attributeValueChangeNotificationPackage

PRESENT IF   "an instance supports it";

REGISTERED AS { g774-03MObjectClass 5 };

protectionGroupBeh BEHAVIOUR

DEFINED AS

"A protectionGroup object instance contains zero or more protectionUnit objects for defining a protection switching relationship where one or more standby (i.e. backup) entities provide protection for one or more working (i.e. regular or preferred) entities.

The invokeProtection action can be used to request a lockout, a forced switch, or a manual switch (i.e. normal switch) on one or more protectionUnit instances contained in the protectionGroup object. The releaseProtection action can be used to release a lockout, a forced switch, or a manual switch (i.e. normal switch) on one or more protectionUnit instances contained in the protectionGroup object.

The protectionSwitchReporting notification is emitted from the protectionGroup object to report any protection switch events, such as protection switching (forced switch, manual switch, or automatic switch), protection release (release of forced switch, manual switch, or automatic switch), lockout or release of lockout.

If the attributeValueChangeNotification package is present, then changes to the protectionGroupType attribute, the revertive attribute or the waitToRestoreTime attribute shall cause an attributeValueChange notification to be emitted.

The protectionGroupType attribute shall have the value 'colon' when more than one protectionUnit is protected. Changing the value of this attribute between 'plus' and 'colon' is allowed when only one protected protectionUnit and one protecting protectionUnit are contained by the protectionGroup. The change from 'plus' to 'colon' is only allowed if the underlying resources support M:N protection.

";

## 6.4	Protection UnitXE "Protection Unit"§

protectionUnit MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation X.721 | ISO/IEC 10165-2 : 1992": top;

CHARACTERIZED BY

protectionUnitPkg PACKAGE

BEHAVIOUR protectionUnitBeh;

ATTRIBUTES

protectionUnitId          GET,

protecting                     GET,

reliableResourcePointer       GET,

unreliableResourcePointer     GET;;;

CONDITIONAL PACKAGES

priorityPkg PRESENT IF   "an instance supports it",

"Recommendation M.3100 : 1992": attributeValueChangeNotificationPackage

PRESENT IF       "an instance supports it";

REGISTERED AS { g774-03MObjectClass 6 };

protectionUnitBeh BEHAVIOUR

DEFINED AS

"A protectionUnit object represents a protected (i.e. working, regular, or preferred) unit or a protecting (i.e. backup or standby) unit. For a protecting protectionUnit, the attribute protecting shall have the value TRUE. For a protected protectionUnit, the attribute protecting shall have the value FALSE. The value of the unreliableResourcePointer attribute points to an unreliable resource (e.g. equipment hardware or termination point) that is associated with the protectionUnit instance. The value of the reliableResourcePointer attribute points to a reliable resource (e.g. termination point) that is associated with the protectionUnit instance.

If the attributeValueChangeNotificationPackage is present, then changes to the unreliableResourcePointer attribute, the priority attribute or the reliableResourcePointer attribute shall cause an attributeValueChange notification to be emitted.

";

## 6.5	SDH Multiplex Section Protection GroupXE "SDH Multiplex Section Protection Group"§

sdhMSProtectionGroup MANAGED OBJECT CLASS

DERIVED FROM    protectionGroup;

CHARACTERIZED BY

sdhMSProtectionGroupPkg PACKAGE

BEHAVIOUR sdhMSProtectionGroupBeh;

ATTRIBUTES

protectionSwitchMode     GET-REPLACE;

NOTIFICATIONS

protectionSwitchReporting protectionStatusParameter;;;

CONDITIONAL PACKAGES

protectionMismatchStatusPkg PRESENT IF "the APS protocol is used",

protectionSwitchExercisePkg PRESENT IF "an instance supports it";

REGISTERED AS { g774-03MObjectClass 7 };

sdhMSProtectionGroupBeh BEHAVIOUR

DEFINED AS

"This object class is used specifically for representing a SDH multiplex section protection group in a protection system. Only one protecting protection unit is allowed.

The protectionMismatchStatus indicates a mismatch between the provisioned protectionGroupType of this protection group and the provisioned protectionGroupType of the far-end. It also indicates mismatch of unidirectional versus bidirectional switch provisioning between the two protection groups.

";

## 6.6	SDH Multiplex Section Protection UnitXE "SDH Multiplex Section Protection Unit"§

sdhMSProtectionUnit MANAGED OBJECT CLASS

DERIVED FROM    protectionUnit;

CHARACTERIZED BY

sdhMSProtectionUnitPkg PACKAGE

BEHAVIOUR sdhMSProtectionUnitBeh;

ATTRIBUTES

channelNumber         GET,

protectionStatus              GET,

reliableResourcePointer PERMITTED VALUES

SDHProtASN1.SDHMSResourcePointer,

unreliableResourcePointer  PERMITTED VALUES

SDHProtASN1.SDHMSResourcePointer;;;

CONDITIONAL PACKAGES

sdhPriorityPkg PRESENT IF "this is a 1:n system",

lastAttemptResultPkg PRESENT IF "the APS protocol is used",

extraTrafficControlPkg PRESENT IF "extra traffic may be suspended and resumed";

REGISTERED AS { g774-03MObjectClass 8 };

sdhMSProtectionUnitBeh BEHAVIOUR

DEFINED AS

"This object class is specific to SDH multiplex section protection systems. Instances of this object class are used to represent a relationship between a protectedTTP and an unprotectedCTP. If this is a protecting protection unit, the reliableResourcePointer points to the protectedTTP for extra traffic or NULL if there is no extra traffic.

The channelNumber attribute value represents the number of the channel used by the SDH MSP protocol, if any. 
If the sdhMSProtectionUnit is protecting, then the channelNumber attribute shall have the value 0. If the sdhMSProtectionUnit is protected, the channelNumber attribute shall have a value between 1 and 14.

For Lockout of protecting unit, protected channels 1 to 14 are incapable of being carried on protection.

";

## 6.7	Unprotected Connection Termination PointXE "Unprotected Connection Termination Point"§

unprotectedCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation M.3100 : 1992": connectionTerminationPointBidirectional;

unprotectedCTPSource,

unprotectedCTPSink;

CHARACTERIZED BY

unprotectedCTPBidirectionalPkg PACKAGE

BEHAVIOUR unprotectedCTPBidirectionalBeh;;;

REGISTERED AS { g774-03MObjectClass 9 };

unprotectedCTPBidirectionalBeh BEHAVIOUR

DEFINED AS

"The unprotectedCTPBidirectional object class is a class of objects that represents the unprotected resources in a protection system.

";

unprotectedCTPSink MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation M.3100 : 1992": connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100 : 1992": crossConnectionPointerPackage,

unprotectedCTPSinkPkg PACKAGE

BEHAVIOUR unprotectedCTPSinkBeh;

ATTRIBUTES

unprotectedCTPId    GET;;;

REGISTERED AS { g774-03MObjectClass 10 };

unprotectedCTPSinkBeh BEHAVIOUR

DEFINED AS

"The unprotectedCTPSink object class is a class of objects that represents the unprotected resources in a protection system. An instance of this object class is pointed to by the unreliableResourcePointer attribute in an instance of the protectionUnit object class (or its subclasses). The crossConnectionObjectPointer attribute in an instance of this object class points to its associated protectionUnit object instance which has the unreliableResourcePointer attribute pointing back to the instance of this object class.

";

unprotectedCTPSource MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation M.3100 : 1992": connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100 : 1992": crossConnectionPointerPackage,

unprotectedCTPSourcePkg PACKAGE

BEHAVIOUR unprotectedCTPSourceBeh;

ATTRIBUTES

unprotectedCTPId    GET;;;

REGISTERED AS { g774-03MObjectClass 11 };

unprotectedCTPSourceBeh BEHAVIOUR

DEFINED AS

"The unprotectedCTPSource object class is a class of objects that represents the unprotected resources in a protection system. An instance of this object class is pointed to by the unreliableResourcePointer attribute in an instance of the protectionUnit object class (or its subclasses). The crossConnectionObjectPointer attribute in an instance of this object class points to its associated protectionUnit object instance which has the reliableResourcePointer attribute pointing back to the instance of this object class.

";

# 7	Packages

## 7.1	Extra Traffic ControlXE "Extra Traffic Control"§

extraTrafficControlPkg PACKAGE

BEHAVIOUR extraTrafficControlPkgBeh;

ATTRIBUTES

"Recommendation X.721 | ISO/IEC 10165-2 : 1992": administrativeState     GET-REPLACE;

REGISTERED AS { g774-03Package 1 };

extraTrafficControlPkgBeh BEHAVIOUR

DEFINED AS

"Attribute administrativeState is used to suspend or resume extra traffic on the protecting channel. When in the Locked state, no extra traffic is carried on the protecting channel, however, this does not prevent protected channel traffic from being switched to the protecting channel. When unlocked, extra traffic is resumed on the protecting channel as long as there is no request from a protected channel of higher priority than on the protecting channel.

";

## 7.2	Last Attempt ResultXE "Last Attempt Result"§

lastAttemptResultPkg PACKAGE

ATTRIBUTES

lastAttemptResult    GET;

REGISTERED AS { g774-03Package 2 };

## 7.3	Protection Switch ExerciseXE "Protection Switch Exercise"§

protectionSwitchExercisePkg PACKAGE

ACTIONS

invokeExercise;

REGISTERED AS { g774-03Package 3 };

## 7.4	Protection Mismatch StatusXE "Protection Mismatch Status"§

protectionMismatchStatusPkg PACKAGE

ATTRIBUTES

protectionMismatchStatus    GET;

REGISTERED AS { g774-03Package 4 };

## 7.5	PriorityXE "Priority"§

priorityPkg PACKAGE

ATTRIBUTES

priority        GET-REPLACE;

REGISTERED AS { g774-03Package 5 };

## 7.6	Sdh PriorityXE "Sdh Priority"§

sdhPriorityPkg PACKAGE

ATTRIBUTES

sdhPriority         PERMITTED VALUES SDHProtASN1.SDHMSPriority     GET-REPLACE;

REGISTERED AS { g774-03Package 6 };

# 8	Attributes

## 8.1	Channel NumberXE "Channel Number"§

channelNumber ATTRIBUTE

WITH ATTRIBUTE SYNTAX       SDHProtASN1.Integer;

MATCHES FOR             EQUALITY, ORDERING;

BEHAVIOUR channelNumberBeh;

REGISTERED AS { g774-03Attribute 1 };

channelNumberBeh BEHAVIOUR

DEFINED AS

"This attribute indicates the channel number associated with a protectionUnit.

";

## 8.2	Last Attempt Result

lastAttemptResult ATTRIBUTE

WITH ATTRIBUTE SYNTAX       SDHProtASN1.LastAttemptResult;

MATCHES FOR             EQUALITY;

BEHAVIOUR lastAttemptResultBeh;

REGISTERED AS { g774-03Attribute 2 };

lastAttemptResultBeh BEHAVIOUR

DEFINED AS

"The value of the last exercise result or the result of a protection group/unit diagnostic or switch attempt which performs at least the same functionality as an exercise, is available from his attribute. If a failure ocurs, the availabilityStatus of the protectionGroup object will be set to degraded. The successful completion of a protection group/unit diagnostic or switch attempt which performs at least the same functionality as an exercise causes the availabilityStatus attribute to be set to NULL (i.e available), and the lastAttemptResult value is set to success.

";

## 8.3	Priority

priority ATTRIBUTE

WITH ATTRIBUTE SYNTAX       SDHProtASN1.Integer;

MATCHES FOR             EQUALITY, ORDERING;

BEHAVIOUR priorityBeh;

REGISTERED AS { g774-03Attribute 3 };

priorityBeh BEHAVIOUR

DEFINED AS

"This attribute specifies the priority of the service (e.g. traffic) carried on the resource associated with the protected protectionUnit instance. Valid values for this attribute are integers, where the value 1 indicates the highest priority, and a larger value indicates a lower priority.

For a protecting protectionUnit, the value of this attribute indicates the priority of choice of the protecting protectionUnit relative to other available protecting protectionUnit(s) within the same protectionGroup. The lower the value, the more preferred the protectionUnit is relative to other protectionUnits.

";

## 8.4	Protected Trail Termination Point IdentifierXE "Protected Trail Termination Point Identifier"§

protectedTTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX       SDHProtASN1.NameType;

MATCHES FOR             EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR protectedTTPIdBeh;

REGISTERED AS { g774-03Attribute 4 };

protectedTTPIdBeh BEHAVIOUR

DEFINED AS

"This attribute is used as an RDN for naming instances of the protectedTTP object classes. If the string choice of the syntax is used, then matching on substrings is permitted. If the number choice for the syntax is used, then matching on ordering is permitted.

";

## 8.5	Reported Protection UnitXE "Reported Protection Unit"§

reportedProtectionUnit ATTRIBUTE

WITH ATTRIBUTE SYNTAX       SDHProtASN1.RelativeDistinguishedName;

MATCHES FOR             EQUALITY;

REGISTERED AS { g774-03Attribute 5 };

## 8.6	Protection Group IdentifierXE "Protection Group Identifier"§

protectionGroupId ATTRIBUTE

WITH ATTRIBUTE SYNTAX       SDHProtASN1.NameType;

MATCHES FOR             EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR protectionGroupIdBeh;

REGISTERED AS { g774-03Attribute 6 };

protectionGroupIdBeh BEHAVIOUR

DEFINED AS

"The protectionGroupId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the protectionGroup object class. If the string choice of the syntax is used, then matching on substrings is permitted. If the number choice for the syntax is used, then matching on ordering is permitted.

";

## 8.7	Protection Group TypeXE "Protection Group Type"§

protectionGroupType ATTRIBUTE

WITH ATTRIBUTE SYNTAX       SDHProtASN1.ProtectionGroupType;

MATCHES FOR             EQUALITY;

BEHAVIOUR protectionGroupTypeBeh;

REGISTERED AS { g774-03Attribute 7 };

protectionGroupTypeBeh BEHAVIOUR

DEFINED AS

"This attribute specifies whether the protection scheme used is 1 + 1 (plus) or M:N (colon). M:N includes the cases where M = 1 and/or N = 1.

";

## 8.8	Protection Mismatch StatusXE "Protection Mismatch Status"§

protectionMismatchStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX       SDHProtASN1.ProtectionMismatchStatus;

MATCHES FOR             EQUALITY;

BEHAVIOUR protectionMismatchStatusBeh;

REGISTERED AS { g774-03Attribute 8 };

protectionMismatchStatusBeh BEHAVIOUR

DEFINED AS

"This attribute indicates a mismatch between the local and far-end provisioned values for the protectionGroupType attribute and has a separate indication for a mismatch between the local and far-end provisioned values for the protectionSwitchMode attribute.

";

## 8.9	Protection StatusXE "Protection Status"§

protectionStatus ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.ProtectionStatus;

MATCHES FOR	EQUALITY, SET-COMPARISON,
SET-INTERSECTION;

BEHAVIOUR protectionStatusBeh;

REGISTERED AS { g774-03Attribute 9 };

protectionStatusBeh BEHAVIOUR

DEFINED AS

"This attribute is used to indicate the status of the protection switch in a protectionUnit instance. This attribute is set-valued because some requests are allowed to be pending. The following rule shall be followed: Only one of the values lockout, forcedSwitch, or manualSwitch can be present at the same time, either local or remote. It is also possible to have two or more pending automatic switch requests. The protectionStatus attribute of a protection Unit which provides protection is used to hold the protection request which is actually performed on that protectionUnit.

Locally invoked management operations (e.g. manual, forced, lockout switch completions) can be overridden at the near end or the far end, but may only be released at the near end.

Single-ended systems keep the status of local and far-end switch requests as separate entitities in the protectionStatus attribute.

The remainder of this behaviour provides the allowable protectionStatus attribute values for protected and protecting units for both revertive and non-revertive systems.

The following allowable protectionStatus values are associated with each protected unit:

–	No RequestXE "No Request"§ – No switch request is present on the unit.

–	Manual Switch to Protecting CompleteXE "Manual Switch to Protecting Complete"§ – The unit has completed a Manual Switch.

–	Release failedXE "Release failed"§ – A switch has been released or preempted and a timeout occurs while waiting for a release of associated bridges, or the near-end switch.

–	Automatic Switch (SF) PendingXE "Automatic Switch (SF) Pending"§ – The unit has a Signal Fail condition present and the protecting unit is unavailable.

–	Automatic Switch (SD) PendingXE "Automatic Switch (SD) Pending"§ – The unit has a Signal Degrade condition present and the protecting unit is unavailable.

–	Automatic Switch (SF) CompleteXE "Automatic Switch (SF) Complete"§ – The unit has completed an Automatic Switch to the protecting unit due to a Signal Fail condition.

–	Automatic Switch (SD) CompleteXE "Automatic Switch (SD) Complete"§ – The unit has completed an Automatic Switch to the protecting unit due to a Signal Degrade condition.

–	Automatic Switch (SF) Present, Operate failedXE "Automatic Switch (SF) Present, Operate failed"§ – An automatic switch (SF) request is in progress and a time-out occurs while waiting for completion.

–	Automatic Switch (SD) Present, Operate failedXE "Automatic Switch (SD) Present, Operate failed"§ – An automatic switch (SD) request is in progress and a time-out occurs while waiting for completion.

–	Force Switch Complete, Automatic Switch (SF) PendingXE "Force Switch Complete, Automatic Switch (SF) Pending"§ – The unit has completed a Force Switch. Additionally, the unit has an automatic switch (SF) pending.

–	Force Switch Complete, Automatic Switch (SD) PendingXE "Force Switch Complete, Automatic Switch (SD) Pending"§ – The unit has completed a Force Switch. Additionally, the unit has an automatic switch (SD) pending.

–	Automatic Switch Complete, Wait-to-RestoreXE "Automatic Switch Complete, Wait-to-Restore"§ – The unit has completed an Automatic Switch to the protecting unit.

–	Force Switch CompleteXE "Force Switch Complete"§ – The unit has completed a Force Switch to the protecting unit.

–	Protected Unit Lockout CompletedXE "Protected Unit Lockout Completed"§ – The unit has been locked out from the protecting unit.

–	Protected Unit Lockout complete, Operate FailedXE "Protected Unit Lockout complete, Operate Failed"§ – The unit has been locked out from the protecting unit, and, the previously completed switch could not be released within the expected time-out. When the switch is released, the operate failed status is removed.

Additionally, the non-revertive protected protection unit has the following status values:

–	Do Not RevertXE "Do Not Revert"§ – The protected unit has been switched to the protecting unit and the request to do so has been released. The switch to the protecting unit is maintained.

–	Manual Switch to Protected Unit CompleteXE "Manual Switch to Protected Unit Complete"§ – The unit has completed a Manual Switch from the protecting unit to the protected unit.

–	Force Switch to Protected Unit CompleteXE "Force Switch to Protected Unit Complete"§ – The unit has completed a Force Switch from the protecting unit to the protected unit.

–	Automatic Switch (SF) to Protected Unit CompleteXE "Automatic Switch (SF) to Protected Unit Complete"§ – The protecting unit has a Signal Fail condition present and traffic is now being carried on the protected unit.

–	Automatic Switch (SD) to Protected Unit CompleteXE "Automatic Switch (SD) to Protected Unit Complete"§ – The protecting unit has a Signal Degrade condition present and traffic is now being carried on the protected unit.

–	Automatic Switch (SD) to Protected Unit Complete, Signal Degrade PresentXE "Automatic Switch (SD) to Protected Unit Complete, Signal Degrade Present"§ – The protecting unit has a Signal Degrade condition present and traffic is now being carried on the protected unit. Additionally, the protected unit has a Signal Degrade condition present.

–	Automatic Switch (SF) to Protected Unit Complete, Signal Degrade PresentXE "Automatic Switch (SF) to Protected Unit Complete, Signal Degrade Present"§ – The protecting unit has a Signal Fail condition present and traffic is now being carried on the protected unit. Additionally, the protected unit has a Signal Degrade condition present.

–	Force Switch from Protecting Unit Complete, Automatic Switch (SF) PendingXE "Force Switch from Protecting Unit Complete, Automatic Switch (SF) Pending"§ – The unit has completed a Force Switch from the protecting unit to the protected unit. Additionally, the protected unit has an automatic switch (SF) condition present.

–	Force Switch from Protecting Unit Complete, Automatic Switch (SD) PendingXE "Force Switch from Protecting Unit Complete, Automatic Switch (SD) Pending"§ – The unit has completed a Force Switch from the protecting unit to the protected unit. Additionally, the protected unit has an automatic switch (SD) condition present.

The following allowable protectionStatus values are associated with each protecting unit:

–	No Request – No request is present on the protecting unit.

–	Manual Switch to Protecting Unit CompleteXE "Manual Switch to Protecting Unit Complete"§ – The protected unit has completed a Manual Switch.

–	Automatic Switch Complete (SF) to Protecting UnitXE "Automatic Switch Complete (SF) to Protecting Unit"§ – The protected unit has completed an automatic switch (SF) to the protecting unit.

–	Automatic Switch Complete (SD) to Protecting UnitXE "Automatic Switch Complete (SD) to Protecting Unit"§ – The protected unit has completed an automatic switch (SD) to the protecting unit.

–	Automatic Switch Complete (SD) to Protecting Unit, Protecting Unit Signal DegradedXE "Automatic Switch Complete (SD) to Protecting Unit, Protecting Unit Signal Degraded"§ – The protected unit has completed an automatic switch to the protecting unit. Additionally, the protecting unit has a Signal Degrade condition present. This applies to 1:n systems only.

–	Automatic Switch Complete (SF) to Protecting Unit, Protecting Unit Signal DegradedXE "Automatic Switch Complete (SF) to Protecting Unit, Protecting Unit Signal Degraded"§ – The protected unit has completed an automatic switch to the protecting unit. Additionally, the protecting unit has a Signal Degrade condition present. This applies to 1:n systems only.

–	Protecting Unit SD PresentXE "Protecting Unit SD Present"§ – The protecting unit has a Signal Degrade condition present. This state may also be a result of invalid or toggling APS byte values. This applies to 1:n systems only.

–	Protecting Unit SF PresentXE "Protecting Unit SF Present"§ – The protecting unit has a Signal Fail condition present. This state may also be a result of invalid or toggling APS byte values. This applies to 1:n systems only.

–	Force Switch Complete to Protecting UnitXE "Force Switch Complete to Protecting Unit"§ – The unit has completed a Force Switch of a protected unit to the protecting unit.

–	Force Switch Complete to Protecting Unit, SD Present on Protecting UnitXE "Force Switch Complete to Protecting Unit, SD Present on Protecting Unit"§ – The unit has completed a Force Switch of a protected unit to the protecting unit. Additionally, there is a Signal Degrade present on the protecting unit.

–	Force Switch Complete to Protecting , SF Present on Protecting UnitXE "Force Switch Complete to Protecting , SF Present on Protecting Unit"§ – The unit has completed a Force Switch of a protected unit to the protecting unit. Additionally, there is a Signal Fail (1 + 1 unidirectional systems only) present on the protecting unit.

–	Protecting Unit Locked OutXE "Protecting Unit Locked Out"§ – The protecting unit has been locked out.

–	Protecting Unit Locked Out, Release FailedXE "Protecting Unit Locked Out, Release Failed"§ – A release of a lockout is in progress and a timeout occurs waiting for the lockout condition to clear.

Additionally, the non-revertive protecting unit has the following values:

–	Do Not Revert – The protected unit has been switched to the protecting unit and the request to do so has been released. The switch to the protecting unit is maintained.

–	Manual Switch to Protected Unit Complete – The unit has completed a Manual Switch from the protecting unit to the protected unit.

–	Force Switch to Protected Unit Complete – The protecting unit has completed a forced switch to the protected unit.

–	Force Switch to Protected Unit Complete, Protecting Unit Signal DegradedXE "Force Switch to Protected Unit Complete, Protecting Unit Signal Degraded"§ – The protecting unit has completed a forced switch to the protected unit. Additionally, there is a Signal Degrade condition on the protecting unit.

–	Force Switch to Protected Unit Complete, Protecting Unit Signal FailedXE "Force Switch to Protected Unit Complete, Protecting Unit Signal Failed"§ – The protecting unit shows a forced switch to the protected unit. Additionally, there is a Signal Fail condition on the protecting unit.

–	Automatic Switch (SF) to Protected Unit Complete – The protecting unit has a Signal Fail condition present and traffic is now being carried on the protected unit.

–	Automatic Switch (SD) to Protected Unit Complete – The protecting unit has a Signal Degrade condition present and traffic is now being carried on the protected unit.

–	Automatic Switch Complete (SD) to Protecting, Automatic Switch (SD) to Protected Unit Pending – The protected unit has completed an automatic switch to the protecting unit. Additionally, the protecting unit has a Signal Degrade condition present.

–	Automatic Switch Complete (SF) to Protecting, Automatic Switch (SD) to Protected Unit Pending – The protected unit has completed an automatic switch to the protecting unit. Additionally, the protecting unit has a Signal Degrade condition present.

APS timeout detected by the exerciser, or, during an attempt to invoke a manual or forced switch will not be indicated as APS failure (operate or release fail) in the protectionStatus attribute. Instead, the actionReply for the invoked action will report this. Additionally, attribute lastAttemptResult will provide a record of the failure, until a successful exercise or protection switch causes this attribute to be cleared.

";

## 8.10	Protection Switch ModeXE "Protection Switch Mode"§

protectionSwitchMode ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.ProtectionSwitchMode;

MATCHES FOR	EQUALITY;

BEHAVIOUR protectionSwitchModeBeh;

REGISTERED AS { g774-03Attribute 10 };

protectionSwitchModeBeh BEHAVIOUR

DEFINED AS

"This attribute specifies whether protection switching is done on a unidirectional or bidirectional basis.

";

## 8.11	Protection Unit IdentifierXE "Protection Unit Identifier"§

protectionUnitId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.NameType;

MATCHES FOR	EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR protectionUnitIdBeh;

REGISTERED AS { g774-03Attribute 11 };

protectionUnitIdBeh BEHAVIOUR

DEFINED AS

"The protectionUnitId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the protectionUnit object class. If the string choice of the syntax is used, then matching on substrings is permitted. If the number choice for the syntax is used, then matching on ordering is permitted.

";

## 8.12	ProtectingXE "Protecting"§

protecting ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.Boolean;

MATCHES FOR	EQUALITY;

BEHAVIOUR protectingBeh;

REGISTERED AS { g774-03Attribute 12 };

protectingBeh BEHAVIOUR

DEFINED AS

"This attribute specifies the type of the protectionUnit. A value of TRUE indicates that the protectionUnit is a protecting (i.e. backup or standby) unit. A value of FALSE indicates that the protectionUnit is a protected (i.e. a regular, working, or preferred) unit.

";

## 8.13	Reliable Resource PointerXE "Reliable Resource Pointer"§

reliableResourcePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.ResourcePointer;

MATCHES FOR	EQUALITY;

BEHAVIOUR reliableResourcePointerBeh;

REGISTERED AS { g774-03Attribute 13 };

reliableResourcePointerBeh BEHAVIOUR

DEFINED AS

"The value of the reliableResourcePointer attribute points to the reliable resource(s) (e.g. termination point) that is/are associated with the protectionUnit instance.

";

## 8.14	RevertiveXE "Revertive"§

revertive ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.Boolean;

MATCHES FOR	EQUALITY;

BEHAVIOUR revertiveBeh;

REGISTERED AS { g774-03Attribute 14 };

revertiveBeh BEHAVIOUR

DEFINED AS

"This attribute indicates whether the protection scheme is revertive or not. If the value of this attribute is TRUE, the traffic is returned to the protected protectionUnit instance that initiated the switch after the fault clears and the waitToRestoreTime interval (if any) has expired. If the value of this attribute is FALSE, then after the fault has cleared, traffic does not revert to the protectionUnit that initiated the switch.

";

## 8.15	Sdh PriorityXE "Sdh Priority"§

sdhPriority ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.Integer;

MATCHES FOR	EQUALITY;

BEHAVIOUR sdhPriorityBeh;

REGISTERED AS { g774-03Attribute 15 };

sdhPriorityBeh BEHAVIOUR

DEFINED AS

"SDH priority may be High or Low for protected protection units. This is intended for prioritizing SD and SF conditions on 1:n systems.

";

## 8.16	Unprotected Connection Termination Point IdentifierXE "Unprotected Connection Termination Point Identifier"§

unprotectedCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.NameType;

MATCHES FOR	EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR unprotectedCTPIdBeh;

REGISTERED AS { g774-03Attribute 16 };

unprotectedCTPIdBeh BEHAVIOUR

DEFINED AS

"This attribute is used as an RDN for naming instances of the unprotectedCTP object classes. If the string choice of the syntax is used, then matching on substrings is permitted. If the number choice for the syntax is used, then matching on ordering is permitted.

";

## 8.17	Unreliable Resource PointerXE "Unreliable Resource Pointer"§

unreliableResourcePointer ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.ResourcePointer;

MATCHES FOR	EQUALITY;

BEHAVIOUR unreliableResourcePointerBeh;

REGISTERED AS { g774-03Attribute 17 };

unreliableResourcePointerBeh BEHAVIOUR

DEFINED AS

"The value of the unreliableResourcePointer attribute points to the unreliable resource(s) (e.g. equipment hardware or termination point) that is/are associated with the protectionUnit instance.

";

## 8.18	Wait to Restore TimeXE "Wait to Restore Time"§

waitToRestoreTime ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHProtASN1.Integer;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR waitToRestoreTimeBeh;

REGISTERED AS { g774-03Attribute 18 };

waitToRestoreTimeBeh BEHAVIOUR

DEFINED AS

"This attribute specifies the amount of time, in seconds, to wait after a fault clears before restoring traffic to the protected protectionUnit that initiated the switching.

";

# 9	Actions

## 9.1	Invoke ExerciseXE "Invoke Exercise"§

invokeExercise ACTION

BEHAVIOUR invokeExerciseBeh;

MODE CONFIRMED;

WITH INFORMATION SYNTAX  SDHProtASN1.InvokeExerciseArg;

WITH REPLY SYNTAX  SDHProtASN1.InvokeExerciseReply;

REGISTERED AS { g774-03Action 1 };

invokeExerciseBeh BEHAVIOUR

DEFINED AS

"The invokeExercise action can be used to request a protection exercise routine to be performed on one or more protectionUnit instances contained in the protectionGroup object.

The action argument contains indications of the protected and protecting units to which the request applies. If a protecting unit is identified in the protectedUnits field, or if a protected unit is identified in the protectingUnits field, the action fails.

The protectionEntity field may be absent, indicating that the request applies to all contained protection units.

While an exercise is in progress, the value of the protectionStatus attribute shall continue to indicate No Request. When an exercise routine has completed for any protectionUnit instances, the lastAttemptResult attribute is updated to reflect pass or fail for each exercised protection unit.

";

## 9.2	Invoke ProtectionXE "Invoke Protection"§

NOTE – The definition of Lockout, Forced Switch, and Manual Switch is provided in Recommendation G.783.

invokeProtection ACTION

BEHAVIOUR invokeProtectionBeh;

MODE CONFIRMED;

PARAMETERS invokeProtectionError;

WITH INFORMATION SYNTAX  SDHProtASN1.InvokeProtectionArg;

REGISTERED AS { g774-03Action 2 };

invokeProtectionBeh BEHAVIOUR

DEFINED AS

"The invokeProtection action can be used to request a lockout, a forced switch, or a manual (i.e. normal switch) on one or more protectionUnit instances contained in the protectionGroup object.

The action argument contains a request (Forced Switch, Manual Switch, or Lockout) and indications of the protected and protecting units to which the request applies. If a protecting unit is identified in the protectedUnits field, or if a protected unit is identified in the protectingUnits field, the action fails.

If the request is Forced Switch or Manual Switch, the protectedUnits field shall identify one or more protection units. If only one unit is identified in the protectedUnits field, and there is only one protecting unit in the protection group, the protectingUnits field may be omitted. If the protectingUnits field is present, it shall identify the same number of units as the protectedUnits field.

If the request is Lockout, the protectionEntity field may be absent, indicating that the request applies to all contained protection units. If the protectionEntity field is present, any number of protection units may be identified in the protectedUnits and/or protectingUnits field, and either field may be absent.

For a Lockout request, the specified protected units and/or protecting units are locked out.

For requests which cannot be completed, either because the request is the protecting unit is serving a request of higher priority (in the error parameter of the CMIP APDU, reason: preempted), or failure occurs (failure), or timeout occurs (timeout), the reply shall indicate why the request could not be completed, and the request shall not be made pending. The CMIP error APDU contains the failure reasons.

";

## 9.3	Release ProtectionXE "Release Protection"§

releaseProtection ACTION

BEHAVIOUR releaseProtectionBeh;

MODE CONFIRMED;

PARAMETERS releaseProtectionError;

WITH INFORMATION SYNTAX  SDHProtASN1.ReleaseProtectionArg;

REGISTERED AS { g774-03Action 3 };

releaseProtectionBeh BEHAVIOUR

DEFINED AS

"The releaseProtection action can be used to release a lockout, a forced switch, or a manual (i.e. normal switch) on one or more protectionUnit instances contained in the protectionGroup object.

The action argument contains a request (Forced Switch, Manual Switch, or Lockout) and indications of the protected and protecting units to which the request applies. If a protecting unit is identified in the protectedUnits field, or if a protected unit is identified in the protectingUnits field, the action fails.

If the request is Forced Switch or Manual Switch, the protectedUnits field shall identify one or more protection units, and the protectingUnits field shall be omitted. For each identified protected unit, if it is not switched to a protecting unit, the action fails.

If the request is Lockout, the protectionEntity field may be absent, indicating that the request applies to all contained protection units. If the protectionEntity field is present, any number of protection units may be identified in the protectedUnits and/or protectingUnits field, and either field may be absent.

For a Lockout request, the specified protected units and/or protecting units are no longer locked out. That is, the protected units are now under protection and the protecting units are now capable of providing protection. For release requests which cannot be completed, the reply shall indicate why the request could not be completed.

";

# 10	Notifications

## 10.1	Protection Switch ReportingXE "Protection Switch Reporting"§

protectionSwitchReporting NOTIFICATION

BEHAVIOUR protectionSwitchReportingBeh;

WITH INFORMATION SYNTAX  SDHProtASN1.ProtectionSwitchReportingInfo;

AND ATTRIBUTE IDS

protectionUnit	reportedProtectionUnit,

additionalInfo	"Rec. X.721 | ISO/IEC 10165-2 : 1992": additionalInformation;

REGISTERED AS { g774-03Notification 1 };

protectionSwitchReportingBeh BEHAVIOUR

DEFINED AS

"The protectionSwitchReporting notification is emitted from the protectionGroup object to report any protection switch events. The choice of protectingUnit is utilized for the protectionUnit argument of the notification in all cases except for when reporting 'lockout of working' or 'release of lockout of working'.

";

# 11	Parameters

## 11.1	Invoke Protection ErrorXE "Invoke Protection Error"§

invokeProtectionError   PARAMETER

CONTEXT SPECIFIC-ERROR;

WITH SYNTAX      SDHProtASN1.InvokeProtectionError;

BEHAVIOUR invokeProtectionErrorBeh;

REGISTERED AS { g774-03Parameter 1 };

invokeProtectionErrorBeh BEHAVIOUR

DEFINED AS

"This parameter is included in the error parameter of the CMIP APDU when the invoke protection action fails. The preempted choice means that a higher priority switch request exists on the protecting protection unit. The failure choice indicates that the request has not been performed due to an abnormal condition on the protection system. The timeout choice indicates that the protection switch has not been performed in the required time frame.

";

## 11.2	Release Protection ErrorXE "Release Protection Error"§

releaseProtectionError   PARAMETER

CONTEXT SPECIFIC-ERROR;

WITH SYNTAX      SDHProtASN1.ReleaseProtectionError;

BEHAVIOUR releaseProtectionErrorBeh;

REGISTERED AS { g774-03Parameter 2 };

releaseProtectionErrorBeh BEHAVIOUR

DEFINED AS

"This parameter is included in the error parameter of the CMIP APDU when the release protection action fails. The failure choice indicates that the request has not been performed due to an abnormal condition on the protection system. The timeout choice indicates that the protection switch has not been performed in the required time frame.

";

## 11.3	Protection Status ParameterXE "Protection Status Parameter"§

protectionStatusParameter PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX      SDHProtASN1.ProtectionStatusParameter;

BEHAVIOUR protectionStatusParameterBeh;

REGISTERED AS { g774-03Parameter 3 };

protectionStatusParameterBeh BEHAVIOUR

DEFINED AS

"This parameter is included in the additional info parameters of the protection switching reporting notification.

This notification is sent by the protection group according to the following rules. There are several cases:

–	The switch from protected to protecting or protecting to protected has been done without preempting 
an existing switch. In this case the old and new values of the Protection Status attribute of the 
protecting channel shall be reported in the notification by means of the oldProtectionStatus and newProtectionStatus parameters respectively.

–	A switch is perfomed by preemting an existing one. In this case the old and new values of the Protection Status attribute of the protecting channel shall be reported in the notification by means of the oldProtectionStatus and newProtectionStatus parameters respectively.

–	An auto-switch condition exists on a channel but the auto-switch cannot be served due to the unavailibility of the channel that otherwise protects it. In this case the oldProtectionStatus and newProtectionStatus parameters refer to the protection status attribute value of the channel on which the autoswitch condition arises. The exception is when the channel is already forced or locked out, in which case no notification is sent.

–	A working channel (protected unit) has been locked out or released from lockout without modifying any existing switch. In this case the oldProtectionStatus and newProtectionStatus parameters refer to the protection status attribute value of the working channel which has been locked out.

–	A protection channel (protecting unit) has been locked out or release from lockout without modifying the existing switch. In this case the oldProtectionStatus and newProtectionStatus parameters refer to the protection status attribute value of the protection channel which has been locked out.

–	The protectionSwitchReporting is not sent when the automatic switch condition is toggling between SD, SF and WTR condition. While in the lockout of forced switch state no notification is sent except for ending of release failure.

";

# 12	Name Bindings

## 12.1	Protected Trail Termination PointXE "Protected Trail Termination Point"§

protectedTTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	protectedTTPBidirectional;

NAMED BY SUPERIOR OBJECT CLASS	"Recommendation G.774 : 1992": sdhNE;

WITH ATTRIBUTE	protectedTTPId;

BEHAVIOUR protectedTTPBidirectional-sdhNEBeh;

REGISTERED AS { g774-03NameBinding 1 };

protectedTTPBidirectional-sdhNEBeh BEHAVIOUR

DEFINED AS

"The subordinate managed objects are instantiated when the multiplex section protection switching function is present. Instances of this object class may also be instantiated when other types of protection switching functions, such as path protection, are present.

";

protectedTTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	protectedTTPSink;

NAMED BY SUPERIOR OBJECT CLASS	"Recommendation G.774 : 1992": sdhNE;

WITH ATTRIBUTE 	protectedTTPId;

BEHAVIOUR protectedTTPSink-sdhNEBeh;

REGISTERED AS { g774-03NameBinding 2 };

protectedTTPSink-sdhNEBeh BEHAVIOUR

DEFINED AS

"The subordinate managed objects are instantiated when the multiplex section protection switching function is present. Instances of this object may also be instantiated when other types of protection switching functions, such as path protection, are present.

";

protectedTTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	protectedTTPSource;

NAMED BY SUPERIOR OBJECT CLASS	"Recommendation G.774 : 1992": sdhNE;

WITH ATTRIBUTE 	protectedTTPId;

BEHAVIOUR protectedTTPSource-sdhNEBeh;

REGISTERED AS { g774-03NameBinding 3 };

protectedTTPSource-sdhNEBeh BEHAVIOUR

DEFINED AS

"The subordinate managed objects are instantiated when the multiplex section protection switching function is present. Instances of this object class may also be instantiated when other types of protection switching functions, such as path protection, are present.

";

## 12.2	Protection GroupXE "Protection Group"§

protectionGroup-managedElement NAME BINDING

SUBORDINATE OBJECT CLASS	protectionGroup AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS	"Recommendation M.3100: 1992": managedElement AND SUBCLASSES;

WITH ATTRIBUTE 	protectionGroupId;

REGISTERED AS { g774-03NameBinding 4 };

## 12.3	AugXE "Aug"§

augBidirectional-protectedTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS 	"Recommendation G.774 : 1992": augBidirectional;

NAMED BY SUPERIOR OBJECT CLASS	protectedTTPBidirectional;

WITH ATTRIBUTE	"Recommendation G.774 : 1992": augId;

BEHAVIOUR augBidirectional-protectedTTPBidirectionalBeh;

REGISTERED AS { g774-03NameBinding 5 };

augBidirectional-protectedTTPBidirectionalBeh BEHAVIOUR

DEFINED AS

"The subordinate managed objects are instantiated when the multiplex section protection switching function is present.

";

augSink-protectedTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	"Recommendation G.774 : 1992": augSink;

NAMED BY SUPERIOR OBJECT CLASS	protectedTTPSink;

WITH ATTRIBUTE	"Recommendation G.774 : 1992": augId;

BEHAVIOUR augSink-protectedTTPSinkBeh;

REGISTERED AS { g774-03NameBinding 6 };

augSink-protectedTTPSinkBeh BEHAVIOUR

DEFINED AS

"The subordinate managed objects are instantiated when the multiplex section protection switching function is present.

";

augSource-protectedTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	"Recommendation G.774 : 1992": augSource;

NAMED BY SUPERIOR OBJECT CLASS	protectedTTPSource;

WITH ATTRIBUTE	"Recommendation G.774 : 1992": augId;

BEHAVIOUR augSource-protectedTTPSourceBeh;

REGISTERED AS { g774-03NameBinding 7 };

augSource-protectedTTPSourceBeh  BEHAVIOUR

DEFINED AS

"The subordinate managed objects are instantiated when the multiplex section protection switching function is present.

";

## 12.4	Protection UnitXE "Protection Unit"§

protectionUnit-protectionGroup NAME BINDING

SUBORDINATE OBJECT CLASS	protectionUnit AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS	protectionGroup AND SUBCLASSES;

WITH ATTRIBUTE	protectionUnitId;

REGISTERED AS { g774-03NameBinding 8 };

## 12.5	Unprotected Connection Termination PointXE "Unprotected Connection Termination Point"§

unprotectedCTPBidirectional-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	unprotectedCTPBidirectional;

NAMED BY SUPERIOR OBJECT CLASS	"Recommendation G.774 : 1992": msTTPBidirectional;

WITH ATTRIBUTE	unprotectedCTPId;

BEHAVIOUR unprotectedCTPBidirectional-msTTPBidirectionalBeh;

REGISTERED AS { g774-03NameBinding 9 };

unprotectedCTPBidirectional-msTTPBidirectionalBeh BEHAVIOUR

DEFINED AS

"The subordinate managed objects are instantiated when the multiplex section protection switching function is present.

";

unprotectedCTPSink-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	unprotectedCTPSink;

NAMED BY SUPERIOR OBJECT CLASS	"Recommendation G.774 : 1992": msTTPSink;

WITH ATTRIBUTE	unprotectedCTPId;

BEHAVIOUR unprotectedCTPSink-msTTPSinkBeh;

REGISTERED AS { g774-03NameBinding 10 };

unprotectedCTPSink-msTTPSinkBeh BEHAVIOUR

DEFINED AS

"The subordinate managed objects are instantiated when the multiplex section protection switching function is present.

";

unprotectedCTPSource-msTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	unprotectedCTPSource;

NAMED BY SUPERIOR OBJECT CLASS	"Recommendation G.774 : 1992": msTTPSource;

WITH ATTRIBUTE	unprotectedCTPId;

BEHAVIOUR unprotectedCTPSource-msTTPSourceBeh;

REGISTERED AS { g774-03NameBinding 11 };

unprotectedCTPSource-msTTPSourceBeh BEHAVIOUR

DEFINED AS

"The subordinate managed objects are instantiated when the multiplex section protection switching function is present.

";

# 13	Subordination Rules

None.

# 14	Pointer Constraints

None.

# 15	Supporting ASN.1 ProductionsXE "Supporting ASN.1 Productions"§

SDHProtASN1 { itu(0) recommendation(0) g(7) g774(774) hyphen(127) prot(03) 
informationModel(0) asn1Module(2) sdhmsp(0) }

DEFINITIONS IMPLICIT TAGS  ::=

BEGIN

--  EXPORTS everything  --

IMPORTS

NameType

FROM

ASN1DefinedTypesModule { itu(0) recommendation m gnm(3100) informationModel(0) 
asn1Modules(2) asn1DefinedTypesModule(0) }

RelativeDistinguishedName

FROM

InformationFramework { joint-iso-itu ds(5) modules(1) informationFramework(1) }

ObjectInstance

FROM

CMIP-1 { joint-iso-ccitt ms(9) cmip(1) modules(0) protocol(3) }

AdditionalInformation,

ManagementExtension

FROM

Attribute-ASN1Module { joint-iso-itu ms(9) smi(3) part2(2) asn1Module(2) 1 };

sdhProt OBJECT IDENTIFIER  ::=  { itu(0) recommendation(0) g(7) g774(774) hyphen(127) 
prot(03) informationModel(0) }

g774-03ObjectClass OBJECT IDENTIFIER  ::=  { sdhProt managedObjectClass(3) }

g774-03Attribute OBJECT IDENTIFIER  ::=  { sdhProt attribute(7) }

g774-03NameBinding OBJECT IDENTIFIER  ::=  { sdhProt nameBinding(6) }

g774-03Action OBJECT IDENTIFIER  ::=  { sdhProt action(9) }

g774-03Notification OBJECT IDENTIFIER  ::=  { sdhProt notification(10) }

g774-03Parameter OBJECT IDENTIFIER  ::=  { sdhProt parameter(5) }

g774-03Package OBJECT IDENTIFIER  ::=  { sdhProt package(4) }

--  default value definitions  --

booleanTrueDefault Boolean  ::=  TRUE

--  supporting productions  --

Boolean  ::=  BOOLEAN

Integer  ::=  INTEGER

InvokeExerciseArg  ::=  SEQUENCE {

protectionEntity ProtectionEntity OPTIONAL,  --  if absent, all PUs  --

otherInfo SET OF ManagementExtension OPTIONAL }

InvokeExerciseReply  ::=  SET OF SEQUENCE {

protectionUnit  RelativeDistinguishedName,

result      LastAttemptResult }

LastAttemptResult  ::=  CHOICE {

success	[0]	NULL,  --  default value  --
denied	[1]	NULL,
fail	[2]	RxTxAPS }

RxTxAPS  ::=  SEQUENCE {

rxAPSvalue	[0]	INTEGER,
txAPSvalue	[1]	INTEGER }  --  APS byte 1 in low order byte, byte 2 in high order byte  --

InvokeProtectionArg  ::=  SEQUENCE {

switchType SwitchType,

protectionEntity ProtectionEntity OPTIONAL,  --  if absent, all PUs  --

otherInfo SET OF ManagementExtension OPTIONAL }

ProtectionDirection  ::=  ENUMERATED {

transmit	(0), 
receive	(1),
bidirectional	(2) }

ProtectionEntity  ::=  SEQUENCE {

protectedUnits	[0]	SEQUENCE OF RelativeDistinguishedName OPTIONAL,
protectingUnits	[1]	SEQUENCE OF RelativeDistinguishedName OPTIONAL

}

--  In case of a 1 + 1 non revertive MSP system for a manual switch from the protecting protection unit to the protected one, the protectingUnits field shall be used and shall indicate the protection unit which has the channel number 0. 
If both fields are present they should be compatible sequences.  --

ProtectionGroupType  ::=  ENUMERATED {

plus	(0),  --  1 + 1 (1 plus 1) or hot-standby
colon	(1)  --  M:N (M for N)  --

}

ProtectionMismatchStatus  ::=  SEQUENCE {

uniBi	[0]	BOOLEAN,
plusColon	[1]	BOOLEAN }

ProtectionStatus  ::=  SET OF CHOICE {

noRequest	[0]	NULL,
doNotRevert	[1]	NULL,
manualSwitch	[2]	SEQUENCE {

requestSource	[0]	RequestSource OPTIONAL,
switchStatus	[1]	SwitchStatus,
relatedChannel	[2]	FromAndToProtectionUnit },

autoSwitch	[3]	SEQUENCE {

requestSource	[0]	RequestSource OPTIONAL,
switchStatus	[1]	SwitchStatus,
relatedChannel	[2]	FromAndToProtectionUnit,
autoSwitchReason	[3]	AutoSwitchReason },

forcedSwitch	[4]	SEQUENCE {

requestSource	[0]	RequestSource OPTIONAL,
switchStatus	[1]	SwitchStatus,
relatedChannel	[2]	FromAndToProtectionUnit },

lockout		[5]	SEQUENCE {

requestSource	[0]	RequestSource OPTIONAL,
switchStatus	[1]	SwitchStatus },

releaseFailed	[6]	NULL,

protectionFailCond	[7]	CHOICE {

aPSInvalid	[0]	BOOLEAN,
channelMismatch	[1]	BOOLEAN }	--  invalid or toggling APS byte value 
					--  received or channel mismatch  --

AutoSwitchReason  ::=  CHOICE {

waitToRestore	[0]	NULL,
signalDegrade	[1]	NULL,
signalFail	[2]	NULL }

ProtectionSwitchMode  ::=  ENUMERATED {

bidirectional	(0),
unidirectional	(1) }

ProtectionSwitchReportingInfo  ::=  SEQUENCE {

protectionUnit	ProtectionUnit,

additionalInfo	AdditionalInformation }

ProtectionUnit  ::=  CHOICE {

protectingUnit	[0]	RelativeDistinguishedName,
protectedUnit	[1]	RelativeDistinguishedName }

ProtectionStatusParameter  ::=  SEQUENCE {

oldProtectionStatus ProtectionStatus,

newProtectionStatus ProtectionStatus,

psDirection ProtectionDirection DEFAULT bidirectional }

ReleaseProtectionArg  ::=  SEQUENCE {

switchType SwitchType,

protectionEntity ProtectionEntity,

otherInfo SET OF ManagementExtension OPTIONAL }

RequestSource  ::=  ENUMERATED {

local	(0),
remote	(1) }

ResourcePointer  ::=  CHOICE {

null	NULL,
objectInstances	SEQUENCE OF ObjectInstance }

SDHMSPriority  ::=  Integer(1 .. 2)

--  1 = high priority, 2 = low priority  --

SDHMSResourcePointer  ::=  ResourcePointer(WITH COMPONENTS {

...,

objectInstances(SIZE(1))

} )

SwitchStatus  ::=  ENUMERATED {

pending	(0),
completed	(1),
operateFailed	(2) }

SwitchType  ::=  ENUMERATED {

manual	(0),
forced	(1),
lockout	(2) }

FromAndToProtectionUnit  ::=  CHOICE {

fromProtectionUnitNumber	[0]	RelativeDistinguishedName,
toProtectionUnitNumber	[1]	RelativeDistinguishedName }

--  The fromProtectionUnitNumber is used for protecting protection unit in order to hold the name of the protected protection unit which has been protected by that protecting protection unit. The toProtectionUnitNumber is used for protected protection unit in order to hold the number of the protecting protection unit to which it has been switched to. In case of a MSP 1 + 1 system these parameters are not mandatory.  --

InvokeProtectionError  ::=  ENUMERATED {

preempted	(0),
failure	(1),
timeout	(2) }

ReleaseProtectionError  ::=  ENUMERATED {

failure	(0),
timeout	(1) }

END

Annex A

Naming and Inheritance DiagramsXE "Naming and Inheritance Diagrams"§

(informative)
(This annex does not form an integral part of this Recommendation)

See Figures A.1 and A.2.

XE "SDH MS Protection Inheritance Diagram"§FIGURE  A.1/G.774.03.....[D01]  =  17 cm

XE "SDH MS Protection Naming Diagram"§FIGURE  A.2/G.774.03.....[D02]  =  13.5 cm

Annex B

Examples of MS ProtectionXE "Examples of MS Protection"§

(informative)
(This annex does not form an integral part of this Recommendation)

See Figures B.1 to B.6.

XE "Example of 1 + 1 Linear MS protection, normal case, traffic on channel 1"§FIGURE  B.1/G.774.03.....[D03]  =  14.5 cm

XE "Example of 1 + 1 Linear MS protection, failure of channel 1, traffic switched onto channel 0"§FIGURE  B.2/G.774.03.....[D04]  =  11.5 cm

XE "Example of 1:1 Linear MS protection, normal case, no extra traffic"§FIGURE  B.3/G.774.03.....[D05]  =  11 cm

XE "Example of 1:1 Linear MS protection, failure of working channel (channel 1), traffic switched onto protecting channel"§FIGURE  B.4/G.774.03.....[D06]  =  11 cm

XE "Example of 1:1 Linear MS protection with extra traffic, normal case"§FIGURE  B.5/G.774.03.....[D07]  =  11 cm

XE "Example of 1:1 Linear MS protection with extra traffic, failure of working channel (channel 1), traffic switched onto protecting channel"§FIGURE  B.6/G.774.03.....[D08]  =  11 cm