|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	I.241.3

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

INTEGRATED  SERVICES  DIGITAL  NETWORK (ISDN)

SERVICE  CAPABILITIES

TELESERVICES  SUPPORTED  BY  AN  ISDN: TELEFAX 4

ITU-T  Recommendation  I.241.3

(Extract from the Blue Book)

NOTES

1	ITU-T Recommendation I.241.3 was published in Fascicle III.7 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation I.241.3

Fascicle III.7 - Rec. I.241.3

xe ""§ TELESERVICES SUPPORTED BY AN ISDN: telefax 4

(Melbourne, 1988)

3	xe ""§Telefax 4

The prose description of the Telefax 4 service is an extract of Recommendation F.184. (If more detail is required this Recommendation should be referred to.) Therefore it does not strictly follow Step 1.1 of the service description method. Closer alignment with the method requires further study.

3.1	Definition

Telefax 4 is an international service enabling subscribers to exchange office correspondence in the form of documents containing facsimile coded information, automatically via the ISDN.

3.2	Description

3.2.1	General description

The Telefax 4 service provides a basic level of compatibility between all terminals participating in the service. It offers bidirectional communication between two users via the ISDN using 64 kbit/s digital signals over the B-channel.

There are three classes of Group 4 facsimile terminals:

Class I	-	In this class the minimum requirement terminal is a terminal able to send and receive documents containing facsimile encoded information (in accordance with Recommendation T.6 and the T.400-Series).

Class II	-	In this class the minimum requirement terminal is a terminal able to transmit documents that are facsimile encoded (in accordance with Recommendation T.6 and the T.400-Series). In addition, the terminal must be capable of receiving documents which are facsimile coded (in accordance with Recommendation T.6 and the T.400-Series), teletex coded (in accordance with the basic coded character repertoire as defined in Recommendation T.61) and also mixed-mode documents (in accordance with Recommendations of the T.400-Series).

Class III	-	In this class the minimum requirement terminal is a terminal that is capable of generating, transmitting and receiving facsimile coded documents (in accordance with Recommendation T.6 and the T.400-Series), teletex coded documents (in accordance with the basic coded character repertoire as defined in Recommendation T.61) and mixed-mode documents (in accordance with Recommendations of the T.400-Series).

The basic element of the correspondence between users is the page which is the smallest unit of text treated as an entity. No restrictions shall exist concerning the operator procedures for generation of the text or the positioning of text within the reproducible area on a page.

3.2.2	Operation – General

The Telefax 4 service in each country and the interconnection between countries or networks shall use automatic switching so that it is possible for any Telefax 4 subscriber to reach any other Telefax 4 subscriber using fully automatic selection.

It is a requirement to allow the through-connection of a call between Group 4 facsimile terminals connected to a private automatic branch exchange (or similar systems) and those connected to public exchanges used for the Group 4 facsimile service.

Two-way alternate (TWA) communication is a capability of the Telefax 4 service, which also includes one-way communication (OWC); the calling subscriber will have full control of the Group 4 facsimile call.

3.3.	Procedures

3.3.1	Provision/withdrawal

The national and international Telefax 4 service shall be open continuously.

Telefax 4 terminals for which call numbers are published in the directories shall, in principle, be available to accept calls continuously.

In order to facilitate the 24-hour duration of service it is permitted to use a centralized storage in the network to realize the receiving memory capability of the terminal.

3.3.2	Call phases

The operations for each call may be divided into the following three phases:

a)	preparation:

–	preparation of the information to be transmitted;

b)	transmission:

–	call establishment (automatic);

–	pre-information phase (see Note);

–	information transfer (see Note);

–	post information phase (see Note);

–	call clearing.

Note - During these parts of the transmission phase the network must be transparent with respect to control procedures.

c)	output:

–	displaying the message either by immediate printing or from a storage medium upon control by the operator.

Note - The information may consist of one or more Telefax 4 documents each consisting of one or more Telefax pages.

The control procedures as specified in Recommendation T.62 and Recommendations of the T.400-Series shall be used as end-to-end communication procedures between terminals in the service.

The low layer protocols and the network independent basic transport protocol for Telefax 4 are specified in Recommendations T.70 and T.90.

The network-dependent control procedures for the Telefax 4 service are those that are defined for ISDN.

3.3.3	Call identification line

The Telefax 4 procedures include the exchange of reference information prior to sending any document. This reference information includes identification of the parties to the call as well as the date and time. Also, supplementary reference information is exchanged during a call to allow reference to an individual document or page for error recovery or other purposes. Date and time have to be provided by the network and sent to the calling terminal in the call set-up phase.

This reference information, taken together, is defined to be printable on a single line called the call identification line. Use of this information is a local decision except in recovering from an interrupted transmission. In the case of automatic linking, the use of this information is for further study.

For the format of the call identification line see Recommendation F.200.

3.3.4	Error protection

To ensure call integrity, error protection will be provided by Telefax 4 control procedures (see Recommendations T.62, T.70 and T.90). Besides the error detection and correction mechanism in layer 2 (and 3) an additional error detection and correction mechanism is provided in the session layer. By this mechanism, errors of the higher layer functions (e.g. command/response sequence error) and transmission errors, which are not corrected by the lower layers, will be corrected, for example, by retransmission of one or several pages.

The error rate on the pre-information, information and post-information phases should not exceed 1 × 10-6.

3.4	Network capabilities for charging

This Recommendation does not cover charging principles. Future Recommendations in the D-Series are expected to contain that information.

It shall be possible to charge the subscriber accurately for the service.

3.5	Interworking requirements

3.5.1	Within the Telefax 4 service, interworking between terminals connected to different networks is required for:

a)	Telefax 4 (ISDN) - Telefax 4 (CSPDN: Circuit switched public data network)

b)	Telefax 4 (ISDN) - Telefax 4 (PSPDN: Packet switched public data network)

c)	Telefax 4 (ISDN) - Telefax 4 (PSTN: Public switched telephone network)

In the case of international interworking between Group 4 facsimile terminals connected to dissimilar networks, Recommendation X.300 shall apply. For international interworking between PSTN and ISDN, a (separate) Telefax 4 interworking unit may be necessary.

International routes between ISDNs for the Telefax 4 service shall be capable of supporting user data rates up to 64 kbit/s.

3.5.2	Intercommunication with other services

3.5.2.1	Intercommunication between basic mode and mixed mode Teletex terminals and Classes I, II and III Group 4 facsimile terminals connected to the Telefax 4 service is shown in Table 1/I.241.3.

In both the Teletex and Telefax 4 services, the equipment providing mixed mode should enable a direct exchange of documents in accordance with Recommendations T.6, T.61 and of the T.400-Series.

TABLE 1/I.241.3

Current status of direct intercommunication 
for Teletex and Group 4 facsimile terminals on the same network

| To From | Facsimile Group 4, Class I | Facsimile Group 4, Class II | Facsimile Group 4, Class III | Teletex basic mode | Teletex mixed mode | Teletex processable mode 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Facsimile Group 4, Class I | F | F | F |  |  |  |
| Facsimile Group 4, Class II | F | F | F |  |  |  |
| Facsimile Group 4, Class III | F | T, F, MM | T, F, MM | T | T, MM | T |
| Teletex basic mode |  | T | T | T | T | T |
| Teletex mixed mode |  | T, MM | T, MM | T | T, MM | T |
| Teletex processable mode 1 |  | T | T | T | T | T, PM1 |

T:	Basic Teletex document with character coded information only.

F:	Group 4 facsimile document with facsimile coded information only.

MM:	Mixed-mode document with character and facsimile coded information.

PM1:	Processable mode document with character coded information only.

3.5.2.2	Intercommunication is desirable between terminals of the Telefax 4 service and terminals of services other than Telefax 4 provided over ISDN and other public switched networks.

Intercommunication possibilities between Telefax 4 terminals and Telefax 3 terminals have to be provided (see also Recommendation F.180):

a)	Telefax 4 (ISDN) - Telefax 3 (PSTN)

b)	Telefax 4 (ISDN) - Telefax 3 (ISDN, via terminal adaptors)

In case a) Telefax 4 terminals use specific service features in ISDN. Intercommunication should be supported by ISDN-PSTN interworking units.

In case b) Telefax 3 terminals and Telefax 4 terminals which are to be connected in the PSTN can also be connected to the ISDN via terminal adaptors.

3.6	xe ""§Interaction with supplementary services

Each supplementary service description identifies the applicability with this teleservice.

For the ISDN, the international supplementary services for the Telefax 4 service in the circuit mode using a B-channel are:

i)	closed user groupxe "closed user group"§;

ii)	multiple subscriber numberxe "multiple subscriber number"§;

iii)	user-to-user signallingxe "user-to-user signalling"§;

iv)	calling line identification presentationxe "calling line identification presentation"§;

v)	calling line identification restrictionxe "calling line identification restriction"§;

vi)	connected line identification presentationxe "connected line identification presentation"§;

vii)	connected line identification restrictionxe "connected line identification restriction"§;

viii)	direct-dialling-inxe "direct-dialling-in"§.

The use of other supplementary services is for further study.

Supplementary services for Telefax 4 with a packet mode of operation are for further study.

3.7	Attributes and values of attributes of the Telefax 4 service

a)	LOW LAYER ATTRIBUTES

Information transfer attributes

|  |  | Circuit-mode bearer capability | Packet-mode bearer capability |
| --- | --- | --- | --- |
| 1. | Information transfer mode | circuit | packet |
| 2. | Information transfer rate | 64 kbit/s | maximum throughput of a given virtual circuit is less than or equal to the maximum bit rate of the user information access channel and the throughput class of the virtual circuit |
| 3. | Information transfer capability | unrestricted (Note 1) | unrestricted |
| 4. | Structure | unstructured (Note 2) | service data unit integrity |
| 5. | Establishment of communication | demand | demand (VC)/ permanent (PVC) |
| 6. | Symmetry | bidirectional symmetric | bidirectional symmetric |
| 7. | Communication configuration | point-to-point | point-to-point |

Access attributes

|  |  | Circuit-mode bearer capability | Packet-mode bearer capability |
| --- | --- | --- | --- |
| 8. | Access channel: | B for user information D for signalling | User information over virtual circuit within B- or D-channel. When D-channel is used, maximum packet size and quality of service may be restricted. Signalling may be provided via D-channel and/or virtual circuit within B-channel. |
| 9. | Access protocol |  |  |
| 9.1 | Signalling access protocol layer 1: | Rec. I.430/I.431 | Rec. I.430/I.431 |
| 9.2 | Signalling access protocol layer 2: | Rec. I.440/I.441 | Rec. I.440/I.441, X.31 |
| 9.3 | Signalling access protocol layer 3: | Rec. I.450/I.451 | Rec. I.450/I.451, X.31 |
| 9.4 | Information access protocol layer 1: | Rec. I.430/I.431 | Rec. I.430/I.431 |
| 9.5 | Information access protocol layer 2: | Rec. X.75 (SLP) | Rec. X.25 (LAPB) |
| 9.6 | Information access protocol layer 3: | ISO 8208 | Rec. X.25 (PLP) |

b)	HIGH LAYER ATTRIBUTES

10.	Type of user information  :		Telefax 4

11.	Layer 4 protocol functions:		T.70

12.	Layer 5 protocol functions:		T.62

13.	Layer 6 protocol functions:		T.400-Series (Note 4)

13.1	Resolution [pixels per inch (ppi)]:	200 ´ 200 standard;

240 ´ 240, 300 ´ 300, 400 ´ 400 optional

14.	Layer 7 protocol functions:		T.503, T.521, T.563

c)	GENERAL ATTRIBUTES

15.	Supplementary services provided:	see § 3.6

16.	Quality of service:			for further study

17.	Interworking possibilities:		see § 3.5

18.	Operational and commercial:		for further study

Note 1 - The interworking arrangements with networks having a restricted 64 kbit/s information transfer capability require further study.

Note 2 - Even if no structure is required the network may provide 8 kHz integrity.

Note 3 - User information transferred via virtual channel on the D-channel is for further study.

Note 4 - Further study is required to identify a more precise reference in the T.400-Series of Recommendations.

3.8	Recommended support of Telefax 4 by an ISDN

a)	Overall support: A

b)	Variations of non-dominant attributes:

1)	Information transfer mode

Note - In the interim period, the circuit mode method of operation is preferred.

–	circuit: A

–	packet: A

2)	Establishment		Symmetry	Communication		Support1
			of communication			configuration

_____________________________________________________________

demand		bidirectional	pt-pt			E
							symmetric

3)	Access

| Signalling and OAM (Note 1) |  | User information |  | Support |
| --- | --- | --- | --- | --- |
| Channel and rate | Protocols | Channel and rate | Protocols |  |
| Circuit mode |  |  |  |  |
| D(16) | I.430, I.440, I.441, I.450, I.451 (Note 2) | B(64) | I.430, X.75 (SLP), ISO 8208 | A |
| D(64) | I.431, I.440, I.441, I.450, I.451 (Note 2) | B(64) | I.431, X.75 (SLP), ISO 8208 | A |
| Packet mode |  |  |  |  |
| D(16) | I.430, I.440, I.441, I.450, I.451, X.31 | B(64) or D (16) | I.430, X.25 LAPB, X.25 (PLP) | A |
| D(64) | I.431, I.440, I.441, I.450, I.451, X.31 | B(64) | I.431, X.25 LAPB, X.25 (PLP) | FS |
| VC in B(64) | for further study | B(64) | FS | FS |

Note 1 - Definition of protocols for OAM is for further study.

Note 2 - Demand services only. Others are for further study.

3.9	Dynamic description

The circuit mode dynamic description appears in Recommendation I.220.

The definition of E (essential) and A (additional) can be found in Recommendation I.240.