|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	I.241.4

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

INTEGRATED  SERVICES  DIGITAL  NETWORK (ISDN)

SERVICE  CAPABILITIES

TELESERVICES  SUPPORTED  BY  AN  ISDN: 
MIXED MODE

ITU-T  Recommendation  I.241.4

(Extract from the Blue Book)

NOTES

1	ITU-T Recommendation I.241.4 was published in Fascicle III.7 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation I.241.4

Fascicle III.7 - Rec. I.241.4

xe ""§ TELESERVICES SUPPORTED BY AN ISDN: mixed mode

(Melbourne, 1988)

4	xe ""§Mixed mode

The prose definition of the mixed mode service is an extract of Recommendation F.230.

4.1	Definition

This service provides combined text and facsimile communication for end-to-end transfer of documents containing mixed information of text and fixed images. The high layer attributes are based on the CCITT Recommendations for Teletex and Telefax 4.

4.2	Description

For further study.

4.3	Procedures

For further study.

4.4	Network capabilities for charging

This Recommendation does not cover charging principles. Future Recommendations in the D-Series are expected to contain that information.

It shall be possible to charge the subscriber accurately for the service.

4.5	Interworking requirements

For further study.

4.6	Interaction with supplementary services

For further study.

4.7	Attributes and values of attributes of the mixed mode service

a)	LOW LAYER ATTRIBUTES

Information transfer attributes

|  |  | Circuit-mode bearer capability | Packet-mode bearer capability |
| --- | --- | --- | --- |
| 1. | Information transfer mode | circuit | packet |
| 2. | Information transfer rate | 64 kbit/s | maximum throughput of a given virtual circuit is less than or equal to the maximum bit rate of the user information access channel and the throughput class of the virtual circuit |
| 3. | Information transfer capability | unrestricted (Note 1) | unrestricted |
| 4. | Structure | unstructured (Note 2) | service data unit integrity |
| 5. | Establishment of communication | demand | demand (VC), permanent (PVC) |
| 6. | Symmetry | bidirectional symmetric | bidirectional symmetric |
| 7. | Communication configuration | point-to-point | point-to-point |

Access attributes

|  |  | Circuit-mode bearer capability | Packet-mode bearer capability |
| --- | --- | --- | --- |
| 8. | Access channel: | B for user information D for signalling | user information over virtual circuit within B- or D-channel. When D-channel is used, maximum packet size and quality of service may be restricted. Signalling may be provided via D-channel and/or virtual circuit within B-channel (Note 3) |
| 9. | Access protocol |  |  |
| 9.1 | Signalling access protocol layer 1: | Rec. I.430/I.431 | Rec. I.430/I.431 |
| 9.2 | Signalling access protocol layer 2: | Rec. I.440/I.441 | Rec. I.440/I.441, X.31 |
| 9.3 | Signalling access protocol layer 3: | Rec. I.450/I.451 | Rec. I.450/I.451, X.31 |
| 9.4 | Information access protocol layer 1: | Rec. I.430/I.431 | Rec. I.430/I.431 |
| 9.5 | Information access protocol layer 2: | Rec. X.75 (SLP) | Rec. X.25 (LAPB) |
| 9.6 | Information access protocol layer 3: | ISO 8208 | Rec. X.25 (PLP) |

b)	HIGH LAYER ATTRIBUTES

10.	Type of user information  :		mixed mode

11.	Layer 4 protocol functions:		X.224, X.214

12.	Layer 5 protocol functions:		X.225, X.215

13.	Layer 6 protocol functions:		T.61, X.226, X.216

13.1	Resolution [pixels per inch (ppi)]:	300 ´ 300

240 ´ 240
								400 ´ 400 optional,

600, 1200

14.	Layer 7 protocol functions:		T.501, T.522, T.561

c)	GENERAL ATTRIBUTES

15.	Supplementary services provided:	for further study

16.	Quality of service:		for further study

17.	Interworking possibilities:		ISDN Teletex, ISDN Telefax 4

(others for further study)

18.	Operational and commercial:	for further study

Note 1 - The interworking arrangements with networks having restricted 64 kbit/s information transfer capability require further study.

Note 2 - Even if no structure is required the network may provide 8 kHz integrity.

Note 3 - User information transferred via virtual channel on the D-channel is for further study.

4.8	Recommended support of mixed mode by an ISDN

a)	Overall support: A

b)	Variation of non-dominant attributes:

1)	Information transfer mode

–	circuit: A

–	packet: A

2)	Establishment		Symmetry	Communication		Support1

of communication			configuration

______________________________________________________________

demand		bidirectional	pt-pt			E

symmetric

3)	Access

| Signalling and OAM (Note 1) |  | User information |  | Support |
| --- | --- | --- | --- | --- |
| Channel and rate | Protocols | Channel and rate | Protocols |  |
| Circuit mode |  |  |  |  |
| D(16) | I.430, I.440, I.441, I.450, I.451 (Note 2) | B(64) | I.430, X.75 (SLP), ISO 8208 | A |
| D(64) | I.431, I.440, I.441, I.450, I.451 (Note 2) | B(64) | I.431, X.75 (SLP), ISO 8208 | A |
| Packet mode |  |  |  |  |
| D(16) | I.430, I.440, I.441, I.450, I.451, X.31 | B(64) or D (16) | I.430, X.25 LAPB, X.25 (PLP) | FS |
| D(64) | I.431, I.440, I.441, I.450, I.451, X.31 | B(64) | I.431, X.25 LAPB, X.25 (PLP) | FS |
| VC in B(64) | for further study | B(64) | for further study | FS |

Note 1 - Definition of protocols for OAM is for further study.

Note 2 - Demand services only. Others are for further study.

4.9	Dynamic description

The circuit mode dynamic description appears in Recommendation I.220.

The definition of E (essential) and A (additional) can be found in Recommendation I.240.