|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	I.241.5

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

INTEGRATED  SERVICES  DIGITAL  NETWORK (ISDN)

SERVICE  CAPABILITIES

TELESERVICES  SUPPORTED  BY  AN  ISDN: 
VIDEOTEX

ITU-T  Recommendation  I.241.5

(Extract from the Blue Book)

NOTES

1	ITU-T Recommendation I.241.5 was published in Fascicle III.7 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation I.241.5

Fascicle III.7 - Rec. I.241.5

xe ""§ TELESERVICES SUPPORTED BY AN ISDN: VIDEOTEX

(Melbourne, 1988)

5	xe ""§Videotex

The prose description for the Videotex service in ISDN is for further study and is intended to be based on Recommendation F.300.

5.1	Definition

The Videotex service in the ISDN is an enhancement of the existing Videotex service with retrieval and mailbox functions for text (alpha) and graphic information.

5.2	Description

For further study.

5.3	Procedures

For further study.

5.4	Network capabilities for charging

This Recommendation does not cover charging principles. Future Recommendations in the D-Series are expected to contain that information.

It shall be possible to charge the subscriber accurately for the service.

5.5	Interworking requirements

For further study.

5.6	Interaction with supplementary services

For further study.

5.7	Attributes and values of attributes of the Videotex service

a)	LOW LAYER ATTRIBUTES

Information transfer attributes

|  |  | User to Videotex centre | Videotex centre to Videotex centre, external computers |
| --- | --- | --- | --- |
| 1. | Information transfer mode: | circuit (Note 1) | circuit/packet |
| 2. | Information transfer rate: | 64 kbit/s | further study |
| 3. | Information transfer capability: | unrestricted | further study |
| 4. | Structure: | further study | further study |
| 5. | Establishment of communication: | demand | demand/permanent |
| 6. | Symmetry: | bidirectional symmetric | bidirectional symmetric |
| 7. | Communication configuration: | point-to-point | demand/permanent point-to-point, multipoint |

Access attributes

| 8. | Access channel: | B for user information (Note 2) D for signalling |
| --- | --- | --- |
| 9. | Access protocol |  |
| 9.1 | Signalling access protocol layer 1: | Rec. I.430/I.431 |
| 9.2 | Signalling access protocol layer 2: | Rec. I.440/I.441 |
| 9.3 | Signalling access protocol layer 3: | Rec. I.450/I.451 |
| 9.4 | Information access protocol layer 1: | Rec. I.430/I.431 |
| 9.5 | Information access protocol layer 2: | Rec. X.75 (SLP) |
| 9.6 | Information access protocol layer 3: | ISO 8208 |

b)	HIGH LAYER ATTRIBUTES

10.	Type of user information: Videotex

11.	Layer 4 protocol functions			ü

12.	Layer 5 protocol functions			ï

13.	Layer 6 protocol functions			ï

13.1	Resolution [pixels per inch (ppi)]			ý	For further study

13.2	Graphic mode			ï

14.	Layer 7 protocol functions			ï

þ

c)	GENERAL ATTRIBUTES: for further study

Note 1 - The use of packet-mode is for further study.

Note 2 - The use of the D-channel for Videotex information is for further study.

5.8	Recommended support of Videotex by an ISDN

a)	Overall support: A

b)	Variations of non-dominant attributes:

1)	Information transfer mode

–	circuit		user terminal-to-Videotex centre				A

videotex centre to Videotex centre, external computer		A

–	packet		videotex centre to Videotex centre, external computer		A

2)	Establishment		Symmetry	Communication		Support1
		of communication				configuration	________________________________________________________________________

demand		ü

permanent	ý		bidirectional	pt-pt			E

(Note)		þ		symmetric

demand		ü

permanent	ý		bidirectional	multipoint		A

(Note)		þ		symmetric	(Note)

Note – Between Videotex centres and to external computers.

3)	Access

| Signalling and OAM (Note 1) |  | User information |  | Support |
| --- | --- | --- | --- | --- |
| Channel and rate | Protocols | Channel and rate | Protocols |  |
| Circuit mode |  |  |  |  |
| D(16) | I.430, I.440, I.441, I.450, I.451 (Note 2) | B(64) | I.430, X.75 (SLP), ISO 8208 | A |
| D(64) | I.431, I.440, I.441, I.450, I.451 (Note 2) | B(64) | I.431, X.75 (SLP), ISO 8208 | A |
| Packet mode |  |  |  |  |
| D(16) | FS | B(64) or D (16) | FS | FS |
| D(64) | FS | B(64) | FS | FS |
| VC in B(64) | FS | B(64) | FS | FS |

Note 1 - Definition of protocols for OAM is for further study.

Note 2 - Demand services only. Others are for further study.

5.9	Dynamic description

The circuit mode dynamic description appears in Recommendation I.220.

The definition of E (essential) and A (additional) can be found in Recommendation I.240.