|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	I.413

TELECOMMUNICATION	(03/93)
STANDARDIZATION  SECTOR
OF  ITU

INTEGRATED  SERVICES  DIGITAL
NETWORK  (ISDN)

ISDN  USER-NETWORK  INTERFACES

B-ISDN  USER-NETWORK  INTERFACE

ITU-T  Recommendation  I.413

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation I.413 was revised by the ITU-T Study Group XVIII (1988-1993) and was approved by the WTSC (Helsinki, March 1-12, 1993).

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector.

In order not to delay publication of this Recommendation, no change has been made in the text to references containing the acronyms “CCITT, CCIR or IFRB” or their associated entities such as Plenary Assembly, Secretariat, etc. Future editions of this Recommendation will contain the proper terminology related to the new ITU structure.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation I.413     (03/93)

Page

1	Introduction		1

2	Reference configuration at the user-network interface		1

2.1	Functional groups and reference points		1

2.2	Examples of physical realizations		2

2.3	Basic characteristics of the interfaces at TB and SB reference points		2

2.4	Relationship between ISDN interfaces		6

2.5	Application of B-ISDN model to functional groups		6

3	Physical layer information flows and interface functions		7

3.1	Relation with other sublayers or entities		7

3.2	Modes of operation		8

3.3	Interface function		8

4	UNI related OAM functions		9

Annex  A – Alphabetical list of abbreviations used in this Recommendation		9

Recommendation I.413

Recommendation I.413     (03/93)

B-ISDN  USER-NETWORK  INTERFACEXE "B-ISDN  USER-NETWORK  INTERFACE"§

(Geneva, 1991; revised at Helsinki, 1993)

# 1	Introduction

This Recommendation gives the reference configuration for the B-ISDN user-network interface (UNI) and examples of physical realizations. It describes physical layer information flows according to the B-ISDN Protocol reference model and identifies interface functions. It also addresses OAM issues as they relate to the reference configuration at the user access and to the interface specifications.

# 2	Reference configuration at the user-network interfaceXE "Reference configuration at the user-network interface"§

## 2.1	Functional groups and reference points

The reference configurations defined in Figure 1/I.411 for ISDN basic access and primary access are considered general enough to be applicable to all aspects of the B-ISDN accesses.

Figure 1 shows the B-ISDN reference configurations which contain the following:

–	functional groups: B-NT1, B-NT2, B-TE1, TE2, B-TE2 and B-TA;

–	reference points: TB, SB and R.

Figure 1/I.413 [D01]  =   10 cm

In order to clearly illustrate the broadband aspects, the notations for reference points and for functional groups with broadband capabilities are appended with the letter B (e.g. B-NT1, TB). The broadband functional groups are equivalent to the functional groups defined in Recommendation I.411. Interfaces at the R reference point may or may not have broadband capabilities.

Interfaces at reference points SB and TB will be standardized. These interfaces will support all ISDN services.

## 2.2	Examples of physical realizations

Figure 2 gives examples of physical configurations illustrating combinations of physical interfaces at various reference points. The examples cover configurations that could be supported by standardized interfaces at reference points SB and TB. Other configurations may also exist. For example, physical configurations of B-NT2 may be distributed or use shared medium to support local area network (LAN) emulation and other applications.

Figure 3 illustrates possible physical configurations, but does not preclude alternative configurations. Whether a single interface at the SB reference point can cover different configurations as illustrated on Figure 3 is for further study.

Figures 2a) and 2b) show separate interfaces at the SB and TB reference points; Figures 2c) and 2d) show an interface at SB but not at TB; Figures 2e) and 2f) show an interface at TB but not at SB; Figures 2g) and 2h) show separate interfaces at S, SB and TB; Figures 2i) and 2j) show interfaces at SB and TB which are coincident.

Additionally, Figures 2b), 2d), 2f), 2h) and 2j) show an interface at reference point R.

## 2.3	Basic characteristics of the interfaces at TB and SB reference pointsXE "Basic characteristics of the interfaces at TB and SB reference points"§

### 2.3.1	Characteristics of the interfaces at 155.520 Mbit/sXE "Characteristics of the interfaces at 155.520 Mbit/s"§

#### 2.3.1.1	Interface at TB reference pointXE "Interface at TB reference point"§

There are two options for the interface at the physical layer: a cell-based physical layer and an SDH-based physical layer. The ATM layer is common to these two options.

There is only one interface per B-NT1 at the TB reference point. The operation of the physical medium is point-to-point in the sense that there is one sink (receiver) in front of one source (transmitter).

When point-to-multipoint configurations at TB are used as an option at ATM and higher layers, their implications at these layers and the physical layer have to be studied.

#### 2.3.1.2	Interface at the SB reference pointXE "Interface at the SB reference point"§

There are two options for the interface at the physical layer: a cell-based physical layer and an SDH-based physical layer. The ATM layer is common to both options.

One or more SB interfaces per B-NT2 are present. The interface at the SB reference point is point to point at the physical layer in the sense that there is only one sink (receiver) in front of one source (transmitter) and may be point to multipoint at the other layers.

#### 2.3.1.3	Relationship between interfaces at SB and TBXE "Relationship between interfaces at SB and TB"§

Configurations described in Figures 2i) and 2j) require that the interface specifications at TB and SB have a high degree of commonality, in order to ensure that a simple broadband terminal may be connected directly to the TB interface.

The feasibility of achieving the needed commonality requires further study.

Figure 2/I.413 [D02] = 21    PAGE PLEINE

FIGURE 3/I.413. SUITE..[D03] = 21 CM   PAGE PLEINE

### 2.3.2	Characteristics of the interfaces at 622.080 Mbit/sXE "Characteristics of the interfaces at 622.080 Mbit/s"§

#### 2.3.2.1	Interface at the TB reference pointXE "Interface at the TB reference point"§

There are two options for the interface at the physical layer: a cell-based physical layer and an SDH-based physical layer. The ATM layer is common to both options.

There is only one interface per B-NT1 at the TB reference point. The operation of the Physical medium is point to point in the sense that there is one sink (receiver) in front of one source (transmitter).

When point to multipoint configurations at TB are used as an option at ATM and higher layers, their implications at these layers and the Physical layer have to be studied.

## 2.4	Relationship between ISDN interfacesXE "Relationship between ISDN interfaces"§

Figures 2g) and 2h) show configurations where B-ISDN and ISDN interfaces may occur at SB and S respectively. In this case B-NT2 functionalities have to ensure the interface capabilities for both S and SB. Other configurations for supporting terminals at the interface at the S reference point may exist.

## 2.5	Application of B-ISDN model to functional groupsXE "Application of B-ISDN model to functional groups"§

### 2.5.1	General

Lists of functions for each functional group are given below. Each particular function is not necessarily restricted to a single functional group. For example, “interface termination” functions are included in the function lists of B-NT1, B-NT2 and B-TE. The function lists for B-NT1, B-NT2, B-TE and B-TA are not exhaustive. Not all specific functions in a functional group need to be present in all implementations.

A complete description of the functional groups remains for further study.

### 2.5.2	Network termination 1 (B-NT1) for B-ISDNXE "Network termination 1 (B-NT1) for B-ISDN"§

This functional group includes functions broadly equivalent to layer 1 of the OSI reference model. Examples of B-NT1 functions are:

–	line transmission termination;

–	transmission interface handling;

–	OAM functions.

Additional functions specific to the transmission system may be required. The means of powering of implementations of the B-NT1 is for further study. When the B-NT1 terminates cell-based OAM flows, cell delineation is required.

### 2.5.3	Network termination 2 (B-NT2) for B-ISDNXE "Network termination 2 (B-NT2) for B-ISDN"§

This functional group includes functions broadly equivalent to layer 1 and higher layers of the Recommendation X.200 reference model. B-NT2 can be null in the case of commonality between TB and SB.

Examples of B-NT2 functions are:

–	adaptation functions for different media and topologies (MA functions);

–	functions of a distributed B-NT2;

–	cell delineation;

–	concentration;

–	buffering;

–	multiplexing/demultiplexing;

–	resource allocation;

–	usage parameter control;

–	adaptation layer functions for signalling (for internal traffic);

–	interface handling (for the TB and SB interfaces);

–	OAM functions;

–	signalling protocol handling;

–	switching of internal connections.

B-NT2 implementations may be concentrated or distributed. In a specific access arrangement, the B-NT2 may consist only of physical connections. When present, implementations of the B-NT2 are locally powered.

### 2.5.4	Terminal equipment (B-TE) for B-ISDNXE "Terminal equipment (B-TE) for B-ISDN"§

This functional group includes functions broadly belonging to layer 1 and higher layers of the Recommendation X.200 reference model.

Examples of B-TE functions are:

–	user/user-and-user/machine dialogue and protocol;

–	interface termination and other layer 1 functions;

–	protocol handling for signalling;

–	connection handling to other equipments;

–	OAM functions.

The possibility of powering the B-TE via the SB interface is for further study.

#### 2.5.4.1	Terminal equipment type 1 (B-TE1) for B-ISDNXE "Terminal equipment type 1 (B-TE1) for B-ISDN"§

This functional group includes functions belonging to the B-TE functional group with an interface that complies with the B-ISDN SB and/or TB interface Recommendations.

#### 2.5.4.2	Terminal equipment type 2 (B-TE2) for B-ISDNXE "Terminal equipment type 2 (B-TE2) for B-ISDN"§

This functional group includes functions belonging to the functional group B-TE but with a broadband interface that complies with interface Recommendations other than the B-ISDN interface Recommendations or interfaces not included in CCITT Recommendations.

### 2.5.5	Terminal adaptor (B-TA) for B-ISDNXE "Terminal adaptor (B-TA) for B-ISDN"§

This functional group includes functions broadly belonging to layer 1 and higher layers of the Recommendation X.200 reference model that allow a TE2 or a B-TE2 terminal to be served by a B-ISDN user-network interface.

# 3	Physical layer information flows and interface functionsXE "Physical layer information flows and interface functions"§

Physical layer functions and physical layer primitives are defined in Recommendation 4.2/I.321. The present clause defines the information flows between the physical medium (PM), the transmission convergence sublayer (TC) and their adjacent entities (ATM layer and management plane). The information flows identified here do not imply any physical realization. Information flows identified in this clause may not be exhaustive.

## 3.1	Relation with other sublayers or entities

The information flows specified in the following subclauses will be provided in Recommendation I.321 by primitives. Further study is required to specify these primitives.

### 3.1.1	Information exchanged between the PM and the TC sublayersXE "Information exchanged between the PM and the TC sublayers"§

a)	From the PM sublayer to the TC sublayer:

The PM sublayer provides at least the following information to the TC sublayer:

–	a flow of logical symbols (e.g. bits),

–	associated timing information.

b)	From the TC sublayer to the PM sublayer:

The TC sublayer provides at least the following information to the PM sublayer:

–	a flow of logical symbols (e.g. bits),

–	associated timing information.

### 3.1.2	Information exchanged between the physical layer and the ATM layerXE "Information exchanged between the physical layer and the ATM layer"§

a)	From the physical layer to the ATM layer 1):

The physical layer provides at least the following information  to the ATM layer:

–	valid cells (excluding idle cells and physical layer OAM cells),

–	associated timing (e.g. presence of data and clock information).

b)	From the ATM layer to the physical layer:

–	assigned and unassigned cells if any available,

–	associated timing (e.g. presence of data and clock information).

In case no cells are available, no data are transferred and the physical layer inserts idle cells to build up the data flow to be transmitted.

### 3.1.3	Information exchanged between the physical layer and the management planeXE "Information exchanged between the physical layer and the management plane"§

a)	From the physical layer to the management plane:

–	loss of incoming signal;

–	indication of received errors or indication of degraded error performance.

Detection of bit errors may be based on received unexpected code violations or other bit error detecting schemes.

In addition, other information may be provided to the management plane. This is for further study.

b)	From the management plane to the physical layer:

For further study.

## 3.2	Modes of operation

Normal mode: “fully active”.

Other modes, e.g. emergency mode in case of power failure or deactivated mode in order to save power are for further study.

## 3.3	Interface functionXE "Interface function"§

### 3.3.1	Data transferXE "Data transfer"§

The user information together with the information for connection related functions (e.g. signalling) are carried in ATM cells. OAM information related to the physical layer is carried in transmission overheads or PL-OAM cells, depending on the transmission structure used (SDH-based or cell-based) and on the functional entity concerned.

### 3.3.2	Timing

Bit timing has to be provided according to Recommendation I.432.

### 3.3.3	Bit sequence independence

The PM and the associated transmission system payload have to provide bit sequence independence.

### 3.3.4	Provision of power feeding

For further study.

### 3.3.5	Activation/deactivation

For further study.

# 4	UNI related OAM functionsXE "UNI related OAM functions"§

The following OAM functions associated with the UNI have been identified and are described in Recommendation I.610:

1)	transmission and reception of maintenance signals [e.g. alarm indication signal (AIS) and far end receive failure (FERF)];

2)	performance monitoring;

3)	control communication provisions.

Some overhead capacity has to be allocated to these functions, but the exact implementation depends on the physical layer option selected, i.e. whether it is SDH-based or cell-based. Implementation of these maintenance functions is described in Recommendation I.432. The definition of maintenance signals required for the supervision of cell delineation and header error performance is for further study.

Annex  A

Alphabetical list of abbreviations used 
in this Recommendation

(This annex forms an integral part of this Recommendation)

AIS		Alarm indication signal

B-NT		Network termination for B-ISDN

B-NT1		Network termination 1 for B-ISDN

B-NT2		Network termination 2 for B-ISDN

B-TA		Terminal adaptor for B-ISDN

B-TE		Terminal equipment for B-ISDN

FERF		Far end receive failure

LAN		Local area network

MA		Medium adaptor

TA		Terminal adaptor

TC		Transmission convergence sublayer

UNI		User-network interface

1) 	The physical layer will provide a clock to the ATM layer. This clock is derived from the line rate of the physical layer (e.g. a 	bit clock at 155.52 Mbit/s for the user-network interface standardized in Recommendation I.432).

For certain applications, there may be a need to indicate to the ATM layer that an idle cell has been discarded by the physical 	layer. Two possible applications are generic flow control (GFC) and the shaping of traffic flows.