|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	J.25
			(ex CMTT.605)

TELECOMMUNICATION	(05/86)
STANDARDIZATION  SECTOR
OF  ITU

TELEVISION  AND  SOUND  TRANSMISSION

ESTIMATION  OF  TRANSMISSION PERFORMANCE  OF  SOUND-PROGRAMME CIRCUITS  SHORTER  OR  LONGER  THAN  
THE  HYPOTHETICAL  REFERENCE  CIRCUIT

ITU-T  Recommendation  J.25

(Formerly Recommendation ITU-R CMTT.605)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation J.25 (formerly Recommendation ITU-R CMTT.605) was elaborated by the former ITU-R Study Group CMTT. See Note 1 below.

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector (ITU-R).

Conforming to a joint decision by the World Telecommunication Standardization Conference (Helsinki, March 1993) and the Radiocommunication Assembly (Geneva, November 1993), the ITU-R Study Group CMTT was transferred to ITU-T as Study Group 9, except for the satellite news gathering (SNG) study area which was transferred to ITU-R Study Group 4.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1990

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation J.251)

Recommendation J.25     (05/86)

ESTIMATION  OF  TRANSMISSION  PERFORMANCE  OF  SOUND-PROGRAMME CIRCUITS  SHORTER  OR  LONGER  THAN  THE  HYPOTHETICAL  
REFERENCE CIRCUIT

(1982; revised 1986)

The CCIR

UNANIMOUSLY  RECOMMENDS

that for the estimation of transmission performance of sound-programme circuits shorter or longer than the hypothetical reference circuit the following rules be applied:

Laws of addition

1.	Comments on the use of the laws of addition

The definition of a circuit in terms of a single multiple of the hypothetical reference circuit is impossible if the number of audio-to-audio sections and the length of the circuit differ from those of the hypothetical reference circuit by different ratios, i.e. if n/3  ¹  L /l where

n :	number of audio-to-audio sections,

L :	length of the circuit,

l :	2500 km.

In such cases two definitions of the circuit in terms of the hypothetical reference circuit should be used, one for those parameters primarily a function of the circuit configuration, and a second for those parameters (e.g. continuous random noise) primarily a function of the length of the circuit.

2.	Law pertaining to circuit configuration

For the first definition of the circuit in terms of the hypothetical reference circuit use the following equation for all parameters in Table II except for “continuous random noise”.

Let D3  :	design objective or the parameter derived therefrom and indicated in Table II, permitted value for three homogeneous sections of the hypothetical reference circuit,

and Dn :	performance, or the parameter mentioned above, to be estimated for n sections.

The following expressions apply:

–	for absolute values and logarithmic deviations:

(1)

–	for absolute levels:

(2)

–	for logarithmic ratios:

(3)

h has the value 1, 3/2 or 2 in accordance with Table II: h  =  1 gives linear or arithmetic law of addition, h  =  3/2 gives the “three-halves power” law of addition and h  =  2 gives the quadratic (r.m.s.) law of addition.

Calculated values of   and    are given in Table I.

3.	Law pertaining to circuit length

For the second definition of the circuit in terms of the hypothetical reference circuit use the following equation for “continuous random noise levels” only. When distance is considered the law of addition becomes:

(4)

where Dn, D3, L and l are as defined in § 1 and § 2. If l < 280 km, set l  =  280 km in (4).

Note – The values calculated as per this Recommendation provide only indications of the probable performance. They should be used with caution when studying the design of equipment, because the laws of addition are not precisely known for every type of impairment. Further studies are necessary, especially to cover the case of mixed analogue-and-digital circuits.

TABLE  I

|  | Formula (1) |  |  | Formulas (2) and (3) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| n |  |  |  |  |  |  |
|  | h = 1 | h = 3/2 | h = 2 | h = 1 | h = 3/2 | h = 2 |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 | 0.33 0.67 1.00 1.33 1.67 2.00 2.33 2.67 3.00 3.33 3.67 4.00 4.33 4.67 5.00 | 0.48 0.76 1.00 1.21 1.41 1.59 1.76 1.92 2.08 2.23 2.38 2.52 2.66 2.79 2.92 | 0.58 0.82 1.00 1.15 1.29 1.41 1.53 1.63 1.73 1.83 1.91 2.00 2.08 2.16 2.24 | – 9.5 dB – 3.5 0.0 2.5 4.4 6.0 7.4 8.5 9.5 10.5 11.3 12.0 12.7 13.4 14.0 | – 6.4 dB – 2.3 0.0 1.7 3.0 4.0 4.9 5.7 6.4 7.0 7.5 8.0 8.5 8.9 9.3 | – 4.8 dB – 1.8 0.0 1.2 2.2 3.0 3.7 4.3 4.8 5.2 5.6 6.0 6.4 6.7 7.0 |

TABLE  II

|  | Parameter | D3 expressed in | Applicable formula | h (tentative value) |
| --- | --- | --- | --- | --- |
| Parameters | Insertion gain (1.0 kHz) |  |  |  |
| relevant to mono and | Adjustment error | dB | 1 | 2 |
| stereo circuits | Variation during 24 h | dB | 1 | 2 |
|  | Gain/frequency response | dB | 1 | 3/2 |
|  | Group delay/frequency response referred to minimum | ms | 1 | 1 |
|  | Maximum weighted noise level | dBq0ps | 4 | 2 |
|  | Single tone interference level | dBm0 | 2 | 3/2 |
|  | Ratio of disturbing modulation by power supply to reference signal | dB | 3 | 3/2 |
|  | Non-linear distortion | % | 1 | 3/2 |
|  | Error in reconstituted frequency | Hz | 1 | 3/2 |
|  | Intelligible crosstalk ratio | dB | 3 | 3/2 |
|  | Error in amplitude/amplitude response | dB | 1 | 3/2 |
| Additional | Difference in gain between A and B channels | dB | 1 | (1) |
| parameters relevant to | Phase difference between A and B channels | degree | 1 | (1) |
| stereo circuits | Intelligible crosstalk ratio between A and B channels | dB | 3 | 3/2 |
|  | Non-linear crosstalk ratio between A and B channels | dB | 3 | 3/2 |
| (1) Some administrations have found h = 3/2 appropriate. Tests conducted in Japan [CCIR, 1978-82], however, showed better agreement with quadratic law of addition (h = 2) than 3/2 law of addition. |  |  |  |  |

REFERENCES

CCIR Documents

[1978-82]: CMTT/233 (Japan).

1) 	Formerly Recommendation ITU-R CMTT.605.