|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	J.54
			(ex CMTT.660)

TELECOMMUNICATION	(05/86)
STANDARDIZATION  SECTOR
OF  ITU

TELEVISION  AND  SOUND  TRANSMISSION

TRANSMISSION  OF  ANALOGUE  
HIGH-QUALITY  SOUND-PROGRAMME  SIGNALS  ON  MIXED  ANALOGUE-AND-DIGITAL  CIRCUITS  USING  384  kbit/s CHANNELS

ITU-T  Recommendation  J.54

(Formerly Recommendation ITU-R CMTT.660)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation J.54 (formerly Recommendation ITU-R CMTT.660) was elaborated by the former ITU-R Study Group CMTT. See Note 1 below.

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector (ITU-R).

Conforming to a joint decision by the World Telecommunication Standardization Conference (Helsinki, March 1993) and the Radiocommunication Assembly (Geneva, November 1993), the ITU-R Study Group CMTT was transferred to ITU-T as Study Group 9, except for the satellite news gathering (SNG) study area which was transferred to ITU-R Study Group 4.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1990

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation J.541)

Recommendation J.54     (05/86)

TRANSMISSION  OF  ANALOGUE  HIGH-QUALITY  SOUND-PROGRAMME
SIGNALS ON  MIXTED ANALOGUE-AND-DIGITAL  CIRCUITS
USING  384  kbit/s  CHANNELS

(1986)

The CCIR,

CONSIDERING

(a)	that high-quality sound-programme signals on mixed analogue-and-digital circuits must satisfy the performance requirements of Recommendation 505;

(b)	that the number of analogue-to-digital and digital-to-analogue conversions as well as the number of digital coding techniques should be kept to a minimum, in order for the international sound programme connection to provide the same or higher degree of performance than possible with a homogeneous analogue connection;

(c)	that the CCITT has recommended a 384 kbit/s H0 bearer channel in Recommendation I.412 which has the capacity to transport sound-programme signals on an integrated services digital network (ISDN);

(d)	that digital-to-digital conversions using the same encoding law on tandem sound-programme circuits with different primary rates should not introduce any signal degradation,

UNANIMOUSLY  RECOMMENDS

that the transmission of sound-programme signals on 384 kbit/s channels, with an analogue interface at the input and output of the international connection, should satisfy the analogue-and-digital requirements defined in § 1 and 2 respectively of this Recommendation.

Note 1 – Equipment characteristics for the coding of analogue high-quality sound-programme signals using 384 kbit/s channels have been defined in CCITT Recommendation J.41. Characteristics for equipment offering digital access at 384 kbit/s into 2048 kbit/s are given in CCITT Recommendations G.735 and G.737.

Note 2 – Other coding techniques are proposed and are the subject of Question 18/CMTT, for example, coding of analogue sound-programme signals at 316 kbit/s, insertion into a 320 kbit/s channel and multiplexing of six high-quality sound-programme channels within 2048 kbit/s. Techniques which may be used by bilateral agreement of the administrations concerned are listed in Table I of Report 647.

1.	Analogue requirements

Transmission of high-quality sound-programme signals on mixed analogue-and-digital circuits shall satisfy the performance requirements of Recommendation 505.

1.1	Single codec performance

The analogue-to-analogue transmission performance of a single codec shall be equivalent to one-third the performance characteristics of the hypothetical reference circuit (see Recommendation 505). Where applicable, the laws of addition given in Recommendation 605 can be used to calculate the single codec performance with n  =  3 analogue-to-analogue sections.

1.2	Tandem codec performance

The hypothetical reference circuit (HRC) given in Recommendation 502 is divided into three sections of equal length which could be either analogue or digital. Each digital section should only contain one encoder and one decoder. Therefore, in a mixed analogue-and-digital HRC, Table I indicates the number of codecs which may be connected in tandem.

TABLE  I – Number of tandem codecs on a mixed
analogue-and-digital sound programme circuit

| Analogue sections | Tandem codecs |
| --- | --- |
| 0 1 2 3 | 3 2 1 0 |

2.	Digital requirements

Digital-to-digital conversions using the same encoding law as given in Table II or Table III of this Recommendation on tandem sound-programme circuits with different primary rates should not introduce any signal degradation. Digital-to-digital conversions using different encoding laws, as given in Tables II and III, should cause minimum signal degradation.

TABLE  II – Coding table for instantaneous companding of sound-programme signals *

| Normalized analogue input (1) | Normalized analogue output (1) | Compressed digital code | Segment No. | Effective resolution (bits) |
| --- | --- | --- | --- | --- |
| 8160 to 8192 4096 to 4128 | 8176 4112 | 895 768 | 1 | 9 |
| 4080 to 4096 2048 to 2064 | 4088 2056 | 767 640 | 2 | 10 |
| 2040 to 2048 1024 to 1032 | 2044 1028 | 639 512 | 3 | 11 |
| 1020 to 1024 512 to 516 | 1022 514 | 511 384 | 4 | 12 |
| 510 to 512 256 to 258 | 511 257 | 383 256 | 5 | 13 |
| 255 to 256 128 to 129 | 255.5 128.5 | 255 128 | 6 | 14 |
| 127 to 128 0 to 1 | 127.5 0.5 | 127 0 |  |  |

* The positive half of the coding table only is shown. (1) The top code word of ± 8192 corresponds to an overload level of +15 dBm0s at the 0 dB insertion loss frequency (2.1 kHz) of the CCITT Recommendation J.17 pre-emphasis circuit with 6.5 dB loss at 800 Hz.

TABLE  III – Coding table for near-instantaneous companding of sound-programme signals

| Range | Normalized analogue input (1) | Normalized analogue output (1) | Compressed digital code | Effective resolution (bits) |
| --- | --- | --- | --- | --- |
| 4 | +8176 to +8192 0 to +16 –16 to 0 –8192 to –8176 | +8184 +8 –8 –8184 | +511 0 –1 –512 | 10 |
| 3 | + 4088 to + 4096 0 to +8 –8 to 0 – 4096 to – 4088 | + 4092 + 4 – 4 – 4092 | +511 0 –1 –512 | 11 |
| 2 | +2044 to +2048 0 to + 4 – 4 to 0 –2048 to –2044 | +2046 +2 –2 –2046 | +511 0 –1 –512 | 12 |
| 1 | +1022 to +1024 0 to +2 –2 to 0 –1024 to –1022 | +1023 +1 –1 –1023 | +511 0 –1 –512 | 13 |
| 0 | +511 to +512 0 to +1 –1 to 0 –512 to –511 | +511.5 + 0.5 – 0.5 –511.5 | +511 0 –1 –512 | 14 |
| (1) The top code word of ±8192 corresponds to an overload level of +12 dBm0s at the 0 dB insertion loss frequency (2.1 kHz) of the CCITT Recommendation J.17 pre-emphasis circuit with 6.5 dB loss at 800 Hz. |  |  |  |  |

2.1	Sampling frequency

The sampling frequency shall be 32 kHz. The associated tolerance shall be ±5  ´  10 –5 as specified in CCITT Recommendations G.732 and G.733 for primary PCM multiplex equipment. This sampling frequency conforms with that given in Recommendation 606.

2.2	Encoding method

The recommended encoding laws are based on a uniformly quantized 14-bit per sample PCM technique with companding and shall employ either method (a) or method (b) with the appropriate rules of precedence as defined below:

(a)	eleven-segment 14-to-11 bit instantaneous A-law companding. The companding characteristic is illustrated in Fig. 1;

(b)	five-range 14-to-10 bit near-instantaneous companding. The companding characteristic is illustrated in Fig. 2.

Digital paths between administrations which have adopted different systems should carry signals encoded according to method (a). Where both administrations have adopted the same method, that method should be used on digital paths between them. Any necessary transcoding will be done by administrations using method (b).

FIGURE 1/660...[D01]

FIGURE 2/660...[D02]

2.3	Coding table

The coding table for the instantaneous A-law compandor is specified in Table II and the coding table for the near-instantaneous compandor is specified in Table III.

2.4	Bit error protection

2.4.1	Error detection

Error detection shall be used to detect erroneous sound programme samples which can be caused by random bit errors, burst errors, frame loss or frame slips.

2.4.2	Error concealment and correction

Error concealment or correction techniques shall be used to ensure that the subjective quality of the sound-programme signal is not degraded below grade 4.5 as defined in Recommendation 562. This quality shall be maintained for a random bit error ratio of 10 –6 as defined in CCITT Recommendation G.821. Further study is necessary to define a requirement for burst errors (see Report 648).

2.4.3	Frame loss/recovery

Frame loss/recovery on digital channels causes interruptions to sound-programme signals. The tolerable limits for these interruptions require further study (see Reports 642 and 647).

2.5	Jitter

Jitter characteristics from the digital interface can produce impairments to the decoded analogue signal after passing through a digital system. This requires further study as outlined in Report 648.

2.6	Transmission bit rate

2.6.1	Monophonic transmission

The transmission bit rate for one sound-programme channel shall be 384 kbit/s, including all ancillary bits as necessary.

2.6.2	Stereophonic transmission

Two separate 384 kbit/s channels shall be used to form a stereophonic (stereo-pair) sound-programme circuit. Both sound-programme channels shall be routed together over the same transmission path to avoid transmission delay differences.

2.7	Network access

Sound-programme signals coded into 384 kbit/s digital channels should interface to the ISDN at 1544 kbit/s or 2048 kbit/s as specified in CCITT Recommendation I.412.

1) 	Formerly Recommendation ITU-R CMTT.660.