|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	J.61
			(ex CMTT.567)

TELECOMMUNICATION	(06/90)
STANDARDIZATION  SECTOR
OF  ITU

TELEVISION  AND  SOUND  TRANSMISSION

TRANSMISSION  PERFORMANCE  OF TELEVISION  CIRCUITS  DESIGNED  FOR  USE IN  INTERNATIONAL  CONNECTIONS

ITU-T  Recommendation  J.61

(Formerly Recommendation ITU-R CMTT.567)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation J.61 (formerly Recommendation ITU-R CMTT.567) was elaborated by the former ITU-R Study Group CMTT. See Note 1 below.

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector (ITU-R).

Conforming to a joint decision by the World Telecommunication Standardization Conference (Helsinki, March 1993) and the Radiocommunication Assembly (Geneva, November 1993), the ITU-R Study Group CMTT was transferred to ITU-T as Study Group 9, except for the satellite news gathering (SNG) study area which was transferred to ITU-R Study Group 4.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1990

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation J.611)

Recommendation J.61    (06/90)

transmission  performance  of  television  circuits
designed  for  use  in  international  connections

(1978; revised in 1982, 1986 and 1990)

The CCIR,

CONSIDERING

the need for a Recommendation concerning analogue television transmissions over long distances, common to the CCIR and CCITT,

UNANIMOUSLY  RECOMMENDS

that, taking account of the definitions in Parts A and B and the measurement methods in Part C and its Annexes, the transmission performance of international television circuits should satisfy the objectives for design given in Parts D and E.

Introduction

The Joint CCIR/CCITT Study Group for Television and Sound Transmission (CMTT) has studied problems which occur when transmitting television signals of various standards over long distances.

The CMTT decided to study unified test methods and transmission performance which can be recommended for circuits intended for transmission of signals conforming to the majority of television standards.

This Recommendation is intended for use where circuits will be required at various times to transmit television signals of the 525-line and the 625-line standards.

However, in view of the comprehensive nature of the Recommendation, it is appropriate that it also be applicable to circuits which are required to transmit television signals of only one standard. Accordingly, the document addresses the different requirements of 525-line, 625-line and multi-standard needs where necessary.

The assumption is made that the circuit does not contain satellite systems using line-rate energy dispersal or systems which employ digital transmission techniques. If it does, it is probable that additional objectives will be required.

The Recommendation contains five parts listed below:

Part A:	Definitions of a connection and circuits

Part B:	Definition of parameters

Part C:	Measurement methods and test signals

Part D:	Design objectives and tolerances applicable to the hypothetical reference circuit

Part E:	Performance of circuits shorter or longer than the hypothetical reference circuit.

Note – References and Bibliography are detailed at the end of the particular Part or Annex to which they are relevant.

PART A – DEFINITION  OF  AN  INTERNATIONAL  TELEVISION  CONNECTION
AND  DEFINITION  OF  THE  TERRESTRIAL  AND  COMMUNICATION  SATELLITE
HYPOTHETICAL  REFERENCE  CIRCUITS

A.1	Definitions

A.1.1	Definition of an international television connection (Fig. 1)

–	Point A, to be considered as the sending end of the international television connection, may be the point at which the programme originates (studio or outside location), a switching centre or the location of a standards converter.

–	Point D, to be considered as the receiving end of the international television connection, may be a programme-mixing or recording centre, a broadcasting station, a switching centre or the location of a standards converter.

–	The local television circuit AB connects point A to the sending terminal station, point B, of the international television circuit.

FIGURE 1/567-3...[D01]

–	The international television circuit, BC, comprises a chain of national and international television links. The precise locations (e.g. within buildings), to be regarded as the points B and C, will be designated by the authorities concerned.

–	The local television circuit CD connects point C, the receiving terminal station of the international television circuit, to the point D.

–	The combination AD, of the international television circuit, BC, and the local television circuits AB and CD, constitutes the international television connection.

The requirements given in subsequent parts of this Recommendation refer to the performance of international television circuits only; no requirements have been laid down for the local television circuits, AB and CD.

A.1.2	Definition of the terrestrial hypothetical reference circuit (Fig. 2a)

The main features of the television terrestrial hypothetical reference circuit, which is an example of an international television circuit (BC in Fig. 1) and which may be of either radio or cable type, are:

–	the overall length between video terminal points is 2500 km;

–	two intermediate video points (M and M¢) divide the circuit into three sections of equal length;

–	the three sections are lined up individually and then interconnected without any form of overall adjustment or correction;

–	the circuit does not contain a standards converter or a synchronizing pulse regenerator, or an equipment for the insertion of signals in the line/field blanking interval.

FIGURE 2a/567-3...[D02]

A.1.3	Definition of the hypothetical reference circuit in the fixed satellite service (Fig. 2b)

A hypothetical reference circuit for a system in the fixed satellite service which may form part of an international television circuit (BC in Fig. 1) is defined as follows:

–	it consists of one Earth station-satellite-Earth station system;

–	it includes one pair of modulation and demodulation equipments for translation from the baseband to the radio-frequency carrier, and from the radio-frequency carrier to the baseband, respectively;

–	it does not include a standards converter or a synchronizing-pulse regenerator, or equipment for the insertion of signals in the line/field blanking interval.

FIGURE 2b/567-3...[D03]

PART B – DEFINITIONS OF PARAMETERS

This part defines terms which are necessary for the understanding of the Recommendation itself. The definitions are necessary either because many of them do not appear in any recognized technical vocabulary or because they are general definitions which have been given meanings which are particular to television transmission. Their use in this limited context should not be taken as a restriction on their use in the International Electrotechnical Vocabulary (IEV) or other vocabularies where they may be given wider definitions, i.e. not limited to television.

B.1	Waveform terminology

The following terms concerning the components and values of a composite colour video signal are illustrated in Fig. 3:

A :	the non-useful d.c. component,

B :	the useful d.c. component, integrated over a complete frame period,

C :	the picture d.c. component, integrated over the active line period (Tu),

D :	the instantaneous value of the luminance component,

E :	the instantaneous signal value with respect to the bottom of the synchronizing pulses,

F :	the peak signal amplitude (positive or negative with respect to blanking level),

G :	the peak amplitudes of chrominance components,

H :	the peak-to-peak signal amplitude,

J :	the difference between black level and blanking level (set-up),

K :	the peak-to-peak amplitude of the colour burst,

L :	the nominal value of the luminance component,

M :	the peak-to-peak amplitude of a monochrome composite video signal (M  =  L  +  S),

S :	the amplitude of the synchronizing pulses,

Tsy :	duration of line synchronizing pulse,

Tlb :	duration of line blanking period,

Tu :	duration of active line period,

Tb :	duration of breezeway,

Tfp :	duration of front porch,

Tbp :	duration of back porch.

The amplitudes L, S and M are used as reference amplitudes for the video signal. The amplitudes defined by B, C, D, E, F, G, H and J above, may be expressed as percentages of the value L.

Average picture level (APL) is the mean value of C over a complete frame period (excluding blanking periods) expressed as a percentage of L.

FIGURE 3/567-3...[D04]

B.2	Requirements at points of video interconnection

B.2.1	Nominal impedance (Z0)

At points of video interconnection, the input and output impedance (Z0) of each section should be specified, subject to bilateral agreement, as either unbalanced or balanced with respect to earth.

B.2.2	Return loss

The return loss, relative to Z0, of an impedance Z is, in the frequency domain:

In the time domain, it is expressed by the symbolic formula:

where A1 is the peak-to-peak amplitude of the incident signal and A2 is the peak-to-peak amplitude of the reflected signal. Numerically, the result is the same as that obtained by the frequency domain method if the return loss is independent of frequency.

B.2.3	Polarity and d.c. component

The polarity of the signal should be “positive”, that is to say, such that black-to-white transitions are positive-going.

The useful d.c. component, B in Fig. 3, which is related to the average luminance of the picture, may or may not be contained in the signal and need not be transmitted or delivered at the output.

A non-useful d.c. component, A in Fig. 3, may be present in the signal (for example, due to d.c. supplies). Limits for this component need to be specified for the terminated and unterminated conditions.

B.2.4	Nominal signal amplitude

The nominal signal amplitude is the peak-to-peak amplitude of the monochrome video signal that includes the synchronizing signal and luminance signal component set to peak-white (M in Fig. 3).

B.3	Transmission performance requirements

The definitions in § B.3.2 and the subsequent sub-sections assume that the circuit has nominal insertion gain as defined in § B.3.1 below.

B.3.1	Insertion gain

Insertion gain is defined as the ratio, expressed in decibels, of the peak-to-peak amplitude of a specified test signal at the receiving end to the nominal amplitude of that signal at the sending end, the peak-to-peak amplitude being defined as the difference between the amplitudes measured at defined points of the signal used.

B.3.2	Noise

B.3.2.1	Continuous random noise

The signal-to-noise ratio for continuous random noise is defined as the ratio, expressed in decibels, of the nominal amplitude of the luminance signal, L in Fig. 3, to the r.m.s. amplitude of the noise measured after band limiting. A signal-to-weighted-noise ratio is defined as a ratio, expressed in decibels, of the nominal amplitude of the luminance signal, L in Fig. 3, to the r.m.s. amplitude of the noise measured after band limiting and weighting with a specified network.

The measurement should be made with an instrument having, in terms of power, a defined time constant or integrating time.

B.3.2.2	Low-frequency noise

The signal-to-noise ratio for low-frequency noise is defined as the ratio, expressed in decibels, of the nominal amplitude of the luminance signal, L, in Fig. 3, to the peak-to-peak amplitude of the noise after band limiting to include only the spectrum 500 Hz to 10 kHz.

B.3.2.3	Periodic noise

The signal-to-noise ratio for periodic noise is defined as the ratio, expressed in decibels, of the nominal amplitude of the luminance signal, L in Fig. 3, to the peak-to-peak amplitude of the noise. Different values are specified for noise at a single frequency between 1 kHz and the upper limit of the video frequency band and for power-supply hum including lower-order harmonics.

B.3.2.4	Impulsive noise

The signal-to-noise ratio for impulsive noise is defined as the ratio, expressed in decibels, of the nominal amplitude of the luminance signal, L in Fig. 3, to the peak-to-peak amplitude of the impulsive noise.

B.3.3	Cross-talk from another television channel

The signal-to-cross-talk ratio is defined as the ratio, expressed in decibels, of the nominal amplitude of the luminance signal (L in Fig. 3) to the peak-to-peak amplitude of the interfering signal.

B.3.4	Non-linear distortion

In a television circuit the transmission characteristic may not be completely linear. The extent of the non-linear distortion which is produced will depend primarily on:

–	the average picture level, as defined in § B.1;

–	the instantaneous value of the luminance signal voltage (D in Fig. 3);

–	the amplitude of the chrominance signal (G in Fig. 3).

There would, in general, be little purpose in defining completely the non-linear characteristics of a transmission circuit. It is necessary, therefore, to limit the number of measured quantities by restricting them to those which are recognized as being directly correlated with picture quality. Additionally, the test conditions should be restricted by introducing a systematic classification in the definition of the quantities to be measured. Examples of distortions not included in this classification are given in the documentation [CCIR, 1970-74a and b].

The nature of the video signal is such that, in terms of picture quality, the impairment due to the effect of circuit non-linearity on the synchronizing signal is different from the effect of circuit non-linearity on the picture signal.

Furthermore, the non-linearity may affect the luminance and chrominance signals individually or cause interaction between them. This leads to the following system of classification of non-linear distortions:

| Non-Linear distortion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Synchronizing signal (§ B.3.4.2) |  |  |  |  |  | Picture signal (§ B.3.4.1) |  |  |  |
| Luminance signal |  |  |  |  |  | Chrominance signal |  |  |  |
| Amplitude distortion |  |  |  | Amplitude distortion |  | Phase distortion |  |  |  |
| Due to luminance amplitude (§ B.3.4.1.1) |  | Due to chrominance amplitude (intermodulation) (§ B.3.4.1.4) |  |  |  | Due to chrominance amplitude (§ B.3.4.1.2) |  | Due to luminance amplitude (differential phase) (§ B.3.4.1.3) |  |
|  |  |  | Due to chrominance amplitude (§ B.3.4.1.2) |  | Due to luminance amplitude (differential gain) (§ B.3.4.1.3) |  |  |  |  |

The above classification applies for steady-state conditions during a time span which is long in relation to the field period. In this case, the concept of average picture level has a precise significance. If these conditions are not fulfilled, for example, if a sudden change in the APL is introduced, additional non-linear effects may be produced, the extent of which will depend on the long-time transient response of the circuit. This aspect requires further study (see Study Programme 13B/CMTT, Report 636 and [CCIR, 1970-74c, d, and e]).

Additional non-linearity may also occur if a sudden change in signal amplitude occurs [CCIR, 1970-74a].

B.3.4.1	Picture signal

B.3.4.1.1	Luminance signal

For a particular value of average picture level, the non-linear distortion of the luminance signal is defined as the departure from proportionality between the amplitude of a small step function at the input to the circuit and the corresponding amplitude at the output, as the initial level of the step is shifted from blanking level to white level.

B.3.4.1.2	Chrominance signal

Gain

For fixed values of luminance signal amplitude and average picture level, the non-linear gain distortion of the chrominance signal is defined as the departure from proportionality between the amplitude of the chrominance sub-carrier at the input to the circuit and the corresponding amplitude at the output, as the amplitude of the sub-carrier is varied from a specified minimum to a maximum value.

Phase

For fixed values of luminance signal amplitude and average picture level, the non-linear phase distortion of the chrominance signal is defined as the variation in the phase of the chrominance sub-carrier at the output, as the amplitude of the sub-carrier is varied from a specified minimum to a maximum value.

B.3.4.1.3	Intermodulation from the luminance signal into the chrominance signal

Differential gain

If a constant small amplitude of chrominance sub-carrier, superimposed on a luminance signal, is applied to the input of the circuit, the differential gain is defined as the change in the amplitude of the sub-carrier at the output as the luminance varies from blanking level to white level, the average picture level being maintained at a particular value.

Differential phase

If a constant small amplitude of chrominance sub-carrier without phase modulation, superimposed on a luminance signal, is applied to the input of the circuit, the differential phase is defined as the change in the phase of the sub-carrier at the output as the luminance varies from blanking level to white level, the average picture level being maintained at a particular value.

B.3.4.1.4	Intermodulation from the chrominance signal into the luminance signal

If a luminance signal of constant amplitude is applied to the input of a circuit, the intermodulation is defined as the variation of the amplitude of the luminance signal at the output resulting from the superimposition on the input signal of a chrominance signal of specified amplitude, the average picture level being maintained at a particular value.

B.3.4.2	Synchronizing signal

B.3.4.2.1	Steady-state distortion

If a video signal of specified average picture level and containing synchronizing pulses of nominal amplitude (S in Fig. 3) is applied to the input of the circuit, the steady state non-linear distortion is defined as the departure from nominal of the mid-point amplitude of the synchronizing pulses at the output.

B.3.4.2.2	Transient distortion

If the average picture level of the video signal is stepped from a low value to a high value, or from a high value to a low value, the transient non-linear distortion is defined as the maximum instantaneous departure from the nominal value of the mid-point amplitude of the synchronizing pulses at the output.

B.3.5	Linear distortion

Linear distortions are those which can be caused by linear networks. Such distortions do not depend on the average picture level, or the amplitude, or the position of the test signals.

In the case of networks which are affected by a small amount of non-linearity, measurements can still be carried out. However, as the results can be somewhat affected by the average picture level and the amplitude and position of the test signals, it is good practice, when presenting the results, to specify the measurement conditions.

Linear distortions can be measured either in the time domain or in the frequency domain.

The quantities which can be measured in the two domains may be classified as shown below.

| Linear-distortion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Time domain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Frequency domain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Chrominance-luminance inequalities (§ B.3.5.3) |  |  |  |  |  |  |  |  |  |  | Chrominance waveform distortion (§ B.3.5.2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Gain/frequency characteristics (§ B.3.5.4.1) |  |  |  |  |  |  |  |  |  |  |  |  |  | Group-delay/frequency characteristics (§ B.3.5.4.2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gain inequality (§ B.3.5.3.1) |  |  |  | Delay inequality (§ B.3.5.3.2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Luminance waveform distortion (§ B.3.5.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Long-time waveform distortion (§ B.3.5.1.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Field-time waveform distortion (§ B.3.5.1.2) |  |  |  |  |  |  |  |  |  |  |  |  |  | Line-time waveform distortion (§ B.3.5.1.3) |  |  |  |  |  |  |  |  |  |  |  |  |  | Short-time waveform distortion (§ B.3.5.1.4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

B.3.5.1	Waveform distortion of the luminance signal

The distortion of the video waveform due to a television circuit will in general be represented by a continuous function in the time domain.

In practice, however, the form of the video signal and the effects on a displayed picture are such that the resulting impairments may be classified by considering four different time scales which are comparable to the durations of many fields (long-time waveform distortion), one field (field-time waveform distortion), one line (line-time waveform distortion), and one picture element (short-time waveform distortion).

In considering each of these time scales, therefore, impairments appropriate to the other three are excluded by the measurement method.

B.3.5.1.1	Long-time waveform distortion

If a video test signal, simulating a sudden change from a low average picture level to a high one or a high average picture level to a low one, is applied to the input of a circuit, long-time waveform distortion is present if the blanking level of the output signal does not accurately follow that of the input. This failure may be either in exponential form or, more frequently, in the form of damped very low-frequency oscillations.

B.3.5.1.2	Field-time waveform distortion

If a square-wave signal with a period of the same order as one field and of nominal luminance amplitude is applied to the input of the circuit, the field-time waveform distortion is defined as the change in shape of the square wave at the output. A period at the beginning and end of the square wave, equivalent to the duration of a few lines, is excluded from the measurement.

B.3.5.1.3	Line-time waveform distortion

If a square-wave signal with a period of the same order as one line and of nominal luminance amplitude is applied to the input of the circuit, the line-time waveform distortion is defined as the change in shape of the square wave at the output. A period at the beginning and end of the square wave, equivalent to a few picture elements, is excluded from the measurement.

B.3.5.1.4	Short-time waveform distortion

If a short pulse (or a rapid step-function) of nominal luminance amplitude and defined shape is applied to the input of the circuit, the short-time waveform distortion is defined as the departure of the output pulse (or step) from its original shape. The choice of the half-amplitude duration of the pulse (or the rise-time of the step) will be determined by the nominal cut-off frequency, fc, of the television system. (See Report 624.)

B.3.5.2	Chrominance waveform distortion

If a test signal in the form of an amplitude-modulated sub-carrier is applied to the input of a circuit, chrominance waveform distortion is defined as the change in the shape of the envelope and phase of the modulated sub-carrier of the output test signal.

B.3.5.3	Chrominance-luminance inequalities

B.3.5.3.1	Gain inequality

If a test signal having defined luminance and chrominance components is applied to the input of the circuit, the gain inequality is defined as the change in amplitude of the chrominance component relative to the luminance component between the input and output of the circuit.

B.3.5.3.2	Delay inequality

If a composite signal, consisting of a defined luminance test signal in fixed amplitude and time relationship with a chrominance sub-carrier modulated by the same luminance test signal, is applied to the input of the circuit and if the luminance signal at the output is compared with the modulation envelope of the chrominance signal, then the delay inequality of the circuit is defined as the change in relative timing of corresponding parts of the two waveforms between input and output.

B.3.5.4	Steady state characteristics

B.3.5.4.1	The gain/frequency characteristic of the circuit is defined as the variation in gain between the input and output of the circuit over the frequency band extending from the field repetition frequency to the nominal cut-off frequency of the system, relative to the gain at a suitable reference frequency.

B.3.5.4.2	The group delay/frequency characteristic of the circuit is defined as the variation in group delay between the input and the output of the circuit, over the frequency band extending from the field repetition frequency to the nominal cut-off frequency of the system, relative to the delay at a suitable reference frequency. It is for practical reasons, an approximation to the slope (derivative) of the phase/frequency characteristic of the circuit.

REFERENCES

CCIR Documents

[1970-74]: a. CMTT/188 (Germany (Federal Republic of)); b. CMTT/189 (Germany (Federal Republic of)); c. CMTT/5 (United Kingdom); d. CMTT/21 (USA); e. CMTT/40 (Germany (Federal Republic of)).

PART C  –  MEASUREMENT  METHODS  AND  TEST  SIGNALS

C.1	Introduction

Section numbering in this part is related to the section numbering of Part B.

The test signal elements contained in Annex I may be combined in any suitable way to form test signals. Unless otherwise specified, the average picture level of test signals so obtained should be 50%. It should be noted that some practical circuits require the presence of synchronizing signals for correct functioning.

Test signals can be used either as repetitive signals or, with certain exceptions, as insertion test signals in connection with active lines chosen to give the required average picture level. During programme periods however, due consideration must be given to the effects of the variations of the average picture level upon measurements made with insertion test signals.

For full-field tests, some administrations may wish to use full-field sequences containing the same signals as specified for use as insertion test signals (Recommendation 473). In this case, measurement methods should be those specified in Annex III to Part C of this Recommendation.

The measurements described in § C.3.2 to C.3.5 are valid provided that the insertion gain of the circuit is within the stated requirements.

Circuit non-linearity may introduce into the received signals spectral components which are not present in the original test signals and which are not related to picture impairment. In such cases, it is suggested that a phase-corrected low-pass filter be inserted before the measuring equipment to eliminate spurious out-of-band components. An example of a filter suitable for 625-line measurements is described elsewhere [CCIR, 1970-74a].

C.2	Measurements of equipment and signal characteristics at points of video interconnection

C.2.1	Nominal impedance

At points of video interconnection, the input and output impedance will be specified. The actual impedances will be measured, in terms of departure from the nominal value, by the return loss.

C.2.2	Return loss

Return loss may be measured in the time domain or in the frequency domain. If the return loss to be measured is independent of frequency both methods will yield the same numerical result.

To measure return loss in the time domain, test signal elements A, B1, B2 or B3, and F shall be used. The return loss is the ratio of the incident to the reflected test signal element, both being measured in peak-to-peak terms. The return loss for each of the above four test signal elements shall be equal to, or greater than, the value specified in Part D.

To measure return loss in the frequency domain, any of several well-established methods may be used. The return loss at all frequencies within the nominal bandwidth of the television system shall be equal to, or greater than, the value specified in Part D.

Note – Care should be taken to ensure that any spectral components produced by the test signal source above the nominal cut-off frequency, fc, of the television system are attenuated by at least 40 dB relative to components below fc.

C.2.3	Non-useful d.c. component

A signal consisting of synchronizing pulses and blanking-level is used. The potential of the blanking-level with respect to earth is measured with a d.c.-coupled instrument.

C.2.4	Nominal signal amplitude

The nominal signal amplitude at points of interconnection is specified in Part D. Conformity with this specification should be assessed by measurement of a composite video signal which contains element B2 or B3.

C.3	Measurements of transmission performance

C.3.1	Insertion gain

The signal element used is B3 for 625-line systems and B2 or B3 for 525-line systems. The amplitude L is measured between the centre of the bar (point b2 in Fig. 4) and blanking level (point b1 in Fig. 4). The resulting value of the received signal must remain inside the limits specified in Part D.

FIGURE 4/567-3...[D05]

C.3.2	Noise

C.3.2.1	Continuous random noise

Measuring equipment

In general, measurements should be made with r.m.s.-reading instruments. Depending on the type of instrument to be used, the circuit will carry either no signal or a specified repetitive signal. The latter case may be used if clamping devices have to be activated. For power measurement, the measuring instrument should have, an effective time constant or integrating time of approximately 1 s.

In some cases it may be desirable to precede the noise measuring equipment by a sub-carrier notch filter, so as to eliminate any sub-carrier periodic noise component from the random noise measurement. Consideration must however be given to the effect of such a filter upon the accuracy of measurement.

When the measurements are made by assessing the quasi peak-to-peak amplitude of the noise, administrations are asked to determine the peak factor appropriate for their measuring methods and to express the results in terms of r.m.s. noise amplitude.

Band limiting

The measuring instrument should be preceded by band-limiting filters (see § 1 and 2 of Annex II to Part C of this Recommendation). The lower band limit is such that power-supply hum and microphonic noise are excluded. The upper limit is so selected as to eliminate noise which occurs outside the wanted band of the video signal.

If the circuit carries a signal, band limiting may be necessary, using a 200 kHz high-pass filter, as described in Annex III to Part C of this Recommendation.

Weighting

The measuring instrument should also be preceded by a unified weighting network (see § 3 of Annex II to Part C of this Recommendation).

1) 	Formerly Recommendation ITU-R CMTT.567.