2.	Combined high-pass, low-pass filter (fc  =  10 kHz)

Recommendation J.61	(06/90)

The high-pass section is used in series with the low-pass filter described in § 1 for measuring continuous random noise.

The low-pass section is used to measure power-supply hum.

TABLE  OF  VALUES

| Component | Value | Tolerance |
| --- | --- | --- |
| C1 | 139 000 |  |
| C2 | 196 000 | ± 5% |
| C3 | 335 000 |  |
| C4 | 81 200 |  |
| L1 | 0.757 |  |
| L2 | 3.12 | ± 2% |
| L3 | 1.83 |  |
| L4 | 1.29 |  |

Note 1 – Inductances are given in mH, capacitances in pF.

Note 2 – The Q-factor of each inductor should be equal to, or greater
than, 100 at 10 kHz.

FIGURE 20/567-3...[D22]

3.	Unified weighting network for random noise

3.1	Network configuration

FIGURE 21/567-3...[D23]

3.2	Insertion loss A

dB

at high frequencies: A¥  ®  20 log (1  +  a)

where:

t  =  245 ns; a  =  4.5	(A¥  ®  14.8 dB)

FIGURE 22/567-3...[D24]

3.3	Noise weighting factors in a 5 MHz band

Flat noise: 7.4 dB	Triangular noise: 12.2 dB

4.	Examples of differentiating and shaping network for luminance non-linearity measurement

Note that the networks shown below have equivalent transfer characteristics.

4.1	Non-constant resistance form

FIGURE 23/567-3...[D25]

4.2	Constant resistance form

FIGURE 24/567-3...[D26]

4.3	Step response of staircase differentiating network

FIGURE 25/567-3...[D27]

5.	Thomson filter for use in measurement of line-time waveform distortion

FIGURE 26/567-3...[D28]

TABLE  OF  VALUES

| Component | Value (f¥ = 3.3 MHz) |
| --- | --- |
| C1 C2 C3 C4 C5 L1 L2 L3 L4 | 147.7 4044 141.6 1057 310.5 2.948 0.5752 5.767 5.664 |

Note 1 – f¥ is the frequency of the first zero of the output/input transfer function.

Note 2 – Inductances are given in µH, capacitances in pF.

Note 3 – For further details see MacDiarmid and Phillips, Proc. IEE, Vol. 105B, 440.

ANNEX  III  TO  PART  C

METHODS  OF  MEASUREMENT  USING  INSERTION  TEST  SIGNALS

# 1.	Introduction

The international insertion test signals (ITS) are described in Recommendation 473. They may be used for out-of-service checks to give results that are closely related to the methods given in the body of this Recommendation or for in-service checks.

Such measurements may give results which differ from those obtained with full-field test signals because:

–	the test signal elements may not be identical with those used for full-field tests or their arrangement may be different;

–	the measurement result may depend upon the content of the preceding line(s);

–	the average picture level (APL) depends upon the nature of the programme signal;

–	measurements made on the basis of a single test-line per field may not be fully representative of the performance of any circuit in which half-field-rate dispersal is employed (e.g. in satellite circuits).

To reduce changes in the measurement results due to errors affecting one or several lines it is desirable for the line preceding the test-line(s) to carry a signal having a mean-level of approximately 50%. Such a signal may be a line-bar at 50% of white level or a data signal waveform having a mean-level of approximately 50%.

When measurements with insertion test signals are effected outside programme transmission periods they should be associated with the normal low, mean and high values of average picture level.

Because the methods of measurement specified in Recommendation 569 may in some cases differ from those of this Annex, measurements made by automatic measuring equipment may give results which differ from those obtained normally in accordance with this Annex.

2.	Measurement methods

The references given in brackets below refer to the section numbering of Part C.

2.1	Measurements which are different with insertion test signals

Insertion gain (§ C.3.1) – Luminance bar amplitude

The signal element used is the luminance bar (B2) in line 17.

The amplitude L of the luminance bar is defined as the difference in level between the mid-point of the luminance bar (b2 in Figs. 27 and 28) and a specified reference point (b1 in Figs. 27 and 28).

Continuous random noise (§ C.3.2.1)

Measurements are made using either a specified line, which is effaced at the insertion point (lines 22 and 335 for 625-line signals) or on the top of the luminance bar.

Band limiting and noise weighting are specified in § C.3.2.1. In some cases, the measuring instrument may have to be preceded by alternative band limiting filters, for example, when sampling techniques are used and low frequency energy is transferred into the band to be measured. Another problem is described in [CCIR, 1974-78] which also requires different band limiting filters. In such cases upper limiting shall be done by the filter according to § 1 of Annex II to Part C of this Recommendation. Lower limiting shall be done by a first order 200 kHz high-pass filter with a slope of 20 dB per decade. The lower band limit is such that power supply hum, microphonic noise and pulses of line frequency are excluded. The upper limit is so selected as to eliminate noise which occurs outside the wanted band of the video signal.

Note – Table I of Recommendation 569 shows the effects of using a 200 kHz high-pass filter on the measurement of continuous random noise.

Luminance non-linearity (§ C.3.4.1.1)

§ 3.4.1.1 of Part C applies, except that the test signal element used for 525-line colour transmissions may be D2.

Line-time waveform distortion (§ C.3.5.1.3)

The signal element used is the luminance bar (B2) in line 17.

The magnitude of the bar-top distortion is the maximum departure in the level in the interval between b4 and b3 (Figs. 27 and 28) expressed as a percentage of the bar amplitude.

As shown in Figs. 27 and 28 the first and the last 1 ms are neglected in this measurement.

The magnitude of the base-line distortion is obtained from the difference between the level at the point:

–	400 ns for 625-line systems,

–	500 ns for 525-line systems,

after the half-amplitude point of the trailing edge of the luminance bar and the level at the reference point (b1 in Figs. 27 and 28) expressed as a percentage of the luminance bar amplitude.

The distortion is to be measured after bandwidth limitation as in § C.3.5.1.3.

Recommendation J.61    (06/90)§