2.2	Measurements which are, in principle, the same with insertion test signals

The following measurements take the luminance-bar amplitude, as defined in § 2.1 above, as the reference level:

–	chrominance-luminance intermodulation (§ C.3.4.1.4);

–	short-time waveform distortion. (The reference point for the pulse amplitude is also b1 in Figs. 27 and 28) (§ C.3.5.1.4);

–	chrominance-luminance gain inequality (§ C.3.5.3.1).

2.3	Measurements which are identical with insertion test signals

–	return loss (§ C.2.2);

–	cross-talk from another television channel (§ C.3.3);

–	chrominance non-linearity (§ C.3.4.1.2);

–	differential gain and differential phase (§ C.3.4.1.3);

–	synchronizing signal amplitude (§ C.3.4.2);

–	chrominance waveform distortion (§ C.3.5.2);

–	chrominance-luminance delay inequality (§ C.3.5.3.2);

–	steady-state gain/frequency characteristic (§ C.3.5.4.1).

2.4	Measurements which are impracticable with insertion test signals

–	non-useful d.c. component (§ C.2.3);

–	periodic noise (§ C.3.2.2);

–	impulsive noise (§ C.3.2.3);

–	long-time waveform distortion (§ C.3.5.1.1);

–	field-time waveform distortion (§ C.3.5.1.2);

–	steady state delay/frequency characteristic (§ C.3.5.4.2).

FIGURE 27/567-3...[D29]

FIGURE 28/567-3...[D30]

REFERENCES

CCIR Documents

[1974-78]: CMTT/246 (Germany (Federal Republic of)).

BIBLIOGRAPHY

CCIR Documents

[1974-78]: CMTT/36 (United Kingdom); CMTT/57 (European Broadcasting Union); CMTT/59 (European Broadcasting Union); CMTT/76 (Germany (Federal Republic of)); CMTT/77 (Germany (Federal Republic of)).

ANNEX IV TO PART C

SHORT-TIME  WAVEFORM  DISTORTION  –  THE  K-RATING  METHOD  OF  ASSESSMENT

# 1.	Introduction

This Annex briefly describes the K-rating method of assessment of short-time waveform distortion which provides a compact method of expressing the results of the measurements outlined in § C.3.5.1.4. It is based on a deleted Recommendation (Annex II to Recommendation 451, Geneva, 1974) which in turn was based on published papers by [Lewis, 1954] and [MacDiarmid, 1959]. A more recent method, called S-rating, for assessing measurements of the bar-edge response on 525-line systems in a broadly similar manner, has been described [Siocos and Chouinard, 1979].

The K-rating method, as originally described, was in fact two methods ideally giving the same results:

–	the routine-test method, and

–	the acceptance-test method.

Recommendation J.61    (06/90)§

The routine-test method is based on parameters which can easily be measured on an oscilloscope to give results quickly. The acceptance-test method, based on the response to a T sine-squared pulse, is more rigorous and well suited to the analysis of systems and networks in addition to acceptance tests on hardware. The rating method has been devised so that equal K-values obtained for the various parameters correspond approximately to equal subjective impairments on pictures.

Section 2 shows how the performance objectives and tolerances for short-time waveform distortion can be expressed using the routine-test K-rating method. Section 3, for completeness, outlines how the acceptance-test method could be used.

2.	Routine-test method

For the first two parameters, the response to the 2T sine-squared pulse (B1) and one of the bar elements (B2 or B3) are used. The third parameter is not normally measured on circuits and equipments for the transmission of composite colour signals. It is included here for possible future use on circuits for colour signals in analogue component form. The test-signal element required is a T sine-squared pulse, where T = 1/2Fc (Fc is the nominal bandwidth of the channel-under-test).

2.1	2T pulse response

For a particular value of K(2T), a mask of the type shown in Figs. 29a or 29b is required. The tolerances on the response at the time intervals shown in Fig. 29a correspond to ± 4K at ± 200 ns, ± 2K at ± 400 ns and ± K at ± 800 ns and beyond, with the same values at the greater times in Fig. 29b.

For the masks illustrated in Figs. 29a and 29b:

2.2	2T pulse/bar ratio

The 2T pulse/bar ratio (P/B) is related to K(P/B) by:

2.3	T pulse response

This measurement is not necessary when the circuit has to meet the close tolerances on chrominance-luminance gain and delay inequalities required for composite colour signals. In other cases, the tests using only the 2T pulse leave distortions in the upper half of the transmission band virtually untested, so that a test using the T pulse becomes necessary.

Limits to the response to the T pulse cannot be specified rigidly because the spectrum of the T pulse extends far beyond the nominal upper frequency limit of the circuit, and the response must therefore contain irrelevant information. A partial solution has been found by the insertion of a phase-corrected low-pass filter with a sharp cut-off at the edge of the nominal channel band, between the channel under test and the oscilloscope. The filter is first measured using a local test signal. The pulse-to-bar ratio is then, say y (y will be in the region of 0.82). The channel under test is then connected to the filter and the pulse-to-bar ratio measured. From this, the T-pulse rating is, approximately:

Delay errors near the edge of the channel pass-band can also affect the T-pulse K-rating. An estimate of the effects of such errors can be obtained from the change, caused by the channel, in the first pre- and post-overshoots measured at the filter output. The change in overshoot (normalized to the pulse amplitude) is, approximately, 3K(T).

3.	Acceptance-test method

From the measured T-pulse response and the measured or assumed response of the measuring equipment itself, the “filtered impulse response” is derived and expressed in the form of a normalized time series [Lewis, 1954]. The “main” term of this series represents the ideal or non-distorting part, and the “echo” terms represent the distorting parts. The amplitudes of the echo terms should meet the following four sets of limits giving four values of K.

Let the time series representing the filtered impulse response be:

and assume that this has already been normalized so that B0  =  1; let the serial product of B(rT) and the series [½, 1, ½] be

where:

then:

and:

The series C(rT) represents fairly closely the response to a 2T pulse. K1 is thus approximately equivalent to K(2T) in the routine test method. K2 places limits on the 2T pulse/bar ratio and is approximately equivalent to K(P/B) in the routine-test method. K3 places limits on the pulse/bar ratio of the response to a hypothetical pulse-and-bar test signal in which the pulse is an ideal filtered impulse and is approximately equivalent to K(T) in the routine-test method. K4 places an upper limit on the average amplitude, ignoring signs, of the 16 central echo terms, to protect against rarely-met distortions such as a long train of echoes whose magnitudes are not great enough individually to reach one of the other limits. It has no routine-test equivalent.

REFERENCES

LEWIS, N. W. [1954] Waveform responses of television links. Proc. IEE, Vol. 101, Part III, 258-270.

MACDIARMID, I. F. [1959] Waveform distortion in television links. POOEJ, Vol. 52, 108-114 and 188-195.

SIOCOS, C. A. and CHOUINARD, G. [June, 1979] Subjective impairment units in relation with oscilloscope graticules for evaluating short-time linear waveform distortion of the luminance signal in 525-line television. IEEE Trans. Broadcasting, Vol. BC-25, 2, 63-71.

PART  D  –  DESIGN  OBJECTIVES  AND  TOLERANCES  APPLICABLE
TO  HYPOTHETICAL  REFERENCE  CIRCUITS

# D.1	Introduction

The purpose of this Part is to provide design objectives and tolerances for the transmission performance characteristics described in § 2 and 3 of Part B. The design objectives and tolerances shown are applicable to circuits which are required to transmit 525- and/or 625-line television signals, these being either monochrome signals or colour signals coded in accordance with such systems as NTSC, PAL or SECAM, as described in Report 624. International circuits using equipment designed by the time this Recommendation is adopted may have characteristics different from those in this part.

Section numbering in this part is related to section numbers of Part B. Sections which are not necessarily relevant to monochrome transmission are D.3.4.1.2, D.3.4.1.3, D.3.4.1.4, D.3.5.2 and D.3.5.3.

This Recommendation does not contain any explicit definition of the bandwidth required for the transmission of colour television signals having at various times any of the standards defined in Report 624. As some of these standards require a bandwidth of 6 MHz, this is the only figure that can be considered to be fully satisfactory. However, because this bandwidth would cause considerable difficulty in countries using standards requiring appreciably smaller bandwidths, it is proposed that, for international circuits which may be required to transmit signals on any standard, it will be sufficient to define the performance of these circuits up to a frequency of 5.5 MHz, unless otherwise specified in the manner indicated in § D.3.5.4. It is noted, however, that countries using 6 MHz bandwidth may have to be protected from interference lying in the unspecified frequency band (from 5.5 to 6 MHz), which for them is in-band interference by means of a phase-corrected low-pass filter. An example of a suitable filter is given elsewhere [CCIR, 1970-74a].

D.2	Objectives and tolerances at points of video interconnection

D.2.1	Nominal impedance

At video interconnection points, the input and output impedance (Z0) of each section should be either unbalanced to earth with a nominal value of 75 W resistive, or balanced to earth with a nominal value of 124 W resistive.

D.2.2	Return loss

At video interconnection points, the return loss, relative to Z0, of a measured impedance Z should not be less than 30 dB.

D.2.3	Non-useful d.c. component

At video interconnection points, any non-useful d.c. component should not exceed 2.75 V, when terminated in the nominal impedance, or 5.5 V in an open circuit.

D.2.4	Nominal signal amplitude

The nominal peak-to-peak amplitude of the monochrome video signal (M in Fig. 3) is 1.0 V.

The nominal peak-to-peak amplitude of a composite colour video signal (H in Fig. 3) will depend upon the characteristics of the particular colour television system employed (see the equations given in Item 2.9 of Table II of Report 624), but for circuits required at various times to transmit all systems covered by that Report, a maximum value of 1.25 V should be assumed.

D.3	Transmission performance objectives and tolerances

Tolerances proposed here are expected to apply for most of the time but may be exceeded for part of the time. Further study is required [CCIR, 1970-74b].

It is assumed that any earth stations in the circuit will be operating with a G/T ratio of no less than 40.7 dB and will be transmitting sound on a separate carrier. The tolerances do not necessarily apply to stations operating under different conditions.

D.3.1	Insertion gain

The insertion gain, after initial or routine adjustment, should be 0 ± 0.5 dB.

D.3.1.1	Variations in the insertion gain

Any variation of insertion gain with time should not exceed the following limits:

–	short period variations (e.g. 1 s): ± 0.3 dB;

–	medium period variations (e.g. 1 h): ± 0.5 dB.

D.3.2	Noise

D.3.2.1	Continuous random noise

When the noise is band limited and weighted in accordance with Part C of this Recommendation, the signal-to-weighted-noise ratio should not fall below 53 dB for more than 1% of any month, nor below 45 dB for more than 0.1% of any month.

Note 1 – Where colour television is concerned, noise measurements using the unified network can only be considered as giving a valid indication of the subjective impairment due to the noise in cases where the noise power per unit bandwidth at 5 MHz does not exceed that at 1 MHz by more than about 11 dB. This condition will be met in the majority of cases with existing transmission systems and only the recommended weighting network will be used for operational purposes. For new systems that would not meet this condition, designers should verify by other means that signal-to-weighted-noise ratio is satisfactory and that the recommended weighting network gives satisfactory results (see Recommendation 568).

Note 2 – There may be administrations which need, for national purposes, values of signal-to-noise ratio which differ from 53 dB.

Note 3 – At present satellite circuits may not be able to meet the performance objective for continuous random noise. Currently achievable values of signal-to-noise ratios are given in Report 965.

D.3.2.2	Low-frequency noise

It is not possible to give objectives for low-frequency noise at the present time. A signal-to-noise ratio of 43 dB has been suggested by one administration for System M. Contributions on this parameter are invited from other administrations.

D.3.2.3	Periodic noise

For power supply hum including lower-order harmonics, the signal-to-noise ratio should not be less than 35 dB. For single-frequency noise between 1 kHz and 5.5 MHz, the signal-to-noise ratio should not be less than 55 dB.

Note – Circuits which are only required to transmit 525-line signals need only be tested up to 4.2 MHz.

D.3.2.4	Impulsive noise

For impulsive noise of a sporadic or infrequently-occurring nature, the signal-to-noise ratio should not be less than 25 dB.

D.3.3	Cross-talk from another television channel

If the cross-talk is substantially undistorted, the signal-to-cross-talk ratio should not be less than 58 dB. If the cross-talk is substantially “differentiated” (i.e. cross-talk voltage proportional to frequency), the signal-to-cross-talk ratio should not be less than 50 dB.

D.3.4	Non-linear distortion

In this section the terms “low average picture level” and “high average picture level” are used. For low average picture level, either 10% or 12.5% is acceptable. For high average picture level, either 87.5% or 90% is acceptable.

The figures shown for a sending level of +3 dB are included only as a guide for designers and require further study. The question as to whether +3 dB is the optimum test signal level for use in specifying circuit overload performance also requires further study.

D.3.4.1	Picture signal

D.3.4.1.1	Luminance signal

On circuits designed for colour television transmission, the distortion should not exceed 5% at high or low average picture level. The figure applicable to a test signal sent at 3 dB above normal level is 10% at the same average picture levels. On circuits designed for monochrome television transmission only, these objectives should be 12% and 24% respectively.

D.3.4.1.2	Chrominance signal

For System M, the following figures apply:

Chrominance gain non-linearity. The distortion should not exceed 4% at high or low average picture level. In addition, when the signal is sent at 3 dB above the normal amplitude, the distortion should not exceed 8% at the same average picture levels.

Chrominance phase non-linearity. The distortion should not exceed 4° at high or low average picture level. In addition, when the signal is sent at 3 dB above the normal amplitude, the distortion should not exceed 8° at the same average picture levels.

Figures for other systems require further study.

D.3.4.1.3	Intermodulation from the luminance signal into the chrominance signal

Differential gain

Differential gain should not exceed the following figures at high or low average picture level:

For 3.58 MHz: x or y or x  +  y : 10%

For 4.43 MHz: x or y : 10%; x  +  y : 12%.

The figures applicable to a signal sent at +3 dB are double those shown above.

Differential phase

Differential phase should not exceed the following figures at high or low average picture level:

For 3.58 MHz: x or y or x  +  y : 5°

For 4.43 MHz: x or y : 5°; x  +  y : 6°.

The figures applicable to a signal sent at +3 dB are double those shown above.

D.3.4.1.4	Intermodulation from the chrominance signal into the luminance signal

The distortion should not exceed ± 3% at low or high average picture levels. The figure applicable to a test signal sent at 3 dB above normal amplitude is ± 6% at the same average picture levels.

D.3.4.2	Synchronizing signal

D.3.4.2.1	Steady-state distortion

The distortion should not exceed ± 10% at high or low average picture level. The figure applicable to a test signal sent at 3 dB above normal amplitude is ± 20% at the same average picture levels.

D.3.4.2.2	Transient distortion

It is not possible to give limits for transient distortion at this time (see Report 636).

D.3.5	Linear distortion

D.3.5.1	Waveform distortion of the luminance signal

D.3.5.1.1	Long-time waveform distortion

It is not possible to give limits for long-time waveform distortion at this time (see Report 636).

D.3.5.1.2	Field-time waveform distortion

Field-time waveform distortion should not exceed ± 6%.

Note – This objective applies to circuits which do not contain waveform clamping devices.

D.3.5.1.3	Line-time waveform distortion

Line-time waveform distortion should not exceed ± 3%. This figure applies to a measurement on the top of the bar. A limit for base-line distortion requires further study.

D.3.5.1.4	Short-time waveform distortion

The sine-squared pulse-to-bar ratio should lie within the limits 100 ± 12%, corresponding to K(P/B) = 3%.

The pulse lobes should lie within the limits shown in Fig. 29a for 625-line systems and in Fig. 29b for 525-line systems, corresponding to K(2T) = 3%.

FIGURE 29a/567-3...[D31]

FIGURE 29b/567-3...[D32]