|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	J.75

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEVISION  AND  SOUND  TRANSMISSION

INTERCONNECTION  OF  SYSTEMS  FOR  TELEVISION  TRANSMISSION  ON  COAXIAL  PAIRS  AND  ON  RADIO - RELAY  LINKS

ITU-T  Recommendation  J.75

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation J.75 was published in Fascicle III.6 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation J.75

Fascicle III.6 - Rec. J.75

INTERCONNECTION OF SYSTEMS FOR TELEVISION TRANSMISSION ON

COAXIAL PAIRS AND ON RADIO-RELAY LINKS

1	Television transmission only

Direct video transmission over long, e.g. more than about 15 km, coaxial cables is unsatisfactory, because of the likelihood of picking up interference and the difficulties of low-frequency equalization; it is therefore necessary to transmit the television signal as a modulated carrier transmission, usually with a vestigial sideband.

On the other hand, the television signal can be transmitted directly in the baseband of a radio-relay system as a video signal. In general it is advantageous to do so, since this minimizes distortion and enables a better signal-to-noise ratio to be obtained as compared with a modulated signal with vestigial sideband, transmitted in the baseband. This procedure is recommended by the CCIR.

Interconnection between television channels on radio-relay and cable systems will therefore normally take place at video frequencies.

Levels and impedances at interconnection points should then conform to Recommendation J.61.

Exceptionally, in special cases, the video signal can be transmitted over short cables, or a vestigial-side-band television signal can be transmitted on short radio-relay links, to allow direct interconnection at line frequencies (radio-relay link baseband). Special arrangements may be necessary in such cases in respect of signal level, pre-emphasis and pilots, to maintain the recommended standard of transmission performance.

2	Telephony and television transmission, alternatively or simultaneously, on coaxial pairs or radio-relay links

2.1	Interconnection between a coaxial cable system having alternative transmission of telephony and television and a radio-relay link with the same alternative transmission

It is recommended that the following conditions should be met at the interconnection point:

-	For telephony transmission, the frequency arrangements, the relative power levels of the telephone channels and the frequency of the pilots should be as indicated in Recommendation G.423 [1].

-	For television transmission, interconnection should generally be made at video frequencies. Levels and impedances at interconnection points should then conform to Recommendation J.61.

2.2	Interconnection between a coaxial system having simultaneous telephony and television transmission and a radio-relay link with the same simultaneous transmission

On all radio-relay links designed for such simultaneous transmission, it is intended to transmit video-frequency television signals in the lower part of the baseband and telephony signals in the upper part. Since these arrangements are incompatible with those which are recommended by the CCITT for simultaneous telephony and television transmission on coaxial cables (Recommendation J.73), it will normally be possible to consider interconnection at video frequencies only for the television channel, and interconnection at group, supergroup, mastergroup or supermastergroup points for telephony.

However, by agreement between the Administrations concerned, direct interconnection may be achieved, in special cases, on a short system (on cable or radio), by using a frequency allocation recommended for the other type of system.

Reference

[1]	CCITT Recommendation Interconnection at the baseband frequencies of frequency-division multiplex radio-relay systems, Vol. III, Fascicle III.2, Rec. G.423.