|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	M.520

TELECOMMUNICATION
STANDARDIZATION SECTOR
OF  ITU

MAINTENANCE:

INTERNATIONAL  TRANSMISSION  SYSTEMS  (ANALOGUE)

ROUTINE  MAINTENANCE  ON  INTERNATIONAL  GROUP,  
SUPERGROUP,  ETC.,  LINKS

ITU-T  Recommendation  M.520

(Extract from the Blue Book)

NOTES

1	ITU-T Recommendation M.520 was published in Fascicle IV.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation M.520

Fascicle IV.1  –  Rec. M.520

ROUTINE  MAINTENANCE  ON  INTERNATIONAL  GROUP,
SUPERGROUP,  ETC.,  LINKS

1	Type of routine tests

Only measurements of the pilot level are made on international group, supergroup, etc. links. These do not involve other stations. Therefore, Administrations are free to decide on the methods and periodicities. In order to ensure that the performance limits of the links laid down in Recommendation M.160 are met, the following tests are recommended for consideration.

2	xe ""§Links without an automatic regulator

At control stations routine measurements should be made of the pilot level. The periodicity of these routines may be weekly or monthly depending on the complexity of the routing and constitution of the link.

3	xe ""§Links with an automatic regulator

At control stations where a regulator is installed, the level at the input and output of the regulator, if these measurement points are provided by the equipment, may be measured every six months.

4	xe ""§Continuous recording of pilot level

In addition to the above it is useful to be able to take continuous pilot-recordings as required to detect fault conditions which do not trigger the normal alarm systems.