|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	M.830

TELECOMMUNICATION
STANDARDIZATION SECTOR
OF  ITU

MAINTENANCE:

INTERNATIONAL  TELEGRAPH  SYSTEMS
AND  PHOTOTELEGRAPH  TRANSMISSION

ROUTINE  MEASUREMENTS  
TO  BE  MADE  ON  INTERNATIONAL
VOICE-FREQUENCY  TELEGRAPH
LINKS

ITU-T  Recommendation  M.830

(Extract from the Blue Book)

NOTES

1	ITU-T Recommendation M.830 was published in Fascicle IV.2 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation M.830

Fascicle IV.2 - Rec. M.830

ROUTINE  MEASUREMENTS  TO  BE  MADE  ON  INTERNATIONAL
VOICE-FREQUENCY  TELEGRAPH  LINKS

1	The routine maintenance measurementsxe " routine maintenance measurements"§ to be made in the two directions of transmission are measurements of level and overall loss/frequency distortionxe " overall loss/frequency distortion"§ using a measurement signal of –10 dBm0 and noise.

The measuring frequencies are as follows:

–	circuits providing an 18-channel telegraph system: 300, 400, 600, 800, 1020, 1400, 1900, 2400, 2600 Hz;

–	circuits providing a 24-channel telegraph system: 300, 400, 600, 800, 1020, 1400, 1900, 2400, 3000, 3200, 3400 Hz.

2	If the nominal overall loss/frequency distortionxe " overall loss/frequency distortion"§ exceeds the limits given in Recommendation M.810, any faults existing should first be removed, and the link should then be readjusted to within the limits given in Recommendation M.810.

3	Weighted and unweighted noise measurementsxe "Weighted and unweighted noise measurements"§ should be made on the voice-frequency telegraph link at the time of the routine measurements of level as given in Recommendation M.820.