|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	M.1375

TELECOMMUNICATION	(05/96)
STANDARDIZATION  SECTOR
OF  ITU

MAINTENANCE:  INTERNATIONAL  DATA
TRANSMISSION  SYSTEMS

MAINTENANCE  OF  INTERNATIONAL  DATA
TRANSMISSION  SYSTEMS

ITU-T  Recommendation  M.1375

(Previously “CCITT  Recommendation”)

FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation M.1375 was revised by ITU-T Study Group 4 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 12th of May 1996.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation M.1375     (05/96)

Page

1	General		1

1.1	Introduction		1

1.2	Terminology		1

1.3	Performance limits and objectives		1

1.4	Maintenance records		1

2	Fault reporting procedures		1

2.1	Control and sub-control status		1

2.2	Fault information exchange		2

3	Fault localization		2

4	Maintenance tests		2

4.1	General		2

4.2	Maintenance intervention tests		5

4.3	Returning to service tests		5

4.4	Loopback tests		5

5	In-service monitoring		5

6	Temporary service restoration		6

6.1	Mechanisms for implementing temporary service restoration		6

6.2	Time to restore service		6

6.3	Restoration procedure		6

6.4	Returning to normal routing		7

6.5	Information that should be exchanged		7

References		7

Recommendation M.1375

Recommendation M.1375     (05/96)

MAINTENANCE  OF  INTERNATIONAL  DATA  TRANSMISSION  SYSTEMS

(Malaga-Torremolinos, 1984; revised in 1988, 1993 and 1996)

# 1	General

## 1.1	Introduction

The requirements of this Recommendation are intended to ensure effective maintenance of international data transmission systems.

## 1.2	Terminology

Recommendation M.1300 [6] provides general descriptions of international data transmission links and international data transmission systems.

Terminologies and definitions relating to this Recommendation are provided in Recommendation M.60 [1].

## 1.3	Performance limits and objectives

Recommendation M.1340 [7] covers all performance limits and objectives associated with this Recommendation. For all performance tests described in this Recommendation, the Errored Second (ES) and Severely Errored Second (SES) limits should be met simultaneously for the test result to be considered acceptable.

## 1.4	Maintenance records

Administrations should ensure that up-to-date information is held on file to assist with maintenance activities. In particular, Administrations should aim to hold all information highlighted in clause 2/M.1370 [8].

# 2	Fault reporting procedures

## 2.1	Control and sub-control status

At the bringing-into-service stage, Administrations will have agreed a dual control or a control and sub-control relationship for the terminal stations of an international data transmission system (see clause 5/M.1300 [6]).

Where a control and sub-control relationship exists, responsibilities will be as described in Recommendations M.80 [2], M.90 [3], M.1012 [4] and M.1013 [5].

Where a dual control relationship has been established, Administrations should have agreed an effective mechanism for coordinating maintenance activities. Under this arrangement, to avoid confusion, it is essential that the terminal stations inform each other of maintenance activities as rapidly as possible.

## 2.2	Fault information exchange

In general, fault information relating to an international data transmission system can originate from four sources:

–	a customer (due to the failure or poor performance of an international leased circuit);

–	a lower order system1) operator (due to the failure or poor performance of a lower order system that is supported as a channel on this system);

–	the system itself (through alarms or distant operator);

–	a network or higher order system1) operator (with notification of the failure or poor performance of the higher order network that supports this system or recognition of a transmit problem from this system).

Where fault information is received directly from a customer, reference should additionally be made to Recommendation M.1385 [15].

During fault localization and clearance, fault information exchange should be as indicated in Figure 1.

Where a fault is passed from one operator to another, or between Administrations, as much information as possible should be exchanged including:

–	the name, title and contact details for the person reporting the fault;

–	time of fault report, recorded in UCT;

–	designation of the faulty system (see Recommendation M.1400 [9]);

–	symptoms of the fault;

–	the observed duration of the fault prior to a report being made;

–	any associated information that may assist with fault clearance.

When a fault is cleared, appropriate clearance information should be passed to stations that had been involved with initial reporting and localization.

# 3	Fault localization

Figure 1 provides a systematic and coordinated procedure for efficient fault localization.

Initial localization should aim to identify as quickly as possible whether the fault is due to the international data transmission link or the terminal multiplexing equipment2). Subsequent localization should aim to identify the specific link section or equipment that has failed.

Where fault localization is not achieved in a reasonable time, Administrations may invoke an agreed escalation procedure (see Recommendation M.1560 [10]) to assist progress.

# 4	Maintenance tests

## 4.1	General

Existing maintenance records should provide details of test equipment, test points, test patterns and performance limits that are to be used for the maintenance of specific international data transmission systems. Where information is incomplete or out of date, reference should be made to Recommendations M.1340 [7] and M.1370 [8] as appropriate.

FIGURE 1/M.1375...[D01] = PAGE PLEINE

FIGURE 1/M.1375 (fin)...[D02] = PAGE PLEINE

Maintenance tests should be kept as short as possible to avoid significantly extending out-of-service time.

For certain international data transmission system and link configurations, dedicated modems will be provided for fault location and testing purposes.

Where an international data transmission system has yet to be fully utilized, that is, where it has spare capacity, it may be possible to connect test equipment to monitor the performance of one channel. The data from such tests may provide a useful indication of the overall performance of the international data transmission system.

## 4.2	Maintenance intervention tests

Several test measurement configurations are possible, see Figure 1/M.2110 [11] for guidance. Maintenance intervention testing should be of a duration that is appropriate to the nature of fault report that has been received.

Where a report suggests that a system has failed completely, a short test of basic integrity should be performed. Such tests, which are interpreted on a simple pass or fail basis, should normally be limited to a duration of 15 minutes with no Severely Errored Seconds (SES) being observed.

Where a fault report suggests that there is an overall degradation of service, but that the system has not failed completely, longer duration tests (1 hour or 24 hours) should be performed as appropriate. It may be possible for some tests to be performed via protected monitoring points with the system still in service. This may be particularly useful when the fault is believed to be in the data receive direction. Where an out-of-service test is necessary, full account should be taken of the traffic that is being supported. On some occasions it may be appropriate to defer testing until such time as it is most convenient for customers.

## 4.3	Returning to service tests

Returning to service tests should be of a duration that is appropriate to the nature of the fault that has been cleared and should include observation of any network alarm facilities that may be available. Where a fault had caused a general degradation of performance, a returning to service test with a short duration (e.g. 15 minutes) may not be appropriate and a longer term test (e.g. 1 hour or 24 hours) should be used.

## 4.4	Loopback tests

On certain occasions it may be possible and convenient to perform a data transmission performance test by utilizing a loopback. However, it should be appreciated that the results of such tests, are not directly comparable with any previous measurements made in a single direction of transmission.

Care must be taken to avoid the simultaneous operation of loopbacks if the system configuration is such that erroneous results would occur. Once the need for a loopback no longer exists, then care should be taken to ensure that it is removed.

# 5	In-service monitoring

On some international data transmission systems it may be possible to assess overall performance by implementing a cyclic redundancy check within certain nominally spare bits of a frame alignment signal (e.g. CRC4 and CRC6 – see Recommendation G.706 [12]) or by monitoring an additional dedicated service channel.

The criteria for the evaluation of the errored second and severely errored second parameters in relation to in-service performance monitoring are given in Annex B/M.2100.

There is a need for further study of many issues relating to the support of in-service performance monitoring for international data transmission systems. However, degraded and unacceptable performance limit thresholds have been derived and are presented in Recommendation M.1340 (see Tables 4/M.1340 and 5/M.1340). The degraded performance limit threshold relates to a 24-hour monitoring period and the unacceptable threshold to a 15-minute monitoring period.

The action to be taken in the event that a threshold is exceeded remains for further study. However, in general, where the unacceptable performance limit threshold is exceeded, immediate corrective action should be taken (e.g. temporary service restoration). Where the degraded performance limit threshold is exceeded immediate corrective action may not be appropriate. However, the performance of the affected international data transmission system should be reviewed on a regular basis to establish whether a fault exists. See also 2.3/M.2120 for further guidance with respect to performance limit thresholds.

Where an international data transmission link is comprised of a number of discrete monitored link sections, the degraded and unacceptable performance limit thresholds should apply to that part of the international data transmission link that is actually monitored. In this case, the allocation for a particular monitored path element will be less than the allocation for the overall international data transmission link.

# 6	Temporary service restoration

Temporary service restoration may be utilized when a network failure is localized to the international network and this is affecting circuits supported by an international data transmission system. Typical international network failures that may require the use of temporary service restoration include the loss of an undersea cable system or satellite system.

Temporary service restoration should only be used when it is clear that normal international network restoration will not be achieved in a reasonable time (see Annex D/M.1560).

## 6.1	Mechanisms for implementing temporary service restoration

Two basic mechanisms have been identified as follows:

1)	To utilize a dedicated temporary restoration facility

The dedicated restoration facility will typically be an international data transmission system. If both of the following criteria are met, the dedicated temporary restoration facility may be utilized:

a)	an international network failure has occurred and it is observed that normal restoration will not be achieved in a reasonable time;

b)	a dedicated temporary restoration facility is available.

2)	To utilize available temporary restoration capacity

The temporary restoration capacity would typically be found within other international data transmission systems that connect to the same destination, although probably follow a different physical route within the international network. If each of the following criteria are met, the temporary restoration capacity may be utilized:

a)	an international network failure has occurred and it is observed that normal restoration will not be achieved in a reasonable time;

b)	any dedicated temporary restoration facility that may have been provided is not available;

c)	spare restoration capacity (at the channel level) is available;

d)	the Administrations involved have the technical capability and operating procedures to support the temporary switching of channels between international data transmission systems.

## 6.2	Time to restore service

A reasonable temporary service restoration time will be set by the Administrations involved following bilateral agreement. See Recommendation M.1560.

## 6.3	Restoration procedure

The following outline procedure is for guidance only. Detailed operating procedures will need to be agreed between the Administrations involved taking account of the network infrastructure that is available.

When an international data transmission system failure occurs and the basic restoration criteria have been met (see 6.1 above), the administrations involved will confirm that some form of temporary service restoration is to be used and agree how this is to be achieved. Service can be restored using the following procedure:

a)	confirm that the international data transmission link is faulty;

b)	implement restoration switching;

c)	confirm that the affected leased circuits have been successfully restored.

## 6.4	Returning to normal routing

When service can be restored via its normal route, this should be done as planned outage (see Recommendation M.1540 [16]).

## 6.5	Information that should be exchanged

Dependent upon the proposed restoration method that is to be used, Administrations should ensure that suitable preliminary information has been exchanged (e.g. designations and routings for any international data transmission systems that may be used for dedicated service restoration).

## References

[1]	ITU-T Recommendation M.60 (1993), Maintenance terminology and definitions.

[2]	CCITT Recommendation M.80 (1988), Control stations.

[3]	CCITT Recommendation M.90 (1988), Sub-control stations.

[4]	CCITT Recommendation M.1012 (1988), Circuit control station for leased and special circuits.

[5]	CCITT Recommendation M.1013 (1988), Sub-control station for leased and special circuits.

[6]	CCITT Recommendation M.1300 (1992), International data transmission systems operating in the range 2.4 kbit/s to 2048 kbit/s.

[7]	ITU-T Recommendation M.1340 (1996), Performance allocations and limits for international data transmission links and systems.

[8]	ITU-T Recommendation M.1370 (1993), Bringing-into-service of international data transmission systems.

[9]	CCITT Recommendation M.1400 (1992), Designations for international network.

[10]	CCITT Recommendation M.1560 (1992), Escalation procedure for international leased circuits.

[11]	CCITT Recommendation M.2110 (1992), Bringing into service international digital paths, sections and transmission systems.

[12]	CCITT Recommendation G.706 (1991), Frame alignment and Cyclic Redundancy Check (CRC) procedures relating to basic frame structures defined in Recommendation G.704.

[13]	ITU-T Recommendation G.736 (1993), Characteristics of a synchronous digital multiplex equipment operating at 2048 kbit/s.

[14]	CCITT Recommendation X.50 (1993), Fundamental parameters of a multiplexing scheme for the international interface between synchronous data networks.

[15]	ITU-T Recommendation M.1385 (1980), Maintenance of international leased circuits that are supported by international data transmission systems.

[16]	ITU-T Recommendation M.1540 (1994), Exchange of information for planned outages of transmission systems.

1	)	The terminology covering lower order systems and higher order systems relates to the hierarchical transmission data rate of the terminal multiplexing equipments that are interconnected to support service. A typical lower and higher order system relationship would exist where a 64 kbit/s international data transmission system (see Recommendation X.50 [14]) is supported as a channel on a 2048 kbit/s international data transmission system (see Recommendation G.736 [13]).

2)	Multiplexing equipments include digital cross connect equipments as defined in Recommendation M.60 [1].