|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	M.3641

TELECOMMUNICATION	(10/94)
STANDARDIZATION  SECTOR
OF  ITU

MAINTENANCE

INTEGRATED  SERVICES  DIGITAL  NETWORKS

MANAGEMENT  INFORMATION  MODEL  FOR  THE  MANAGEMENT  OF  THE  DATA  LINK  AND  NETWORK  LAYER  OF  THE  ISDN D-CHANNEL

ITU-T  Recommendation  M.3641

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation M.3641 was prepared by ITU-T Study Group 4 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 15th of October 1994.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Overview of the Model		1

2	Managed object classes		4

2.1	Access port ISDN		4

2.2	D Channel		4

2.3	D-channel log		4

2.4	Error frames log record		4

2.5	Invalid frames log record		4

2.6	Managed data link D-channel		4

2.7	Managed data link current data		5

2.8	Managed data link history data		5

2.9	Managed network layer D-channel		5

2.10	Managed network layer current data		5

2.11	Managed network layer history data		5

2.12	Message call reference error log record		6

2.13	Other special message log record		6

2.14	Special message type log record		6

2.15	Timer expiry network log record		6

2.16	Unexpected frames log record		6

3	Packages		6

3.1	Access port ISDN		6

3.2	D-channel		7

3.3	D-channel log		7

3.4	Error frames log record		7

3.5	Invalid frames log record		7

3.6	Invalid frames current data		7

3.7	Invalid frames history data		8

3.8	Invalid frames notification		8

3.9	Message call reference error current data		8

3.10	Message call reference error history data		8

3.11	Message call reference error log record		9

3.12	Message call reference error notification		9

3.13	Managed data link D-channel		9

3.14	Managed data link current data		9

3.15	Managed data link history data		10

3.16	Managed network layer D-channel		10

3.17	Managed network layer current data		10

3.18	Managed network layer history data		11

3.19	Other special message current data		11

3.20	Other special message log record		11

3.21	Other special message notification		12

Page

3.22	Other special message history data		12

3.23	Special message type log record		12

3.24	Timer expiry network log record		12

3.25	Unexpected frames log record		12

4	Name bindings		13

4.1	Access port ISDN – Managed element		13

4.2	D-channel log – Managed element		13

4.3	D-channel – Access port ISDN		13

4.4	Managed data link D-channel – D-channel		13

4.5	Managed network layer D-channel – D-channel		14

4.6	Managed data link current data – Managed data link D-channel		14

4.7	Managed network layer current data – Managed network layer D-channel		14

5	Attributes		15

5.1	Call Reference Flag		15

5.2	Call Reference Value		15

5.3	Cause Value		15

5.4	Data Link State		16

5.5	Error Frames		16

5.6	Frame Type		16

5.7	Invalid Frames		16

5.8	Managed data link D-channel identification		16

5.9	Managed network layer D-channel identification		16

5.10	Message call reference error		17

5.11	Message call reference errors		17

5.12	Messages too short		17

5.13	Messages with general information element missing		17

5.14	Messages with mandatory information element missing		18

5.15	Message type		18

5.16	Message with wrong protocol discriminator		18

5.17	Number of frames received		18

5.18	Number of frames retransmitted		18

5.19	Number of frames transmitted		18

5.20	Number of frames with frame check sequence error		19

5.21	Number of invalid frames		19

5.22	Number of timer 303 expiries		19

5.23	Number of timer 310 expiries		19

5.24	Oldest record age		19

5.25	Other special message		20

5.26	Received		20

5.27	Relating To Active Call		20

5.28	Restart messages received		20

5.29	Restart messages transmitted		20

5.30	Special message type		20

5.31	Status messages received		21

5.32	Status messages transmitted		21

Page

5.33	TEI value		21

5.34	Timer expiry network		21

5.35	Type Of Error Frame		21

5.36	Type Of Timer		21

5.37	Unexpected frames		22

6	Notifications		22

6.1	Error frames		22

6.2	Invalid frames		22

6.3	Message call reference error		23

6.4	Oldest record less than limit		23

6.5	Other special message		23

6.6	Special message type		24

6.7	Timer expiry network		24

6.8	Unexpected frames		24

7	ASN.1 Type definitions module		25

8	Support objects from other ITU-T Recommendations		27

9	Functional unit		27

9.1	Definition of the fpmMonitorDchannel functional unit		27

9.2	Negotiation of the fpmMonitorDchannel functional unit		29

10	Conformance		29

11	References		30

# ABSTRACT

This Recommendation provides the management information model of the Q3-interface in a TMN [8] for the management of the D-channels in an ISDN switch. In particular, it focuses on the fault and performance management of the data link and network layer of ISDN D-channels. Managed objects are specified by using the OSI-management templates of Recommendation X.722 [5]. These specifications are based on the prosaic description of the D-channel management aspects as provided by Recommendation M.3640.

# KEYWORDS

Fault Management, Management of ISDN D-channel, Performance Management.

Recommendation M.3641

Recommendation M.3641     (10/94)

management  information  model  for  the  management  of  the  data  link  and  network  layer  of  the  isdn  d-channel

(Geneva, 1994)

# 1	Overview of the Model

This clause provides a conceptual framework for understanding how the various managed object classes are used to provide the performance monitoring services described in this Recommendation. The managed objects defined in this Recommendation are derived from the managed object class definitions in Recommendations Q.822 [2] and M.3100 [6]. They are based on the prosaic description of the D-channel management aspects as provided by Recommendation M.3640 [1] and supersedes the definitions as contained in the appendix of M.3640 [1].

The naming schema for the management of the ISDN D-channel data link and network layer is  shown in Figure 1.

FIGURE 1/M.3641...[D01] = 3 CM  (118%)

The monitored entities are the protocol machine objects mgdDataLinkDChannel for layer 2 and mgdNetwLayerDChannel for layer 3 contained in the D-channel objects.

The inheritance hierarchy for the management of the ISDN D-channel data link and network layer is shown in Figure 2.

FIGURE 2/M.3641...[D02] = 3 CM  (118%)

The Current PM data is collected for the Monitored Objects by instances of mgdDataLinkCurrentData and mgdNetwLayerCurrentData object classes. Instances of the current data object classes are contained by the monitored object. The details on the behaviour of the current data objects regarding the collection and emission of reports are defined in Recommendation Q.822 [2]. mgdDataLink and mgdNetwLayer history data objects, defined as subclasses of historyData in Q.822 [2] are contained in the respective current data objects. The history objects store the information collected during the performance interval.

The specific objects in the model are:

–	accessPortISDN Object – This object reflects the management view of an ISDN access port in a switch and is used for naming the D-channel objects1) .

–	dChannel Object – This object reflects the management view of an ISDN D channel in a switch and is used for naming the mgdDataLinkDChannel and mgdNetwLayerDChannel objects1).

–	dChannelLog Object – This object is the managed object for logging in case the protocol machine has detected one of the following events which are defined in clause 7: an operational state change, the receipt of an invalid, error or unexpected frame, the receipt of a special message or other special message, a message call reference error or a timer expiry.

–	errorFramesLogRecord Object – This object represents the logged information in the dChannelLog object in case of detection of the receipt of error frames.

–	invalidFramesLogRecord Object – This object represents the logged information in the dChannelLog object in case an invalid frame other than a frame with an FCS error was received.

–	messageCallReferenceErrorLogRecord Object – This object represents the logged information in the dChannelLog object in case a message with a call reference error was detected.

–	mgdDataLinkDChannel Object – This object is the managed object for which the performance measurements are being collected. It represents the resource Layer 2 protocol machine.

–	mgdNetwLayerDChannel Object – This object is the managed object for which the performance measurements are being collected. It represents the resource Layer 3 protocol machine.

–	mgdDataLinkCurrentData Object – This object contains the measurements for the layer 2 protocol machine being monitored for a specified time interval (e.g. 15 minutes).

–	mgdNetwLayerCurrentData Object – This object contains the measurements for the layer 3 protocol machine being monitored for a specified time interval (e.g. 15 minutes).

–	mgdDataLinkHistoryData Object – This object will contain a copy of the performance attributes that are present in the mgdDataLinkCurrentData object at the end of the current interval (e.g. 15 minutes). A new instance of this object class is created at the end of each interval.

–	mgdNetwLayerHistoryData Object – This object will contain a copy of the performance attributes that are present in the mgdnetwlayerCurrentData object at the end of the current interval (e.g. 15 minutes). A new instance of this object class is created at the end of each interval.

–	other Special MessageTypeLogRecord Object – This object represents the logged information in the dChannelLog object in case a STATUS or RESTART message was received or transmitted.

–	specialMessageTypeLogRecord Object – This object represents the logged information in the dChannelLog object in case of the receipt or transmission of a DISCONNECT, RELEASE or RELEASE COMPLETE message.

–	timerExpiryNetworkLogRecord Object – This object represents the logged information in the dChannelLog object in case of the expiry of timers T308, T309, T316 or T317.

–	unexpectedFramesLogRecord Object – This object represents the logged information in the dChannelLog object in case of the receipt of unexpected frames.

In addition, the generic thresholdData and scanner [X.738] support objects as defined in Recommendation Q.822 [2] and log and eventLogRecord support objects as defined in Recommendation X.721 [4] are included in the model.

# 2	Managed object classes

## 2.1	Access port ISDN2)

accessPortISDN	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100 : 1992": trailTerminationPointBidirectional; 
	CHARACTERIZED BY	accessPortISDN-package;

REGISTERED AS	{ m3641ManagedObjectClass 1 };

## 2.2	D Channel2)XE "D Channel"§

dChannel	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100 : 1992": connectionTerminationPointBidirectional;

CHARACTERIZED BY	dChannel-package;

REGISTERED AS	{ m3641ManagedObjectClass 2 };

## 2.3	D-channel logXE "D-channel log"§

dChannelLog	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":log;

CHARACTERIZED BY	dChannelLog-package;

REGISTERED AS	{ m3641ManagedObjectClass 3 };

## 2.4	Error frames log recordXE "Error frames log record"§

errorFramesLogRecord	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":eventLogRecord;

CHARACTERIZED BY	errorFramesLogRecord-package;

REGISTERED AS	{ m3641ManagedObjectClass 4 };

## 2.5	Invalid frames log recordXE "Invalid frames log record"§

invalidFramesLogRecord	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":eventLogRecord;

CHARACTERIZED BY	invalidFramesLogRecord-package;

REGISTERED AS	{ m3641ManagedObjectClass 5 };

## 2.6	Managed data link D-channel XE "Managed data link D-channel"§

mgdDataLinkDChannel	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":top;

CHARACTERIZED BY	mgdDataLinkDChannel-package;

CONDITIONAL PACKAGES

invalidFramesNotification-package PRESENT IF

"the events of invalid frame received are to be logged3).";

REGISTERED AS	{ m3641ManagedObjectClass 6 };

## 2.7	Managed data link current dataXE "Managed data link current data"§

mgdDataLinkCurrentData	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation Q.822 : 1993":currentData;

CHARACTERIZED BY	"Recommendation Q.822 : 1993":thresholdPkg,
		mgdDataLinkCurrentData-package;

CONDITIONAL PACKAGES

invalidFramesCurrentData-package PRESENT IF

"the events of invalid frame received are to be counted4).";

REGISTERED AS	{ m3641ManagedObjectClass 7 };

## 2.8	Managed data link history dataXE "Managed data link history data"§

mgdDataLinkHistoryData	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation Q.822 : 1993":historyData;

CHARACTERIZED BY	mgdDataLinkHistoryData-package;

CONDITIONAL PACKAGES

invalidFramesHistoryData-package PRESENT IF

"the events of invalid frame received are to be counted4)" ;

REGISTERED AS	{ m3641ManagedObjectClass 8};

## 2.9	Managed network layer D-channelXE "Managed network layer D-channel"§

mgdNetwLayerDChannel	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":top;

CHARACTERIZED BY	mgdNetwLayerDChannel-package;

CONDITIONAL PACKAGES

otherSpecialMessageNotification-package PRESENT IF

"the events of other special message received are to be logged5).",

messageCallRefErrorNotification-package PRESENT IF

"the events of message Call Reference Error detected are to be logged6).";

REGISTERED AS	{ m3641ManagedObjectClass 9 };

## 2.10	Managed network layer current dataXE "Managed network layer current data"§

mgdNetwLayerCurrentData	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation Q.822 : 1993":currentData;

CHARACTERIZED BY	"Recommendation Q.822 : 1993":thresholdPkg,

mgdNetwLayerCurrentData-package;

CONDITIONAL PACKAGES

otherSpecialMessageCurrentData-package PRESENT IF

"the events of other special message received are to be counted5).",

messageCallRefErrorCurrentData-package PRESENT IF

"the events of message Call Reference Error detected are to be counted6).";

REGISTERED AS	{ m3641ManagedObjectClass 10 };

## 2.11	Managed network layer history dataXE "Managed network layer history data"§

mgdNetwLayerHistoryData	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation Q.822 : 1993":historyData;

CHARACTERIZED BY	mgdNetwLayerHistoryData-package;

CONDITIONAL PACKAGES

otherSpecialMessageHistoryData-package PRESENT IF

"the events of other special message received are to be counted5).",

messageCallRefErrorHistoryData-package PRESENT IF

"the events of message Call Reference Error detected are to be counted7).";

REGISTERED AS	{ m3641ManagedObjectClass 11 };

## 2.12	Message call reference error log recordXE "Message call reference error log record"§

messageCallRefErrorLogRecord	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":eventLogRecord;

CHARACTERIZED BY	messageCallRefErrorLogRecord-package;

REGISTERED AS	{ m3641ManagedObjectClass 12 };

## 2.13	Other special message log recordXE "Other special message log record"§

otherSpecialMessageLogRecord	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":eventLogRecord;

CHARACTERIZED BY	otherSpecialMessageLogRecord-package;

REGISTERED AS	{ m3641ManagedObjectClass 13 };

## 2.14	Special message type log recordXE "Special message type log record"§

specialMessageTypeLogRecord	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":eventLogRecord;

CHARACTERIZED BY	specialMessageTypeLogRecord-package;

REGISTERED AS	{ m3641ManagedObjectClass 14 };

## 2.15	Timer expiry network log recordXE "Timer expiry network log record"§

timerExpiryNetworkLogRecord	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":eventLogRecord;

CHARACTERIZED BY	timerExpiryNetworkLogRecord-package;

REGISTERED AS	{ m3641ManagedObjectClass 15 };

## 2.16	Unexpected frames log recordXE "Unexpected frames log record"§

unexpectedFramesLogRecord	MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation X.721 : 1992":eventLogRecord;

CHARACTERIZED BY	unexpectedFramesLogRecord-package;

REGISTERED AS	{ m3641ManagedObjectClass 16 };

# 3	Packages

## 3.1	Access port ISDN

accessPortISDN-package		PACKAGE

BEHAVIOUR

accessPortISDNBehaviour BEHAVIOUR

DEFINED AS

"This package is contained by the access port ISDN object which reflects the management view of the termination of an ISDN D-channel on a switch."

;;

REGISTERED AS	{ m3641Package 1 };

## 3.2	D-channel

dChannel-package	PACKAGE

BEHAVIOUR

dChannelBehaviour BEHAVIOUR

DEFINED AS

"This package is contained by the D-channel object which reflects the management view of an ISDN D-channel on a switch."

;;

REGISTERED AS	{ m3641Package 2 };

## 3.3	D-channel log

dChannelLog-package	PACKAGE

BEHAVIOUR

dChannelLogBehaviour BEHAVIOUR

DEFINED AS

"This package contains the notification which will be sent by the dChannelLog object instance in case the D-channel Log is full and the oldest record in this log is younger than the value indicated by attribute oldestRecordAge."

;;

ATTRIBUTES

oldestRecordAge	GET-REPLACE;

NOTIFICATIONS

oldestRecordLessThanLimit;

REGISTERED AS	{ m3641Package 3 };

## 3.4	Error frames log record

errorFramesLogRecord-package	PACKAGE

BEHAVIOUR

errorFramesLogRecordBehaviour BEHAVIOUR
		DEFINED AS

"This managed object is used to represent logged information from notifications received from the mgdDataLinkDChannel managed object in case of detection of the receipt of error frames.";;

ATTRIBUTES
		errorFrames	GET;

REGISTERED AS	{ m3641Package 4 };

## 3.5	Invalid frames log record

invalidFramesLogRecord-package	PACKAGE

BEHAVIOUR

invalidFramesLogRecordBehaviour BEHAVIOUR

DEFINED AS

"This managed object is used to represent logged information from notifications received from the mgdDataLinkDChannel managed object in case an invalid frame other than a frame with an FCS error is received.";;

ATTRIBUTES

invalidFrames	GET;

REGISTERED AS	{ m3641Package 5 };

## 3.6	Invalid frames current data

invalidFramesCurrentData-package	PACKAGE

BEHAVIOUR

invalidFramesCurrentDataBehaviour BEHAVIOUR

DEFINED AS

"This package contains the noOfInvalidFrames attribute. When this attribute exceeds a threshold, a qualityofServiceAlarm notification is issued. The threshold corresponding to this attribute is contained in a thresholdData managed object. The mgdDataLinkHistoryData object corresponds to the mgdDataLinkCurrentData object according to the performance management model of Recommendation Q.822 [2].";;

ATTRIBUTES

noOfInvalidFrames	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET;

REGISTERED AS	{ m3641Package 6 };

## 3.7	Invalid frames history data

invalidFramesHistoryData-package	PACKAGE

BEHAVIOUR

invalidFramesHistoryDataBehaviour BEHAVIOUR

DEFINED AS

"This package contains the history data of the noOfInvalidFrames attribute.";;

ATTRIBUTES

noOfInvalidFrames	GET;

REGISTERED AS	{ m3641Package 7 };

## 3.8	Invalid frames notification

invalidFramesNotification-package	PACKAGE

BEHAVIOUR

invalidFramesNotificationBehaviour BEHAVIOUR

DEFINED AS

"This package contains the notification which will be sent in case an invalid frame other than a frame with an FCS error is received.";;

NOTIFICATIONS

invalidFrames;

REGISTERED AS	{ m3641Package 8 };

## 3.9	Message call reference error current data

messageCallRefErrorCurrentData-package	PACKAGE

BEHAVIOUR

messageCallRefErrorCurrentDataBehaviour BEHAVIOUR

DEFINED AS

"This package contains an attribute that counts the number of messages with a call reference error. When this attribute exceeds a threshold, a qualityofServiceAlarm notification is issued. The threshold corresponding to this attribute is contained in a thresholdData managed object. The mgdNetwLayerHistoryData object corresponds to the mgdNetwLayerCurrentData object according to the performance management model of Recommendation Q.822 [2].";;

ATTRIBUTES

messageCallReferenceErrors	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero
	GET;

REGISTERED AS	{ m3641Package 9 };

## 3.10	Message call reference error history data

messageCallRefErrorHistoryData-package	PACKAGE

BEHAVIOUR

messageCallRefErrorHistoryDataBehaviour BEHAVIOUR

DEFINED AS

"This package contains the history data of the messageCallReferenceErrors attribute.";;

ATTRIBUTES

messageCallReferenceErrors	GET;

REGISTERED AS	{ m3641Package 10 };

## 3.11	Message call reference error log record

messageCallRefErrorLogRecord-package	PACKAGE

BEHAVIOUR

messageCallRefErrorLogRecordBehaviour BEHAVIOUR

DEFINED AS

"This managed object is used to represent logged information from notifications received from the mgdNetwLayerDChannel managed object in case a message with a call reference error was detected.";;

ATTRIBUTES

messageCallRefError	GET;

REGISTERED AS	{ m3641Package 11 };

## 3.12	Message call reference error notification

messageCallRefErrorNotification-package	PACKAGE

BEHAVIOUR

messageCallRefErrorNotificationBehaviour BEHAVIOUR

DEFINED AS

"This package contains the notification which will be sent by the mgdNetwLayerDChannel object instance in case a message with a call reference error was detected."

;;

NOTIFICATIONS

messageCallRefError;

REGISTERED AS	{ m3641Package 12 };

## 3.13	Managed data link D-channel

mgdDataLinkDChannel-package	PACKAGE

BEHAVIOUR

mgdDataLinkDChannelBehaviour BEHAVIOUR

DEFINED AS

"This managed object class reflects the management information for the data link layer protocol. The major part of this information is for fault and performance management.";;

ATTRIBUTES

mgdDataLinkDChannelId	GET,

"Recommendation X.721 : 1992":operationalState	GET;

NOTIFICATIONS

errorFrames,

unexpectedFrames,

"Recommendation X.721 : 1992":stateChange;

REGISTERED AS	{ m3641Package 13 };

## 3.14	Managed data link current data

mgdDataLinkCurrentData-package	PACKAGE

BEHAVIOUR

mgdDataLinkCurrentDataBehaviour BEHAVIOUR

DEFINED AS

"When one of the attributes in this package exceeds a threshold, a qualityofServiceAlarm notification is issued. The thresholds corresponding to the attributes are contained in a thresholdData managed object.

The mgdDataLinkHistoryData object is corresponding to the mgdDataLinkCurrentData object according to the performance management model of Recommendation Q.822 [2].";;

ATTRIBUTES

noOfFramesWithFCSERROR	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

noOfFramesReceived	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

noOfFramesTransmitted	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

noOfFramesRetransmitted	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET;

REGISTERED AS	{ m3641Package 14 };

## 3.15	Managed data link history data

mgdDataLinkHistoryData-package	PACKAGE

BEHAVIOUR

mgdDataLinkHistoryDataBehaviour BEHAVIOUR

DEFINED AS

"The managed object, characterized by this package contains the history data of the performance attributes for layer two of an ISDN D-channel.";;

ATTRIBUTES

noOfFramesWithFCSERROR	GET,

noOfFramesReceived	GET,

noOfFramesTransmitted	GET,

noOfFramesRetransmitted	GET;

REGISTERED AS	{ m3641Package15 };

## 3.16	Managed network layer D-channel

mgdNetwLayerDChannel-package	PACKAGE

BEHAVIOUR

mgdNetwLayerDChannelBehaviour BEHAVIOUR

DEFINED AS

"This managed object class represents the management view of the basic call control function in the network layer of the D-channel.";;

ATTRIBUTES

mgdNetwLayerDChannelId	GET,

"Recommendation X.721 : 1992":operationalState	GET;

NOTIFICATIONS

timerExpiryNetwork,

specialMessageType,

"Recommendation X.721 : 1992":stateChange;

REGISTERED AS	{ m3641Package 16 };

## 3.17	Managed network layer current data

mgdNetwLayerCurrentData-package	PACKAGE

BEHAVIOUR

mgdNetwLayerCurrentDataBehaviour BEHAVIOUR

DEFINED AS

"When one of the attributes in this package exceeds a threshold, a qualityofServiceAlarm notification is issued. The thresholds corresponding to the attributes are contained in a thresholdData managed object.

The mgdNetwLayerHistoryData object is corresponding to the mgdNetwLayerCurrentData object according to the performance management model of Recommendation Q.822 [2].";;

ATTRIBUTES

messageWithWrongPdi	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

messagesTooShort	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

messagesWithGenInfoElemMissing	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

messagesWithMandInfoElemMissing	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

noOfT303Expiries	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

noOfT310Expiries	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET;

REGISTERED AS	{ m3641Package 17 };

## 3.18	Managed network layer history data

mgdNetwLayerHistoryData-package	PACKAGE

BEHAVIOUR

mgdNetwLayerHistoryDataBehaviour BEHAVIOUR

DEFINED AS

"The managed object, characterized by this package contains the history data of the attributes it has.";;

ATTRIBUTES

messageWithWrongPdi	GET,

messagesTooShort	GET,

messagesWithGenInfoElemMissing	GET,

messagesWithMandInfoElemMissing	GET,

noOfT303Expiries	GET,

noOfT310Expiries	GET;

REGISTERED AS	{ m3641Package 18 };

## 3.19	Other special message current data

otherSpecialMessageCurrentData-package	PACKAGE

BEHAVIOUR

otherSpecialMessageCurrentDataBehaviour BEHAVIOUR

DEFINED AS

"This package contains attributes that count the receipt or transmission of STATUS and RESTART messages. When one of these attributes exceeds a threshold, a qualityofServiceAlarm notification is issued. The thresholds corresponding to these attributes are contained in a thresholdData managed objet. The mgdNetwLayerHistoryData object is corresponding to the mgdNetwLayerCurrentData object according to the performance management model of Recommendation Q.822 [2].";;

ATTRIBUTES

sTATUSMessagesReceived	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

sTATUS MessagesTransmitted	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

rESTARTMessagesReceived	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET,

rESTARTMessagesTransmitted	REPLACE-WITH-DEFAULT

DEFAULT VALUE ASN1TypeModule.defaultZero

GET;

REGISTERED AS	{ m3641Package 19 };

## 3.20	Other special message log record

otherSpecialMessageLogRecord-package	PACKAGE

BEHAVIOUR

otherSpecialMessageLogRecordBehaviour BEHAVIOUR

DEFINED AS

"This managed object is used to represent logged information from notifications received from the mgdNetwLayerDChannel managed object in case a status or restart message is received or transmitted.";;

ATTRIBUTES

otherSpecialMessage	GET;

REGISTERED AS	{ m3641Package 20 };

## 3.21	Other special message notification

otherSpecialMessageNotification-package	PACKAGE

BEHAVIOUR

otherSpecialMessageNotificationBehaviour BEHAVIOUR

DEFINED AS

"This package contains the notification which will be sent by the mgdNetwLayerDChannel object instance in case a status or restart message is received or transmitted."

;;

NOTIFICATIONS

otherSpecialMessage;

REGISTERED AS	{ m3641Package 21 };

## 3.22	Other special message history data

otherSpecialMessageHistoryData-package	PACKAGE

BEHAVIOUR

otherSpecialMessageHistoryDataBehaviour BEHAVIOUR

DEFINED AS

"This package contains the history data of the attributes it has.";;

ATTRIBUTES

sTATUSMessagesReceived	GET,

sTATUSMessagesTransmitted	GET,

rESTARTMessagesReceived	GET,

rESTARTMessagesTransmitted	GET;

REGISTERED AS	{ m3641Package 22};

## 3.23	Special message type log record

specialMessageTypeLogRecord-package	PACKAGE

BEHAVIOUR

specialMessageTypeLogRecordBehaviour BEHAVIOUR

DEFINED AS

"This managed object is used to represent logged information from notifications received from the mgdNetwLayerDChannel managed object in case of the receipt or transmission of a DISCONNECT, RELEASE or RELEASE COMPLETE message.";;

ATTRIBUTES

specialMessageType	GET;

REGISTERED	 AS	{ m3641Package 23 };

## 3.24	Timer expiry network log record

timerExpiryNetworkLogRecord-package	PACKAGE

BEHAVIOUR

timerExpiryNetworkLogRecordBehaviour BEHAVIOUR

DEFINED AS

"This managed object is used to represent logged information from notifications received from the mgdNetwLayerDChannel managed object in case of detection of the expiry of timers T308, T309, T316 or T317.";;

ATTRIBUTES

timerExpiryNetwork	GET;

REGISTERED AS	{ m3641Package 24 };

## 3.25	Unexpected frames log record

unexpectedFramesLogRecord-package	PACKAGE

BEHAVIOUR

unexpectedFramesLogRecordBehaviour BEHAVIOUR

DEFINED AS

"This managed object is used to represent logged information from notifications received from the mgdDataLinkDChannel managed object in case of receipt of unexpected frames.";;

ATTRIBUTES

unexpectedFrames	GET;

REGISTERED AS	{ m3641Package 25 };

# 4	Name bindings

## 4.1	Access port ISDN – Managed elementXE "Access port ISDN – Managed element"§

accessPortISDN-managedElement	NAME BINDING

SUBORDINATE OBJECT CLASS	accessPortISDN AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS	"Recommendation M.3100 : 1992":managedElement 							AND SUBCLASSES;

WITH ATTRIBUTE	"Recommendation M.3100 : 1992":tTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS	{ m3641NameBinding 1 };

## 4.2	D-channel log – Managed element

dChannelLog-managedElement	NAME BINDING

SUBORDINATE OBJECT CLASS	dChannelLog AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS	"Recommendation M.3100 : 1992":managedElement 
AND SUBCLASSES;

WITH ATTRIBUTE		"Recommendation X.721 : 1992":logId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS	{ m3641NameBinding 2 };

## 4.3	D-channel – Access port ISDNXE "D-channel – Access port ISDN"§

dChannel-accessPortISDN	NAME BINDING

SUBORDINATE OBJECT CLASS	dChannel AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS	accessPortISDN AND SUBCLASSES;

WITH ATTRIBUTE		cTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS	{ m3641NameBinding 3 };

## 4.4	Managed data link D-channel – D-channelXE "Managed data link D-channel – D-channel"§

mgdDataLinkDChannel-dChannel	NAME BINDING

SUBORDINATE OBJECT CLASS	mgdDataLinkDChannel AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS	dChannel AND SUBCLASSES;

WITH ATTRIBUTE	mgdDataLinkDChannelId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS	{ m3641NameBinding 4};

## 4.5	Managed network layer D-channel – D-channelXE "Managed network layer D-channel – D-channel"§

mgdNetwLayerDChannel-dChannel	NAME BINDING

SUBORDINATE OBJECT CLASS		mgdNetwLayerDChannel AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS	dChannel AND SUBCLASSES;

WITH ATTRIBUTE	mgdNetwLayerDChannelId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS	{ m3641NameBinding 5 };

## 4.6	Managed data link current data – Managed data link D-channel

mgdDataLinkCurrentData-mgdDataLinkDChannel	NAME BINDING

SUBORDINATE OBJECT CLASS	mgdDataLinkCurrentData AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS	mgdDataLinkDChannel AND SUBCLASSES;

WITH ATTRIBUTE	"Recommendation X.738 : 1992":scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS	{ m3641NameBinding 6 };

## 4.7	Managed network layer current data – Managed network layer D-channelXE "Managed network layer current data – Managed network layer D-channel"§

mgdNetwLayerCurrentData-mgdNetwLayerDChannel	NAME BINDING

SUBORDINATE OBJECT CLASS	mgdNetwLayerCurrentData AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS	mgdNetwLayerDChannel AND SUBCLASSES;

WITH ATTRIBUTE	"Recommendation X.738 : 1992":scannerId";

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS	{ m3641NameBinding 7 };

The naming of mgdDataLinkHistoryData instances by mgdDataLinkCurrentData instances is specified in Recommendation Q.822 [2] by means of the name binding between currentData and historyData instances. This is also true for naming of mgdNetwLayerHistoryData instances by mgdNetwLayerCurrentData instances. The naming of thresholdData instances by managedElement instances is also specified in Recommendation Q.822 [2].

The naming of eventForwardingDiscriminator instances by managedElement instances is specified in Recommendation M.3100 [6].

The naming of the following generic and the following D-channel specific object instances by dChannelLog instances is specified in Recommendation X.721 [4] by means of the name binding between log and logRecord instances:

–	objectCreationRecord;

–	objectDeletionRecord;

–	attributeValueChangeRecord;

–	stateChangeRecord;

–	alarmRecord;

–	errorFramesLogRecord;

–	invalidFramesLogRecord;

–	messageCallRefErrorLogRecord;

–	otherSpecialMessageLogRecord;

–	specialMessageTypeLogRecord;

–	timerExpiryNetworkLogRecord;

–	unexpectedFramesLogRecord.

# 5	Attributes

## 5.1	Call Reference FlagXE "Call Reference Flag"§

callReferenceFlag	ATTRIBUTE

WITH ATTRIBUTE SYNTAX	ASN1TypeModule.CallReferenceFlag;
	MATCHES FOR	Equality;

BEHAVIOUR

callReferenceFlagBehaviour	BEHAVIOUR
	DEFINED AS 
		"This attribute represents the value of the call reference flag.";;

REGISTERED AS	{ m3641Attribute 1 };

## 5.2	Call Reference ValueXE "Call Reference Value"§

callReferenceValue	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.CallReferenceValue;
	MATCHES FOR			Equality;

BEHAVIOUR

callReferenceValueBehaviour	BEHAVIOUR
	DEFINED AS 
		"This attribute represents the incorrect call reference value.";;

REGISTERED AS	{ m3641Attribute 2 };

## 5.3	Cause ValueXE "Cause Value"§

causeValue	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.CauseType;
MATCHES FOR	Equality;

BEHAVIOUR

causeValueBehaviour	BEHAVIOUR
DEFINED AS

"This attribute represents the octets three to five as defined in Recommendation Q.931, which includes the cause value, location and diagnostic field.";;

REGISTERED AS	{ m3641Attribute 3 };

## 5.4	Data Link StateXE "Data Link State"§

dataLinkState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.DataLinkState;

MATCHES FOR	Equality;

BEHAVIOUR

dataLinkStateBehaviour	BEHAVIOUR

DEFINED AS

"This attribute represents the state of the data link.";;

REGISTERED AS	{ m3641Attribute 4 };

## 5.5	Error FramesXE "Error Frames"§

errorFrames	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.ErrorFrames;

MATCHES FOR	Equality;

BEHAVIOUR

errorFramesAttrBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the specific error information that is stored in the errorFramesLogRecord.";;

REGISTERED AS	{ m3641Attribute 5 };

## 5.6	Frame TypeXE "Frame Type"§

frameType	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.FrameType;

MATCHES FOR	Equality;

BEHAVIOUR

frameTypeBehaviour	BEHAVIOUR

DEFINED AS

"This attribute represents the type of the unexpected frame.";;

REGISTERED AS	{ m3641Attribute 6 };

## 5.7	Invalid FramesXE "Invalid Frames"§

invalidFrames 	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.InvalidFrames;

MATCHES FOR	Equality;

BEHAVIOUR

invalidFramesAttrBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the specific error information that is stored in the invalidFramesLogRecord.";;

REGISTERED AS	{ m3641Attribute 7 };

## 5.8	Managed data link D-channel identificationXE "Managed data link D-channel identification"§

mgdDataLinkDChannelId	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.NameType;

MATCHES FOR 	Equality, ordering;

BEHAVIOUR

mgdDataLinkDChannelIdBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the identification of the mgdDataLinkDChannel managed object class.";;

REGISTERED AS	{ m3641Attribute 8 };

## 5.9	Managed network layer D-channel identificationXE "Managed network layer D-channel identification"§

mgdNetwLayerDChannelId	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.NameType;

MATCHES FOR	Equality, ORDERING;

BEHAVIOUR

mgdNetwLayerDChannelIdBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the identification of the mgdNetwLayerDChannel managed object class.";;

REGISTERED AS	{ m3641Attribute 9 };

## 5.10	Message call reference errorXE "Message call reference error"§

messageCallRefError	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.MessageCallRefError;

MATCHES FOR	Equality;

BEHAVIOUR

messageCallRefErrorAttrBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the specific error information that is stored in the messageCallRefErrorLogRecord.";;

REGISTERED AS	{ m3641Attribute 10 };

## 5.11	Message call reference errors

messageCallReferenceErrors	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

messageCallReferenceErrorsBehaviour	BEHAVIOUR

DEFINED AS "A call reference is used to specify a call. This attribute represents the number of messages with call reference errors. In the following situations, the call reference error is not correct:

| Messages | Call reference value |
| --- | --- |

| All messages except: SETUP, STATUS, RESUME | Which is not relating to an active call or a call in progress |
| --- | --- |

| SETUP, RESUME | Which is not relating to an active call or a call in progress and with a call reference flag set to “1” |
| --- | --- |

| SETUP | Which is relating to an active call or a call in progress |
| --- | --- |

| All messages except: RESTART, RESTART ACK, STATUS | Which is the global call reference |
| --- | --- |

";;

REGISTERED AS	{ m3641Attribute 11 };

## 5.12	Messages too shortXE "Messages too short"§

messagesTooShort	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

messagesTooShortBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents a counter for the number of messages that are too short  to contain a complete message type.";;

REGISTERED AS	{ m3641Attribute 12 };

## 5.13	Messages with general information element missingXE "Messages with general information element missing"§

messagesWithGenInfoElemMissing	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

messagesWithGenInfoElemMissingBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents a counter for counting the number of general 	information element errors. The counter is raised when an information element is out of sequence or when a duplicated information element was detected, see 5.8.5/Q.931.";;

REGISTERED AS	{ m3641Attribute 13 };

## 5.14	Messages with mandatory information element missingXE "Messages with mandatory information element missing"§

messagesWithMandInfoElemMissing	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

messagesWithMandInfoElemMissingBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents a counter for counting the number of messages with mandatory information element errors. The counter is raised when a mandatory information element is missing or when a mandatory information element content error was detected, see 5.8.5 and 5.8.6/Q.931.";;

REGISTERED AS	{ m3641Attribute 14 };

## 5.15	Message typeXE "Message type"§

messageType	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.MessageType;

MATCHES FOR	Equality;

BEHAVIOUR

messageTypeBehaviour	BEHAVIOUR

DEFINED AS

"This attribute represents the type of layer 3 message.";;

REGISTERED AS	{ m3641Attribute 15 };

## 5.16	Message with wrong protocol discriminatorXE "Message with wrong protocol discriminator"§

messageWithWrongPdi	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

messageWithWrongPdiBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents a counter. This counter holds the number of messages receved with a protocol discriminator that is not valid, i.e. a protocol discriminator other than "Q.931 user-network call control message". The counter has an associated threshold.";;

REGISTERED AS	{ m3641Attribute 16 };

## 5.17	Number of frames receivedXE "Number of frames received"§

noOfFramesReceived	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

noOfFramesReceivedBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the total number of frames received for a D-channel.";;

REGISTERED AS	{ m3641Attribute 17 };

## 5.18	Number of frames retransmittedXE "Number of frames retransmitted"§

noOfFramesRetransmitted	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

noOfFramesRetransmittedBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the total number of frames retransmitted for a D-channel.";;

REGISTERED AS	{ m3641Attribute 18 };

## 5.19	Number of frames transmittedXE "Number of frames transmitted"§

noOfFramesTransmitted	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

noOfFramesTransmitted	BEHAVIOUR

DEFINED AS "This attribute represents the total number of frames transmitted for a D-channel.";;

REGISTERED AS	{ m3641Attribute 19 };

## 5.20	Number of frames with frame check sequence errorXE "Number of frames with frame check sequence error"§

noOfFramesWithFCSERROR	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

noOfFramesWithFCSERRORBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents a counter for the total number of frames received with FCS-error for a D-channel.";;

REGISTERED AS	{ m3641Attribute 20 };

## 5.21	Number of invalid framesXE "Number of invalid frames"§

noOfInvalidFrames	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

noOfInvalidFramesBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the number of invalid frames received.";;

REGISTERED AS		{ m3641Attribute 21 };

## 5.22	Number of timer 303 expiriesXE "Number of timer 303 expiries"§

noOfT303Expiries	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

noOfT303ExpiriesBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the number of expiries of T303. The counter is raised after the second expiry. This implies that the user did not respond after a SETUP was sent to the user on a point-to-point link.";;

REGISTERED AS	{ m3641Attribute 22 };

## 5.23	Number of timer 310 expiriesXE "Number of timer 310 expiries"§

noOfT310Expiries	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

noOfT310ExpiriesBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the number of expiries of T310. T310 expires after receipt of a call proceeding, no response has reached the network.";;

REGISTERED AS	{ m3641Attribute 23 };

## 5.24	Oldest record ageXE "Oldest record age"§

oldestRecordAge	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.OldestRecordAge

MATCHES FOR	Equality, ORDERING;

BEHAVIOUR

oldestRecordAgeBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the time when the notification oldestRecordLessThanLimit will be emitted by object dChannelLog.";;

REGISTERED AS	{ m3641Attribute 24 };

## 5.25	Other special messageXE "Other special message"§

otherSpecialMessage	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.OtherSpecialMessage;

MATCHES FOR	Equality;

BEHAVIOUR

otherSpecialMessageAttrBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the specific error information that is stored in the otherSpecialMessageLogRecord.";;

REGISTERED AS	{ m3641Attribute 25 };

## 5.26	ReceivedXE "Received"§

received	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.Received;

MATCHES FOR	Equality;

BEHAVIOUR

receivedBehaviour	BEHAVIOUR

DEFINED AS

"This attribute indicates whether a message was received or not, i.e. sent.";;

REGISTERED AS	{ m3641Attribute 26 };

## 5.27	Relating To Active CallXE "Relating To Active Call"§

relatingToActiveCall	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.RelatingToActiveCall;

MATCHES FOR	Equality;

BEHAVIOUR

relatingToActiveCallBehaviour	BEHAVIOUR

DEFINED AS

"This attribute indicates whether a call reference relates to an active call.";;

REGISTERED AS	{ m3641Attribute 27 };

## 5.28	Restart messages receivedXE "Restart messages received"§

rESTARTMessagesReceived	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

rESTARTMessagesReceivedBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the number of RESTART messages 	received.";;

REGISTERED AS	{ m3641Attribute 28 };

## 5.29	Restart messages transmittedXE "Restart messages transmitted"§

rESTARTMessagesTransmitted	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

rESTARTMessagesTransmittedBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the number of RESTART messages transmitted.";;

REGISTERED AS	{ m3641Attribute 29 };

## 5.30	Special message typeXE "Special message type"§

specialMessageType		ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.SpecialMessageType;

MATCHES FOR	Equality;

BEHAVIOUR

specialMessageTypeAttrBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the specific error information that is stored in the specialMessageTypeLogRecord";;

REGISTERED AS	{ m3641Attribute 30 };

## 5.31	Status messages receivedXE "Status messages received"§

sTATUSMessagesReceived	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

sTATUSMessagesReceivedBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the number of STATUS messages received.";;

REGISTERED AS	{ m3641Attribute 31 };

## 5.32	Status messages transmittedXE "Status messages transmitted"§

sTATUSMessagesTransmitted	ATTRIBUTE

DERIVED FROM "Recommendation X.721 : 1992":counter;

BEHAVIOUR

sTATUSMessagesTransmittedBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the number of STATUS messages transmitted.";;

REGISTERED AS	{ m3641Attribute 32 };

## 5.33	TEI valueXE "TEI value"§

tEIValue ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.TEIValue;

MATCHES FOR	Equality;

BEHAVIOUR

tEIValueBehaviour	BEHAVIOUR

DEFINED AS

"This attribute represents the TEI Value of the terminal involved.";;

REGISTERED AS	{ m3641Attribute 33 };

## 5.34	Timer expiry networkXE "Timer expiry network"§

timerExpiryNetwork	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.TimerExpiryNetwork;

MATCHES FOR	Equality;

BEHAVIOUR

timerExpiryNetworkAttrBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the specific error information that is stored in the timerExpiryNetworkLogRecord.";;

REGISTERED AS	{ m3641Attribute 34 };

## 5.35	Type Of Error FrameXE "Type Of Error Frame"§

typeOfErrorFrame	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.TypeOfErrorFrame;

MATCHES FOR	Equality;

BEHAVIOUR

typeOfErrorFrameBehaviour	BEHAVIOUR

DEFINED AS

"This attribute represents the type of error when a frame error was detected.";;

REGISTERED AS	{ m3641Attribute 35 };

## 5.36	Type Of TimerXE "Type Of Timer"§

typeOfTimer	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.TypeOfTimer;

MATCHES FOR	Equality;

BEHAVIOUR

typeOfTimerBehaviour	BEHAVIOUR

DEFINED AS

"This attribute represents the type of the timer expired.";;

REGISTERED AS	{ m3641Attribute 36 };

## 5.37	Unexpected framesXE "Unexpected frames"§

unexpectedFrames	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	ASN1TypeModule.UnexpectedFrames;

MATCHES FOR	Equality;

BEHAVIOUR

unexpectedFramesAttrBehaviour	BEHAVIOUR

DEFINED AS "This attribute represents the specific error information that is stored in the unexpectedFramesLogRecord.";;

REGISTERED AS	{ m3641Attribute 37 };

# 6	Notifications

## 6.1	Error frames

errorFrames	NOTIFICATION

BEHAVIOUR	
		errorFramesBehaviour BEHAVIOUR

DEFINED AS

"This notification is issued when a protocol error is detected. It will contain one of the following messages:

•	The frames received with a control field that is undefined or not implemented: framesWithBadControlfield.

•	The frames received with an information field that is not permitted: framesWithBadInfoField.

•	The supervisory frames with incorrect length: supervisoryFramesWithBadLength.

•	The unnumbered frames with incorrect length: unnumberedFramesBadLength.

•	The frames received with an invalid receive sequence number (i.e. invalid N(R)) framesWithInvalidReceiveSeqNumber.

•	The frames received with an information field that exceeds the maximum established length. A parameter is defined for the maximum established length. The default value is 260 octets: framesWithInfoFieldTooLong.

•	The Frame Reject (FRMR) frames received: fRMRReceived.

•	The Disconnect Mode (DM) frames received in response to a Set Asynchronous Balanced Mode Extended (SABME) frame: dMFramesSentInResponseToSABME.

•	The proper responses (i.e. unnumbered acknowledgement or disconnect mode frames not received to establish or restore the link after N200 SABME frames were sent: notProperResponseAfterN200SABMEs.

•	The Disconnect Mode (DM) frames received in response to a Set Asynchronous Balanced Mode Extended (SABME) frame: dMFramesReceivedInResponseToSABME.

It also contains the TEI value to identify the terminal involved.";;

WITH INFORMATION SYNTAX 	ASN1TypeModule.ErrorFrames

AND ATTRIBUTE IDS

tEIValue			tEIValue,

typeOfErrorFrame	typeOfErrorFrame;

REGISTERED AS { m3641Notification 1 };

## 6.2	Invalid frames

invalidFrames	NOTIFICATION

BEHAVIOUR

invalidFramesBehaviour BEHAVIOUR

DEFINED AS

"This notification is emitted on receipt of an invalid frame. The notification contains the invalid frame. An invalid frame is a frame with any of the following:

•	is not properly bounded by two flags;

•	has fewer than six octets between flags of frames that contain sequence numbers and fewer than five octets between flags of frames that do not contain sequence numbers;

•	does not consist of an integral number of octets prior to zero bit insertion or following zero bit extraction;

•	contains a single address field;

•	contains a service access point identifier, which is not supported by the receiver.

";;

WITH INFORMATION SYNTAX	ASN1TypeModule.InvalidFrames;

REGISTERED AS	{ m3641Notification 2 };

## 6.3	Message call reference error

messageCallRefError	NOTIFICATION

BEHAVIOUR

messageCallRefErrorBehaviour BEHAVIOUR

DEFINED AS

"This notification is issued when a message with a call reference error was detected. A call reference is used to specify a call. In the following situations, the call reference error is not correct:

| Messages | Call reference value |
| --- | --- |

| All messages except: SETUP, STATUS, RESUME | Which is not relating to an active call or a call in progress |
| --- | --- |

| SETUP, RESUME | Which is not relating to an active call or a call in progress and with a call reference flag set to “1” |
| --- | --- |

| SETUP | Which is relating to an active call or a call in progress |
| --- | --- |

| All messages except: RESTART, RESTART ACK, STATUS | Which is the global call reference |
| --- | --- |

It also contains the TEI value to identify the terminal involved.";;

WITH INFORMATION SYNTAX	ASN1TypeModule.MessageCallRefError

AND ATTRIBUTE IDS

messageType	messageType,

relatingToActiveCall	relatingToActiveCall,

callReferenceFlag	callReferenceFlag,

callReferenceValue	callReferenceValue,

tEIValue		tEIValue;

REGISTERED AS	{ m3641Notification 3 };

## 6.4	Oldest record less than limitXE "Oldest record less than limit"§

oldestRecordLessThanLimit	NOTIFICATION

BEHAVIOUR

oldestRecordLessThanLimitBehaviour BEHAVIOUR

DEFINED AS

"This notification is emitted when the oldest record in the full D-Channel Log is less old than the value indicated by attribute oldestRecordAge of object dChannelLog.";;

WITH INFORMATION SYNTAX 	ASN1TypeModule.OldestRecordLessThanLimit;

REGISTERED AS	{ m3641Notification 4 };

## 6.5	Other special message

otherSpecialMessage	NOTIFICATION

BEHAVIOUR

otherSpecialMessageBehaviour BEHAVIOUR

DEFINED AS

"This notification is issued upon the receipt or transmission of a STATUS or RESTART message. The notification contains the information whether the message was received or transmitted and the message concerned. It also contains the TEI value to identify the terminal involved. In case of a STATUS message, it also contains the contents of the cause field. The cause field contains the octets three to five as defined in Recommendation Q.931. This includes the cause value, location and the diagnostic field.";;

WITH INFORMATION SYNTAX	ASN1TypeModule.OtherSpecialMessage

AND ATTRIBUTE IDS

messageType	messageType,

received	received,

tEIValue	tEIValue,

causeValue	causeValue;

REGISTERED AS	{ m3641Notification 5 };

## 6.6	Special message type

specialMessageType		NOTIFICATION

BEHAVIOUR

specialMessageTypeBehaviour BEHAVIOUR

DEFINED AS

"This notification is issued upon the receipt or transmission of a DISCONNECT, RELEASE and RELEASE COMPLETE message. This notification is only issued when the cause value indicates an other than normal event. The notification contains the information whether the message was received or transmitted and the message concerned. It also contains the TEI value to identify the terminal involved. It also contains the contents of the cause field. The cause field contains the octets three to five as defined in Recommendation Q.931. This includes the cause value, location and the diagnostic field.";;

WITH INFORMATION SYNTAX	ASN1TypeModule.SpecialMessageType

AND ATTRIBUTE IDS

messageType	messageType,

received	received,

tEIValue	tEIValue,

causeValue	causeValue;

REGISTERED AS	{ m3641Notification 6 };

## 6.7	Timer expiry networkXE "Timer expiry network"§

timerExpiryNetwork NOTIFICATION

BEHAVIOUR

timerExpiryNetworkBehaviour BEHAVIOUR

DEFINED AS

"This notification is issued upon expiry of timers:

T308:	A DISCONNECT was received from the user. The network answers with a RELEASE. T308 expires when no answer was received from the user after that. After the second expiry the B-channel will be placed in a maintenance condition.

T309:	The data link failure has not been recovered from a data link layer malfunction.

T316:	No RESTART ACKNOWLEDGE was received after a RESTART was transmitted. After "n" expiries the Operations System shall be notified.

T317:	After a RESTART was received, the restart procedure did not work. It also contains the TEI value to identify the terminal involved.";;

WITH INFORMATION SYNTAX	ASN1TypeModule.TimerExpiryNetwork

AND ATTRIBUTE IDS

tEIValue		tEIValue,

typeOfTimer	typeOfTimer;

REGISTERED AS	{ m3641Notification 7 };

## 6.8	Unexpected frames

unexpectedFrames	NOTIFICATION

BEHAVIOUR

unexpectedFramesBehaviour BEHAVIOUR

DEFINED AS

"This notification is emitted on receipt of an unexpected frame. The notification contains the unexpected frame. In Table 9/Q.921, an overview of unexpected frames is given. It also contains the TEI value to identify the terminal involved.";;

WITH INFORMATION SYNTAX 	ASN1TypeModule.UnexpectedFrames

AND ATTRIBUTE IDS

frameType	frameType,

dataLinkState	dataLinkState,

tEIValue	tEIValue;

REGISTERED AS	{ m3641Notification 8 };

# 7	ASN.1 Type definitions module

This clause contains the abstract syntax in ASN.1 [10] for the management information defined in this Recommendation.

ASN1TypeModule { itu(0) recommendation(0) m(13) m3641(3641) informationModel(0) asn1Module(2) asn1TypeModule(0) }

DEFINITIONS IMPLICIT TAGS  ::=

BEGIN

--  EXPORTS everything

IMPORTS

NameType FROM ASN1DefinedTypeModule { itu(0) recommendation(0) m(13) gnm(3100) informationModel(0) asn1Module(2) asn1DefinedTypesModule(0)};

m3641InformationModel OBJECT IDENTIFIER  ::=  { itu(0) recommendation(0) m(13) m3641(3641) informationModel(0) }

m3641StandardSpecificExtension OBJECT IDENTIFIER  ::=  { m3641InformationModel standardSpecificExtension (0) }

m3641ManagedObjectClass OBJECT IDENTIFIER  ::=  { m3641InformationModel managedObjectClass (3) }

m3641Package OBJECT IDENTIFIER  ::=   { m3641InformationModel package(4) }

m3641NameBinding OBJECT IDENTIFIER  ::=  { m3641InformationModel nameBinding (6) }

m3641Attribute OBJECT IDENTIFIER  ::=  { m3641InformationModel attribute (7) }

m3641Notification OBJECT IDENTIFIER  ::=  { m3641InformationModel notification (10) }

defaultZero DefaultZero  ::= 0

CallReferenceFlag  ::=  INTEGER

CallReferenceValue  ::=  INTEGER

CauseType  ::=  OCTETSTRING

--  The causeType contains the octets three to five as defined in Recommendation Q.931.

--  This includes the cause value, location and the diagnostic field.

DataLinkState ::= ENUMERATED {

awaitingEstablishment	(1),

awaitingRelease	(2),

multipleFrameEstablishedMode	(3),

multipleFrameTimeRecoveryMode	(4),

tEIAssigned	(5),

tEIUnassigned	(6) }

DefaultZero  ::=	INTEGER

ErrorFrames  ::=	SEQUENCE {

tEIValue	TEIValue,

typeOfErrorFrame	TypeOfErrorFrame}

FrameType  ::=	ENUMERATED {

dISC	(1),

dM	(2),

fRMR	(3),

i	(4),

rEJ	(5),

rNR	(6),

rR  	(7),

sABME	(8),

uA	(9),

uI	(10),

xID	(11) }

InvalidFrames  ::= BITSTRING

MessageCallRefError  ::=  SEQUENCE {

messageType 	MessageType,

relatingToActiveCall	RelatingToActiveCall,

callReferenceFlag	CallReferenceFlag,

callReferenceValue	CallReferenceValue,

tEIValue	TEIValue }

MessageType  ::=  ENUMERATED {

aLERTING	(1),

cALLpROCEEDING	(2),

cONGESTIONcONTROL	(3),

cONNECT	(4),

cONNECTaCKNOWLEDGE	(5),

dISCONNECT	(6),

fACILITY	(7),

iNFORMATION	(8),

nOTIFY	(9),

pROGRESS	(10),

rELEASE	(11),

rELEASEcOMPLETE	(12),

rESTART	(13),

rESUME	(14),

rESUMEaCKNOWLEDGE	(15),

rESUMErEJECT	(16),

sETUP	(17),

sETUPaCKNOWLEDGE	(18),

sTATUS	(19),

sTATUSeNQUIRY	(20),

sUSPEND	(21),

sUSPENDaCKNOWLEDGE	(22),

sUSPENDrEJECT	(23),

uSERiNFORMATION	(24) }

OtherSpecialMessage  ::=  SEQUENCE {

messageType	MessageType,

received	Received,

tEIValue	TEIValue,

causeValue	CauseType	OPTIONAL }

OldestRecordAge  ::=  GeneralizedTime

OldestRecordLessThanLimit  ::= BOOLEAN

Received  ::=  BOOLEAN

RelatingToActiveCall  ::=  BOOLEAN

SpecialMessageType  ::=  SEQUENCE {

messageType	MessageType,

received	Received,

tEIValue	TEIValue,

causeValue	CauseType }

TEIValue  ::=  INTEGER

TimerExpiryNetwork  ::=  SEQUENCE {

tEIValue	TEIValue,

typeOfTimer	TypeOfTimer }

TypeOfErrorFrame  ::=  ENUMERATED {

framesWithBadInfoField	(1),

framesWithBadControlField	(2),

supervisoryFramesWithBadLength	(3),

unnumberedFramesBadLength	(4),

framesWithInvalidReceiveSeqNumber	(5),

framesWithInfoFieldTooLong	(6),

fRMRReceived	(7),

dMFramesSentInResponseToSABME 	(8),

notProperResponseAfterN200SABMEs	(9),

dMFramesReceivedInResponseToSABME 	(10)}

TypeOfTimer  ::=  ENUMERATED	{

t308	(0),

t309 	(1),

t316 	(2),

t317 	(3) }

UnexpectedFrames  ::=  SEQUENCE {

frameType	FrameType,

dataLinkState	DataLinkState,

tEIValue	TEIValue }

END

# 8	Support objects from other ITU-T Recommendations

The following support object classes (or their subclasses) should be used to support the ISDN D-channel performance and fault management functions specified in this Recommendation:

•	eventForwardingDiscriminator (defined in Recommendation X.721 [4];

•	eventLogRecord (defined in Recommendation X.721 [4]);

•	thresholdData (defined in Recommendation Q.822 [2]);

•	log (defined in Recommendation X.721 [4]).

The following system management functions define management capabilities which are used in the management information model of the data link and network layer:

•	Object management function [11];

•	State management function [12];

•	Alarm reporting function [13];

•	Event report management function [14];

•	Log control function [15];

•	Summarization function [7];

•	Metric Objects and Attributes [16].

# 9	Functional unit

## 9.1	Definition of the fpmMonitorDchannel functional unitXE "fpmMonitorDchannel functional unit"§

One functional unit is defined in this Recommendation for the management of fault and performance monitor object classes:

•	fpmMonitorDchannel functional unit - This functional unit supports the following seven specific Event Reporting Services:

–	Error Frames Reporting ServiceXE "Error Frames Reporting Service"§;

–	Invalid Frames Reporting ServiceXE "Invalid Frames Reporting Service"§;

–	Message with call reference error Reporting ServiceXE "Message with call reference error Reporting Service"§;

–	Special Message Reporting ServiceXE "Special Message Reporting Service"§;

–	Other Special Message Reporting ServiceXE "Other Special Message Reporting Service"§;

–	Timer Expiry Reporting ServiceXE "Timer Expiry Reporting Service"§;

–	Unexpected Frames Reporting ServiceXE "Unexpected Frames Reporting Service"§.

It requires support of the pmMonitor and pmControl functional units defined in Recommendation Q.822 [2].

To provide the mapping between the parameters for the event report (result of the notification) and CMIS M-EVENT-REPORT [3], [9],  the seven specific Event Reporting Services are defined as follows.

The Error, Invalid, Unexpected Frames Reporting Service, Timer Expiry, Message with call reference error, Special Message or Other Special Message Reporting Service allow a managed system to report that a protocol error, timer expiry or special message is detected.

Table 1 contains the parameters for these specific Event Reporting Services. The Event Information parameter will indicate which Event Reporting Service is invoked (in this example the Error Frames Reporting Service).

TABLE  1/M.3641

D-channel specific Event Reporting Service Parameters
in case of errorFrames Event Type

| Parameter Name | REQ/IND | RSP/CNF |
| --- | --- | --- |

| Invoke Identifier | P | P |
| --- | --- | --- |

| Mode | P | – |
| --- | --- | --- |

| Managed Object Class | P | P |
| --- | --- | --- |

| Managed Object Instance | P | P |
| --- | --- | --- |

| Event Type | M | C(=) |
| --- | --- | --- |

| Event Time | P | – |
| --- | --- | --- |

| Event Information |  |  |
| --- | --- | --- |

| ErrorFrames | M | – |
| --- | --- | --- |

| Current Time | – | P |
| --- | --- | --- |

| Event reply | – | C |
| --- | --- | --- |

| Errors | – | P |
| --- | --- | --- |

NOTE – The meaning of the symbols “M”, “C” and “(=)” used in this table are the same as used in the tables for the CMISE description [3]. The symbol “P” means “subject to the constraints imposed on the parameter by Rec. X.710 ¦ ISO/IEC 9595”.

The following parameters are defined for use in the specific Event Reporting Services:

•	Invoke Identifier: see Invoke Identifier [3].

•	Mode: see Mode [3]. The values for this parameter are either confirmed or unconfirmed.

•	Managed Object Class: see Managed Object Class [3]. This parameter shall indicate the mgdDataLinkDChannel or mgdNetwLayerDChannel object class.

•	Managed Object Instance: see Managed Object Instance [3].

•	Event Type: This parameter specifies the type of event being reported and the value shall be as indicated by Table 2.

TABLE  2/M.3641

Event Type values for the D-channel specific  Event Reporting Services

| Event reporting service | Event type value |
| --- | --- |

| Error Frames | errorFrames |
| --- | --- |

| Invalid Frames | invalidFrames |
| --- | --- |

| Unexpected Frames | unexpectedFrames |
| --- | --- |

| Timer Expiry | timerExpiryNetwork |
| --- | --- |

| Message with call reference error | messageCallRefError |
| --- | --- |

| Special Message | specialMessageType |
| --- | --- |

| Other Special Message | otherSpecialMessage |
| --- | --- |

The Event Type parameter may be included in the response if the Event Reply parameter is present in the response.

•	Event Time: see Event Time [3].

•	Event Information: see Event Information [3]. This parameter contains a sub-parameter which depends on the Event Type parameter. The sub-parameter specifies the following information as indicated by Table 3.

•	Current Time: see Current Time [3].

•	Event Reply: see Event Reply [3]. The inclusion of this parameter in the response is conditional upon the successful receipt of the event report in the confirmed mode.

•	Errors: see Errors [3]. This parameter shall be included in a failure confirmation.

TABLE  3/M.3641

Information included in Event Information Subparameters

| Sub-parameter Name | Information |
| --- | --- |

| ErrorFrames | TEI value, type of error frame received |
| --- | --- |

| InvalidFrames | Incorrectness of frame received |
| --- | --- |

| UnexpectedFrames | Type of frame, datalink state, TEI value |
| --- | --- |

| TimerExpiryNetwork | TEI value, type of timer |
| --- | --- |

| MessageCallRefError | Message type, indication whether call was active or not, call reference flag and value, TEI value |
| --- | --- |

| SpecialMessageType | Message type, special message was sent or received, TEI value, cause value |
| --- | --- |

| OtherSpecialMessage | Message type, special message was sent or received, TEI value, cause value |
| --- | --- |

## 9.2	Negotiation of the fpmMonitorDchannel functional unitXE "Negotiation of the fpmMonitorDchannel functional unit"§

This Recommendation assigns the following object identifier value

{ itu(0) recommendation (0) m(13) m3641(3641) functionalUnitPackage(1) } as a value for the ASN.1 type FunctionalUnitPackageID defined in Rec. X.701 | ISO/IEC 10040 [17] to use for negotiation the following functional unit:

0 fpmMonitorDchannel

where the number (0) identifies the bit position in the BITSTRING assigned to the functional unit.

In addition, the functional units identified in Recommendation Q.822 [2] are to be used in order to monitor and control the objects defined in this Recommendation.

The mechanism for negotiating the functional units is described in X.701 | ISO/IEC 10040 [17].

# 10	Conformance

A system claiming conformance to this Recommendation shall comply to the general static and dynamic conformance statements as included in Recommendation Q.822 [2]. In addition,  the system shall comply to the conformance statements of Recommendation Q.822 [2] which address conformance to support managed object definitions. Finally, the system shall comply to the following ISDN D-channel specific conformance statement:

The system shall:

–	support the role of manager or agent or both, with respect to the fpmMonitorDchannel functional unit defined in this Recommendation;

–	For each dChannel managed object it should be possible to instantiate exactly one instance of the mgdDataLinkDChannel and mgdNetwLayerDChannel object.

–	For each mgdDataLinkDChannel and mgdNetwLayerDChannel managed object it should be possible to instantiate at least one instance of the mgdDataLinkCurrentData or mgdNetwLayerCurrentData object.

# 11	References

[1]	CCITT Recommendation M.3640, Management of the D-channel – Data link and network layer.

[2]	ITU-T Recommendation Q.822: COM11\R\R002EV2, Stage 1, stage 2 and stage 3 description for the Q3 interface – Performance management.

[3]	CCITT Recommendation X.710, Common management information service definition for CCITT applications.

[4]	CCITT Recommendation X.721, Definition of management information.

[5]	CCITT Recommendation X.722, Guidelines for the definition of managed objects.

[6] 	CCITT Recommendation M.3100, Generic network information model.

[7] 	ITU-T Recommendation X.738, Summarization function.

[8]	CCITT Recommendation M.3010, Principles for a telecommunication management network.

[9]	CCITT Recommendation X.711, Common management information protocol specification for CCITT applications.

[10]	CCITT Recommendation X.208, Specification of abstract syntax notation one (ASN.1).

[11]	CCITT Recommendation X.730, Object management function.

[12]	CCITT Recommendation X.731, State management function.

[13]	CCITT Recommendation X.733, Alarm reporting function.

[14]	CCITT Recommendation X.734, Event report management function.

[15]	CCITT Recommendation X.735, Log control function.

[16]	ITU-T Recommendation X.739, Metric objects and attributes.

[17]	CCITT Rec. X.701 | ISO/IEC 10040:1992, Information technology – Open Systems Interconnection – Systems management overview.

1	)	The accessPort ISDN and D-channel objects are currently for further study and will be further specified in Q-Series Recommendations.

2	)	The accessPort ISDN and D-channel objects are currently for further study and will be further specified in Q-Series Recommendations.

3	)	This case is one of two alternatives (see Recommendation M.3640): the events of invalid frame received may be counted or logged.

4	)	This case is one of two alternatives (see Recommendation M.3640): the events of invalid frame received may be counted or logged.

5	)	This case is one of two alternatives (see Recommendation M.3640): the events of other special message received may be counted or logged.

6	)	This case is one of two alternatives (see Recommendation M.3640): the events of message Call Reference Error detected may be counted or logged.

7	)	This case is one of two alternatives (see Recommendation M.3640): the events of message Call reference Error detected may be counted or logged.