|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	P.33

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TELEPHONE  TRANSMISSION  QUALITY

SUBSCRIBERS'  LINES  AND  SETS

SUBSCRIBER  TELEPHONE  SETS  CONTAINING  EITHER  LOUDSPEAKING  RECEIVERS  OR  MICROPHONES  ASSOCIATED  WITH  AMPLIFIERS

ITU-T  Recommendation  P.33

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation P.33 was published in Volume V of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation P.33

Volume V - Rec. P.33

SUBSCRIBER TELEPHONE SETS CONTAINING

EITHER LOUDSPEAKING RECEIVERS OR

MICROPHONES ASSOCIATED WITH AMPLIFIERS

(Mar del Plata, 1968; amended at Geneva, 1972 and 1980)

The CCITT,

considering

(a)	that an increasing number of loudspeaker sets is being used in the telephone network,

(b)	the complex nature of factors introduced by this equipment and affecting telephone transmission performance,

(c)	the need to help Administrations to determine the conditions in which the use of such equipment may be authorized in telephone networks,

makes the following recommendation:

(1)	In order to avoid overload of carrier systems, the mean long-term power of speech currents should not exceed the mean absolute power level assumed for system design. In Recommendation G.223 [1] the value adopted for this mean power level is – 15 dBm0 (mean power = 31.6 microwatts). Loudspeaker telephones having a sending sensitivity that complies with Recommendation P.34 can be assumed to fulfil this Recommendation. Furthermore, in order to avoid excessive crosstalk from high-level speech currents and/or inadequate received volume from low-level speech currents, care should be taken to ensure that the variation of speech currents is not substantially greater than that from modern handset telephones.

(2)	Administrations should take the necessary precautions so that the person listening may be able to break the sending circuit if oscillations occur, or provide for suitable methods so that a device controlled by the voice may prevent oscillations.

Reference

[1]	CCITT Recommendation Assumptions for the calculation of noise on hypothetical reference circuits for telephony, Vol. III, Rec. G.223.