|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	P.311

TELECOMMUNICATION	(04/95)
STANDARDIZATION  SECTOR
OF  ITU

TELEPHONE  TRANSMISSION  QUALITY

TRANSMISSION  CHARACTERISTICS
OF  WIDEBAND  HANDSET  TELEPHONES

ITU-T  Recommendation  P.311

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation P.311 was prepared by ITU-T Study Group 12 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 18th of April 1995.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation P.311     (04/95)

Page

1	Scope		1

2	Normative References		1

3	Abbreviations and definitions		1

4	Sending characteristics		2

4.1	Levels		2

4.2	Sensitivity/frequency characteristics		2

4.3	Noise		2

4.4	Distortion		3

4.5	Discrimination against out-of-band input signals		3

5	Receiving characteristics		3

5.1	Levels		3

5.2	Sensitivity/frequency characteristics		4

5.3	Noise		5

5.4	Distortion		5

5.5	Spurious out-of-band receiving signals		5

6	Sidetone characteristics		5

6.1	Talker sidetone		5

6.2	Sidetone distortion		5

7	Echo path loss characteristics		5

7.1	Weighted terminal coupling loss		5

7.2	Stability loss		5

8	Delay		6

Annex A – Objective measurement methods for wideband handset telephones		6

A.1	Introduction		6

A.2	Electrical interface specifications		6

A.2.1	Digital interface		6

A.2.2	Wideband reference codec specification		6

A.2.3	Analogue interface		7

A.2.4	Definition of 0 dBr point		7

A.3	Electro-acoustic measurement considerations		7

A.3.1	Artificial mouths and ears		7

A.3.2	Stimulus signals		7

A.4	Measurement methods		7

A.4.1	Sending sensitivity/frequency response and loudness rating		7

A.4.2	Receiving sensitivity/frequency response and loudness rating		7

A.4.3	Sidetone measurements		8

A.4.4	Terminal coupling loss		8

A.4.5	Stability loss		8

A.4.6	Distortion		8

A.4.7	Out-of-band signals		9

A.4.8	Noise			10

A.4.9	Delay			10

# SUMMARY

This Recommendation provides preliminary audio performance requirements for wideband audio (7 kHz) handset telephones. Associated test methods for verifying wideband audio performance are contained in Annex A.

Requirements and test methods are specified for the major audio transmission parameters affecting wideband audio, including levels, frequency response, noise, distortion, spurious signals, sidetone, echo path and delay. Wideband audio represents a considerable departure from traditional telephony, offering significantly improved quality. However, since this is a new technical area, the requirements in this Recommendation are not yet complete, and studies are ongoing in ITU-T Study Group 12.

Recommendation P.311

Recommendation P.311     (04/95)

Transmission  characteristics  of  wideband  handset  telephones

(Geneva, 1995)

# 1	Scope

This Recommendation provides audio performance requirements and test methods for handset telephones capable of transmitting an audio bandwidth extending beyond the conventional telephony bandwidth of 300 to 3400 Hz, to a bandwidth of approximately 150 to 7000 Hz. Such telephones are known as wideband audio telephones, and will make use of digital encoding schemes such as in Recommendation G.722 [1]. Wideband audio telephones are expected to be used in new services such as high quality audio conferencing, video conferencing and multimedia applications.

The requirements listed in this Recommendation are primarily applicable to telephones using G.722 encoding at 64 kbit/s, but should also be used as the basis of requirements for other wideband audio encoding schemes. This is still under study in ITU-T Study Group 12.

# 2	Normative References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision: all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	CCITT Recommendation G.722 (1988), 7 kHz audio coding within 64 kbit/s.

[2]	ITU-T Recommendation P.31 (1993), Transmission characteristics for digital telephones.

[3]	ITU-T Recommendation P.51 (1993), Artificial mouth.

[4]	ITU-T Recommendation P.57 (1993), Artificial Ears.

[5]	ITU-T Recommendation P.64 (1993), Determination of sensitivity/frequency characteristics of local telephone systems.

[6]	ITU-T Recommendation P.66 (1993), Methods for evaluating the transmission performance of digital telephone sets.

[7]	ITU-T Recommendation P.79 (1993), Calculation of loudness ratings for telephone sets.

[8]	IEC Publication 651, Sound level meters.

[9]	CCITT Recommendation G.122 (1988), Influence of national systems on stability, talker echo and listener echo in international connections.

# 3	Abbreviations and definitions

For the purposes of this Recommendation, the following definitions and abbreviations are used:

acoustic Reference Level ARL: Defines as the acoustic level at MRP which results in a –10 dBm0 output at the digital interface.

DRP	Eardrum Reference Point

ERP	Ear Reference Point

Lmest	Sidetone Path Loss

LRGP	Loudness Rating Guard Ring Position

LSTR	Listener Sidetone Rating

MRP	Mouth Reference Point

RLR	Receiving Loudness Rating

SLR	Sending Loudness Rating

STMR	Sidetone Masking Rating

TCL	Terminal Coupling Loss

TCLw	Weighted Terminal Coupling Loss

# 4	Sending characteristics

## 4.1	Levels

The electro-acoustic gain in the send directionXE "electro-acoustic gain in the send direction"§ should be adjusted in terms of a narrow-band loudness rating calculated according to Recommendation P.79 [7] (calculated over the range 200 to 4000 Hz). When measured in this manner, the provisional Send Loudness Rating (SLR) shall be + 8 dB (consistent with Recommendation P.31 [2]).

NOTE – The overload point for wideband audio is defined as + 9 dBm0.

## 4.2	Sensitivity/frequency characteristics

The sending sensitivity/frequency characteristicXE "sending sensitivity/frequency characteristic"§ from the Mouth Reference Point (MRP) to the digital interface shall fall within a mask which can be drawn between the points given in Table 1, also shown in Figure 1. All sensitivities are dB on an arbitrary scale.

TABLE  1/P.311

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 0100 | 4 | – ¥ |
| 0125 | 4 | –7 |
| 0200 | 4 | – 4 |
| 1000 | 4 | – 4 |
| 5000 | (Note) | – 4 |
| 6300 | 9 | –7 |
| 8000 | 9 | – ¥ |
| NOTE – The limit lies on a straight line drawn between the given values on a logarithmic (frequency) – linear (dB) scale. |  |  |

## 4.3	NoiseXE "Noise"§

With the microphone muted acoustically (equivalent to an ambient noiseXE "ambient noise"§ level of < 30 dBA), the noise produced by the apparatus in the sending direction at the digital interface shall not exceed – 68 dBm0 (A-weighted).

FIGURE 1/P.311...[D01] = 8.5 CM

## 4.4	DistortionXE "Distortion"§

The distortion of the apparatus in the sending direction shall be measured in terms of the total distortion (harmonic and quantizing) arising from the application of 200 Hz, 1 kHz and 6 kHz tones applied separately. The limits shall be as shown in Table 2.

TABLE  2/P.311

| Input level | Signal-to-distortion ratio limit (dB) |  |  |
| --- | --- | --- | --- |
| (dB re ARL) | 200 Hz | 1 kHz | 6 kHz |
| +18 to –20 | 29.0 | 35.0 | 29.0 |
| –30 | 25.0 | 26.5 | 25.0 |
| – 46 | 11.0 | 12.5 | 11.0 |
| NOTE – These limits only apply up to the maximum sound pressure level which can be produced by the artificial mouth (+10 dBPa). |  |  |  |

## 4.5	Discrimination against out-of-band input signalsXE "Discrimination against out-of-band input signals"§

The level of any in-band image frequenciesXE "in-band image frequencies"§ at the output resulting from application of input signals above 8 kHz shall be attenuated by at least 25 dB compared to the output level of a 1 kHz input signal.

# 5	Receiving characteristics

## 5.1	Levels

The electro-acoustic gain in the receive directionXE "electro-acoustic gain in the receive direction"§ should be adjusted in terms of a narrow-band loudness rating calculated according to Recommendation P.79 (calculated over the range 200 to 4000 Hz). When measured this way, the provisional Receive Loudness Rating (RLR) is +7 dB.

This is derived from the RLR of +2 dB specified in Recommendation P.31, with 3 dB loss added to account for the effective loudness gain going from narrow-band to wideband, and a further 2 dB loss to account for the loudness loss introduced by the Type 3.2 artificial ear specified in Recommendation P.57 [4].

It is anticipated that wideband audio telephones will incorporate a receive volume control, and the above requirement is taken to apply at the nominal volume level.

NOTE – The overload point for wideband audio is defined as + 9 dBm0.

## 5.2	Sensitivity/frequency characteristics

The receiving sensitivity/frequency characteristicsXE "receiving sensitivity/frequency characteristics"§ from the digital interface to the ear reference point shall fall within a mask which can be drawn between the points given in Table 3, also shown in Figure 2. All sensitivities are dB on an arbitrary scale.

TABLE  3/P.311

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 0100 | 4 | – ¥ |
| 0160 | 4 | –7 |
| 0200 | 4 | – 4 |
| 1000 | 4 | – 4 |
| 5000 | 4 | – 4 |
| 6300 | 4 | –7 |
| 8000 | 4 | – ¥ |

FIGURE 2/P.311...[D02] = 8.5 CM

## 5.3	Noise

The noise produced by the apparatus in the receiving direction at the nominal receiving level specified in 5.1 shall not exceed –59 dBPa (A).

## 5.4	Distortion

The distortion of the apparatus in the receiving direction shall be measured in terms of the total distortion (harmonic and quantizing) arising from the application of 200 Hz, 1 kHz and 6 kHz tones applied separately. The limits shall be as shown in Table 4.

TABLE  4/P.311

| Receive level | Signal-to-distortion ratio limit (dB) |  |  |
| --- | --- | --- | --- |
| (dBm0) | 200 Hz | 1 kHz | 6 kHz |
| +8 to –30 | 29.0 | 35.0 | 29.0 |
| – 40 | 25.0 | 26.5 | 25.0 |
| –56 | 11.0 | 12.5 | 11.0 |

## 5.5	Spurious out-of-band receiving signalsXE "Spurious out-of-band receiving signals"§

The level of any spurious out-of-band signals arising from application of in-band signals at a level of 0 dBm0 shall be attenuated by the following amounts relative to the output level of a 1 kHz sine wave applied at an input of 0 dBm0:

9 kHz	50 dB

14 kHz and above	60 dB

# 6	Sidetone characteristicsXE "Sidetone characteristics"§

## 6.1	Talker sidetoneXE "Talker sidetone"§

The nominal value of Sidetone Masking Rating (STMR), measured with the receive volume control set to provide an RLR of +7 dB, shall be 13 dB.

## 6.2	Sidetone distortionXE "Sidetone distortion"§

The signal to third harmonic distortion ratio generated by the sidetone path shall be greater than 20 dB.

# 7	Echo path loss characteristicsXE "Echo path loss characteristics"§

## 7.1	Weighted terminal coupling lossXE "Weighted terminal coupling loss"§

With the handset suspended in the free air, the value of TCLw, normalized to SLR + RLR = 15 dB, shall be at least 35 dB.

## 7.2	Stability lossXE "Stability loss"§

With the handset lying on a hard surface with the transducers facing that surface, the attenuation from receive digital input to transmit digital output shall be at least 6 dB at all frequencies in the range 100 Hz to 8 kHz.

# 8	DelayXE "Delay"§

The total audio group delay of the sending and receiving parts shall be less than 7 ms. Note that this value allows for the 4 ms delay inherent in G.722 codec.

Annex  A

Objective measurement methods for wideband handset telephonesXE "Objective measurement methods for wideband handset telephones"§

(This annex forms an integral part of this Recommendation)

## A.1	Introduction

This annex describes methods which may be used to measure the performance of wideband handset telephones, that is, telephones capable of transmitting an audio bandwidth extending beyond the conventional telephony bandwidth of 300 to 3400 Hz, to a bandwidth of approximately 150 to 7000 Hz.

## A.2	Electrical interface specifications

Wideband audio will be implemented by a digital encoding scheme such as Recommendation G.722, and will therefore require a suitable interface for test purposes. In general, there are two approaches for evaluating the transmission performance of a wideband digital telephone, the direct approach and the reference codec approach. The direct approach is in principle the most accurate although the use of the reference codec approach may sometimes be advantageous. Detailed requirements for the direct approach are not yet available, so for the time being the same approach may be followed as for making measurements on narrow-band digital telephones according to Recommendation P.66 [6], see Figure A.1.

FIGURE 3/P.311...[D03] = 5 CM

### A.2.1	Digital interfaceXE "Digital interface"§

The interface of the test equipment connected to the terminal under test shall be capable of providing the signalling and supervision necessary for the terminal to be working in all test modes.

### A.2.2	Wideband reference codec specificationXE "Wideband reference codec specification"§

The reference codec and its audio parts shall comply with Recommendation G.722 [1]. Tests shall be carried out with the codec operating in Mode 1.

### A.2.3	Analogue interfaceXE "Analogue interface"§

Measurements shall be carried out by connecting the measurement instrumentation to the test points A and B of the reference codec (see Figure 2/G.722, ). For compatibility with existing telephone instrumentation, 600 ohm balanced electrical interfaces shall be implemented.

### A.2.4	Definition of 0 dBr pointXE "0 dBr point"§

A/D conversion: A 0 dBm0 signal generated by a 600 ohm source will give the digital sequence whose equivalent analogue level is 9 dB below the maximum full-load capacity of the codec.

D/A conversion: A digital sequence whose equivalent analogue level is 9 dB below the maximum full-load capacity of the codec will generate 0 dBm across a 600 ohm termination.

## A.3	Electro-acoustic measurement considerations

### A.3.1	Artificial mouths and earsXE "Artificial mouths and ears"§

The Mouth Reference Point (MRP) and Ear Reference Point (ERP) used for wideband audio measurements are defined in Annex A/P.64.

The Loudness Rating Guarding Position (LRGP) is defined in Annex C/P.64.

The artificial mouth specified in Recommendation P.51 shall be used for making wideband transmit measurements.

NOTE – If the Brüel and Kjaer type 4227 artificial mouth is used, the rounded face plate is recommended.

For making handset receive measurements, the Type 3.2 artificial ear described in Recommendation P.57 is recommended. The low leakage option should be used.

If the geometry of the handset does not allow the use of the Type 3.2 artificial ear, then the Type 3.3 artificial ear shall be used. The force against the ear shall be as specified in Recommendation P. 57 [4].

Sound pressure levels should be referred to ERP using the correction factors given in Tables 1a/P.57 and 1b/P.57.

### A.3.2	Stimulus signalsXE "Stimulus signals"§

In general, a speech-like stimulus signalXE "speech-like stimulus signal"§ is preferred for testing, but care should be taken that the signal contains sufficient high-frequency components to obtain an adequate signal-to-noise ratio for the measurement. If pink noise is used, it should be band-limited to the range 100 Hz to 8 kHz, using a band-pass filter with at least 24 dB/octave slope in the transition band and a minimum of 25 dB out-of-band attenuation.

An on/off modulation (250 ms “ON” and 150 ms “OFF”) shall be applied if echo control or automatic noise detection mechanisms are involved. If modulated signals are used, excitation levels are referred to the ON component of the signals.

## A.4	Measurement methods

### A.4.1	Sending sensitivity/frequency response and loudness ratingXE "Sending sensitivity/frequency response and loudness rating"§

The handset is mounted in the LRGP. The earpiece is coupled to the artificial ear.

The testing signal shall be applied at the MRP as described in Recommendation P.64 at a sound pressure level of – 4.7 dBPa.

The output signal is measured at the output interface of the reference codec.

The Send Loudness Rating (SLR) is calculated according to clause 3/P.79.

### A.4.2	Receiving sensitivity/frequency response and loudness ratingXE "Receiving sensitivity/frequency response and loudness rating"§

The handset is mounted in the LRGP and the earpiece is coupled to the artificial ear.

A test signal source is connected to the input of the reference codec at a level of – 20 dBm0.

Sound pressure measurements made in the artificial ear shall be referred to the ERP using the correction characteristic specified in Recommendation P.57.

The Receive Loudness Rating (RLR) is calculated according to clause 3/P.79. A correction for ear Leakage (Le) shall NOT be applied.

### A.4.3	Sidetone measurements

For the sidetone tests, the input of the reference codec shall be terminated in 600 ohm.

#### A.4.3.1	Talker sidetone

The handset is mounted in the LRGP and the earpiece is coupled to the artificial ear. The test signal is applied at the MRP at a sound pressure level of – 4.7 dBPa.

Measurements of the sound pressure in the artificial ear are made at 1/3 octave points at the frequencies specified in Table 3/P.79, corrected to sound pressure levels at ERP, and STMR is then calculated according to clause 4 /P.79. No correction should be made for Le.

### A.4.4	Terminal coupling lossXE "Terminal coupling loss"§

The handset is suspended in free air in such a way that the inherent mechanical coupling of the handset is not affected. The test space shall be practically free-field (anechoic) down to a lowest frequency of 100 Hz, and be such that the test handset lies totally within the free-field volume. This is met where the reverberation distance is greater than 50 cm. The ambient noise level shall be less than – 64 dBPa (A).

The attenuation from digital input to digital output is measured at 1/12th octave frequencies form 100 Hz to 8000 Hz. The input signal shall be 0 dBm0. The weighted terminal coupling loss is calculated according to the method in Annex B.4/G.122 [9], (trapezoidal rule) over the frequency range 100 Hz to 8 kHz.

### A.4.5	Stability lossXE "Stability loss"§

With an input signal of 0 dBm0, the attenuation from digital input to digital output is measured at 1/12th octave intervals for frequencies from 100 Hz to 8 kHz under the following conditions:

a)	The handset, with the speech transmission circuit fully active shall be positioned on one inside surface that is part of three perpendicular plane, smooth and hard surfaces. Each surface shall extend 500 mm from the apex of the corner. One surface shall be marked with a diagonal line extending from the corner and with a reference position 250 mm from the corner formed by the three surfaces, as shown in Figure 14/P.66 [6].

b)	The handset shall be positioned on the defined surface as follows:

1)	the mouthpiece and earcap shall face towards the surface,

2)	the handset shall be placed centrally above the diagonal line, with the earcap nearer to the apex of the corner,

3)	the extremity of the handset shall coincide with the perpendicular to the reference point, as shown in Figure 14/P.66 [6].

### A.4.6	Distortion

#### A.4.6.1	Sending

The handset is mounted at LRGP and the earpiece is coupled to the artificial ear. A sinusoidal signal at the measurement frequency is applied at MRP. The level of this signal is adjusted until the output of the terminal is –10 dBm0. This acoustic level is by definition the Acoustic Reference Level (ARL). The test signal is applied at the following levels, with the proviso that the sound pressure level at MRP shall not exceed + 6 dBPa:

– 46, – 40, –35, –30, – 24, – 20, –17, –10, –5, 0, 5, 10, 15, 18 dB relative to ARL.

The ratio of the signal to total distortion power at the output of the reference codec is measured.

#### A.4.6.2	Receiving

The handset is mounted at LRGP and the earpiece is coupled to the artificial ear. A sinusoidal signal at the measurement frequency is applied to the electrical input of the reference codec at the following levels:

–56, –50, – 45, – 40, –34, –30, – 27, – 20, –15, –10, –5, 0, 5, 8 dBm0.

The ratio of signal to total distortion power measured at 1 kHz is incremented by 6 dB to allow for the frequency characteristics of the ear canal transmission.

#### A.4.6.3	Sidetone

The handset is mounted at LRGP and the earpiece is coupled to the artificial ear. A sinusoidal tone of – 4.7 dBPa is applied at the MRP at frequencies of 200 Hz, 315 Hz, 500 Hz, 1000 Hz and 2000 Hz. At each frequency, the third harmonic distortion of the acoustic signal in the artificial ear is measured.

The measured sound pressure levels are algebraically added to the correction factors below (see Table A.1).

TABLE  A.1/P.311

| Frequency (Hz) | Correction factor (dB) |
| --- | --- |
| 0200 | +1 |
| 0315 | +2 |
| 0500 | +3 |
| 1000 | +8 |
| 2000 | –3 |

### A.4.7	Out-of-band signals

#### A.4.7.1	Sending

The artificial mouth is fed by a 1 kHz sine wave. The sound pressure level delivered under free-field conditions at the MRP shall be – 4.7 dBPa.

The handset is mounted at LRGP. The reference level is measured at the output interface of the reference codec.

The handset is then placed in a free field where an acoustic signal at 9 kHz, 10 kHz, 12 kHz, 13 kHz and 15 kHz is generated in turn.

The generated field shall approximate a plane acoustic wave, parallel to the earphone reference plane with a propagation direction towards the microphone of the handset.

The microphone input signal is measured by a calibrated probe microphone (diameter < 3.2 mm) placed near the centre of the acoustic input port of the handset closest to the centre of the artificial mouth opening when the handset is mounted at LRGP.

The level of each image frequency is measured at the output interface of the reference codec.

NOTE – Plane wave propagation conditions are considered to be adequately reproduced for the purposes of this measurement by positioning the acoustic centre of the sound source at least 500 mm from the earphone reference plane and on the perpendicular to the plane through the monitored acoustic input port.

#### A.4.7.2	Receiving

The handset is mounted at LRGP and the earpiece is coupled to the artificial ear. For input signals at frequencies of 200 Hz, 350 Hz, 500 Hz, 1000 Hz, 2000 Hz, 3500 Hz, 5000 Hz, and 7000 Hz, applied at 0 dBm0 at the input port of the reference codec, the level of spurious out-of-band image signals at frequencies up to 16 kHz is measured selectively in the artificial ear and corrected to ERP.

The correction factors for referring the sound pressure level at ERP in the frequency range between 8 kHz and 16 kHz are still under study, but the following values may be used (see Table A.2).

TABLE  A.2/P.311

| Frequency (kHz) | Correction (dB) |
| --- | --- |
| 09.0 | 14.0 |
| 09.5 | 21.0 |
| 10.0 | 18.0 |
| 10.7 | 14.0 |
| 11.3 | 13.0 |
| 12.0 | 11.0 |
| 12.7 | 05.0 |
| 13.5 | 02.0 |
| 14.3 | 04.0 |
| 15.1 | 00.0 |
| 16.0 | –2.0 |

### A.4.8	Noise

#### A.4.8.1	Sending

With the handset mounted at LRGP and the earpiece coupled to the artificial ear in a quiet environment [ambient noise less than –64 dBPa (A)], the noise level at the digital output is measured with apparatus including A-weighting according to IEC Publication 651 [8].

#### A.4.8.2	Receiving

The handset is mounted at LRGP and the earpiece is coupled to the artificial ear. The input port of the reference codec is terminated in 600 ohm. The 1/3 octave noise spectrum is measured in the artificial ear and the correction factors of Table 1a, P.57 [4] are used for referring to ERP. The A-weighted noise level at ERP is then calculated.

### A.4.9	Delay

The handset is mounted at LRGP (see Annex C/P.64 [5]). The earpiece is coupled to the artificial ear. The delay in send and receive directions shall be measured separately from MRP to the digital interface (Ds) and from digital interface to ERP (Dr).

The acoustic input level shall be ARL, as defined in clause 3.

For each of the nominal frequencies (F0) given in Table A.3 in turn, the audio group delay at each value of F0 is derived from the phase measurements at the corresponding frequencies F1 and F2.

TABLE  A.3/P.311

Frequencies for audio group delay measurement

| F0 (Hz) | F1 (Hz) | F2 (Hz) |
| --- | --- | --- |
| 1000 | 0990 | 1010 |
| 6000 | 5990 | 6010 |

The measurement configuration is given in Figure A.2.

FIGURE A.2/P.311...[D04] = 5.5 CM
XE "Configuration for audio group delay measurement"§

For each value of F0, the audio group delay is evaluated according to the following procedure:

1)	output the frequency F1 from the frequency-response analyser;

2)	measure the phase shift in degrees between CH1 and CH2 (P1);

3)	output the frequency F2 from the frequency-response analyser;

4)	measure the phase shift in degrees between CH1 and CH2 (P2);

5)	compute the audio group delay (in milliseconds) from the formula:

D  =

6)	calculate the absolute average of the 2 values.

The measured phases P2 and P1 shall be used as original values. When using this formula a negative audio group delay at individual frequencies is possible. Care shall be taken that this real effect is not confused with measurements effect caused by passing 360°.

The audio group delay introduced by the artificial mouth shall be measured by mounting the artificial ear microphone, or equivalent, at the MRP. The audio group delay of all additional test equipment shall be determined.

The audio group delay is calculated from the formula:

where DE is the audio group delay of the test equipment.