|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.12

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

GENERAL  RECOMMENDATIONS  ON  TELEPHONE  SWITCHING  AND  SIGNALLING

INTERNATIONAL  AUTOMATIC  AND  
SEMI-AUTOMATIC  WORKING

OVERFLOW - ALTERNATIVE  ROUTING - REROUTING - AUTOMATIC  REPEAT  ATTEMPT

ITU-T  Recommendation  Q.12

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.12 was published in Fascicle VI.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.12

Fascicle VI.1 - Rec. Q.12

OVERFLOW - ALTERNATIVE ROUTING - REROUTING -
AUTOMATIC REPEAT ATTEMPT

1	When a call cannot find a free circuit in one group of circuits (first choice), technical arrangements can be made to route the call automatically via another group of circuits (second choice), at the same exchange; this process is called overflow. There may also be overflow, at the same exchange, from a second choice group of circuits to a third choice group of circuits, etc.

2	When the group of circuits over which the overflow traffic is routed involves at least one exchange not involved in the previous choice route, the process is called alternative routing.

3	It should be noted that overflow can occur without alternative routing for cases such as, when there are in one relation two groups of circuits, one group reserved for one-way operation and the other group used for both-way operation. In this case, when all one-way circuits are busy, the call can overflow to the both-way circuit group.

4	When congestion occurs at a transit exchange, arrangements can be made in some signalling systems, at the outgoing international exchange on receipt of a busy-flash signal or a congestion signal sent by the transit exchange, to reroute the call automatically from the outgoing international exchange over another route. This process is called re-routing. The use of rerouting is not envisaged in the International Routing Plan.

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

5	When a difficulty is encountered in the setting-up of a connection - such as double seizure on both-way circuits or error detection - arrangements can be provided to make another attempt to set up the connection for that call from the point where the first attempt took place. This process is called automatic repeat attempt.

An automatic repeat attempt may take place:

-	on the same circuit; or

-	on another circuit of the same group of circuits; or

-	on a circuit in another group of circuits.