|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.28

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

GENERAL  RECOMMENDATIONS  ON  TELEPHONE  SWITCHING  AND  SIGNALLING

INTERNATIONAL  AUTOMATIC  AND  
SEMI-AUTOMATIC  WORKING

DETERMINATION  OF  THE  MOMENT  OF  THE  CALLED  SUBSCRIBER'S  ANSWER  IN  THE  AUTOMATIC  SERVICE

ITU-T  Recommendation  Q.28

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.28 was published in Fascicle VI.1 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.28

Fascicle VI.1 - Rec. Q.28

DETERMINATION OF THE MOMENT OF THE CALLED

SUBSCRIBER'S ANSWER IN THE AUTOMATIC SERVICE

1	Arrangements should be made in the national signalling system of the incoming country to determine (in the outgoing international exchange) the moment when the called subscriber replies; this information is necessary in the international service for the purposes of:

-	charging the calling subscriber [1];

-	measuring the call duration [2].

2	Where subscribers in an outgoing country have direct access to an operator's position (in a manual exchange, for instance) in a public exchange of an incoming country, arrangements should be made in the national network of the incoming country to ensure that - in the outgoing country - the calling subscriber is charged, and the call duration measured, only from the moment when the called subscriber replies. This means that an answer signal is not sent when the operator in a public exchange of the incoming country replies. These provisions are set out in detail for CCITT standardized systems (see Recommendation Q.102).

References

[1]	CCITT Recommendation Chargeable duration of calls, Vol. II, Rec. E.230.

[2]	CCITT Recommendation Basic technical problems concerning the measurement and recording of call durations, Vol. II, Rec. E.260.