|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.123

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No.4

SIGNAL  RECEIVER

ITU-T  Recommendation  Q.123

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.123 was published in Fascicle VI.2 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.123

Fascicle VI.2 - Rec. Q.123

3.2  SIGNAL RECEIVER1)

3.2.1	Operating limits of the signal receiver

The signal receiver shall operate in the conditions specified under 3.2.5 to received signals that meet the following three conditions.

a)	The signal frequencies shall be within the following limits:

"x" frequency: 2040 ± 15 Hz

"y" frequency: 2400 ± 15 Hz.

b)	The absolute power level N of each unmodulated received signal frequency shall be within the limits:

- 18 + n £ N ³ n dBm;

where n is the relative power level at the signal receiver input.

These limits give a margin2) of ± 9 dB on the nominal absolute level of each received signal at the input to the signal receiver.

c)	The absolute level of the two unmodulated signal frequencies may differ from each other, but the received level of the 2400 Hz signal shall not be more than 3 dB above, nor more than 6 dB below the received level of the 2040 Hz signal.

The tolerances given in §§ a, b and c above are to allow for variations at the sending end and for variations in line transmission.

3.2.2	Non-operate conditions for the signal receiver

a)	Selectivity

The signal receiver shall not operate to a signal having an absolute power level at the receiving end within the limits specified in § 3.2.1 when the frequency differs by more than 150 Hz from the nominal value of 2040 Hz or of 2400 Hz.

b)	Maximum sensitivity of the receiver

The signal receiver shall not operate to a signal of 2040 ± 15 Hz or 2400 ± 15 Hz whose absolute power level at the point of connection of the receiver is (-26 - 9 + n) dBm, n being the relative power level at this point.

This limit is 26 decibels below the nominal absolute level of the signal current at the input to the signal receiver.

3.2.3	Efficiency of the guard circuit

The signal receiver must be protected by a guard circuit against false operation due to speech currents, circuit noise or other currents of miscellaneous origin circulating in the line.

The purpose of the guard circuit is to prevent:

a)	signal imitation (signals are imitated if the duration of the resulting direct current pulses at the output of the signal receiver is long enough to be recognized as signals by the switching equipment);

b)	operation of the splitting device from interfering with speech.

To minimize signal imitation by speech currents it is advisable that the guard circuit be tuned.

To minimize signal interference by low frequency noise, it is advisable that the response of the guard circuit falls off towards the lower frequencies and that the sensitivity of the guard circuit at 200 Hz be at least 10 dB less than that at 1000 Hz.

An indication of the efficiency of the guard circuit is given by the following:

a)	during 10 hours of speech, normal speech currents should not, on the average, cause more than one simultaneous operation of the receiver relays for each of the two signalling frequencies lasting more than 55 ms (the minimum recognition time of a compound signal element is 60 ms);

b)	the number of false splits of the speech path caused by speech currents should not cause an appreciable reduction in transmission quality of the circuit.

3.2.4	Guard circuit limits

A. - Steady noise

Considering:

a)	that when there is noise on a circuit an over-sensitive guard circuit might give rise to signalling difficulties and, in particular, inhibit the response of the signal receiver;

b)	that unweighted noise of a level -40 dBm0 (100 000 pW) and uniform spectrum energy may arise during end-to-end signalling over a multilink chain of System No. 4 circuits;

it is recommended that, for either one or two signalling currents (each being within the limits of the level specified in § 3.2.1) the signal receiver should satisfy the conditions indicated in § 3.2.5 for the distortion of signals in the presence of noise of a level of -40 dBm0 and uniform spectrum energy over the frequency range 300 to 3400 Hz.

B. - Surges

A guard circuit with an excessive hangover time may cause difficulties in receiving a signal, for example when it has been immediately preceded by surges, and it is therefore recommended that the following condition should be fulfilled:

If a disturbing current of a frequency corresponding to the maximum sensitivity of the guard circuit and having an absolute power level of (-10 + n) dBm at the relative level point n where the receiver is connected, ceases 30 ms before the application of a signal satisfying the limits defined in § 3.2.1, the lengths of the received signals must remain within the limits specified in § 3.2.5.

3.2.5	Distortion of received signals

When the signal frequencies and levels are within the limits specified in § 2.3.1, the following conditions should be met:

1.	a)	the delay in the start of a received pulse consisting of one of the two signalling frequencies should be less than 20 ms;

b)	the delay in reproducing the beginning of a signal consisting of a combination of the two frequencies x and y (compound signal) should be less than 20 ms; this delay is defined as the interval between the moment when the beginning of the compound signal arrives at the signal receiver input and the moment of the beginning the reproduction of the two frequencies x and y as a direct current signal output of the signal receiver;

2.	the change of signal length in the presence of the noise defined in § 2.3.4 should be less than:

a)	5 ms when the signal receiver receives an isolated pulse at one frequency only, with a minimum duration of 25 ms;

b)	8 ms when the signal receiver receives a compound pulse of the two frequencies with a minimum duration of 50 ms; this change is defined as the difference between the simultaneous reception of the two received frequencies at the input of the receiver and the simultaneous reproduction of the two components as a direct current signal at the output of the signal receiver;

c)	6 ms when the signal receiver receives a pulse of current of a single frequency with a minimum duration of 80 ms, preceded by a compound signal element (separated or not by an interval of silence of 5 ms maximum). Consequently the change in the duration of a signal suffix3) , measured from the moment when the prefix 3) signal mentioned under b), will be less than 6 + 8 = 14 ms.

1	)	See also Recommendation Q.112.

2	)	See 2.1.2 of Recommendation Q.112.

3	)	See the definition of prefix and suffix signals under § 2.3.1 of Recommendation Q.121.