|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.136

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No. 4

LOOP  TRANSMISSION  MEASUREMENTS

ITU-T  Recommendation  Q.136

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.136 was published in Fascicle VI.2 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.136

Fascicle VI.2 - Rec. Q.136

5.4  LOOP TRANSMISSION MEASUREMENTS

A permanent loop will be connected between the GO and RETURN paths of an international circuit at its incoming end when the circuit is free, so that transmission tests can be made independently of the signalling conditions.

The loop between the GO and RETURN paths shall be connected in such a manner that the level diagrams of each of the two paths will be respected when the circuit is free (loop established); the loop may therefore include an attenuation pad of the required value.

The loop at the incoming end of the international circuit should be disconnected when a seizing signal is received. The loop must be disconnected within 35 ms so as to ensure that the part of a seizing signal which passes round the loop and which is returned to the outgoing end cannot be recognized as a signal.