|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.260

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No. 6

SIGNAL  UNIT  FORMATS  AND  CODES

MANAGEMENT  SIGNALS

ITU-T  Recommendation  Q.260

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.260 was published in Fascicle VI.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.260

Fascicle VI.3 - Rec. Q.260

3.4     MANAGEMENT SIGNALS

3.4.1	General

Management signals may include:

-	network-management signals,

-	network-maintenance signals,

-	signalling-network-management signals,

i.e. signals concerned with the management of the signalling network and of the speech circuit network.

These signals may be transferred by means of one-unit messages or multi-unit messages.

3.4.1.1	Basic format of management signals

The basic format of a one-unit management message is shown in Figure 12/Q.260.

The management information field, bits 10-20, may be subdivided as required. When a band number is included in the management signal unit, it is placed in bits 10-16.

For some management signals which relate to a group or sub-group of circuits, the band number in bits 10 to 16 and the management information in bits 17-20. This is detailed under the type of signal.

3.4.1.2	Format of a multi-unit management message

The format of the initial signal unit of a multi-unit management message is given in Figure 12/Q.260. The use of a special code, 0 0 0 0 in the signal information field (bits 6-9) distinguishes an initial signal unit from a one-unit management message.

The format of a subsequent signal unit of a multi-unit management message is shown in Figure 13/Q.260.

3.4.1.3	Codes for management signals

a)	Heading

The heading code 1 1 1 0 1 is used for one-unit management messages and for the initial signal unit of a multi-unit management message. The heading code 0 0 is used for subsequent signal units of multi-unit messages.

b)	Signal information

Signal information codes are assigned as follows:

0 0 0 1		network-management and network-maintenance signal units

0 0 1 0		spare

0 0 1 1		spare

0 1 0 0		spare

0 1 0 1		signalling-network-management signal unit

0 1 1 0		spare (reserved for regional and/or national use)

0 1 1 1		spare (reserved for regional and/or national use)

1 0 0 0		spare

1 0 0 1		spare

1 0 1 0		spare

1 0 1 1		MBS (see Recommendation Q.259)

1 1 0 0		SCU (see Recommendation Q.259)

1 1 0 1		SYU (see Recommendation Q.259)

1 1 1 0		spare (reserved for regional and/or national use)

1 1 1 1		spare (reserved for regional and/or national use)

Signal information code 0 0 0 0 indicates that the signal unit is the initial signal unit of a multi-unit message.

The spare international signal information codes may be assigned to either management signals or signalling-system-control signals.

3.4.2	Network-management and network-maintenance signals

3.4.2.1	Network-management signals

The heading field is coded 1 1 1 0 1. Signal information fields are coded 0 0 0 0 for multi-unit management messages and 0 0 0 1 for one unit network-management and network-maintenance signals. Figure 12/Q.260 and Figure 13/Q.260 apply. Multi-unit management messages and corresponding management information are established in three categories:

1)	destination hard to reach,

2)	all circuits busy, and

3)	switching centre congestion.

For coding see § 3.4.2.4 b).

3.4.2.2	Network maintenance signals

Network maintenance signals may be sent as one-unit or multi-unit messages under heading code 1 1 1 0 1.

3.4.2.3	Codes for one-unit network-management and network-maintenance signals

a)	Heading

The heading code 1 1 1 0 1 is used.

b)	Signal information

The signal information code 0 0 0 1 is used.

c)	Band number

The band number (bits 10-16) indicates the group or subgroup of circuits to which the signal refers.

d)	Management or maintenance information is given in bits 17-20

0 0 0 0		spare

0 0 0 1		spare

0 0 1 0		spare

0 0 1 1		spare

0 1 0 0		spare

0 1 0 1		spare

0 1 1 0		spare

0 1 1 1		spare

1 0 0 0		spare

1 0 0 1		spare

1 0 1 0		spare

1 0 1 1		spare

1 1 0 0		spare

1 1 0 1		spare

1 1 1 0		reset-band-acknowledgement, all circuits idle

1 1 1 1		reset-band

3.4.2.4	Codes used in multi-unit management messages

a)	Initial signal unit

-	The five-bit heading code 1 1 1 0 1 is used.

-	The signal information code 0 0 0 0 is used.

-	The band number is used to designate the group or subgroup of circuits to which the signals apply when appropriate. The remaining bits are used for management or maintenance information.

b)	Management information - initial signal unit

Management or maintenance information is given in bits 17-20.

0 0 0 0		destination hard to reach

0 0 0 1		all circuits busy

0 0 1 0		switching centre congestion

0 0 1 1		spare

0 1 0 0		spare

0 1 0 1		spare

0 1 1 0		spare

0 1 1 1		spare

1 0 0 0		spare

1 0 0 1		spare

1 0 1 0		spare

1 0 1 1		spare

1 1 0 0		spare

1 1 0 1		spare

1 1 1 0		spare

1 1 1 1		reset-band acknowledgement

c)	Subsequent signal unit

-	The heading code 0 0 is used.

-	The length indicator is coded as appropriate (see Recommendation Q.257, § 3.1.3.4).

-	The management information is inserted.

d)	Management information - subsequent signal units

The format of subsequent signal units is determined by the management information coded in bits 17-20 in b) above.

e)	First subsequent signal unit

ISU bits 17-20	Bits 5-20 in first SSU			Bits 5-20 in second SSU

0 0 0 0		XXXX	XXXX	XXXX	XXXX		XXXX	XXXX	XXXX	XXXX

(5-8)	(9-12)	(13-16)	(17-20)		(5-8)	(9-12)	(13-16)	(17-20)

ISC		Reason	D1	D2		D3	D4	D5	D6

Code

The digits shown as D1 to D6 represent destination code assignments which can follow address patterns or can be selected by bilateral agreement or both. The "reasons" given in the first SSU, bits (9-12) are:

0 0 0 0		ABR (Answer/Bid Ratio) below arbitrary threshold

0 0 0 1		ABR below high threshold

0 0 1 0		ABR below medium threshold

0 0 1 1		ABR below low threshold

0 1 0 0-1 1 1 1	spare

0 0 0 1		XXXX	XXXX	XXXX	XXXX		XXXX	XXXX	XXXX	XXXX

(5-8)	(9-12)	(13-16)	(17-20)		(5-8)	(9-12)	(13-16)	(17-20)

ISC		Reason	D1	D2		D3	D4	D5	D6

Code

Reasons	are as follows:

0 0 0 0		all circuits busy threshold exceeded

0 0 0 1		low congestion

0 0 1 0		medium congestion

0 0 1 1		high congestion

0 1 0 0-1 1 1 1	spare

0 0 1 0		XXXX	XXXX	1 1 1 1	1 1 1 1

(5-8)	(9-12)	(13-16)	(17-20)
		ISC		Reason	Not	Not
		Code			used	used

Reasons are as follows:

0 0 0 0			moderate congestion - level 1

0 0 0 1			serious congestion - level	2

0 0 1 0			unable to process calls - level 3

0 0 1 1-1 1 1 1	spare

0 0 1 1		spare

0 1 0 0		spare

0 1 0 1		spare

0 1 1 0		spare

0 1 1 1		spare

1 0 0 0		spare

1 0 0 1		spare

1 0 1 0		spare

1 0 1 1		spare

1 1 0 0		spare

1 1 0 1		spare

1 1 1 0		spare

1 1 1 1		circuit status indicators - codes to indicate for each circuit in the band the circuit status. Bit 5 refers to the first circuit, (circuit No. 0 0 0 0), and so on to bit 20 which refers to circuit No. 1 1 1 1, the last circuit. Code 0 indicates circuit available for service, 1 circuit should be blocked. However, if all circuits are idle (available), the LSU coding in § 3.4.2.3 d) for "all circuits idle" applies. Also see § 9.5.1.

3.4.3	Signalling-network-management signals

3.4.3.1	Format of a signalling-network-management signal

The format of a one-unit signalling-network-management message is given in Figure 14/Q.260.

3.4.3.2	Codes for the signalling network-management signal unit parts

a)	Heading

The heading code 1 1 1 0 1 is used.

b)	Signal information

The signal information code 0 1 0 1 is used.

c)	Band number

The band number (bits 10-16) indicates the group or sub-group of circuits to which the signal refers (see Recommendation Q.257, § 3.1.3.3).

d)	Management information

The codes used in the management information field are allocated as follows:

0 0 0 0		spare

0 0 0 1		spare

0 0 1 0		spare

0 0 1 1		spare

0 1 0 0		spare

0 1 0 1		transfer-prohibited

0 1 1 0		transfer-allowed

0 1 1 1		spare

1 0 0 0		transfer-allowed-acknowledgement

1 0 0 1		spare

1 0 1 0		spare

1 0 1 1		spare

1 1 0 0		spare

1 1 0 1		spare

1 1 1 0		spare

1 1 1 1		spare