|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.264

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No. 6

SIGNALLING  PROCEDURES

POTENTIAL  FOR  AUTOMATIC  REPEAT  ATTEMPT  AND  RE - ROUTING

ITU-T  Recommendation  Q.264

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.264 was published in Fascicle VI.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.264

Fascicle VI.3 - Rec. Q.264

4.4     POTENTIAL FOR AUTOMATIC REPEAT ATTEMPT AND RE-ROUTING

4.4.1	Automatic repeat attempt

The potential for automatic repeat attempt as defined in Recommendation Q.12 is provided in System No. 6. Backward signals are included to provide information on which to base a decision as to whether or not it would be advantageous to invoke an automatic repeat attempt.

An automatic repeat attempt will be made:

-	upon failure of the continuity check (§ 4.1.4 above),

-	on receipt of the confusion signal (while setting up a call) (§ 4.7.6.4 below),

-	on detection of double seizing (at the non-control exchange) (§ 4.3.5 above),

-	in some cases on receipt of a message-refusal signal (§ 4.6.2.3 below), and

-	on receipt of a blocking signal after sending an initial address message and before any backward signals have been received (§ 4.6.1 below).

4.4.2	Automatic repeat attempt and re-routing

The potential for automatic repeat attempt or re-routing on receipt of the circuit-group-congestion, the switching-equipment-congestion or the call-failure signals is provided.