|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.278

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No. 6

SIGNALLING  LINK

SYNCHRONIZATION

ITU-T  Recommendation  Q.278

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.278 was published in Fascicle VI.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.278

Fascicle VI.3 - Rec. Q.278

6.8     SYNCHRONIZATION

6.8.1	General

The SYU will contain, in addition to the 8 check bits, a 16 bit pattern for bit and signal unit synchronization and a 4 bit number for block synchronization. The same 16-bit pattern will appear in every SYU. The 4-bit number will describe the position of the SYU within its block (see Recommendation Q.259, § 3.3.3.2).

Each signalling terminal requires 2 counters of up to 8 bits capacity, to keep a record of the blocks completed and acknowledged.

The block-completed counter (BCC) indicates the sequence number of the last block transmitted by the terminal. The last 3 bits of this number are also sent to the ACU of the block and occupy the bit positions reserved for the block-completed sequence number (BCSN).

The block-acknowledged counter (BAC) is up-dated using the block-acknowledged sequence number (BASN) in the incoming ACUs and therefore indicates the sequence number of the block being acknowledged by the last received ACU. In order to keep it up to date even when ACUs are detected in error, the block-acknowledged counter is incremented whenever the twelfth signal unit of a block is received in error. in the case where the block-acknowledged sequence number does not have the expected value, the block-acknowledged counter will be up-dated as follows:

-	If the BASN has the same value as in the previous ACU, the BAC will not be incremented;

-	If the BASN has an unexpected value greater than the previous BASN, then the least significant three bits of the BAC are replaced by the latest BASN;

-	If the BASN has a value less than the previous BASN, then the BAC is incremented by eight and the least significant three bits replaced by the latest BASN.

If the terminal is in multiblock synchronization, and if a jump in the BASN of greater than 2 or equal to or less than -1 occurs, then multiblock synchronization must be checked immediately.

The counters are set to zero during normal synchronization and are checked periodically by using the multiblock monitoring procedure.

If the capacity of the counters is exceeded by the number of blocks in the error control loop, the signalling link is not capable of being used.

Some variations of the synchronization procedures in the specification may be incompatible with Green Book synchronization procedures.

6.8.2	Normal synchronization

This synchronization procedure is used whenever a signalling link is brought into service, either initially or after a total loss of synchronism.

Normal synchronism will be established in the following manner. Each terminal will emit either:

-	a series of blocks containing eleven SYUs, plus one ACU, or

-	a series of blocks of faulty-link information as covered in Recommendation Q.293, § 8.6.1, when  changeover has been requested.

In both cases ACUs are transmitted initially with the acknowledgement indicators set to 1 and the block-completed and block-acknowledged sequence number set to 0.

The instant of commencement of emission at the terminals is immaterial.

After bit synchronism has been established in the demodulator, the incoming bit stream will be monitored to find the SYU pattern. Once this pattern is found and verified the sequence number can be determined and the ACU position located.

In due course, three consecutive ACUs should be correctly received with block-acknowledged sequence number set to 0.

At this time the acknowledgement indicators in the next outgoing ACU are set to reflect the detected errors in the signal units of the associated received block. Both sequence numbers in the ACU remain at 0.

The reception of at least two consecutive ACUs with block-acknowledged sequence numbers set to 0 which check correctly and acknowledge one or more signal units as correct indicates that both terminals are in bit, signal unit and block synchronism.

At this time the one-minute proving period is started and block sequence numbering is initiated as follows:

the block-completed counter and block-completed sequence number in the next outgoing ACU are set to 1. Thereafter the counter and the block-completed sequence number in the ACU are incremented by 1 each time an ACU is transmitted. The block-acknowledged sequence number in outgoing ACUs is now up-dated from the block-completed sequence number of the appropriate received ACU.

When the terminal receives an ACU with a block-acknowledged sequence number other than 0, the block-acknowledged counter is set to this number. Thereafter, the counter is up-dated by the appropriate block-acknowledged sequence number each time an ACU is received.

When the block-acknowledged counter is incremented for the first time the number of blocks in the error control loop may be determined by subtracting the contents of the block-acknowledged counter from the contents of the block-completed counter. Should the result be negative, then the counters should be reset and block sequence numbering should be restarted.

The counter capacity is insufficient if the block-completed counter recycles before the block-acknowledged counter advances.

If, and only if, the initial synchronization procedure has indicated more than eight blocks in the error control loop, should the multi-block-monitoring procedure be used once every cycle of the block-completed counter. In this case the multi-block monitoring procedure should also be used for block resynchronization (see § 6.8.4 below).

Whenever a multi-block-monitoring signal is received it must be acknowledged by a multi-block-acknowledgement signal within the time required to send 40 signal units.

When the multi-block-acknowledgement signal is received, the multi-block and block numbers are compared with the contents of the block-acknowledged counter. If the received number is within minus four to plus three of the contents of the block-acknowledged counter then it is assumed that multi-block synchronism exists.

When a multi-block-acknowledgement signal is not received in response to a multi-block-monitoring signal transmitted no action need be taken. However, if a multi-block-monitoring signal is acknowledged as being received in error or if the ACU is in error then the multi-block monitoring procedure may be restarted.

If the signal unit error rate is acceptable at the end of the one-minute proving period, two load-transfer signals are emitted in the case of a regular link, or two standby-ready signals for synchronized reserve links. Acknowledgement of these signals by the other terminal is as covered in Recommendation Q.293, §§ 8.6.2 and 8.8. Signalling traffic may then be offered to regular links, while synchronized reserve links may be marked as ready for service.

The one-minute and the emergency proving periods and the load-transfer signalling sequence are omitted for non-synchronized reserve links when a changeover is made from the regular link as covered in Recommendation Q.293, § 8.6.1.

Bit synchronism is maintained by the transition between dibits for the analogue modem or by the receive clock on digital links; loss of synchronism will result in signal units failing to check correctly; however, incorrect signal units are more likely to result from line interference than loss of synchronism. Monitoring of the bit stream should result in the recognition of the 16 bit pattern of an SYU and enable synchronism to be restored if it had been lost.

6.8.3	Signal unit resynchronization

Loss of signal unit synchronism will result in continuous failure of signal units to check. When the signalling terminal receives consecutive signal units in error, it may take unilateral action to resynchronize to the incoming bit stream. In any ACUs transmitted during this procedure, all the indicator bits must be set to 1 and the block-acknowledged number and the block-completed number must be incremented as in normal operation. When synchronism is re-established on the incoming channel the indicators are set according to the incoming signal units, i.e. normal operation is resumed. The signal unit error rate monitor must continue to count signal units in error throughout this procedure.

During unilateral resynchronization, means must be provided to ensure that false resynchronization is kept to a level that is compatible with the dependability requirements (Recommendation Q.276). For this reason signal units should be checked to see that synchronization is valid.

6.8.4	Block resynchronization

Equipment must be provided to detect loss of block synchronism.

Loss of block synchronism will be recognized when a valid signal unit, which is not an ACU, is received in the 12th position in a block.

Loss of block synchronism may also be recognized by any of the following:

a)	an ACU is received in other than the 12th position in a block;

b)	the block-completed sequence number is not the one expected (see Note 2);

c)	an SYU sequence number is not the one expected.

Loss of block synchronism will not be recognized prior to the initial incrementing of the block-acknowledged counter, during either initial synchronization or after a total loss of synchronism as specified in § 6.8.2.

When loss of block synchronism has been recognized - by any of the four events described above - the terminal will stop sending telephone signals and send only SYUs and repeated ACUs (see Recommendation Q.279).

When the terminal has identified the signal unit position in a block either by recognizing the SYU number or by identifying an ACU, and has subsequently recognized two consecutive ACUs with correctly advancing block-completed sequence numbers, synchronism is deemed to have been regained.

After successful block-synchronization, the block being transmitted is completed with SYUs and an ACU. At least one complete block of 11 SYUs shall be sent before resuming normal traffic.

The first ACU sent after synchronization has been regained will have the following characteristics:

a)	the indicator bits are all set to 1;

b)	the blocked-completed sequence number is set to the next in sequence;

c)	the block-acknowledged sequence number will correspond to the latest received ACU.

Upon resynchronization, a terminal may receive an ACU with an acknowledged block number which differs from that expected. All messages sent in unacknowledged blocks should be retransmitted.

After the completion of block resynchronization, multi-block synchronism should be checked, if applicable.

When block synchronism cannot be regained within 350 ms, the link is considered to have failed and resynchronization according to § 6.8.2 is commenced. The relevant link security procedures of Recommendation Q.293 will be initiated where appropriate (e.g. changeover, emergency restart, etc.). In the case of a link that is not carrying signalling traffic, resynchronization should commence without waiting for 350 ms, that is, unilateral block resynchronization should be dispensed with.

Note 1 - An all-zero signal, i.e. a signal unit consisting of 20 zeros with the correct check bits, may cause a discontinuity in the transmitted signal unit sequence.

A receiving terminal which can recognize such a signal may, optionally, take steps to ensure that synchronism is not lost. In this case, the zero signal units should be treated as if they were in error, causing the error counter to be stepped, but no request for retransmission should be sent. Thus if zero signal units are received too frequently, a changeover or emergency restart will be initiated.

Note 2 - If an unexpected ACU with both the BASN and BCSN equal to zero is received, reset the block counters, restart the block sequence numbering as in § 6.8.2 and count the ACU as being in error.

6.8.5	Multi-block resynchronization

If the multi-block and block numbers in a multi-block-acknowledgement signal unit are not within minus four to plus three of the contents of the block-acknowledged counter a new multi-block-monitoring signal is sent. If the result of the second measurement is not within the above limit, multi-block synchronism has been lost. However, if the results of the measurements are the same, multi-block synchronism can be regained by up-dating the contents of the block-acknowledged counter to the obtained result.

When the second multi-block-monitoring signal is sent the terminal will send only SYUs and ACUs for three blocks. Normal traffic is then resumed and all messages transmitted in the interval between the two multi-block-monitoring signals are retransmitted.

If multiblock synchronism cannot be regained, the link is considered to have failed and resynchronization according to § 6.8.2 is commenced. The block counters will be reset and the block sequence numbering restarted. The relevant link security procedures of Recommendation Q.293 will be initiated where appropriate (e.g. change over, emergency restart. etc.).