|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.285

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No. 6

SIGNAL  TRAFFIC  CHARACTERISTICS

SIGNAL  PRIORITY  CATEGORIES

ITU-T  Recommendation  Q.285

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.285 was published in Fascicle VI.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.285

Fascicle VI.3 - Rec. Q.285

7.1     SIGNAL PRIORITY CATEGORIES

7.1.1	Rules for signal priority

The following rules for establishing priority categories must be followed in normal operation; within any of the priority categories, signals are transmitted in order of their arrival at the output buffer (see Recommendation Q.251, § 1.1.l):

a)	Acknowledgement signal units (12th signal unit of each block) have absolute priority for emission at their fixed predetermined position;

b)	Faulty link information (Recommendation Q.293, § 8.6.1) has priority over all other signals;

c)	The answer signal, charge, the answer signal, no charge and the multi-block-monitoring and multi-block-acknowledgement signals have priority over other waiting telephone signals and signalling-system-control signals except those cited in a) and b) above;

d)	All other telephone signals, one-unit or multi-unit messages, and all other signalling-system-control signals, except synchronization signal units, have priority over management or other signals concerned with the bulk handling of traffic;

Note - In the event that a management signal concerns the bulk restoration of service, e.g. RSB, RBA, TFA, TAA, this signal may take priority over other telephone or signalling system control signals.

e)	Any signal which is to be retransmitted will take precedence over other waiting signals in the same priority category;

f)	Management signals have priority over synchronization signal units;

g)	Synchronization signal units have no priority.

7.1.2	Break-in

a)	Potential for a priority one-unit message to break into a multi-unit message is provided in the design of the format, but initially this feature will not apply except for ACU;

b)	If a multi-unit message is used for a management signal, potential for break-in by a lone signal unit should be retained as a future option. However, there is no intent to provide means for a multi-unit message to apply break-in to another multi-unit message.

c)	In the rare event that a SYU breaks into a multi-unit message (e.g. owing to severe processor overload), the multi-unit message may be accepted as valid.