|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.292

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  No. 6

SECURITY  ARRANGEMENTS

RESERVE  FACILITIES  PROVIDED

ITU-T  Recommendation  Q.292

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.292 was published in Fascicle VI.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.292

Fascicle VI.3 - Rec. Q.292

8.4     RESERVE FACILITIES PROVIDED

The reserve facilities provided can be subdivided into three groups following below in the order of ready availability:

a)	quasi-associated reserve signalling links,

b)	full-time reserved transfer links,

c)	nominated direct circuits.

Within each group, one or more arrangements can be distinguished which differ in the preparatory actions to be taken to bring the reserve facility into active service.

The choice of the particular facilities to be used can be governed by several factors, e.g. the possibility of using quasi-associated signalling links, the number of circuits served, the geographical distance between the System No. 6 exchanges, etc. The choice of method(s), therefore, will be made by the Administrations involved according to the circumstances which apply.

As a matter of principle, the reserve facility to be used should follow a route different from the route of the regular signalling link.

8.4.1	Quasi-associated reserve signalling links

The method of using a quasi-associated signalling link as a reserve facility is directly derived from the principles accepted for System No. 6 (Recommendation Q.253).

This method assumes an adequate signalling network and requires prior agreements on its adoption between the Administration(s) through whose signal transfer point(s) the signalling traffic may overflow.

Methods of controlling quasi-associated signalling are described in Recommendation Q.266, § 4.6.2.

8.4.2	Full-time reserved transfer links

A transfer link is permanently assigned to provide the reserve signalling link.

The following arrangements can be distinguished:

a)	Load sharing

Both transfer links are equipped with modems or interface adaptors and signalling terminals and are in use on the basis of duplicate working with load sharing. Each link in this method is the reserve for the signal load on the other link.(See also Recommendation Q.293, § 8.9.)

The circuits shall be assigned identical labels on both links and each circuit shall be assigned to one of the parallel signalling links as its regular link. The exchange must be capable of accepting signalling traffic for the labels over either link at any time.(See also Recommendation Q.293, § 8.9.)

b)	Synchronized reserve

The transfer link is equipped with modems or interface adaptors and signalling terminals, thus forming a reserve signalling link.

The link is not in use, but its channels are synchronized.

c)	Non-synchronized reserve

The transfer link is not equipped with modems or interface adaptors and signalling terminals. A switching operation is thus required to convert the transfer link into a signalling link, before synchronizing of the signalling channels can start.

Arrangements a) and b) are considered to be more usual than c) and will no doubt be the general rule in the case of a full-time reservation of a transfer link. However, for international exchanges at which very many signalling links terminate, Administrations may prefer not to use the arrangements a) and b) above but to pool available modems, interface adaptors and signalling terminals for common use to a number of reserve transfer links.

8.4.3	Nominated direct circuits

A nominated direct circuit is permanently assigned to be converted into a signalling link, when required. The following arrangements can be distinguished:

a)	Speech circuit reserve

The nominated circuit is normally in speech (or other service) condition. Switching action and synchronizing must be performed when the transfer link of the circuit is required for the reserve signalling link. The switching action is allowed only when the transfer link is not in use. For this reason, Administrations should ensure that the nominated speech circuit has a high probability of being free (for example, by using a last-choice circuit).

The available modems and signalling terminals may be pooled for common use to a number of speech circuit groups.

b)	TASI-through reserve (analogue only)

The nominated circuit is a TASI-through circuit. The circuit is not to be used for speech. When it is required to open up a reserve signalling link, data are applied in the normal way. These data will be sufficient to operate the speech detector at each end and cause TASI channels to be associated with the circuit for as long as the data are applied.

Arrangement b) cannot be ranked as a general solution since it depends on having a TASI system between the two international exchanges involved.

8.4.4	Link-sets, signalling routes, signalling route sets and opposite signalling route sets

a)	Link sets, signalling routes, and signalling route sets

A regular link and reserve links directly connecting two System No. 6 exchanges, a System No. 6 exchange and an STP or two STPs, and which provide signalling for the same 2048 circuit labels are termed a link set. Where quasi-associated signalling facilities are provided, the security arrangements for a band of speech circuits will consist of one or more link sets. The different signalling paths so formed are known as signalling routes. The list of signalling routes in their priority order is known as a signalling route set. One signalling route set is allocated for all bands which have the same security arrangements.

b)	Opposite signalling route sets

At an STP traffic passes from the originating to destination exchange and vice versa using a signalling route set in each direction. Each of these signalling route sets, which form a complementary pair, is termed an opposite signalling route set with respect to the other.

8.4.5	Choice of reserve facility

When the regular link in a link set has failed and where more than one type of service is provided, signalling should first be restored on a synchronized reserve, such as a load shared or full-time synchronized reserve transfer link in the same link set. If these are not provided or are not available, signalling should then be restored via one or more link sets using quasi-associated signalling. If this option is not provided or is not available, then an attempt should be made to restore signalling on a nonsynchronized reserve such as a full-time nonsynchronized reserve transfer link or nominated direct circuit, within the original link set. Should the failed link be a reserve link, then changeover follows the same priority order as above except that the search should commence on the link below the one that has just failed. Changeover to a proved reserved link of a higher priority is only possible by using the emergency restart procedure. See Recommendation Q.293, § 8.7.

For each band or group of bands the provision of the different types of reserve, the search order to be applied where a number of one type are provided, and the selection order between link sets should be specified by the Administrations concerned.