|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.311

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  R1

LINE  SIGNALLING

2600 Hz  LINE  SIGNALLING

ITU-T  Recommendation  Q.311

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.311 was published in Fascicle VI.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.311

Fascicle VI.4 - Rec. Q.311

2.1     2600 Hz LINE SIGNALLING

The line-signal coding arrangement is based on the application and removal of a single frequency tone (2600 Hz) as shown in Table 1/Q.311.

By taking advantage of the fixed order of occurrence of specific signals, both tone-on and tone-off signals are used to indicate more than one signal condition. For example, in the backward direction tone-on is used to indicate start-dialling (proceed-to-send), and terminating end hang-up (clear-back) signals without conflict. The equipment must retain memory of the preceding signal states and the direction of signals in order to differentiate between tone-on and tone-off signals.