|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.315

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  R1

LINE  SIGNALLING

PCM  LINE  SIGNAL  SENDER  (TRANSMITTER)

ITU-T  Recommendation  Q.315

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.315 was published in Fascicle VI.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.315

Fascicle VI.4 - Rec. Q.315

2.5     PCM LINE SIGNAL SENDER (TRANSMITTER)

2.5.1	Signalling format

The primary multiplex format is shown in Figure 1/Q.315. Per channel, in-slot signalling is accomplished by utilizing bit No. 8 in each time slot of the designated frames (6, 12, etc.) for signalling purposes. Bit No. 8 of each time slot in the intervening frames (1-5, 7-11, etc.) is used for encoding speech. Two signalling channels per speech channel are provided in the format. The multiframe alignment required for signalling purposes is obtained by subdividing the 8 kbit/s framing pulse stream into two 4 kbit/s streams, one for terminal framing and one for signalling framing (S-bits). The relationship of the framing and multiframing signals to the signalling bits is given in Table 2/Q.315. Since only one line signalling channel is required for System R1, the same signalling information is sent over both signalling channels A and B.

2.5.2	Transmitted signal duration

The transmitted signal durations are given in Table 1/Q.311 of Recommendation Q.311.

2.5.3	Transmitting line split

Since signalling is out-band, no transmitting line split is required.

TABLE 2/Q.315

Multiframe structure

| Frame number | Frame alignment | Multiframe alignment | Bit number(s) in each channel time slot |  | Signalling |
| --- | --- | --- | --- | --- | --- |
|  | signal | signal (S-bit) | For character signal | For signalling | channel |
| 1 2 3 4 5 6 7 8 9 10 11 12 | 1 - 0 - 1 - 0 - 1 - 0 - | - 0 - 0 - 1 - 1 - 1 - 0 | 1 to 8 1 to 8 1 to 8 1 to 8 1 to 8 1 to 7 1 to 8 1 to 8 1 to 8 1 to 8 1 to 8 1 to 7 | - - - - - 8 - - - - - 8 | A B |

Note 1 - The sequence shown is repetitive.

Note 2 - For System R1 the same signalling information is sent on signalling channels A and B.