|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.320

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  R1

REGISTER  SIGNALLING

SIGNAL  CODE  FOR  REGISTER  SIGNALLING

ITU-T  Recommendation  Q.320

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.320 was published in Fascicle VI.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.320

Fascicle VI.4 - Rec. Q.320

3.1     SIGNAL CODE FOR REGISTER SIGNALLING

3.1.1	General

1)	Either semi-automatic working (with automatic machine or direct operator access), or automatic working (with automatic machine access) may be used for outgoing traffic. With automatic machine access the incoming address signals are stored in a register until sufficient address information is received to route the call properly, at which time a free circuit may be selected and a connect (seizing) line signal sent. Subsequent to the recognition of a delay-dialling line signal and a start-dialling (proceed-to-send) line signal a KP (start-of-pulsing) signal followed by the address and ST (end-of-pulsing) signals are transmitted. The KP signal, which is nominally 100 ms in duration, prepares the receiving equipment to accept subsequent register signals. The transmission of the KP signal should be delayed by a minimum of 140 ms, but not more than 300 ms, after recognition of the start-dialling line signal.

2)	Link-by-link register signalling applies.

3)	Register signalling is in a forward direction only and shall be in accordance with the two-out-of-six multifrequency code shown in Table 3/Q.320. Three of the 15 possible codes are unused in international service and are available for special purposes.

4)	The receiving equipment must furnish a two-and-two only frequency check on each received signal to ensure its validity.

3.1.2	Sending sequence of register signals

1)	The sending sequence of address signals conforms to the sequence indicated in Recommendation Q.107. However, for traffic within an integrated world numbering zone (e.g. Zone 1) the language or discriminating digit and country codes may have no application and may not be sent. In Zone 1, the sequence of signals sent from the operator or subscriber is as follows:

a)	Semi-automatic working for calls to a subscriber within Zone 1:

i)	KP;

ii)	national (significant) number of the called subscriber;

iii)	ST.

b)	Semi-automatic working for calls to operators within Zone 1:

i)	KP;

ii)	special decimal numbers; 2)

iii)	ST.

c)	Automatic working for calls to a subscriber within Zone 1:

i)	national (significant) number of the called subscriber.

2)	The sending sequence of register signals shall conform to Table 3/Q.320, noting the following:

a)	a KP (start-of-pulsing) signal shall precede the sequence of signals in all cases;

b)	the ST (end-of-pulsing) signal shall follow the sequence of signals in all cases.

TABLE 3/Q.320

Register signal code of System R1

| Signals | Frequencies (compound) Hz |
| --- | --- |
| KP (start-of-pulsing) Digit 1 Digit 2 Digit 3 Digit 4 Digit 5 Digit 6 Digit 7 Digit 8 Digit 9 Digit 0 ST (end-of-pulsing) Spare Spare Spare | 1100 + 1700 700 + 900 700 + 1100 900 + 1100 700 + 1300 900 + 1300 1100 + 1300 700 + 1500 900 + 1500 1100 + 1500 1300 + 1500 1500 + 1700 700 + 1700 900 + 1700 1300 + 1700 |

As used in this Recommendation, the term register includes traditional registers in electromechanical exchanges and also the equivalent receiving device, memory and logic in stored program exchanges.

2	)	The special numbers used to reach operators are by agreement between Administrations.