|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.322

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  R1

REGISTER  SIGNALLING

MULTIFREQUENCY  SIGNAL  SENDER

ITU-T  Recommendation  Q.322

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.322 was published in Fascicle VI.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.322

Fascicle VI.4 - Rec. Q.322

3.3     MULTIFREQUENCY SIGNAL SENDER

3.3.1	Signalling frequencies 700, 900, 1100, 1300, 1500 and 1700 Hz. A signal shall consist of a combination of any two of these six frequencies. The frequency variation shall not exceed ± 1.5% of each nominal frequency.

3.3.2	Transmitted signal level -7 ± 1 dBm0 per frequency. The difference in transmitted level between the two frequencies comprising a signal shall not exceed 0.5 dB.

3.3.3	Signal frequency leak and modulation products. The level of the signal leak current transmitted to the line should be at least:

a)	50 dB below the single frequency level when a multifrequency signal is not being transmitted;

b)	30 dB below the transmitted signal level of either of the two frequencies when a multifrequency signal is being transmitted. The modulation products of a signal shall be at least 30 dB below the transmitted level of either of the two frequencies comprising the signal.

3.3.4	Signal durations

KP signal: 100 ± 10 ms.

All other signals: 68 ± 7 ms.

Interval between all signals: 68 ± 7 ms.

3.3.5	Compound signal tolerance

The interval of time between the moments when the two frequencies comprising a signal are sent must not exceed 1 ms. The interval of time between the moments when the two frequencies cease must not exceed 1 ms.