|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	Q.411

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

SPECIFICATIONS  OF  SIGNALLING  SYSTEM  R2

LINE  SIGNALLING,  ANALOGUE  VERSION

LINE  SIGNALLING  CODE

ITU-T  Recommendation  Q.411

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation Q.411 was published in Fascicle VI.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation Q.411

Fascicle VI.4 - Rec. Q.411

2.1  LINE SIGNALLING CODE

2.1.1	General

The System R2 line signalling, analogue version, is intended for use on carrier circuits. The line signals are transmitted link-by-link. The code for the transmission of line signals is based on the tone-on-idle signalling method. It is required that thFascicle VI.4 - Rec. Q.411§Fascicle VI.4 - Rec. Q.411§Fascicle VI.4 - Rec. Q.411§e circuits on which the system is employed are equipped in each direction of transmission with a signalling channel outside the speech frequency band. When the circuit is in the idle state, a low-level signalling tone is sent continuously in both directions over the signalling channels. The tone is removed in the forward direction at the moment of seizure and in the backward direction when the called subscriber answers.

The connection is released when the signalling tone is restored in the forward direction; release causes the tone to be restored in the backward direction. If the called party is the first to clear, the signalling tone is restored in the backward direction first. It is then restored in the forward direction either when the caller clears or when a certain interval has elapsed after recognition of the signalling tone in the backward direction. This signalling method, requiring only simple equipment, provides rapid signal recognition and retransmission. The signal transfer speed provided by continuous type signalling compensates for the need of signal repetition inherent in link-by-link transmission.

The signalling system is specified for one-way operation of 4-wire carrier circuits.

2.1.2	Line conditions

Tone-on or tone-off denotes a certain line signalling condition. The line thus has two possible conditions in each direction, i.e. a total of four line signalling conditions. Taking into account the time sequence, the circuit may resume one of the six characteristic states shown in Table 1/Q.411.

TABLE 1/Q.411

| State | Line signalling condition |  |
| --- | --- | --- |
| of the circuit | Forward | Backward |
| 1. Idle 2. Seized 3. Answered 4. Clear-back 5. Release 6. Blocked | Tone-on Tone-off Tone-off Tone-off Tone-on Tone-on | Tone-on Tone-on Tone-off Tone-on Tone-on or off Tone-off |

The transition from one signalling condition to another corresponds to the transfer of a line signal according to the definitions in § 1. To change from the release state to the idle state additional criteria (timing) are necessary to ensure a defined sequence corresponding to the transfer of the release-guard signal (see § 2.2.2.6 below).